3GPP TSG-RAN WG3 Meeting #129	R3-255222

Bengaluru, India, 25 – 29 August 2025

Agenda item:	8.1

Source:	Nokia

Title:	Handling of Emergency Call back and Paging

WID/SID:	NR_newRAT-Core, NR_redcap-Core

Document for:	Discussion and Decision

# 1	Introduction

At RAN3#129 the LS in R3-255022 (S2-2505938) [1] has been received from SA2:

SA WG2 thanks RAN3 for the LS on emergency call back and paging. SA2 could not reach consensus to agree  to update our specifications. SA2 would request some clarifications from RAN3. For normal sessions, it is possible for SMF to keep the PDU session while the NG-RAN releases the RRC connection, e.g. AN release due to User Inactivity timer expired.

However there is different understanding in SA2 regarding the case that emergency PDU session exists. Hence SA2 have the following questions:

Q1： Is it possible to have different behaviour in NG-RAN node depending on the presence of the emergency PDU session to decide when to release RRC connection, compared to when there is no emergency PDU session?

Q2:   If yes, when the emergency PDU session exist, whether it is required to configure the NG-RAN to not release RRC connection until the emergency PDU session is released by the SMF?

This paper describes this issue and proposes a way forward and a Reply LS.

# 2	Discussion

Background

The topic was started two meetings ago and it is first useful to recall the conclusions reached so far.

At RAN3#127 RAN3 received the LS from RAN2 in R3-250006 reporting an issue if eDRX is configured/used in RRC inactive state for an emergency PDU session.

The issue is that between the emergency call and the call back the gNB could send the UE to RRC inactive with long eDRX if it has been configured with eDRX by the AMF, leading to unacceptable delay for the callback.

At RAN3#127bis, RAN3 received a first response from SA2 in the LS R3-251521 indicating that AMF can make gNB aware of an emergency PDU session through the ARP.

Accordingly, RAN3 made following answer to SA2, RAN2 at RAN3#127bis in R3-252402 [2]:

RAN3 thanks SA2 for the LS on emergency call back and paging.

RAN3 has discussed the scenario raised by RAN2 and checked the response from SA2. According to SA2’s clarification, RAN3 agrees that the NG-RAN is able to be aware of the presence of emergency PDU session when PDU session resources are established in the NG-RAN node based on the received special ARP value from CN. Regarding the open issues, RAN3 has endorsed the attached CRs to TS 38.300.

The RAN3 stage 2 CR for TS 38.300 endorsed at RAN3#127bis was in R3-252400 and it was further revised at RAN3#128 in R3-253764 [3] to specify this requirement in TS 38.300 along the lines of SA2, RAN2 view. It should be noticed that the text does not mandate gNB to send the UE to any particular RRC state as shown below:

NOTE:	If emergency PDU session resources are established, the gNB should not configure the Extended DRX when sending the UE to RRC_INACTIVE state. The gNB recognizes the emergency PDU session resources based on special ARP value of a QoS flow as specified in TS 23.501 [3].

At the same time, RAN2 answered SA2 at last RAN2 meeting and sent the Reply LS which was received by RAN3 in R3-253853 (R2-2504847) [4] which confirms RAN3 CR as follows:

RAN2 thanks SA2 for the reply LS on emergency service call back and paging.

RAN2 further discussed this issue and confirm the feasibility of the solution mentioned in reply LS, that RAN can be aware of emergency PDU session based on the reserved ARP value.

In addition to the endorsed solution from RAN3 (i.e. the gNB should not configure eDRX for RAN paging when releasing the UE to RRC_INACTIVE state), RAN2 also agreed that no new UE behavior will be specified for this case.

It is up to the gNB implementation whether to release the UE to RRC_IDLE or RRC_INACTIVE or keep the UE in RRC_CONNECTED state for fast emergency callback.

2. Actions:

To SA2, RAN3 group:

ACTION: RAN2 kindly requests SA2 and RAN3 to take the above information into account.

As clearly emphasized by RAN2 it is up to gNB implementation to decide whether to release the UE to RRC Idle or RRC Inactive or keep the UE in RRC connected state.

Response to SA2

The new received LS from SA2 in [1] is in response to RAN3 LS sent in R3-252402 [2].

In line with our previous response and agreement, and also RAN2 confirmation, the following answers can be given to SA2:

Q1： Is it possible to have different behaviour in NG-RAN node depending on the presence of the emergency PDU session to decide when to release RRC connection, compared to when there is no emergency PDU session?

Answer: it is possible to have different behaviour in NG-RAN node depending on the presence of the emergency PDU session to decide when to release RRC connection but a said by previous LS from RAN3 and confirmed by RAN2 it is an implementation decision in the gNB.

Q2:   If yes, when the emergency PDU session exist, whether it is required to configure the NG-RAN to not release RRC connection until the emergency PDU session is released by the SMF?

Answer: It is up to gNB implementation to decide the RRC state of the UE.

Proposal 1: agree the response LS in [5].

# 3	Conclusion

This paper has recalled the past discussions on emergency call and paging and proposed to reply to new SA2 LS aligned with previous RAN3 answers and in alignment with RAN2 answer.

Proposal 1: agree the response LS in [5].

# 4	References

- R3-255022, Reply LS on Emergency Callback and paging, S2-2505938, RAN3#129

- R3-252402, Reply LS on Emergency Call back and paging, RAN3#127, to SA2

- R3-253764, Correction of emergency call and eDRX, RAN3#128

- R3-253853, Reply LS on Emergency Call back and paging, RAN3#128 from RAN2 R2-2504847

- R3-255223, Response LS on Emergency Call back and paging