3GPP TSG-RAN WG3 Meeting #129	R3-255240

Bengaluru, India, 25th - 29th Aug, 2025

Title: 	(TP to BLCR for TS 38.473, TS 38.423 and TS 38.420) Discussion on on-demand SIB1 for UEs in idle or inactive mode

Source: 	Huawei

Agenda item:	17.3

Document Type:	Other

# Introduction

In the previous RAN3 meetings, the following agreements on OD-SIB1 were made:

Agreement at RAN3#127 meeting It is agreed that after Cell A gNB has received and accepted the UL WUS Configuration request from the NES Cell gNB, it broadcasts in SIBx at the first opportunity. It is agreed from the signalling point of view, in UL WUS Configuration Provision Request (naming to be further discussed) from NES Cell gNB to Cell A gNB, one option is “start with UL WUS Configuration”. This codepoint/choice is used to trigger Cell A to broadcast UL WUS configuration in SIBx. It is agreed that NES Cell gNB can “stop” (naming to be further discussed) Cell A to broadcast UL WUS configuration. Agreement at RAN3#127bis meeting Step4 is not needed. In the new Class 1 message (Direction NES gNB to Cell A gNB), RAN3 has agreed that: - One Choice “Start with UL WUS Condiguration” - One Choice “Stop” meaning the Cell A gNB will remove/release/discard the UL WUS Configuration and stop the broadcasting, it also means that next time the NES gNB should use “Start with UL WUS Configuration”. Agreement at RAN3#128 meeting One “Provision Request message includes one “OD-SIB1 config R19” referring to the TS 38.331 definition, it is a RRC Container in octet string (presence M) + one NES Cell ID (presence M ) + one Cell-A ID (presence O ) Cell A gNB-CU encoding the SIBxx. The NES gNB-CU sends the indication to NES gNB-DU. The NES gNB-DU MAY go to OD-SIB 1 operation up to gNB-DU decision.

This contribution focuses on the remaining issues, and finalize the work.

# Discussion

## 2.1 Xn impact

### 2.1.1 The class 1 procedure

In the RAN2-129bis meeting, the general guidelines were agreed to keep the specification clear and readable as follows, and the RAN2 running RRC CR in [1] already captured the details.

- RAN2#129bis meeting Define features, not devices. That is, definition of OD-SIB1, OD-SSB, paging adaptation etc in 38.300 and in other specifications refer to 38.300: - Avoid NES, Cell A, UL-WUS - “UE supporting OD-SIB1” where/if needed, and to replace with the UE capability name - UL-WUS is replaced with SIB1 request in e.g. field descriptions - ul-WUS-Config renamed as od-sib1-Config

- It is reasonable for RAN3 to take the same rule for the new defined Class 1 and Class 2 procedures including avoiding NES, Cell A, UL-WUS terminologies etc. Note that hereafter the NES node, Cell A node are used for illustration purpose.

- The exact class1 name is still to be determined, where it is named as UL WUS Configuration Provision procedure in stage 3 BLCR in [2]. Following the above discussion, it can be renamed as On-demand SIB1 Configuration Provision procedure (including request, response, and failure message).

RAN3 to take the above RAN2 guidance for stage 2/stage 3 details, including avoiding NES, Cell A, UL-WUS terminologies etc. Naming the class 1 procedure as On-demand SIB1 Configuration Provision procedure (including request, response, and failure message). The same is true for stage2 TS 38.420.

The UL WUS CONFIGURATION PROVISION REQUEST message is copied below.

#### 9.x.x.x UL WUS CONFIGURATION PROVISION REQUEST This message is sent by the NG-RAN node1 to the peer NG-RAN node2 to request or stop provisioning of an UL WUS configuration in one assisted cell. - Direction: NG-RAN node1 ® NG-RAN node2. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.2.3.1; YES; reject. CHOICE Request Type; M; YES; reject. >Start. >>UL WUS configuration (FFS to align to RAN2 spec); M; OCTET STRING; Includes the XX IE (FFS) of the cell indicated by NR CGI at NG-RAN node1 specified in TS 38.331 [10].; –. >>NR CGI at NG-RAN node1 (Naming FFS); M; NR CGI 9.2.2.7; Indicates the NR cell of the NG-RAN node1 supports the UL WUS configuration/On Demand SIB1(Naming FFS); –. >>NR CGI at NG-RAN node2 (Naming FFS); O; NR CGI 9.2.2.7; Indicates the NR cell of the NG-RAN node2 requested to broadcast the UL WUS configuration (Naming FFS).; –. >Stop. >> NR CGI at NG-RAN node1 (Naming FFS); M; NR CGI 9.2.2.7; Indicates the NR cell of the NG-RAN node1 supports the UL WUS configuration/On Demand SIB1(Naming FFS); –. Interface Instance Indication; O; 9.2.2.39; YES; reject.

- First, the UE UWS configuration can refer to the od-SIB1-Config,  which is included in  the SIBxx as captured in the current RAN2 running RRC CR as follows in [2].

- It can be noticed that in the OD-SIB1-Config-r19 IE, multiple PCIs could be associated with the same SIB1 request configuration and the same carrier frequency. Given that the UL WUS configuration request message is associated with only one NES Cell ID, it is better to specify in the semantic descriptions that the PCI indicated in the OD-SIB1-Config-r19 IE is restricted to the PCI associated with the NR CGI. When Cell A node decides to broadcast the SIBxx, it can combine and merge the multiple per cell level OD-SIB1-Config into the SIBxx.

To refer the OD-SIB1-Config-r19 IE defined in TS 38.331, and specify in the semantic descriptions that the PCI indicated in the OD-SIB1-Config-r19 IE is restricted to the PCI associated with the NR CGI.

- Second, the optional cell A was agreed in the request message. The association between the request and response message association issue needs to be discussed. For example,

- The NES cell node first includes cell A in the request message for the NES cell ID1, before it receives the successful response message,

- The NES cell node then includes a different cell A in the request message, but receives the failure response.

- In this case, the NES Cell node cannot discern which request is succeeded, which one is failed. To solve the problem, there are several options can be considered.

- Option 1: to introduce the message level transaction ID.

- Option 2: to add the optional Cell A ID (i.e., the NR CGI at NG-RAN node2) in the provision response and failure message.

- Option 3: to clarify that at any instance a maximum of one ongoing procedure is allowed.

Option 1 and option2 can allow the parallel transactions.  Option3 allows a single transaction and simple, which can be clarified by a note under the provision procedure.

For the class 1 procedure, RAN3 to take option 3 (to clarify that at any instance a maximum of one ongoing procedure is allowed) as agreement.

### 2.1.2 The class 2 procedure

The UL WUS CONFIGURATION PROVISION STATUS UPDATE message in the XnAP BLCR was captured as follows.

First, as discussed above, the class 2 procedure can be named as On-demand SIB1 Configuration Provision Status Update message.

Second, regarding the provision status, there is a remaining issue whether there is a need for the Cell A gNB itself to send the “resume” or “restart” indication to NES Cell to assist the WUS configuration broadcast again.

Whether CellA can stop and resume the broadcasting of UL WUS configuration?

- We consider this is not much needed. One reason is that Cell A possibly will not know if the NES Cell is currently in OD-SIB1 mode, hence it becomes useless to send “start” or “restart” to the NES Cell. Another reason is that the Cell A may not obtain the latest WUS config in time when the WUS config was updated. .

- Further, it is FFS whether the Cell A ID or the NES Cell ID is included in this message. Considering the Cell A is an optional IE in the class 1 request message, a set of NES cell IDs could be used to indicate the stop OD-SIB1 provision for the indicated cells, without cell A ID.

For the class 2 procedure:

- Naming the class 2 procedure to On-demand SIB1 Configuration Provision Status Update message, including only the “stop” codepoint.

- include a set of NES Cell IDs, instead of the Cell A ID.

## 2.2 F1 impact

### 2.2.1 Supporting the Xn class 1 procedure

At NES cell Node

- In the F1AP BLCR [3], there is FFS on the on-demand SIB1 config IE. As discussed above,  according to the RAN2 RRC running CR in [1], multiple PCIs could be associated with the same SIB1 request configuration and the same carrier frequency in the OD-SIB1-Config-r19 IE. Given that the On-demand SIB1 is included in the served cell information IE, it is better to specify in the semantic descriptions that the PCI indicated in the OD-SIB1-Config-r19 IE is restricted to the PCI associated with the NR CGI.

To remove the FFS, after referring to the OD-SIB1-Config-r19 IE defined in TS 38.331, and specify in the semantic descriptions that the PCI indicated in the OD-SIB1-Config-r19 IE is restricted to the PCI associated with the NR CGI.

- Further, the On-demand SIB1 Cell IE was sent from the CU to the DU with start or stop indicator in the GNB-CU CONFIGURATION UPDATE message, for which there is an editor’s note.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>> If the On-Demand SIB1 Cell IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider the cell indicated by the NR CGI IE for On-demand SIB1 operation. Editor’s note: FFS how to describe the behaviour relative to the On-Demand SIB1 Indicator IE. <<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>> On-Demand SIB1 Cell; 0..1; YES; ignore. >NR CGI; M; 9.3.1.12; -. >On-Demand SIB1 Indicator; M; ENUMERATED (start, stop, ...); -.

- Since the NES Cell CU has received the class 1 successful response message, or the class 2 message from the Cell A CU, the NES Cell CU could indicate whether to enter On-demand SIB1 operation by using the On-Demand SIB1 Indicator IE. Hence, the NES Cell DU could consider the cell indicated by the NR CGI IE to start or stop the On-Demand SIB1 indicator operation as indicated by the On-Demand SIB1 indicator IE.

Remove the editor’s note and take the following procedure texts.

- If the On-Demand SIB1 Cell IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider the cell indicated by the NR CGI IE to start or stop the On-demand SIB1 operation as indicated by the On-Demand SIB1 indicator IE.

At cell A Node

It was already agreed that the Cell A gNB-CU encoding the SIBxx. Currently, the gNB-CU System Information incudes certain SIBs encoded by the gNB-CU to the DU to broadcast. The SIBxx is already included in the gNB-CU System Information, thus there is no stage 3 impact.

The Annex provides the TPs for TS 38.473 and TS 38.423 respectively.

# 3. Conclusion

In this contribution, we discussed the on-demand SIB1 operation related issues and made the following proposals:

The class 1 procedure

- RAN3 to take the above RAN2 guidance for stage 2/stage 3 details, including avoiding NES, Cell A, UL-WUS terminologies etc. Naming the class 1 procedure as On-demand SIB1 Configuration Provision procedure (including request, response, and failure message). The same is true for stage2 TS 38.420.

- To refer the OD-SIB1-Config-r19 IE defined in TS 38.331, and specify in the semantic descriptions that the PCI indicated in the OD-SIB1-Config-r19 IE is restricted to the PCI associated with the NR CGI.

- For the class 1 procedure, RAN3 to take option 3 (to clarify that at any instance a maximum of one ongoing procedure is allowed) as agreement.

The class 2 procedure

For the class 2 procedure:

- Naming the class 2 procedure to On-demand SIB1 Configuration Provision Status Update message, including only the “stop” codepoint.

- include a set of NES Cell IDs, instead of the Cell A ID.

F1 impact for the class 1 procedure

To remove the FFS, after referring to the OD-SIB1-Config-r19 IE defined in TS 38.331, and specify in the semantic descriptions that the PCI indicated in the OD-SIB1-Config-r19 IE is restricted to the PCI associated with the NR CGI.

Remove the editor’s note and take the following procedure texts.

- If the On-Demand SIB1 Cell IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider the cell indicated by the NR CGI IE to start or stop the On-demand SIB1 operation as indicated by the On-Demand SIB1 indicator IE.

# 4. Reference

- R2-25xxxxx Running RRC CR for enhancements for network energy efficiency , RAN2

- R3-253974 (BL CR to 38.423) Introduction of Network Energy Saving Enhancement	Ericsson etc

- R3-255081 (BL CR to 38.473) Introduction of Network Energy Saving Enhancement	Huawei etc

- RAN3_126_agenda_20241122_EOM1.doc

- R3-245737 Summary of offline discussions: Rel-19 Network Energy Saving, Ericsson

# 5. TP for TS 38.473 – on top of R3-255081

<<<<<<<<<<<<<<<<<<<< Change Begins >>>>>>>>>>>>>>>>>>>>

### 8.2.3	F1 Setup

#### 8.2.3.1	General

The purpose of the F1 Setup procedure is to exchange application level data needed for the gNB-DU and the gNB-CU to correctly interoperate on the F1 interface. This procedure shall be the first F1AP procedure triggered for the F1-C interface instance after a TNL association has become operational.

NOTE:	If F1-C signalling transport is shared among multiple F1-C interface instances, one F1 Setup procedure is issued per F1-C interface instance to be setup, i.e. several F1 Setup procedures may be issued via the same TNL association after that TNL association has become operational.

NOTE:	Exchange of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received. This procedure also re-initialises the F1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do.

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: F1 Setup procedure: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the F1 SETUP REQUEST message contains the Mobile IAB-MT User Location Information IE, the gNB-CU shall, if supported, take it into account when reporting UE location information to the AMF for a UE served by the mobile IAB-node.

If the XR Broadcast Information IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, consider the indicated cell does not allow 2Rx XR UEs in case of subsequent outgoing mobility involving XR UEs.

If the NCGI to be Updated List IE is included in the F1 SETUP RESPONSE message, the gNB-DU shall, if supported, change the NCGI of the cell indicated by the Old NCGI IE to the NCGI indicated by the New NCGI IE.

If the Barring Exemption for Emergency Call Information IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU may store the information and consider the indicated cell allows emergency bearer services for UEs who would otherwise consider the cell as barred as specified in TS 38.304 [24].

If the On-demand SIB1 IE is included and set to “Provision” in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, use this information indicated in the od-SIB1-Config IE for coordination of on-demand SIB1 transmission for network energy saving as specified in TS 38.300 [6].

If the On-demand SIB1 IE is included and set to “Stop provision” in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, stop the coordination of on-demand SIB1 transmission as specified in TS 38.300 [6].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.2.4	gNB-DU Configuration Update

#### 8.2.4.1	General

The purpose of the gNB-DU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and the gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

NOTE:	Update of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: gNB-DU Configuration Update procedure: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the GNB-DU CONFIGURATION UPDATE message contains the Mobile IAB-MT User Location Information IE, the gNB-CU shall, if supported, take it into account when reporting UE location information to the AMF for a UE served by the mobile IAB-node.

If the XR Broadcast Information IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, consider the indicated cell does not allow 2Rx XR UEs in case of subsequent outgoing mobility involving XR UEs.

If the Barring Exemption for Emergency Call Information IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store the information and consider the indicated cell allows emergency bearer services for UEs who would otherwise consider the cell as barred as specified in TS 38.304 [24].

If the On-demand SIB1 IE is included and set to “Provision” in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, use this information indicated in the od-SIB1-Config IE for coordination of on-demand SIB1 transmission for network energy saving as specified in TS 38.300 [6].

If the On-demand SIB1 IE is included and set to “Stop provision” in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, stop the coordination of on-demand SIB1 transmission for network energy saving as specified in TS 38.300 [6].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the gNB-CU Name IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-CU. If the Extended gNB-CU Name IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-CU and shall ignore the gNB-CU Name IE if also included.

If the Mobile IAB Barred IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider it as an indication of whether the cell allows mobile IAB-node access.

If the On-demand SIB1 Cell IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that the cell indicated by the NR CGI IE can start or stop the on-demand SIB1 operation as indicated by the On-demand SIB1 indicator IE.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells to be Activated List |  | 0..1 |  | List of cells to be activated or modified | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >>gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.3.1.65 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.3.1.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |
| >>IAB Info IAB-donor-CU | O |  | 9.3.1.105 | IAB-related configuration sent by the IAB-donor-CU. | YES | ignore |
| >>Available SNPN ID List | O |  | 9.3.1.163 | Indicates the available SNPN ID list. If this IE is included, the content of the Available PLMN List IE and Extended Available PLMN List IE if present in the Cells to be Activated List Item IE is ignored. | YES | ignore |
| >>MBS Broadcast Neighbour Cell List | O |  | 9.3.1.226 |  | YES | ignore |
| >>SSBs within the cell to be Activated List | O |  | 9.3.1.326 | List of SSB beams within the cell requested to be activated. | YES | reject |
| <<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| Cells Allowed to be Deactivated List |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed to be Deactivated List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| On-demand SIB1 Cell |  | 0..1 |  |  | YES | ignore |
| >NR CGI | M |  | 9.3.1.12 |  | - |  |
| >On-demand SIB1 Indicator | M |  | ENUMERATED (start, stop, ...) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.10	Served Cell Information

This IE contains cell configuration information of a cell in the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.3.1.12 |  | - |  |
| NR PCI | M |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| 5GS TAC | O |  | 9.3.1.29 | 5GS Tracking Area Code | - |  |
| Configured EPS TAC | O |  | 9.3.1.29a |  | - |  |
| Served PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs in SIB 1 associated to the NR Cell Identity in the NR CGI IE | - |  |
| >PLMN Identity | M |  | 9.3.1.14 |  | - |  |
| >TAI Slice Support List | O |  | Slice Support List 9.3.1.37 | Supported S-NSSAIs per PLMN or per SNPN. | YES | ignore |
| >NPN Support Information | O |  | 9.3.1.156 | Supported NPNs per PLMN. | YES | reject |
| >Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.165 | Additional Supported S-NSSAIs per PLMN or per SNPN. | YES | reject |
| >TAI NSAG Support List | O |  | 9.3.1.273 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| <<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| XR Broadcast Information | O |  | ENUMERATED (true, …) | Corresponds to information provided in the cellBarred2RxXR contained in the SIB1 message as defined in TS 38.331 [8]. | YES | ignore |
| Barring Exemption for Emergency Call Information | O |  | ENUMERATED (true, …) | Corresponds to information provided in the barringExemptEmergencyCall contained in the SIB1 message as defined in 38.331 [10]. | YES | ignore |
| CHOICE on-demand SIB1 | O |  |  |  | YES | ignore |
| >Provision |  |  |  |  |  |  |
| >>On-demand SIB1 Config | M |  | Octet String | Includes the od-SIB1-Config contained in the SIBxx message for the cell indicated by the NR PCI IE as defined in TS 38.331 [8]. | - |  |
| >Stop provision |  |  |  |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMN Ids. Value is 6. |
| maxnoofExtendedBPLMNs | Maximum no. of Extended Broadcast PLMN Ids. Value is 6. |
| maxnoofBPLMNsNR | Maximum no. of PLMN Ids.broadcast in an NR cell. Value is 12. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U Channel IDs in a cell. Value is 16. |
| maxnoofMBSFSAs | Maximum no. of MBS FSAs by a cell. Value is 256. |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

<NO Change>

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>

# 6. TP for TS 38.423 on top of R3-253974

### 8.x.x	On-demand SIB1 Configuration Provision

#### 8.x.x.1	General

The purpose of the On-demand SIB1 Configuration Provision procedure is to enable an NG-RAN node1 to provide On-demand SIB1 configuration information to NG-RAN node2 and request NG-RAN node2 to transmit On-demand SIB1 configuration information.

The procedure is also used to enable an NG-RAN node1 to request NG-RAN node2 to stop transmission of On-demand SIB1 configuration.

The procedure uses non-UE -associated signaling. At any instance a maximum of one ongoing procedure is allowed.

#### 8.x.x.2	Successful Operation

Figure 8.x.x.2-1: On-demand SIB1 Configuration Provision procedure, successful operation

The NG-RAN node1 initiates the procedure by sending the On-demand SIB1 CONFIGURATION PROVISION REQUEST message to the peer NG-RAN node2.

If the On-demand SIB1 Config IE is included in the Request Type IE in the On-demand SIB1 CONFIGURATION PROVISION REQUEST message, the NG-RAN node2 shall, if supported, broadcast On-demand SIB1 Configuration  in SIBxx and reply with the On-demand SIB1 CONFIGURATION PROVISION RESPONSE  message. The NG-RAN node2 stores the On-demand SIB1 configuration information .

If the NR CGI at NG-RAN node2 IE is included in the On-demand SIB1 CONFIGURATION PROVISION REQUEST message, the NG-RAN node2 shall, if supported, broadcast the On-demand SIB1 Configuration information in the cell indicated by the NR CGI at NG-RAN node2 IE.

If the stop is included in the Request Type IE in the On-demand SIB1 CONFIGURATION PROVISION REQUEST  message, the NG-RAN node2 shall stop broadcasting On-demand SIB1 Configuration for the cell given by the NR CGI at NG-RAN node1 IE in SIBxx. NG-RAN node2 removes the received On-demand SIB1 Configuration.

#### 8.x.x.3	Unsuccessful Operation

Figure 8.x.x.3-1: On-demand SIB1 Configuration Provision procedure, unsuccessful operation

If the NG-RAN node2 cannot broadcast On-demand SIB1 Configuration in SIBxx , or a failure occurs during On-demand SIB1 Configuration Provision , the NG-RAN node2 shall send On-demand SIB1 CONFIGURATION PROVISION FAILURE  message. The message shall contain the Cause IE with an appropriate value.

#### 8.x.x.4	Abnormal Conditions

Void.

### 8.x.y	On-demand SIB1 Configuration Transmission Status Update

#### 8.x.y.1	General

This procedure is initiated by an NG-RAN node2 to inform a neighbouring NG-RAN node1 about a previously admitted On-demand SIB1 configuration provision being stopped. The procedure uses non-UE associated signalling.

#### 8.x.y.2	Successful Operation

Figure 8.x.y.2-1: UL WUS Configuration Transmission Status Update procedure, successful operation

The NG-RAN node2 shall indicate that a previously admitted On-demand SIB1 configuration provision in the On-demand SIB1 CONFIGURATION PROVISION STATUS UPDATE message is being stopped for the indicated cells given by the NR CGI list at NG-RAN node1 IE.

#### 8.x.y.3	Unsuccessful Operation

Not applicable.

#### 8.x.y.4	Abnormal Conditions

Void.

*************** Next change ***************

#### 9.x.x.x	ON-DEMAND SIB1  CONFIGURATION PROVISION REQUEST

This message is sent by the NG-RAN node1 to the peer NG-RAN node2 to request or stop provisioning of an On-demand SIB1configuration in one assisted cell.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| CHOICE Request Type | M |  |  |  | YES | reject |
| >Start |  |  |  |  |  |  |
| >>On-demand SIB1 Config | M |  | OCTET STRING | Includes the od-SIB1-Config contained in the SIBxx as specified in TS 38.331 [10]. | – |  |
| >>NR CGI at NG-RAN node1 | M |  | NR CGI 9.2.2.7 | Indicates the NR cell of the NG-RAN node1 supports the On Demand SIB1 | – |  |
| >>NR CGI at NG-RAN node2 | O |  | NR CGI 9.2.2.7 | Indicates the NR cell of the NG-RAN node2 requested to broadcast the On-demand SIB1 configuration. | – |  |
| >Stop |  |  |  |  |  |  |
| >> NR CGI at NG-RAN node1 | M |  | NR CGI 9.2.2.7 | Indicates the NR cell of the NG-RAN node1. | – |  |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

#### 9.1.3.y	ON-DEMAND SIB1 CONFIGURATION PROVISION RESPONSE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that the On-demand SIB1 configuration will be provided by the NG-RAN node2.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NR CGI at NG-RAN node1 | M |  | NR CGI 9.2.2.7 | Indicates the NR cell of the NG-RAN node1 supports the On Demand SIB1 | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

#### 9.1.3.z	ON-DEMAND SIB1 CONFIGURATION PROVISION FAILURE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that the On-demand SIB1 configuration cannot be provided in the requested cell of the NG-RAN node2.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| NR CGI at NG-RAN node1 | M |  | NR CGI 9.2.2.7 | Indicates the NR cell of the NG-RAN node1 supports the On Demand SIB1 | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

#### 9.1.3.t	ON-DEMAND SIB1 CONFIGURATION PROVISION STATUS UPDATE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to report that an admitted UL WUS configuration provision is being stopped.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NR CGI list at NG-RAN node1 |  | 1..< maxnoofCellsinNG-RAN node > |  |  |  |  |
| >NR CGI | M |  | 9.2.2.7 | Indicates the NR cell of RAN node1 supports the on demand SIB1 |  |  |
| Provision Status | M |  | ENUMERATED (stopped, …) | Indicates the status of the UL WUS configuration transmission. | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>

# 6. TP for TS 38.420 on top of R3-253917

### 5.2.4	Energy saving function

This function enables decreasing energy consumption by indication of cell activation/deactivation, SSB beam activation/deactivation over the Xn interface or enabling the neighbour NG-RAN node(s) to broadcast the On-demand SIB1 configuration.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 6.2.5	Energy saving procedures

-	Cell Activation procedure: enables an NG-RAN node to request the activation of a previously deactivated cell or SSB beam hosted in another NG-RAN node.

-    On-demand SIB1 Configuration Provision procedure: enables an NG-RAN node to provide On-demand SIB1 configuration to another NG-RAN node and request the receiving NG-RAN node to start or stop broadcasting the on-demand SIB1 configuration.

-    On-demand SIB1 Configuration Transmission Status Update procedure: enables an NG-RAN node to inform the requesting NG-RAN node that the on-demand SIB1 configuration broadcasting is stopped.

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>