3GPP TSG-RAN WG3 Meeting #129                                                                   R3-255242

Bengaluru, India, Aug 25th – 29th, 2025

Source:	CATT, China Telecom, CMCC, NTT Docomo, Huawei

Title:	Discussion on L3-report based SCell activation

Agenda Item:	9.2

Document for:	Discussion and Approval

1 Introduction

In the Rel-18 NR RRM enhancement topic, RAN4 had some conclusions on unknown FR2 SCell activation enhancement with L3 reporting. RAN4 sent a LS[1] to RAN2 in RAN4#108 meeting (August 2023) for designing the corresponding signaling to support the enhancements.

Following details are captured in the LS to RAN2:

Enhancement 1: network triggering report (of L3 measurement result) during FR2 unknown SCell activation

Enhancement for multiple FR2 unknown SCells’ activation Agreements: - Define requirements for multiple SCell activation (with and without PUCCH SCell) considering at least enhancement of L3 measurement reporting triggered by SCell activation command when all to-be-activated SCells are in the same band. - Note: If requirements are not finalized in RAN4 #109 then they will be deprioritized TCI activation command transmitted is based on L3 measurement results or L1 measurement results? Agreements: - If the L3 report is triggered after SCell activation, the TCI activation command is assumed based on: - the L3 report, if no L1-RSRP report is received before transmitting the TCI activation command. - the L3 report or L1-RSRP report (i.e., up to network implementation), if L3 report and L1-RSRP report is received before transmitting the TCI activation command.

RAN4 introduced an enhancement to support L3-report based TCI activation during FR2 unknown SCell activation. Later RAN4 extended the same framework for both FR1 and FR2 Unknown SCells

When SCell activation command MAC CE is received and UE has valid L3 measurement results available but not reported for the unknown SCell, the L3 measurement report containing SCell measurements will be sent to gNB. In this case, the gNB will receive the L3 SCell measurement report quickly within few milli seconds (note that UE already performed these L3 deactivated SCell measurements at low periodicity but were not reported to gNB before Aperiodic Scell activation) after sending SCell activation MAC-CE command, the gNB can activate TCI state (i.e., to activate a specific beam) for UE’s SCell based on the L3 measurement report so that it can significantly reduce the latency of SCell activation. Without this enhancement, UE will start legacy L1 measurements after receiving the SCell activation command and report L1 measurements, but result in significant delay for SCell TCI state activation, CSI reporting and subsequent scheduling from gNB.

To support this enhancement, RAN2 has agreed to introduce a new report type “reportOnScellActivation”. According to the CR[2] to RRC spec and CR[3] to MAC spec of RAN2, for the unknown SCell, L3 measurement results are reported upon UE receiving the SCell activation command MAC CE in case the new report type is configured to UE.

2 Discussion

According to RAN4, if the L3 report is triggered upon UE receiving the SCell activation command for the unknown SCell, gNB can activate the TCI state based on the L3 report instead of waiting for L1 report. In this way the activation of unknown SCell can be significantly accelerated.

However, in split gNB architecture, the node receiving the L3 measurement report is not that transmitting the MAC-CE command of TCI activation, it is CU to receive the L3 measurement report while DU MAC is responsible for MAC CE based SCell activation and MAC CE based TCI state activation to UE. So, the L3-report based SCell activation in split architecture is not supported at present.

To support this enhancement, coordination between CU and DU is needed, and the function split between CU and DU should be discussed. Note that it is important to support this fast SCell activation enhancement, otherwise L3-report based Scell activation (for both FR1 and FR2) does not work in split gNB architecture and RAN4/RAN2 work will become useless. Without this enhancement, it will take few seconds to activate SCell in split architecture.

Proposal 1: RAN3 to discuss whether to support L3-report based SCell activation in split gNB architecture.

Since it’s gNB-CU to configure the L3 report type reportOnScellActivation to UE, when it receives the L3 measurement report from UE with such report type, it can know that the L3 measurement report is triggered at UE’s reception of SCell activation MAC CE and the L3-report can be used for TCI activation. If this feature is supported in split architecture, CU can send some information to DU for TCI activation, at reception of L3 report with type reportOnScellActivation.

Proposal 2: If this feature is supported in split architecture, CU can send information on L3-report based TCI activation for SCell at reception of L3 report with type reportOnScellActivation.

On what information is sent from CU to DU, there would be several potential options.

Alt. 1: CU sends the selected beam for SCell

Alt. 2: CU simply delivers the L3 report to DU

Alt. 3: CU sends the assistant information for DU selecting the beam of SCell

Alt. 1: CU select the beam for SCell using L3 report

CU uses the L3 report received from UE to select the beam for SCell activation, then indicates the selected beam to gNB-DU. In other words, this alternative allows CU triggers the DU to do TCI activation using the beam selected by the CU. Note that UE shall report all serving cells’ measurement results for this report type when the SCell activation MAC CE is received for any unknown SCell, thus the gNB-CU normally has no idea which SCell is activated by MAC CE (i.e. which SCell’s activation triggers the L3 measurement report), i.e., which SCell’s TCI needs to be activated based on the L3 report. Therefore, CU would have to indicate the selected beam for each UE’s SCell to DU no matter this SCell needs to be activated or not, then DU use the information from CU to activate the TCI towards the SCell as needed.

Alt. 2: CU simply delivers the L3 report to DU

gNB-CU simply deliver the L3 report to gNB-DU when it receives the L3 report with the trigger type reportOnScellActivation. As the SCell activation command for any unknown SCell can trigger the UE’s L3 report for all SCells in case such type of report is configured, the L3 report transferred by CU includes L3 measurement results for all UE’s SCells. The DU can select the beam for TCI activation towards SCell whose activation command triggered the L3 report based on the L3 report. This alternative allows DU to carry out both SSB level and CSI-RS level beam selection for TCI activation as L3 report may include either or both SSB-based and CSI-RS based L3 measurement results.

Alt. 3: CU sends the assistant information for DU selecting beam for SCell

It’s the combination of Alt.1 and Alt.2.

For example, CU sends DU the candidate beam(s) together with the L3 measurement results for these beams. At reception of L3 report from UE, CU can use the L3 report to select the preferred beams for SCell’s TCI activation, then indicates the candidate beams to gNB-DU. Then DU selects the final beam within the candidate beams which are recommended by CU. Similar to Alt.1, CU has no idea which SCell is activated by MAC CE because any unknown SCell activation can trigger the L3 measurement report for all UE’s SCells. As a result, the CU would have to send the assistant information for all UE’s SCells to DU.

Proposal 3: RAN3 can down-select from following alternatives on CU sending DU the information of L3-report based TCI activation for fast SCell activation:

- Alt. 1: CU sends the selected beam for SCell

- Alt. 2: CU simply delivers the L3 report to DU

- Alt. 3: CU sends the assistant information for DU selecting the beam of SCell

Per RAN4’s agreements, in case L3 report is triggered at SCell activation, the TCI activation command for the SCell is assumed based on:

- the L3 report, if no L1-RSRP report is received before transmitting the TCI activation command.

- the L3 report or L1-RSRP report (i.e., up to network implementation), if L3 report and L1-RSRP report is received before transmitting the TCI activation command.

As well known, the L1-RSRP measurement result for SCell is always be reported after the UE receiving the SCell Activation command. In Alt.1, CU makes the final beam selection based on the L3 report and triggers DU to send TCI activation using the selected beam. However, the RAN4’s agreement implies that the beam selected for TCI activation may not be based on the L3 report even if the L3 report is received, in case the gNB also receives the L1 report. It means the DU may not follow the beam selected by CU for TCI activation, e.g., DU may select the beam based on L1 report, which makes the instruction from CU useless. On the contrast, Alt.3 and Alt.2 allow the DU to make the final decision and thus DU can determine whether to use L3 report or L1 report for beam selection. In Alt.3, with the recommended beams and the corresponding L3 measurement results, that DU can select the beam for TCI activation within the candidate beams based on the L3 report or L1 report. In Alt.2, the DU can select within all the beams based on the L3 report or L1 report. Alt.2 and Alt.3 are better aligned with RAN4’s assumption.

Observations:

- For Alt.1/Alt.3, CU may need to send DU the selected beam/assistance information for all UE’s SCells, no matter which SCell is being activated by DU

- Alt.1 requires DU to always follow the CU’s beam selection which is based on L3 report, while Alt.2 and Alt.3 allows DU to determine whether to use L3 report or L1 report for beam selection. Alt.2 and Alt.3 are better aligned with RAN4’s assumption than Alt.1.

For CU sending the information to DU to support L3 report based SCell activation in split architecture, there are two ways to enhance F1AP signalling: one is reusing or enhancing existing procedure/message, the other is introducing new procedure/message.

If only Alt.2 is adopted, enhancing the existing F1AP message can be considered. The UE’s L3 report can be transferred via an RRC container over F1 interface and the impact to existing IE/message is limited. For example, the UE CONTEXT MODIFICATION REQUEST message can be used. A new RRC container can be introduced to the CU to DU RRC Information IE in UE CONTEXT MODIFICATION REQUEST to transfer the L3 report of UE. According to the RRC spec[4], L3 measurement results of all serving cells are reported by UE via measResultServingMOList IE in MeasResults IE. So, this new RRC container can contain the measResultServingMOList IE contained in the MeasResults IE defined by RRC.

New procedure/message can be used in case multiple alternatives are agreed, and it can refer to the siganlling design for L3-report based LTM. In the last meeting, a new F1AP procedure has been introduced to support the L3-report based LTM. And the F1AP message (CU-DU MOBILITY INITIATION REQUEST) is used to convey the selected target cell/beam from CU to DU to trigger cell switch/early synchronization or send the assistance information on multiple candidate cells/beams from CU. Similarly, the F1AP message for L3-report based SCell activation can include the selected beam (if Alt.1 is adopted) by CU to trigger the TCI activation on DU, or include the assistance information of multiple candidate beams (if Alt.3 is adopted) or just UE’s L3 report (if Alt.2 is adopted) from CU for DU to determine the beam of TCI activation.

#### 9.2.2.x	CU-DU MOBILITY INITIATION REQUEST

This message is sent by the gNB-CU to the gNB-DU to trigger cell switch command and/or early synchronization for the UE.

Direction: gNB-CU ® gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| CHOICE Mobility Initiation | M |  |  |  | YES | reject |
| >Mobility Trigger |  |  |  |  |  |  |
| >>Triggering Indication | M |  | BIT STRING (SIZE(8)) | Indicates the triggering of the CU-DU Mobility Initiation procedure. First bit = early UL synchronization, second bit = early DL synchronization, third bit = cell switch, other bits reserved for future use. | - |  |
| >>Cell Switch Information | C-ifTrigger IndicationCellSwitch |  |  |  | YES | ignore |
| >>>Candidate Cell with Beam Information | M |  | 9.3.1.xxx |  | - |  |
| >>Early UL Sync Information | C-ifTrigger IndicationEarlyULSync |  |  |  | YES | ignore |
| >>>Candidate Cell with Beam Information List | M |  | 9.3.1.xxy |  | - |  |
| >>Early DL Sync Information | C-ifTrigger IndicationEarlyDLSync |  |  |  | YES | ignore |
| >>>Candidate Cell with Beam Information List | M |  | 9.3.1.xxy |  | - |  |
| >Assistance Information |  |  |  |  |  |  |
| >>Measurement Quantities | M |  |  |  | - |  |
| >>>Serving Cell Measurements | M |  |  |  | - |  |
| >>>>Serving Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>>>SSB List |  | 1..<maxnoofSSBs> |  |  |  |  |
| >>>>>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>>>>Selected Measurement Quantities | M |  | 9.3.1.xyy |  | - |  |
| >>>Candidate Cell ID List | M | 1 .. < maxnoofCandidateCells> |  |  | - |  |
| >>>>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>>>SSB List |  | 1..<maxnoofSSBs> |  |  |  |  |
| >>>>>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>>>>Selected Measurement Quantities | M |  | 9.3.1.xyy |  | - |  |

Proposal 4: RAN3 consider two ways for CU sending the information to DU on L3 report based fast SCell activation in split architecture, one is reusing or enhancing existing F1AP procedure/message, the other is introducing new F1AP procedure/message.

3 Conclusion

In the previous sections we made the following proposals:

Proposal 1: RAN3 to discuss whether to support L3 measurement report based SCell activation in split gNB architecture.

Proposal 2: If this feature is supported in split architecture, CU can send information on L3-report based TCI activation for SCell at reception of L3 report with type reportOnScellActivation.

Proposal 3: RAN3 down-select from following alternatives on CU sending DU the information of L3-report based TCI activation for fast SCell activation:

- Alt. 1: CU sends the selected beam for SCell

- Alt. 2: CU simply delivers the L3 report to DU

- Alt. 3: CU sends the assistant information for DU selecting the beam of SCell

Observations:

- For Alt.1/Alt.3, CU may need to send DU the selected beam/assistance information for all UE’s SCells, no matter which SCell is being activated by DU

- Alt.1 requires DU to always follow the CU’s beam selection which is based on L3 report, while Alt.2 and Alt.3 allows DU to determine whether to use L3 report or L1 report for selecting beam. Alt.2 and Alt.3 are better aligned with RAN4’s assumption than Alt.1.

Proposal 4: RAN3 consider two ways for CU sending the information to DU on L3 report based fast SCell activation in split architecture, one is reusing or enhancing existing F1AP procedure/message, the other is introducing new F1AP procedure/message.

4 Reference

- R2-2309450, LS on FR2 SCell activation enhancements, RAN2#123bis, RAN4, to RAN2, cc RAN1

- R2-2313936, Introduction of FR2 SCell enhancements, RAN2#124, Apple, CATT, Ericsson, Xiaomi, Qualcomm Incorporated, Huawei, HiSilicon, ZTE

- R2-2313937, Introduction of FR2 SCell enhancements, RAN2#124, Xiaomi, Apple, CATT, Ericsson, Qualcomm Incorporated, Huawei, HiSilicon, ZTE

- 3GPP TS 38.331, Radio Resource Control (RRC) protocol specification, v18.4.0 (2024-12)