3GPP TSG-RAN WG3 Meeting #129                                                              R3-255253

Bengaluru, India, 25 – 29 August 2025

Agenda item: 		12.2

Source: 		Qualcomm Incorporated

Title: 	(TP to TS 38.401) WAB resource coordination

Document for:		Other

# 1	Introduction

The Chair notes of the last RAN3 meeting captured on WAB resource coordination [1]:

There is no consensus on supporting resource coordination by specification in Rel-19.

This contribution discusses on how RAN3 should proceed with WAB resource coordination.

# 2	Discussion

## 2.1	WAB resource coordination

In last meeting, RAN3 had a lengthy discussion on the support of resource coordination for inband operation of WAB. There was agreement that resource coordination for WAB should leverage the resource coordination framework from IAB. However, there was a wide spread of opinions on the features to be used from IAB resource coordination for WAB.

Observation 1: RAN3 has no consensus on the features of IAB resource coordination to be used for WAB.

In the offline discussion on this topic, companies were not able to project a path to define this WAB enhancement within the limited remaining time of the WI and the necessary involvement of RAN1 and RAN2. Many companies felt uncomfortable to extend the WI for this purpose.

Observation 2: In offline discussion, companies were not able to project a path to define this WAB enhancement within the limited remaining time of the WI, the necessary involvement of RAN1 and RAN2, and without extension of the WI.

Finally, operators participating in the effort stated that they would not deploy resource coordination for WAB due to the need for WAB-related enhancements on the BH RAN, which would typically not be available.

Observation 3: Operators participating in the effort stated that they would not deploy resource coordination for WAB due to the need for WAB-related enhancements on the BH RAN.

The Chair notes of the last RAN3 meeting summarized the sentiment of Observations 1 to 3:

There is no consensus on supporting resource coordination by specification in Rel-19.

In this meeting, the WI should be concluded. There is no time left to discuss extensive new features with impact on RAN1 and RAN2, such as resource coordination.

Observation 4: In this meeting, the WI should be concluded. There is no time left to discuss extensive new features with impact on RAN1 and RAN2, such as resource coordination.

RAN3 should therefore deprioritize resource coordination for WAB in this release.

Proposal 1: RAN3 to deprioritize resource coordination for WAB in this release.

## 2.2	Co-location discovery

Without WAB resource coordination, there is no need for WAB co-location discovery.

Observation 5: Without WAB resource coordination, there is no need for co-location discovery.

RAN3#127bis achieved the following agreement on co-location discovery [2]:

Include a WAB-MT Identifier in the XN SETUP REQUEST, XN SETUP RESPONSE, NG-RAN NODE CONFIGURATION UPDATE and NG-RAN NODE CONFIGURATION UPDATE ACK e.g. for colocation discovery for resource multiplexing or for WAB node indication.

In the last meeting, RAN3 already achieved the following agreements on co-location discovery:

The “WAB-MT ID” sent from the WAB-gNB to the BH-gNB consists of the WAB-MT’s C-RNTI assigned by the BH-gNB and the cell id of BH-gNB´s cell serving the WAB MT.

Without co-location discovery, it is not necessary to include a WAB-MT identifier in the Xn messages. However, it is still necessary to include a WAB-MT indicator in these Xn message to allow WAB-nodes to avoid establishment of Xn.

Observation 6: Without co-location discovery, it is sufficient to have a WAB indicator rather than a WAB-MT identifier in the Xn messages.

Proposal 2: Replace WAB-MT identifier with WAB indicator in XN SETUP REQUEST, XN SETUP RESPONSE, NG-RAN NODE CONFIGURATION UPDATE and NG-RAN NODE CONFIGURATION UPDATE ACK messages.

## 2.3	Update to stage-2

Based on the above proposals, the BL CR to TS 38.401 needs to be updated as shown in the Annex.

Proposal 3: Agree TP to BL CR for TS 38.401 in Annex.

# 3	Conclusion

This contribution discussed resource coordination for WAB. The following observations and proposals have been made:

On WAB resource coordination:

Observation 1: RAN3 has no consensus on the features of IAB resource coordination to be used for WAB.

Observation 2: In offline discussion, companies were not able to project a path to define this WAB enhancement within the limited remaining time of the WI, the necessary involvement of RAN1 and RAN2, and without extension of the WI.

Observation 3: Operators participating in the effort stated that they would not deploy resource coordination for WAB due to the need for WAB-related enhancements on the BH RAN.

Observation 4: In this meeting, the WI should be concluded. There is no time left to discuss extensive new features with impact on RAN1 and RAN2, such as resource coordination.

Proposal 1: RAN3 to deprioritize resource coordination for WAB in this release.

On co-location discovery:

Observation 5: Without WAB resource coordination, there is no need for co-location discovery.

Observation 6: Without co-location discovery, it is sufficient to have a WAB indicator rather than a WAB-MT identifier in the Xn messages.

Proposal 2: Replace WAB-MT identifier with WAB indicator in XN SETUP REQUEST, XN SETUP RESPONSE, NG-RAN NODE CONFIGURATION UPDATE and NG-RAN NODE CONFIGURATION UPDATE ACK messages.

On update to stage-2

Proposal 3: Agree TP to BL CR for TS 38.401 in Annex.

# 4	References

[1]  Chair notes, 3GPP TSG RAN WG3 Meeting #128, Wuhan China, April 2025

[2]  Chair notes, 3GPP TSG RAN WG3 Meeting #127bis, St. Julian, Malta, May 2025

# 5	Annex: TP to BLCR for TS 38.401

-------------------------------------------Beginning of changes-------------------------------------------

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

### 6.1.x	Wireless Access Backhaul architecture

A WAB-node consists of a WAB-gNB and a WAB-MT. The WAB-gNB is based on the gNB functionality specified in TS 38.300 [2] and serves UEs by means of a terrestrial NR Uu radio link.

The WAB-MT is served by a BH-RAN-node. The WAB-gNB traffic, including NG, Xn and OAM traffic is transported via backhaul PDU session(s) of the WAB-MT.

NOTE: The use of other types of backhaul, e.g. non-3GPP backhaul, is up to implementation.

The WAB-gNB and the WAB-MT may connect to the same PLMN or to different PLMNs.

Figure 6.1.x-1 shows the WAB architecture for 5GS.

Figure 6.1.x-1: The WAB architecture

In in-band scenarios, backhaul and access of the WAB-node use terrestrial radio links. Resource coordination between access and backhaul links is based on implementation. In out-of-band scenarios, the backhaul can use a terrestrial or a non-terrestrial radio link, while the access uses terrestrial radio link.

The WAB-MT may connect to a public PLMN or an SNPN.

The WAB-gNB may connect to a public PLMN or an SNPN.

Figure 6.1.x-2 shows protocol stacks for NG Control plane and NG User plane transport via the wireless backhaul.

Figure 6.1.x-2: Protocol stacks for NG Control plane and NG User plane transport

Figure 6.1.x-3 shows protocol stacks for Xn Control plane and Xn User plane transport for WAB-node.

Figure 6.1.x-3: Protocol stacks for Xn Control plane and Xn User plane transport

-------------------------------------------Next change-------------------------------------------

# X	Wireless Access Backhaul

## X.1	WAB-node integration procedure

Figure X.1-1 WAB-node integration procedure

Phase 1: WAB-MT setup. The WAB-MT of a WAB-node connects to the network in the same way as a UE by performing RRC connection setup procedure with the BH-RAN-node. The WAB-MT then performs, authorization and authentication with the BH-5GC. After the WAB-MT is authorized, the WAB-MT can establish one or more PDU sessions for backhauling.

Phase 2: WAB-gNB setup. This phase includes the following 3 sub-phases:

Sub-phase 2-1: WAB-gNB initialization. In this phase, the WAB-gNB is configured by the OAM (e.g., with the information needed to establish NG connections towards one or more AMF(s) and the WAB-gNB is service-authorized by the SeGW or by the OAM).

Sub-phase 2-2: NG connection setup. The WAB-gNB establishes NG connection(s) toward the AMF(s). This step follows legacy procedures. After the NG is set up, the WAB-gNB can start serving UE(s).

Sub-phase 2-3: Xn connection setup. If needed, the WAB-gNB can establish Xn connection(s) towards the BH-RAN-node and/or other NG-RAN node(s). If the WAB-gNB includes a WAB-MT indicator in the signalling for Xn connection setup, the BH-RAN-node and/or other NG-RAN node(s) can understand that the peer node is the WAB-gNB of the WAB-node.

## X.2	Configuration of WAB-node

### X.2.1	General

The following configurations of the WAB-node may need to be updated as the node moves:

-	The parameters that enable the WAB-gNB to select and connect to the AMF(s) that serve the UE(s).

-	The parameters that enable the WAB-gNB to connect to, and communicate with, the OAM system.

-	The parameters that the WAB-gNB should broadcast, e.g., the TAC(s), the cell ID(s), the RANAC(s).

A WAB-node may be provisioned with the parameters pertinent to different potential locations of the WAB- node. Alternatively, the OAM can provision configuration parameters to the WAB-node based on the location of the node. In that case, the continuity of OAM connectivity needs to be ensured as the WAB-node moves.

### X.2.2	IP address configuration for the WAB-gNB

A WAB-MT obtains IP address(es) for the PDU sessions in the same manner as a legacy UE.

The IP address(es) of the WAB-MT can be used for the backhauling of WAB-gNB’s NG, Xn and OAM traffic. In case the WAB-gNB uses the IPsec tunnel mode to protect the OAM, NG and/or Xn traffic, the IP address(es) of the WAB-MT is used as outer IP address(es), and the allocation of the inner tunnel IP address(es) is outside of 3GPP scope.

### X.2.3	TAC/RANAC (re-)configuration for a WAB-gNB’s cell

The TAC/RANAC of WAB-gNB’s cell is configured by the OAM. During the mobility of WAB-node, it can be reconfigured by the OAM, or it can remain unchanged in a certain geographical area. The TAC/RANAC of the WAB-gNB’s cell may be the same as, or different than, the TAC/RANAC of the co-located WAB-MT’s serving cell. The TAC/RANAC broadcast by the WAB-gNB’s cell can be changed in order to reflect the WAB-node’s physical location.

## X.3 	NG connection management

Based on the OAM configuration, the WAB-gNB can set up NG interface with an AMF. When disconnecting from an AMF is required, due to inter-AMF mobility of a WAB-gNB, or when the authorization status of the WAB-gNB changes from “authorized” to “not authorized”, the WAB-gNB may request the removal of the NG interface by triggering the NG Removal procedure toward the AMF.

## X.4 	WAB-node authorization

WAB-node authorization includes authorization of the WAB-MT’s backhaul support, and service authorization of the WAB-gNB, as defined in TS 23.501 [3].

Authorization of the WAB-MT is different from the service authorization of the WAB-gNB.

Authorization of the WAB-MT is defined in TS 23.501 [3].

Authorization of the WAB-gNB provides the service authorization, i.e., the right to serve UEs.

When the WAB-gNB’s service authorization status changes from “authorized” to “not authorized”:

- The WAB-gNB attempts to handover, or release, the UEs.

- The NG and Xn connections of the WAB-gNB are removed.

- The PDU sessions of the WAB-MT used for backhauling may be released.

It is expected that WAB-MT’s PLMN/SNPN ensures that backhaul PDU sessions of the WAB-MT are maintained long enough for the WAB-gNB to perform UE handover/release and the removal of NG and Xn connections, as specified in TS 23.501 [3].

## X.5 	User Location Information for UEs served by a WAB-gNB

For UEs served by a WAB-gNB, in addition to the User Location Information (ULI), the WAB-gNB also provides the core network with Additional ULI, which includes a TAI and a NR CGI pertinent to the WAB-gNB’s broadcasted PLMN/SNPN.

If the PLMN/SNPN broadcasted by a WAB-gNB is the same as the PLMN/SNPN serving the WAB-MT, and the WAB-MT connects to the BH-gNB by means of a terrestrial link, the Additional ULI for UEs served by the WAB-gNB includes the TAI and the NR CGI of the cell serving the WAB-MT.

If the PLMN/SNPN serving the WAB-MT is different from the WAB-gNB’s broadcasted PLMN/SNPN, and the WAB-MT connects to the BH-gNB by means of a terrestrial link, the Additional ULI for UEs served by the WAB-gNB is determined by the WAB-gNB, based on the WAB-node’s geo-location.

If the WAB-MT connects to the BH-gNB by means of a non-terrestrial link, the Additional ULI for UEs served by WAB-gNB is determined by the WAB-gNB, based on WAB-node’s geo-location. This applies regardless of whether the PLMN/SNPN serving the WAB-MT is the same as, or different than, the WAB-gNB’s broadcasted PLMN/SNPN.

In case Additional ULI for UEs served by a WAB-gNB changes, e.g., due to WAB-node movement, the WAB-gNB derives the new Additional ULI and reports it via legacy procedures, if required by the core network.

## X.6 	PCI collision avoidance

In WAB-node deployments, the legacy mechanism for PCI collision avoidance can be reused. PCI space can be partitioned by allocating a range of PCIs to WAB-gNB cells.

## X.7	WAB-node mobility

### X.7.1	WAB-MT mobility

The WAB-MT reuses legacy mobility procedures defined for the UE. During the WAB-node’s movement, when the BH PDU session(s) of WAB-MT are re-established, the co-located WAB-gNB may need to update the IP addresses used for its traffic. In case IPsec tunnel mode is used to protect the WAB-gNB’s traffic, MOBIKE (IETF RFC 4555 [29]) can be used to avoid the change of inner IP addresses used for this traffic. Otherwise, following procedures can be used for handling the IP address change of the WAB-gNB’s traffic:

- NG-C and Xn-C can be migrated to the new IP addresses via legacy procedures defined in TS 38.412[36] and TS 38.422[37], respectively.

- NG-U GTP-U tunnels can be migrated via the legacy NGAP PDU Session Resource Modify Indication procedure.

The continuity of OAM connectivity of the WAB-gNB needs to be ensured as the WAB-node moves across the BH network.

### X.7.2	WAB-gNB mobility

#### X.7.2.1 	WAB-gNB mobility with change of UE’s AMF(s)

Due to the WAB-gNB’s mobility, the UEs moving together with the WAB-gNB may have to change AMF.

For the AMF change, a new logical WAB-gNB is instantiated, which establishes NG connection(s) towards one or more new AMF(s). The new logical WAB-gNB may obtain from the OAM the configuration parameters needed to establish the connection(s) to the UE’s new AMF(s), based on, e.g., WAB-node’s location.

The new logical WAB-gNB may activate one or more cell(s) with new TAC, cell ID, and PCI, which depend on the WAB-node’s current location.

Editor’s Note 1:	FFS if the new cell(s) can belong to the WAB-gNB serving the old cell(s).

The UEs are handled as follows:

- A UE in RRC_CONNECTED state is handed over from a cell served by the old logical WAB-gNB to a cell served by the new logical WAB-gNB via NG-based handover with AMF relocation, as defined in TS 23.502 [7], after which the UE’s AMF is changed to the new AMF.

- A UE in RRC_IDLE or RRC_INACTIVE state camping on a cell served by the old logical WAB-gNB reselects a cell served by the new logical WAB-gNB. The reselection may be triggered by the removal of the old logical WAB-gNB’s cells from service. After cell reselection, the UE performs a Mobility Registration Update as defined in TS 23.502 [7], which is triggered by the new TAC broadcasted by the new logical WAB-gNB’s cell. During this Mobility Registration Update, the UE’s AMF is changed to the new AMF.

After all the UEs in RRC_CONNECTED state are handed over, the NG connection(s) between the WAB-node and the old AMF(s) are removed via NG Removal procedure and the old logical WAB-gNB’s cell(s) are removed from service.

-------------------------------------------End of changes-------------------------------------------