3GPP TSG-RAN WG3 Meeting #129                                                              R3-255255

Bengaluru, India, 25 – 29 August 2025

Agenda item: 		17.3

Source: 		Qualcomm Incorporated

Title: 	(TPs to BL CR 38.401) Stage-2 procedures for OD SIB1

Document for:		Other

# 1	Introduction

RAN3 achieved the following agreements related to OD SIB1 [1][2]:

Scenario1: UE obtains the UL WUS configuration from Cell A, and transmits UL WUS to NES Cell. - UL WUS configuration from NES Cell to Cell A via Xn, e.g., before entering on-demand SIB1 mode in the NES Cell or when the UL WUS configuration is changed Focus on case2 in RAN3 first. Case 2 can be supported with the below details: UL WUS configuration is decided by NES Cell DU. UL WUS configuration transmission from NES Cell DU to NES Cell CU. NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface. Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject). The UL WUS configuration will be transferred over Xn via a new class1 defined procedure. Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB. The purpose of the new XnAP procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2. The procedure uses non UE-associated signaling. Cell A stores the UL WUS configuration information after it has received it. A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB. The procedure uses non UE-associated signalling. Reuse the existing IE “Served Cell Information” IE to include UL WUS configuration from gNB-DU to gNB -CU in F1 Setup Request and gNB-DU Configuration Update procedures. It is agreed that after Cell A gNB has received and accepted the UL WUS Configuration request from the NES Cell gNB, it broadcasts in SIBx at the first opportunity. It is agreed from the signalling point of view, in UL WUS Configuration Provision Request (naming to be further discussed) from NES Cell gNB to Cell A gNB, one option is “start with UL WUS Configuration”. This codepoint/choice is used to trigger Cell A to broadcast UL WUS configuration in SIBx. It is agreed that NES Cell gNB can “stop” (naming to be further discussed) Cell A to broadcast UL WUS configuration. In the new Class 1 message (Direction NES gNB to Cell A gNB), RAN3 has agreed that: - One Choice “Start with UL WUS Condiguration” - One Choice “Stop” meaning the Cell A gNB will remove/release/discard the UL WUS Configuration and stop the broadcasting, it also means that next time the NES gNB should use “Start with UL WUS Configuration”. The Xn/F1 TP capturing the above agreements in the tabular are agreed in: R3-252385 (Xn) and R3-252386 (F1). One “Provision Request message includes one “OD-SIB1 config R19” referring to the TS 38.331 definition, it is a RRC Container in octet string (presence M) + one NES Cell ID (presence M ) + one Cell-A ID (presence O ) Cell A gNB-CU encoding the SIBxx. The NES gNB-CU sends the indication to NES gNB-DU. The NES gNB-DU MAY go to OD-SIB 1 operation up to gNB-DU decision

The following discussion aims to define the stage-2 procedures for the supports of OD SIB1. In this context, remaining stage-2 issues are identified.

# 2	Discussion

## 2.1	Class-1 UL WUS CONFIGURATION PROVISION procedure

Based on RAN3 agreements [2], the NES-cell CGI is used as the indicator for an UL WUS configuration across UL WUS CONFIGURATION PROVISION REQUEST, - RESPONSE and -FAILURE messages.

The UL WUS CONFIGURATION PROVISION REQUEST may also include a cell-A CGI. This Cell-A CGI, however, is only optionally included, and therefore, it can only be regarded as a recommendation. This implies that the NES cell gNB leaves it up to the Cell-A gNB, whether the Cell-A gNB wants to use the recommended cell-A, or not. In fact, the Cell-A gNB may select a different or an additional set of cell-As for the transmission of the UL WUS configuration.

The UL WUS CONFIGURATION PROVISION RESPONSE should therefore not include any cell-A CGI indication. Instead, the NES-cell gNB can trust the cell-A gNB that the Cell-A gNB selects an appropriate (set of) cell-A(s).

Observation 1: Since the Cell-A CGI is only optionally included in the UL WUS CONFIGURATION PROVISION REQUEST it can only be regarded as a recommendation, but not as a mandate.

Proposal 1: The UL WUS CONFIGURATION PROVISION RESPONSE to not include a cell A CGI since the NES gNB need not know the cell-As selected by cell-A gNB for the transmission of the NES-cell’s UL WUS configuration.

## 2.2	Class-2 UL WUS CONFIGURATION STATUS UPDATE procedure

Since the NES gNB does not know the cell-A(s) where the UL WUS configuration is transmitted, the Cell-A gNB should not include any indication of Cell-A CGIs in this status update (same as for UL WUS CONFIGURATION PROVISION REQUEST, above).

Proposal 2: The UL WUS PROVISION STATUS UPDATE message to not include (a) cell A CGI(s) since the NES gNB need not know the cell-As selected by Cell-A gNB for the transmission of the NES-cell’s UL WUS configuration.

However, when the NES gNB receives the UL WUS CONFIGURATION STATUS UPDATE, it needs to know to which NES-cell the message refers. The message therefore needs to include the NES-cell CGI.

Proposal 3: The Cell-A gNB needs to include the NES cell ID in the UL WUS CONFIGURATION PROVISION STATUS UPDATE to which the status update refers.

## 2.3	F1 enhancements

RAN3 agreed that it is the Cell-A gNB-CU, who owns the SIB containing UL WUS CONFIGURATION. The Cell-A gNB-CU therefore does not have to ask Cell-A gNB-DU for permission to transmit this SIB. Consequently, no enhancement is needed on the F1 of the Cell-A gNB.

Proposal 4: No enhancements are needed for F1 on Cell-A gNB since it is Cell-A gNB-CU, since Cell-A gNB-CU owns the SIB containing the UL WUS configuration, and therefore, it does not need to ask the Cell-A gNB-DU for permission to transmit this SIB.

RAN3 already agreed on enhancements to F1 on NES gNB. No additional enhancements are needed for F1 on NES gNB.

Proposal 5: No further enhancements needed for F1 on NES gNB.

## 2.4	TP to 38.401

A TP to 38.401 on the introduction for on-demand SIB1 is included in the Annex.

Proposal 6: Agree on TP to TS 38.401 in the Annex.

# 3	Conclusion

This contribution discussed stage-2 procedures for the support of OD SIB1. The following observations and proposals have been made:

Class-1 UL WUS CONFIGURATION PROVISION procedure:

Observation 1: Since the Cell-A CGI is only optionally included in the UL WUS CONFIGURATION PROVISION REQUEST it can only be regarded as a recommendation, but not as a mandate.

Proposal 1: The UL WUS CONFIGURATION PROVISION RESPONSE to not include a cell A CGI since the NES gNB need not know the cell-As selected by cell-A gNB for the transmission of the NES-cell’s UL WUS configuration.

Class-2 UL WUS CONFIGURATION STATUS UPDATE procedure:

Proposal 2: The UL WUS PROVISION STATUS UPDATE message to not include (a) cell A CGI(s) since the NES gNB need not know the cell-As selected by Cell-A gNB for the transmission of the NES-cell’s UL WUS configuration.

Proposal 3: The Cell-A gNB needs to include the NES cell ID in the UL WUS CONFIGURATION PROVISION STATUS UPDATE to which the status update refers.

F1 enhancements:

Proposal 4: No enhancements are needed for F1 on Cell-A gNB since it is Cell-A gNB-CU, since Cell-A gNB-CU owns the SIB containing the UL WUS configuration, and therefore, it does not need to ask the Cell-A gNB-DU for permission to transmit this SIB.

Proposal 5: No further enhancements needed for F1 on NES gNB.

TP to 38.401:

Proposal 6: Agree on TP to TS 38.401 in the Annex.

The stage-3 issues are the following:

TP to BL CR to TS 38.401:

Proposal 9: RAN3 to approve TP for BL CR to TS 38.401 in the Annex.

# Reference

[1]  Chair notes, 3GPP TSG RAN WG3 Meeting #127bis, Wuhan, China, April 2025 and earlier meetings.

[2]  Chair notes, 3GPP TSG RAN WG3 Meeting #128, St Julian, Malta, May 2025

# Annex: TP for BL CR to 38.401

BEGIN OF CHANGE

## 8.x	Procedures for network energy savings

### 8.x.1	Procedures for on-demand SIB1

The high-level functionality of on-demand SIB1 for network energy savings is specified in TS 38.300 [2].

The following describes the inter-gNB signaling procedures and enhancements to intra-gNB signaling for split architecture in support of on-demand SIB1.

Figure 8.x.1-1 shows an example for the procedure, where a gNB (gNB1) requests a peer gNB (gNB2) to start or to stop transmitting the UL WUS configuration(s) of one of its cells.

Figure 8.x.1-1: Procedure for provisioning of UL WUS configuration to peer gNB in support of OD SIB1

The procedure has the following steps:

1. gNB1-DU sends to gNB1-CU the UL WUS configuration for each cell that supports OD SIB1 operation using the F1 SETUP REQUEST or GNB-DU CONFIGURATION UPDATE messages. gNB1-DU further indicates to gNB1-CU the cells that have discontinued support of OD SIB1 operation.

2. gNB1-CU replies with the F1 SETUP RESPONSE or GNB-DU CONFIGURATION UPDATE ACKNOWLEDGEMENT message, respectively.

3. gNB1-CU determines that gNB2 should transmit the UL WUS configuration of one of gNB1-DU’s cells, which supports OD SIB1 operation. Alternatively, gNB1-CU determines that gNB2 should stop transmitting the UL WUS CONFIGURATION for one of gNB1-DU’s cells, which has discontinued support of OD SIB1 operation.

4. gNB1 sends the UL WUS CONFIGURATION PROVISION REQUEST message to gNB2 and potentially other peer gNBs including the UL WUS configuration and the cell ID of the cell that supports OD SIB1 mode. In this case, gNB1 may include a gNB2 cell it recommends for the transmission of this UL WUS configuration. Alternatively, gNB1 uses this message to send an indication to gNB2 and potentially other peer gNBs to stop the transmission of the UL WUS configuration together with the cell ID of the gNB1-DU cell that has discontinued support of OD SIB1.

5. In case gNB2-CU has received an UL WUS configuration for gNB1 cell in step 4, it determines the gNB2 cell(s) that should transmit this UL WUS configuration. Alternatively, in case gNB2-CU has received indication to stop transmitting an UL WUS configuration for a gNB1 cell, gNB2-CU determines the gNB2 cells that presently transmit this UL WUS configuration.

6. gNB2 sends the UL WUS CONFIGURATION PROVISION RESPONSE message to gNB1 to confirm the start or stop of the transmission of the UL WUS configuration for gNB1’s cell.

7. gNB2-CU sends a GNB-CU CONFIGURATION UPDATE message to the gNB2-DU including updated gNB-CU-owned System Information. gNB2-CU may have added or removed an UL WUS configuration from this System Information for one or more gNB2 cells.

8. gNB2-DU replies with the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGEMENT message, and it performs System Information update for the affected cells.

9. Based on the confirmation received from gNB2 in step 6 and potentially other peer gNBs, gNB1-CU determines whether the gNB1-DU cell can enter OD SIB1 operation mode.

10. gNB1-CU informs gNB1-DU about the cells that can operate in OD SIB1 operation mode via the GNB-CU CONFGIGURATION UPDATE message.

NOTE: Before gNB1-CU indicates to gNB1-DU that a cell can enter OD SIB1 operation, it should leave sufficient time for gNB2 to start the transmission of this cell’s UL WUS configuration, based on implementation.

11. gNB1-DU sends the GNB-CU CONFGIGURATION UPDATE ACKNOWLEDGE message to gNB1-CU.

Figure 8.x.1-2 shows an example for the procedure, where a gNB (gNB2) indicates to a peer gNB (gNB1) that it discontinues the transmission of an UL WUS configuration.

Figure 8.x.1-2: Procedure for update of UL WUS transmission status to peer gNB

The procedure has the following steps:

1. gNB2-CU determines that a cell should discontinue the transmission of an UL WUS configuration for a gNB1 cell.

2. gNB2 sends the UL WUS CONFIGURATION STATUS UPDATE message to gNB1 including the cell ID of the gNB1 cell that is using this UL WUS configuration.

3. Based on the information received from gNB2, gNB1-CU determines whether the gNB1 cell indicated in step 2 has to discontinue OD SIB1 operation.

4. In case the affected cell needs to discontinue OD SIB1 operation, gNB1-CU sends an indication to gNB1-DU via the GNB-CU CONFGIGURATION UPDATE message.

5. gNB1-DU replies with the GNB-CU CONFGIGURATION UPDATE ACKNOWLEDGE message and discontinues OD SIB1 mode for the cell indicated by gNB1-CU.

6. gNB2-CU sends a GNB-CU CONFIGURATION UPDATE message to the gNB2-DU, where the UL WUS configuration has been removed from this System Information for the affected gNB2 cells.

7. gNB2-DU replies with the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGEMENT message, and it performs System Information update for the affected cells.

NOTE: Before gNB2-CU sends system information update to gNB2-DU to discontinue the transmission of an UL WUS configuration(s), it should leave sufficient time for gNB1 to deactivate the OD SIB1 operation for the cells affected, based on implementation.

END OF CHANGE