3GPP TSG-RAN3 Meeting #129	R3-255256

Bengaluru, India, 25 – 29 August 2025

Agenda Item:	18.2

Source: 	Qualcomm Incorporated

Title: 	Remaining open issues on LP-WUS support

Document for:	Discussion/Decision

# Background

- (TP To BL CR for TS 38.300) support of homogeneous deployment for LP-WUS in R3-253831 noted, wait for reply LS from SA2

To be continued...

- FFS on the range of LP-WUS CN Subgroup ID IE included in LP-WUSPS Assistance Information pending on RAN1 decision on which subgroup ID(s) would be reserved for common codepoint.

- Whether to capture that the eDRX and LP-WUS may be used together by the gNB for RAN paging and CN paging.

- The issue on UE battery drain during LP-WUS monitoring and very long eDRX cycles can be discussed based on companies’ contributions.

Below are the open topics on LP-WUS feature from RAN3#128 meeting:

In this contribution, we discuss the details for these open issues.

# Discussion

## 2.1 LP-WUS paging loss issue

RAN2 sent an LS reply on paging capability loss issue in [1], R2-2503172, indicating the below

RAN2 further discussed the issue of UE radio paging capability loss mentioned in the LS S2-2412876 and acknowledged that based on the legacy signalling, a gNB implementing Release X may not be able to forward the radio paging capabilities for a feature introduced after Release X, which results in UE missing the paging message.

For UE Radio Paging Capabilities introduced up to Rel-18, RAN2 agreed not to introduce any standard solution, i.e., rely on proper network implementation, as follows.

- We will not change legacy UE capabilities and rely on network implementation

To address the issue for UE Radio Paging Capabilities introduced in Rel-19 onwards, RAN2 made the following agreement:

- Use a “container” using OCTET STRING with content generated by UE when new UE Radio Paging Capabilities are introduced.   This is applicable only to new Rel-19 and future capabilities added to paging capabilities.  This will only work for gNB that support this Rel-19 “container”

In particular for Rel-19 LP-WUS feature, if it is deployed, all the gNBs belonging to a certain area need to support the Rel-19 “container”.

Observation 1: RAN2 agreed in [1] that there is no LP-WUS paging loss issue for network with all the gNBs belonging to a certain area support the Rel-19 UE Radio Paging Capabilities container including the LP-WUS capabilities.

SA2 sent [2],S2-2506087, reply LS to RAN2 LS [1], with below response (truncated from the LS)

The 5GS design means that the UE Radio Capability for Paging is loaded into the AMF only in the cell where the UE attaches (does “initial registration”) or first moves from LTE to NR. After that the UE can move throughout the whole PLMN and normally there is no event that triggers any update of the UE Radio Capability for Paging information stored in the AMF. Hence the “certain area” mentioned by RAN 2 is the whole NR coverage of that PLMN.

It is essential that operators are able to use RAN equipment from different vendors and that competition between those RAN vendors is not inhibited by RAN 2 requiring all RAN vendors to upgrade their equipment before new UE functionality can be used with the first RAN vendor to develop a solution.

Observation 2: As explained in RAN2 LS [1] and SA2 LS reply [2],  if the UE initial registration is performed in a pre-Rel-19 gNB area, then the pre-Rel-19 gNB does not forward the Rel-19 LP-WUS UE Radio Paging Capabilities container to the AMF. Hence, LP-WUS paging loss is expected when the UE moves to the coverage area of Rel-19 gNB which does not receive the container from AMF.

On the LP-WUS paging loss, we can discuss below options in RAN3 and converge on a way forward:

- Ignore SA2 concerns and expect that entire PLMN RAN is upgraded to Rel-19 before LP-WUS feature is deployed.

- Identify technical solutions to address SA2 concerns in this meeting and potentially extending the WI.

- Leave it up to the RAN plenary to decide whether and how to address SA2 concerns, potentially in next release.

Proposal 1: On the LP-WUS paging loss, RAN3 to discuss these options to agree on a way forward: 1. Ignore SA2 concerns (e.g. add a NOTE) 2. Address SA2 concerns in Rel-19 (e.g. extending the WI) 3. RAN plenary to decide (e.g. address in next release).

## 2.2 CN Subgroup ID range

In RAN3#128, RAN3 discussed on LP-WUS CN subgroup ID range, but did not agree on the range of 0 to 30 or 1 to 31 and left an FFS “FFS on the range of LP-WUS CN Subgroup ID IE included in LP-WUSPS Assistance Information pending on RAN1 decision on which subgroup ID(s) would be reserved for common codepoint”

RAN1 made the agreements below on common codepoints

RAN1#120bis agreements

Agreement

For Option 2, at least one codepoint corresponding to each of the subgroups in each PO is supported.

- For codepoint corresponding to more than one subgroups:

  - Alt 2: One codepoint for each PO corresponding to all the subgroups in the PO

Agreement

For Option 2, a common codepoint per PO is always used and the maximum number of subgroups supported per PO is

- 7 for the case where 4 POs are mapped to one LO

- 15 for the case where 2 POs are mapped to one LO

RAN1#121 agreements

Agreement

For PO-to-LO association and codepoint determination, assume NPOLO is the number of POs associated with a LO, NSGPO is the number of subgroups per PO for LP-WUS, and  iSG is the subgroup ID of a UE (0≤iSG<NSGPO).

- The PO index within the LO is defined as

iPO=UE_ID mod N∙Ns+is mod NPOLO,

- The reference PF for the LO of a PO is provided by (SFN for PF) – floor(iPO/NS) * T/N, which is the first PF of the PF(s) associated with the LO.

- For the codepoints,

  - If NSGPO>1,

    - The number of information bits in LP-WUS is log2NPOLO*(NSGPO+1).

    - Alt 2: (the codepoints for each PO are consecutive)

      - The codepoint for subgroup iSG in PO iPO is iPO*(NSGPO+1)+iSG.

      - The codepoint for all the subgroups in PO iPO is (iPO+1)*NSGPO+1-1.

  - If NSGPO=1, the number of information bits in LP-WUS is log2NPOLO, and the codepoint for PO iPO is iPO.

- Here UE_ID (for LP-WUS), N, NS, is and T are defined in TS 38.304

:

As per RAN1 agreements, 1 to 4 common subgroup IDs are used by the RAN for UE-ID based subgrouping based on the number of POs mapped to one LO. In case of 4 POs mapped to 1 LO, common codepoint for each PO derived from the equation will be 7 for PO1, 15 for PO2, 23 for PO3, 31 for PO4. For 2 POs mapped to 1 LO, common codepoint for each PO will be 15 for PO1 and 31 for PO2. For 1 PO mapped to 1 LO, common codepoint is 31.

Observation 3: The common code point used per PO is either ‘7’, ‘15’, ‘23’ or ‘31’ as per RAN1 agreements on multiple POs (1,2 or 4) mapped to one LO.

Observation 4: Number of POs mapped per LO is a gNB configuration that changes on a per cell basis and not known to the Core network.

CN assigns CN based subgrouping IDs to the UEs supporting CN based subgrouping based on the CN subgroup ID range configured by OAM. However, allowing the CN to assign any ID in the range of ‘0 to 31’ and not excluding common subgroup IDs from the CN based subgrouping ID introduces below issues:

- CN Page sent to a specific UE with a CN based subgroup ID conflicting with the common subgroup ID in Observation 3 results in wake up of other UEs as well which are monitoring for common subgroup ID.

- If gNB has to send a RAN page to a subgroup of UEs, then gNB uses the common subgroup ID corresponding to the PO for those UEs. If common subgroup IDs are not reserved, then it is not possible for UE supporting CN based subgrouping to identify it is common page and wake up.

Different cells in the network may support a different PO to LO mapping like 4:1, 2:1 or 1:1 and may only need 4 or 2 or 1 common subgroup IDs. However, CN assigns the CN based subgroup ID across the UEs in the network during Registration and it is not possible to change this assignment based on the cell configuration dynamically. Thus, it is better to be conservative and exclude 4 common subgroup ID values of  ‘7’, ‘15’, ‘23’ or ‘31’ from the “LP-WUS CN Subgroup ID” range.

Observation 5: LP-WUS CN Subgroup ID range exclude the common subgroup IDs to avoid CN assigned subgroup ID conflicting with the common subgroup ID and resulting in either paging loss or unnecessary wakeups.

Observation 6: CN assigns the CN based subgroup ID across the UEs in the network during Registration and it is not possible to change this assignment based on the cell configuration (PO to LO mapping of 4:1, 2:1, or 1:1) dynamically.

Proposal 2: Define the range for “LP-WUS CN Subgroup ID” in LP-WUSPS Assistance Information IE as ‘0 to 31’, with a NOTE that CN subgroup ID values of ‘7’, ‘15’, ‘23’, ‘31’ are not assigned to the UE for CN-assigned subgroup IDs by the network to be in compliance with RAN1 agreements.

Proposal 3: Send an LS to SA2, RAN1 and RAN2 to inform them about the “LP-WUS CN Subgroup ID” range and the excluded common subgroup ID values.

## 2.3 LP-WUS and eDRX support

RAN1/RAN2 TR [4] discussed in the introduction section that LP-WUS is introduced to provide better power savings than long eDRX. Thus, UE will wake up MR only when paged and monitor per the long eDRX.

Observation 7: In R2-2503182, RAN2 informed RAN3 that “LP-WUS is supported with eDRX, FFS on exact impact if any”. RAN2 has not identified any further impacts in the RAN2#130 meeting.

Proposal 4: RAN3 to agree that LP-WUS and eDRX can be used together and there is no impact to RAN3 specifications to support LP-WUS and eDRX together.

# Summary

Below are the observations and proposals discussed in this document.

Observation 1: RAN2 agreed in [1] that there is no LP-WUS paging loss issue for network with all the gNBs belonging to a certain area support the Rel-19 UE Radio Paging Capabilities container including the LP-WUS capabilities.

Observation 2: As explained in RAN2 LS [1] and SA2 LS reply [2],  if the UE initial registration is performed in a pre-Rel-19 gNB area, then the pre-Rel-19 gNB does not forward the Rel-19 LP-WUS UE Radio Paging Capabilities container to the AMF. Hence, LP-WUS paging loss is expected when the UE moves to the coverage area of Rel-19 gNB which does not receive the container from AMF.

Observation 3: The common code point used per PO is either ‘7’, ‘15’, ‘23’ or ‘31’ as per RAN1 agreements on multiple POs (1,2 or 4) mapped to one LO.

Observation 4: Number of POs mapped per LO is a gNB configuration that changes on a per cell basis and not known to the Core network.

Observation 5: LP-WUS CN Subgroup ID range exclude the common subgroup IDs to avoid CN assigned subgroup ID conflicting with the common subgroup ID and resulting in either paging loss or unnecessary wakeups.

Observation 6: CN assigns the CN based subgroup ID across the UEs in the network during Registration and it is not possible to change this assignment based on the cell configuration (PO to LO mapping of 4:1, 2:1, or 1:1) dynamically.

Observation 7: In R2-2503182, RAN2 informed RAN3 that “LP-WUS is supported with eDRX, FFS on exact impact if any”. RAN2 has not identified any further impacts in the RAN2#130 meeting.

Proposal 1: On the LP-WUS paging loss, RAN3 to discuss these options to agree on a way forward: 1. Ignore SA2 concerns (e.g. add a NOTE) 2. Address SA2 concerns in Rel-19 (e.g. extending the WI) 3. RAN plenary to decide (e.g. address in next release).

Proposal 2: Define the range for “LP-WUS CN Subgroup ID” in LP-WUSPS Assistance Information IE as ‘0 to 31’, with a NOTE that CN subgroup ID values of ‘7’, ‘15’, ‘23’, ‘31’ are not assigned to the UE for CN-assigned subgroup IDs by the network to be in compliance with RAN1 agreements.

Proposal 3: Send an LS to SA2, RAN1 and RAN2 to inform them about the “LP-WUS CN Subgroup ID” range and the excluded common subgroup ID values.

Proposal 4: RAN3 to agree that LP-WUS and eDRX can be used together and there is no impact to RAN3 specifications to support LP-WUS and eDRX together.

# References

[1] R2-2503172, Reply LS on paging capability loss issue, source: RAN2, To: SA2, RAN3, CT1

[2] S2-2506087, Response to Reply LS on paging capability loss issue, source: SA2, To: RAN2, RAN3, Cc: TSG RAN, CT1

[3] R2-2503182, LS on LP-WUS subgrouping progress, source: RAN2, To: RAN3, Cc: SA2, SA3

[4] RP-240801 - WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO, Ericsson, MediaTek, Samsung, Sony