3GPP TSG-RAN WG3 Meeting #129	R3-255259

Bengaluru, India, 25 - 29 August 2025

Agenda Item:	8.1

Source:	ZTE Corporation

Title:	Further discussion on emergency call back and paging

Document for:	Discussion

- Introduction

In last SA2 meeting, SA2 has replied RAN3’s LS on emergency call back and paging and provided two questions on this issue[1], which is given as below.

SA WG2 thanks RAN3 for the LS on emergency call back and paging. SA2 could not reach consensus to agree to update our specifications. SA2 would request some clarifications from RAN3. For normal sessions, it is possible for SMF to keep the PDU session while the NG-RAN releases the RRC connection, e.g. AN release due to User Inactivity timer expired. However there is different understanding in SA2 regarding the case that emergency PDU session exists. Hence SA2 have the following questions: Q1： Is it possible to have different behaviour in NG-RAN node depending on the presence of the emergency PDU session to decide when to release RRC connection, compared to when there is no emergency PDU session? Q2: If yes, when the emergency PDU session exist, whether it is required to configure the NG-RAN to not release RRC connection until the emergency PDU session is released by the SMF?

In this contribution, we further provide our views on the reply LS from SA2.

2	Discussion

In the previous meeting, RAN3 has already discussed the emergency PDU session issue and reached the following consensus.

It can be left unspecified whether NG-RAN can release a UE with an emergency PDU session to RRC_INACTIVE state.

In addition, in last RAN3 meeting, RAN3 has received RAN2’s reply LS on this issue in [2]. RAN2 agreed that no new UE behavior will be specified for this case. And RAN2 also indicates that it is up to the gNB implementation whether to release the UE to RRC_IDLE or RRC_INACTIVE or keep the UE in RRC_CONNECTED state for fast emergency callback.

Observation: RAN3 and RAN2 share the view that it is up to the gNB implementation whether to release the UE to RRC_IDLE or RRC_INACTIVE or keep the UE in RRC_CONNECTED state for fast emergency callback.

For the Q1, currently, there is no corresponding description in RAN3’s specifications for the NG-RAN node to decide when to release RRC connection, regardless of the presence of emergency PDU session, which is also aligned with the above conclusion from RAN2.

Proposal 1: There is no specific description for the NG-RAN node to decide when to release RRC connection regardless of the presence of emergency PDU session.

For the Q2, in addition to User Inactivity, there could be many other reasons for the NG-RAN node to release RRC connection, as shown in the TS 38.413.

-----------------------------------------------------------TS 38.413-------------------------------------------------------------------------

| Radio Network Layer cause | Meaning |
| --- | --- |
| Unspecified | Sent for radio network layer cause when none of the specified cause values applies. |
| TXnRELOCOverall expiry | The timer guarding the handover that takes place over Xn has abnormally expired. |
| Successful handover | Successful handover. |
| Release due to NG-RAN generated reason | Release is initiated due to NG-RAN generated reason. |
| Release due to 5GC generated reason | Release is initiated due to 5GC generated reason. |
| Handover cancelled | The reason for the action is cancellation of Handover. |
| Partial handover | Provides a reason for the handover cancellation. The HANDOVER COMMAND message from AMF contained PDU Session Resource to Release List IE or QoS flow to Release List and the source NG-RAN node estimated service continuity for the UE would be better by not proceeding with handover towards this particular target NG-RAN node. |
| Handover failure in target 5GC/ NG-RAN node or target system | The handover failed due to a failure in target 5GC/NG-RAN node or target system. |
| Handover target not allowed | Handover to the indicated target cell is not allowed for the UE in question. |
| TNGRELOCoverall expiry | The reason for the action is expiry of timer TNGRELOCoverall. |
| TNGRELOCprep expiry | Handover Preparation procedure is cancelled when timer TNGRELOCprep expires. |
| Cell not available | The concerned cell is not available. |
| Unknown target ID | Handover rejected because the target ID is not known to the AMF. |
| No radio resources available in target cell | Load on target cell is too high. |
| Unknown local UE NGAP ID | The action failed because the receiving node does not recognise the local UE NGAP ID. |
| Inconsistent remote UE NGAP ID | The action failed because the receiving node considers that the received remote UE NGAP ID is inconsistent. |
| Handover desirable for radio reasons | The reason for requesting handover is radio related. |
| Time critical handover | Handover is requested for time critical reason i.e., this cause value is reserved to represent all critical cases where the connection is likely to be dropped if handover is not performed. |
| Resource optimisation handover | The reason for requesting handover is to improve the load distribution with the neighbour cells. |
| Reduce load in serving cell | Load on serving cell needs to be reduced. When applied to handover preparation, it indicates the handover is triggered due to load balancing. |
| User inactivity | The action is requested due to inactivity on all user data radio bearers (i.e., DRBs and, if applicable, MRBs as per section 16.10.5.2 in TS 38.300 [8]), e.g., NG is requested to be released in order to optimise the radio resources. For L2 U2N Relay UE, this action is requested due to user inactivity on all PDU sessions of L2 U2N Relay UE and its served remote UE(s). |
| Radio connection with UE lost | The action is requested due to losing the radio connection to the UE. |
| Radio resources not available | No requested radio resources are available. |
| Invalid QoS combination | The action was failed because of invalid QoS combination. |
| Failure in the radio interface procedure | Radio interface procedure has failed. |
| Interaction with other procedure | The action is due to an ongoing interaction with another procedure. |
| Unknown PDU Session ID | The action failed because the PDU Session ID is unknown in the NG-RAN node. |
| Unknown QoS Flow ID | The action failed because the QoS Flow ID is unknown in the NG-RAN node. |

-----------------------------------------------------------TS 38.413-------------------------------------------------------------------------

Based on the above cause values, the NG-RAN node may initiates the UE Context Release Request procedure to release RRC connection based on variants of conditions. Therefore, when the emergency PDU session exist, the requirement of configuring the NG-RAN to not release RRC connection until the emergency PDU session is released by the SMF cannot be guaranteed which needs to take all conditions into account and is up to the implementation of the gNB.

Proposal 2: The requirement of configuring the NG-RAN to not release RRC connection can not always be guaranteed in RAN side.

With above two proposals, it is beneficial that RAN3 replies the LS to SA2 to provide RAN3’s further understanding on this issue and RAN3’s answers to the question raised by SA2.

Proposal 3: Reply the LS to SA2 to provide RAN3’s further understanding on this issue.

3	Summary

Observation: RAN3 and RAN2 share the view that it is up to the gNB implementation whether to release the UE to RRC_IDLE or RRC_INACTIVE or keep the UE in RRC_CONNECTED state for fast emergency callback.

Proposal 1: There is no specific description for the NG-RAN node to decide when to release RRC connection regardless of the presence of emergency PDU session.

Proposal 2: The requirement of configuring the NG-RAN to not release RRC connection can not always be guaranteed in RAN side.

Proposal 3:  Reply the LS to SA2 to provide RAN3’s further understanding on this issue.

The corresponding draft LS is given in [3].

4	Reference

[1] R3-255022 (S2-2505938), Reply LS on emergency call back and paging, SA2

[2] R3-253853 (R2-2504847), Reply LS on emergency call back and paging, RAN2

[3] R3-255260  [Draft] Reply LS on emergency call back and paging, ZTE