3GPP TSG-RAN WG3 Meeting #129	R3-255262

Bengaluru, India, 25 - 29 August 2025

Agenda Item:	14.3

Source:	ZTE Corporation

Title:	Remaining issues for NR NTN

Document for:	Discussion

1  Introduction

In previous meetings, the discussion on support of regenerative payload was discussed, and the following agreements were reached[1].

NG Interface management Issue: Option 2 NG Removal/Setup can be used for Regenerative Payload NG interface management. No new NGAP suspend/resume procedures in R19. Inactive UE support: RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment. Support RRC_INACTIVE UE in NTN by implementation in R19. Feeder link switch issue: RAN3 assumes that during the feeder link switch, AMF is not changed for the UEs. Whether there has any standard impact needs to be further checked. The current mechanisms including, e.g. multiple TNLA association, are sufficient for TNL management during feeder link switch. In case of the TNL addresses are changed due to feeder link switch over, existing procedures may be used. Further check the functionality: in case of hard feeder link switch over, the 5GC suspends or resumes the DL signalling/data transmission(s) to the gNB based on the new indicator in RAN configuration update, FFS on the relationship with respect to NG suspend/resume (if any). WA: For regenerative payload, an indication to suspend or resume NG connection may be sent to AMF from gNB in the RAN Configuration Update procedure. Check the use cases including Hard Feederlink Switch, other use cases to be further justified. Supported TAI list of the on-board gNB: The supported TAIs of an on-board gNB could be provided to AMF by OAM/pre-configuration. Others: No enhancements on location based CHO for NTN in R19.

In this contribution, we provide our further views on the remaining issues for NTN in Rel-19.

2  Discussion

2.1  Correction on MBS Broadcast

In current NTN BL CR to 38.413, for the value of maxnoofIntendedAreasforMBS within the IE MBS Service Area Information is 65536, which is given as below[2].

While, in RAN2’s running RRC CR for NR NTN, the value of maxNrofMBS-Area-r19 in SIBXX is set to 32, which is given as below[3].

- Therefore, the value of maxnoofIntendedAreasforMBS in the MBS Intended Service Area List IE over NGAP should be 32.

Proposal 1: Change the value of maxnoofIntendedAreasforMBS within the IE MBS Service Area Information as 32 in BL CR 38.413.

2.2  NG connection interruption

In last meeting, the NG connection interruption issue was discussed, and the following WA was reached as below.

WA: For regenerative payload, an indication to suspend or resume NG connection may be sent to AMF from gNB in the RAN Configuration Update procedure.

The “Downlink NG Transmission Control” IE was introduced over NG to support the hard feeder link switchover. However, in this case, in addition to the DL data/ signalling which should be kept and suspended, the UE Context should be also kept in the AMF. Therefore, it is better to change the newly added IE name as “NG Connection Control” to make it more general.

Proposal 2: Regarding the NG connection interruption, rename the IE as “NG Connection Control” to make it more general.

In addition, currently, the above indication is only used for hard FLSO scenario, while, as we discussed before, this indication can be used to suspend or resume the NG connection, not only for suspending or resuming data and signalling procedures over NG. Meanwhile, the applicable scenario should not be limited to the hard FLSO, and it could be used as the complementation to the NG Removal to help resume the NG interface quickly.

Proposal 3: Regarding the NG connection interruption, extend the usage of NG suspend/resume indication.

3  Conclusion

Proposal 1: Change the value of maxnoofIntendedAreasforMBS within the IE MBS Service Area Information as 32 in BL CR 38.413.

Proposal 2: Regarding the NG connection interruption, rename the IE as “NG Connection Control” to make it more general.

Proposal 3: Regarding the NG connection interruption, extend the usage of NG suspend/resume indication.

The stage 3 TP on Support of MBS Broadcast is given in [4].

The stage 2 and stage 3 TP on NG connection interruption are given in [5] and [6] respectively.

References

- RAN3#128 Chairman Notes

- R3-255068, Support for Regenerative Payload and MBS broadcast in NR NTN, CATT, Thales, Nokia, Nokia Shanghai Bell, Ericsson, Huawei, ZTE, Qualcomm, Samsung, Xiaomi, CMCC, China Telecom, Jio Platforms, LG Electronics,  NEC, ETRI, SES, ESA

- R2-2504656, Running RRC CR for NR NTN phase 3, Ericsson

- R3-255264, (TP to BL CR for 38.413) Support of MBS Broadcast, ZTE Corporation

- R3-255179, (TP to BL CR for 38.300) Support for NG Suspend/Resume, CATT, ZTE,  Nokia, Nokia Shanghai Bell, Qualcomm, Samsung, CMCC, China Telecom, LG Electronics, Xiaomi

- R3-255263, (TP to BL CR for 38.413) Support of Suspend and Resume Indication, ZTE Corporation, CATT, Nokia, Nokia Shanghai Bell, Qualcomm, Samsung, CMCC, China Telecom, LG Electronics, Xiaomi