3GPP TSG-RAN WG3 Meeting #129	R3-255267

Bangalore, India, 25th – 29th August 2025

Agenda Item:	19.2

Source:	Charter Communications, Qualcomm Inc., Fraunhofer HHI, Fraunhofer IIS

Title:                        UE-to-UE CLI mitigation support signaling

Document for:	Discussion and decision

# Introduction

At the RANP #108 meeting RP-251874 was agreed as a revision to the WID on “Evolution of NR duplex operation: Sub-band full duplex (SBFD).

After discussions on the exact wording of the WID, it was finally agreed that the update would be the following additions / modifications shown below:

  - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4]

    - Information exchange among gNBs, including [RAN3]

      - Semi-static cell-specific SBFD time and frequency location configuration

      - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

      - Strongest DL beam information

      - CLI-mitigation request

      - One or more SRS resource configurations

- Notes: Following are assumed for L1 based UE-to-UE CLI measurement and reporting

  - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No RAN1/RAN2/RAN4 specification impact due to SRS configuration information exchange between gNBs

  - Periodic and semi-persistent reporting can be considered in addition to aperiodic reporting

  - For the new reporting quantities, at least wideband reporting is supported

  - The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point

## 1.1 A framework for supporting UE-to-UE CLI mitigation

3GPP will not dictate a particular algorithm to be used for minimizing inter-cell UE-to-UE CLI interference.  That is up to each gNB vendor to implement.

The outline below is only to provide a context for framing the issue and to  describe  the enhancements to the Xn/F1 protocol that are required in order to transport the necessary information between gNBs. While optimizations are possible, they will be up to each equipment vendor implementation choices and the specification shall stipulate only the signaling necessary to enable and support some different implementations.

In general, gNBs can exchange the SRS-Resource configuration(s) of each of their respective cells  either when the Xn interface is being established—or when a relevant update to that information is made or requested. This is similar to what is already specified in the case of gNB-to-gNB CLI (when NZP-CSI RS configuration is exchanged).

For example, when some gNB2 detects that the downlink performance of certain UEs begins to deteriorate—as evidenced by an increased number of NACKs—it may decide to request its neighbor gNB1 to send UE-to-UE CLI measurement assistance information (i.e. a list of the SRS-Resource IE that it uses in its cells for measurement), so that gNB2 can configure its served UEs with appropriate resources for UE-to-UE CLI measurement.

When gNB1 receives a request for UE-to-UE CLI assistance it will send, to gNB2, the list of “SRS-Resource” configuration for all (or a subset) of the UEs served by gNB1. gNB2 would then be able to (selectively) request the UEs it (i.e., gNB2) serves to measure the CLI interference that those UEs can detect against each SRS-Resource received from gNB1.

gNB2 will/may relay back to gNB1 the list of CLI measurement result of the UE(s) it tested (using an existing TS 38.331 IE, that will now be exchanged over Xn); and, optionally, a request to mitigate the corresponding CLI.

Based on the CLI measurement result list received (which includes the corresponding SRS-ResourceId) gNB1 will then be able to identify which of its UE(s) are causing CLI on some UEs of gNB2, and can, if it so desires, request that those particular UEs cut back on their transmitted power or change scheduling resources for these identified aggressor UEs.

# 2 Summary of proposed changes

## 2.1 Summary of proposed changes to Xn protocol

We propose that SRS-Resource used by cells of gNBs for their UEs, be exchanged when a Xn link is established (via Xn message Xn SETUP REQUEST and Xn SETUP RESPONSE messages), or anytime the SRS-Resource configuration is updated (via Xn message NG-RAN NODE CONFIGURATION UPDATE. In particular, we will update the existing Xn “Served Cell Information NR” IE to add a new Xn “SRS-Resource List” IE.

Since gNB2 will be sending back CLI measurement results, our proposal is to reuse the existing Xn CLI INDICATION message—that is currently used to send gNB-to-gNB CLI test results and/or to possibly request gNB-to-gNB CLI mitigation request—to also support UE-to-UE CLI testing results and/or to possibly request UE-to-UE CLI mitigation request.  Towards that end, we propose to rename the existing “CLI Measurement Result Item” IE as “gNB-to-gNB CLI Measurement Result Item” and to add a new IE “UE-to-UE CLI Measurement Result Item”.

We will also add a new Xn IE to the CLI INDICATION message to request neighbors to send UE-to-UE-CLI measurement assistance information.

For details refer to section 5.1 of this document, and the corresponding TP to BL CR mentioned in section 3.

## 2.2 Summary of proposed changes to F1 protocol

Since CU will be sending SRS-Resource(s) to DU, we propose to reuse the existing Xn GNB-CU CONFIGURATION UPDATE message (which is currently already used to exchange information related to gNB-to-gNB CLI) to also exchange UE-to-UE CLI related information.

Similar to what is already done for gNB-to-gNB CLI, we will update the existing F1 “Neighbor Cell Information List Item” IE to add a new F1 “SRS-Resource List” IE.

Since DU (CU) will be sending CLI measurement results to CU (DU), our proposal is to reuse the existing F1 CLI INDICATION message, that is currently already used to send gNB-to-gNB CLI test results and/or possibly request gNB-to-gNB CLI mitigation request, to also support UE-to-UE CLI testing results and/or possibly request UE-to-UE CLI mitigation request.

Towards that end, we propose to rename the existing “CLI Measurement Result Item” IE as “gNB-to-gNB CLI Measurement Result Item” and to add a new IE “UE-to-UE CLI Measurement Result Item”.

We will also add a new F1 IE to the F1 CLI INDICATION message to request neighbors to send UE-to-UE-CLI measurement assistance information.

For details refer to section 5.2 of this document, and the corresponding TP to BL CR mentioned in section 3.

## 2.3 Proposals

Based on above discussion, we have the following summary of proposals.

Proposal 1: gNB2 serving victim UEs will send request to gNB1 serving aggressor UEs requesting a list of SRS-Resource configuration and it is up to gNB2 implementation when to request.

Proposal 2: gNB1 serving aggressor UEs will provide SRS-Resource configuration information to gNB2 serving victim UEs either based on gNB2 request or without request. It is up to gNB1 implementation which UEs SRS-Resource configuration will be provided to gNB2.

Proposal 3: SRS-Resource configuration information between gNBs will be exchanged by means of Xn Setup and  NG-RAN Node Configuration Update procedures.

Proposal 4: gNB2 serving victim UEs will send CLI measurement results to gNB1 along with UE to UE CLI Mitigation Request Indication in Xn CLI Indication Message.

Proposal 5: Introduce equivalent signaling enhancements for F1-AP signaling messages.

# 3 References

[1] RP-251874: Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

at RP#108 in Prague, Czech Republic, June 9-13, 2025.

[2] R3-255276 – TP to BL CR TS 38.300 Introduction of Evolution of NR duplex operation

[3] R3-255277 – TP to BL CR TS 38.423 UE-to-UE CLI mitigation

[4] R3-255278 – TP to BL CR TS 38.473 UE-to-UE CLI mitigation

# 4 Message Flows (Illustrative purposes only)

# 5 Proposed Signaling Changes

## 5.1 Changes to Xn protocol (TS 38.423)

=============================Start of change==========================

#### 9.2.2.11	Served Cell Information NR

This IE contains cell configuration information of an NR cell that a neighbouring NG-RAN node may need for the Xn AP interface.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR-PCI | M |  | INTEGER (0..1007, …) | NR Physical Cell ID | – |  |
| NR CGI | M |  | 9.2.2.7 |  | – |  |
| TAC | M |  | 9.2.2.5 | Tracking Area Code | – |  |
| RANAC | O |  | RAN Area Code 9.2.2.6 |  | – |  |
| Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs contained in the SIB1 message as specified in TS 38.331[10], associated to the NR Cell Identity in the NR CGI IE. | – |  |
| >PLMN Identity | M |  | 9.2.2.4 |  | – |  |
| CHOICE NR-Mode-Info | M |  |  |  | – |  |
| >FDD |  |  |  |  |  |  |
| >>FDD Info |  | 1 |  |  | – |  |
| >>>UL NR Frequency Info | M |  | NR Frequency Info 9.2.2.19 | This IE is ignored for NR operating bands for which uplink range of NREF is not defined in section 5.4.2.3 of TS 38.104 [24]. | – |  |
| >>>DL NR Frequency Info | M |  | NR Frequency Info 9.2.2.19 |  | – |  |
| >>>UL Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.2.20 | This IE is ignored for NR operating bands for which uplink range of NREF is not defined in section 5.4.2.3 of TS 38.104 [24]. | – |  |
| >>>DL Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.2.20 |  | – |  |
| >>>UL Carrier List | O |  | NR Carrier List 9.2.2.63 | If included, the UL Transmission Bandwidth IE shall be ignored. | YES | ignore |
| >>>DL Carrier List | O |  | NR Carrier List 9.2.2.63 | If included, the DL Transmission Bandwidth IE shall be ignored. | YES | ignore |
| >>>gNB-DU Cell Resource Configuration-FDD-UL | O |  | gNB-DU Cell Resource Configuration 9.2.2.95 | Contains FDD UL resource configuration of gNB-DU’s cell. Only applicable if the gNB-DU is an IAB-DU or an IAB-donor-DU. | YES | ignore |
| >>>gNB-DU Cell Resource Configuration-FDD-DL | O |  | gNB-DU Cell Resource Configuration 9.2.2.95 | Contains FDD UL resource configuration of gNB-DU’s cell. Only applicable if the gNB-DU is an IAB-DU or an IAB-donor-DU. | YES | ignore |
| >TDD |  |  |  |  |  |  |
| >>TDD Info |  | 1 |  |  | – |  |
| >>>Frequency Info | M |  | NR Frequency Info 9.2.2.19 |  | – |  |
| >>>Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.2.20 | This IE is ignored if the Transmission Bandwidth asymmetric IE is present. | – |  |
| >>>Intended TDD DL-UL Configuration NR | O |  | 9.2.2.40 |  | YES | ignore |
| >>>TDD UL-DL Configuration Common NR | O |  | OCTET STRING | Includes the tdd-UL-DL-ConfigurationCommon contained in the SIB1 message as defined in TS 38.331 [10] | YES | ignore |
| >>>Carrier List | O |  | NR Carrier List 9.2.2.63 | If included, the Transmission Bandwidth IE shall be ignored. | YES | ignore |
| >>>gNB-DU Cell Resource Configuration-TDD | O |  | gNB-DU Cell Resource Configuration 9.2.2.95 | Contains FDD UL resource configuration of gNB-DU’s cell. Only applicable if the gNB-DU is an IAB-DU or an IAB-donor-DU. | YES | ignore |
| >>>Transmission Bandwidth asymmetric |  | 0..1 |  | Indicates the asymmetric UL and DL transmission bandwidth. | YES | ignore |
| >>>>UL Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.2.20 |  | – |  |
| >>>>DL Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.2.20 |  | – |  |
| >>>SBFD Configuration | O |  | FFS (pending on RAN2 progress) |  | YES | ignore |
| Measurement Timing Configuration | M |  | OCTET STRING | Includes the MeasurementTimingConfiguration inter-node message for the served cell, as defined in TS 38.331 [10]. | – |  |
| Connectivity Support | M |  | 9.2.2.28 |  | – |  |
| Broadcast PLMN Identity Info List NR |  | 0..<maxnoofBPLMNs> |  | This IE corresponds to information provided in the PLMN-IdentityInfoList IE and the NPN-IdentityInfoList IE (if available) in SIB1 as specified in TS 38.331 [10]. All PLMN Identities and associated information contained in the PLMN-IdentityInfoList IE and NPN identities and associated information contained in the NPN-IdentityInfoList IE (if available) are included and provided in the same order as broadcast in the SIB1 message. NOTE: In case of NPN-only cell, the PLMN Identities and associated information contained in the PLMN-IdentityInfoList IE are not included. | YES | ignore |
| >Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs in the SIB1 message, associated to the NR Cell Identity IE. | – |  |
| >>PLMN Identity | M |  | 9.2.2.4 |  | – |  |
| >TAC | M |  | 9.2.2.5 |  | – |  |
| >NR Cell Identity | M |  | BIT STRING (SIZE(36)) |  | – |  |
| >RANAC | O |  | RAN Area Code 9.2.2.6 |  | – |  |
| >Configured TAC Indication | O |  | 9.2.2.39a | NOTE: This IE is associated with the TAC in the Broadcast PLMN Identity Info List NR IE | YES | ignore |
| >NPN Broadcast Information | O |  | 9.2.2.71 | If this IE is included the content of the Broadcast PLMNs IE in the Broadcast PLMN Identity Info List NR IE is ignored. | YES | reject |
| Configured TAC Indication | O |  | 9.2.2.39a | NOTE: This IE is associated with the TAC on top-level of the Served Cell Information NR IE | YES | ignore |
| NPN Broadcast Information | O |  | 9.2.2.71 | If this IE is included the content of the Broadcast PLMNs IE in the top Served Cell Information NR IE is ignored. | YES | reject |
| SSB Positions In Burst | O |  | 9.2.2.64 |  | YES | ignore |
| NR Cell PRACH Configuration | O |  | OCTET STRING | Includes the NR Cell PRACH Configuration IE as defined in section 9.3.1.139 in TS 38.473 [41]. | YES | ignore |
| CSI-RS Transmission Indication | O |  | ENUMERATED (activated, deactivated, ...) | This IE indicates the CSI-RS transmission status of the given cell. If the Additional Measurement Timing Configuration List IE is present, this IE is ignored. | YES | ignore |
| SFN Offset | O |  | 9.2.2.75 |  | YES | ignore |
| Supported MBS FSA ID List |  | 0..<maxnoofMBSFSAs> |  | Shall contain all MBS Frequency Selection Area Identities associated to the NR Cell Identity in the NR CGI IE. | YES | ignore |
| >MBS Frequency Selection Area Identity | M |  | OCTET STRING(3) | Corresponds to information provided in the MBS-FSAI IE as defined in TS 38.331 [10]. | – |  |
| NR-U Channel Info List |  | 0..1 |  |  | YES | ignore |
| >NR-U Channel Info Item |  | 1..<maxnoofNR-UChannelIDs> |  |  | – |  |
| >>NR-U Channel ID | M |  | INTEGER (1.. maxnoofNR-UChannelIDs, …) | Index to uniquely identify the part of the NR-U Channel Bandwidth consisting of a contiguous set of resource blocks (RBs) on which a channel access procedure is performed in shared spectrum. Value 1 represents the first part of the NR-U Channel Bandwidth on which a channel access procedure is performed. Value 2 represents the second part of the NR-U Channel Bandwidth on which a channel access procedure is performed, and so on. | – |  |
| >>NR ARFCN | M |  | INTEGER (0.. maxNRARFCN) | It represents the centre frequency of the NR-U Channel Bandwidth for NR bands restricted to operation with shared spectrum channel access, as defined in TS 37.213 [51]. Allowed values are specified in 38.101-1 [52] in Table 5.4.2.3-2, Table 5.4.2.3-3 and Table 5.4.2.3-4. | – |  |
| >>Bandwidth | M |  | ENUMERATED (10MHz, 20MHz, 40MHz, 60MHz, 80MHz, …,100MHz) |  | – |  |
| Additional Measurement Timing Configuration List | O | 1 .. <maxnoofMTCItems> |  |  | YES | ignore |
| >Measurement Timing Configuration Index | M |  | INTEGER (0..16) | “0” refers to the configuration contained in the Measurement Timing Configuration IE. Any value between “1” and “16” refers to a configuration within the Additional Measurement Timing Configuration List IE. | – |  |
| >CSI- RS MTC Configuration List | M | 1 .. <maxnoofCSIRSconfigurations> |  | This list explicitly expresses the CSI-RS configurations contained in the MTC | – |  |
| >>CSI-RS Index | M |  | INTEGER (0..95) | Index of CSI-RS as in MTC | – |  |
| >>CSI-RS Status | M |  | ENUMERATED (activated, deactivated, …) | This IE indicates the CSI-RS transmission status of the configuration. | – |  |
| >>CSI-RS Neighbour List | O | 1 .. <maxnoofCSIRSneighbourCells> |  | This list expresses the cells and CSI-RSs neighbouring the CSI-RS in the CSI-RS Index IE. | – |  |
| >>>NR CGI | M |  | 9.2.2.7 |  | – |  |
| >>>CSI-RS MTC Neighbour List | O | 1 .. < maxnoofCSIRSneighbourCellsInMTC> |  | This list expresses the CSI-RSs served by the NR CGI, which are neighbouring the CSI-RS of the served cell and contained in the MTC indicated by the neighbouring NR cell. | – |  |
| >>>>CSI-RS Index | M |  | INTEGER (0..95) |  | – |  |
| RedCap Broadcast Information | O |  | BIT STRING (SIZE(8)) | The presence of this IE indicates that the intraFreqReselectionRedCap is broadcast in the SIB1 message of the corresponding cell, see TS 38.331 [10]. Each position in the bitmap indicates which RedCap UEs are allowed access, according to the setting of RedCap barring indicators in the SIB1 message, see TS 38.331 [10]. First bit = 1Rx, second bit = 2Rx, third bit = halfDuplex, other bits reserved for future use. Value '1' indicates 'access allowed'. Value '0' indicates 'access not allowed”. | YES | ignore |
| eRedCap Broadcast Information | O |  | BIT STRING (SIZE(8)) | The presence of this IE indicates that the intraFreqReselection-eRedCap IE is broadcast in SIB1 of the corresponding cell, see TS 38.331 [10]. Each position in the bitmap indicates which eRedCap UEs are allowed access, according to the setting of the barring indicators in SIB1, see TS 38.331 [10]. First bit = 1Rx, second bit = 2Rx, third bit = half-duplex, other bits reserved for future use. Value '1' indicates 'access allowed'. Value '0' indicates 'access not allowed'. | YES | ignore |
| Mobile IAB Cell | O |  | 9.2.2.106 |  | YES | ignore |
| XR Broadcast Information | O |  | ENUMERATED (true, …) | Corresponds to information provided in the cellBarred2RxXR contained in the SIB1 message as defined in TS 38.331 [10]. | YES | ignore |
| Barring Exemption for Emergency Call Information | O |  | ENUMERATED (true, ...) | Corresponds to information provided in the barringExemptEmergencyCall contained in the SIB1 message as defined in 38.331 [10]. | YES | ignore |
| NZP CSI-RS Resources Configuration | O |  | 9.2.2.x2 |  | YES | Ignore |
| SRS-Resource-List | O |  | 9.2.2.x3 |  | YES | Ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of broadcast PLMNs by a cell. Value is 12. |
| maxnoofMBSFSAs | Maximum no. of MBS FSAs by one gNB. Value is 256. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U channel IDs in a cell. Value is 16. |
| maxnoofMTCItems | Maximum no. of measurement timing configurations associated with the neighbour cell. Value is 16. |
| maxnoofCSIRSconfigurations | Maximum number of CSI RS configurations reported in the MTC. Value is 96 |
| maxnoofCSIRSneighbourCells | Maximum number of cells neighbouring a CSI-RS coverage area. Value is 16 |
| maxnoofCSIRSneighbourCellsInMTC | Maximum number of CSI-RS coverage areas neighbouring a specific CSI-RS coverage area. Value is 16 |

=============================Next change============================

#### 9.2.2.x3	SRS-Resource-List

This IE contains a list of SRS-Resource of UEs in the current cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SRS-Resource-List |  | 1 |  |  |
| >SRS-Resource-Item |  | 1..<maxNrofSRS-Resources> |  |  |
| >>SRS-Resource | M |  | OCTET STRING | Includes the SRS-Resource IE as defined in TS38.331. |

| Range bound | Explanation |
| --- | --- |
| maxNrofSRS-Resources | Maximum Nr. of SRS-Resource. Value is 64 |

=============================Next change==============================

#### 9.1.3.y	CLI INDICATION

This message is sent by NG-RAN node1 to NG-RAN node2 to report the results of the CLI measurements, or to request a neighbor gNB to send UE-to-UE CLI measurement assistance information.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| gNB-to-gNB-CLI- Measurement Result | O |  |  |  | YES | ignore |
| >gNB-to-gNB-CLI-Measurement Result Item | O | 0 .. < maxnoofCellsinNG-RANnode > |  |  | YES | ignore |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>SSB index | O |  | INTEGER (0..63) | Strongest DL SSB beam information | – |  |
| >>NZP CSI-RS Resource Indication | O |  | INTEGER (1..64) | Strongest DL NZP CSI-RS beam information. | – |  |
| >>CLI Mitigation Indication | O |  | ENUMERATED (true, …) | Indicates to request CLI mitigation | – |  |
| UE-to-UE-CLI- Measurement Result | O |  |  |  | YES | Ignore |
| >MeasResultCLI-r16 | M |  |  | As defined in TS38.331 | - |  |
| >CLI Mitigation Indication | O |  | ENUMERATED (true, …) | Indicates to request CLI mitigation |  |  |
| UE-to-UE-CLI- MeasurementAssistanceRequest | O |  | ENUMERATED (true, …) | Indication to neighbor gNB to send SRS-Resource configuration info to assist with SRS measurements |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

===========================End of changes=============================

## 5.2 Changes to F1 protocol (TS 38.473)

===========================Start of change=============================

#### 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name |  | Presence |  | Range |  | IE type and reference | Semantics description |  | Criticality |  | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type |  | M |  |  |  | 9.3.1.1 |  |  | YES |  | reject |  |
| Transaction ID |  | M |  |  |  | 9.3.1.23 |  |  | YES |  | reject |  |
| Cells to be Activated List |  |  |  | 0..1 |  |  | List of cells to be activated or modified |  | YES |  | reject |  |
| >Cells to be Activated List Item |  |  |  | 1.. <maxCellingNBDU> |  |  |  |  | EACH |  | reject |  |
| >>NR CGI |  | M |  |  |  | 9.3.1.12 |  |  | - |  |  |  |
| >>NR PCI |  | O |  |  |  | INTEGER (0..1007) | Physical Cell ID |  | - |  |  |  |
| >>gNB-CU System Information |  | O |  |  |  | 9.3.1.42 | RRC container with system information owned by gNB-CU |  | YES |  | reject |  |
| >>Available PLMN List |  | O |  |  |  | 9.3.1.65 |  |  | YES |  | ignore |  |
| >>Extended Available PLMN List |  | O |  |  |  | 9.3.1.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. |  | YES |  | ignore |  |
| >>IAB Info IAB-donor-CU |  | O |  |  |  | 9.3.1.105 | IAB-related configuration sent by the IAB-donor-CU. |  | YES |  | ignore |  |
| >>Available SNPN ID List |  | O |  |  |  | 9.3.1.163 | Indicates the available SNPN ID list. If this IE is included, the content of the Available PLMN List IE and Extended Available PLMN List IE if present in the Cells to be Activated List Item IE is ignored. |  | YES |  | ignore |  |
| >>MBS Broadcast Neighbour Cell List |  | O |  |  |  | 9.3.1.226 |  |  | YES |  | ignore |  |
| >>SSBs within the cell to be Activated List |  | O |  |  |  | 9.3.1.326 | List of SSB beams within the cell requested to be activated. |  | YES |  | reject |  |
| Cells to be Deactivated List |  |  |  | 0..1 |  |  | List of cells to be deactivated |  | YES |  | reject |  |
| >Cells to be Deactivated List Item |  |  |  | 1.. <maxCellingNBDU> |  |  |  |  | EACH |  | reject |  |
| >>NR CGI |  | M |  |  |  | 9.3.1.12 |  |  | - |  |  |  |
| gNB-CU TNL Association To Add List |  |  |  | 0..1 |  |  |  |  | YES |  | ignore |  |
| >gNB-CU TNL Association To Add Item IEs |  |  |  | 1..<maxnoofTNLAssociations> |  |  |  |  | EACH |  | ignore |  |
| >>TNL Association Transport Layer Information |  | M |  |  |  | CP Transport Layer Information 9.3.2.4 | Transport Layer Address of the gNB-CU. |  | - |  |  |  |
| >>TNL Association Usage |  | M |  |  |  | ENUMERATED (ue, non-ue, both, ...) | Indicates whether the TNL association is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.472 [22]. |  | - |  |  |  |
| gNB-CU TNL Association To Remove List |  |  |  | 0..1 |  |  |  |  | YES |  | ignore |  |
| >gNB-CU TNL Association To Remove Item IEs |  |  |  | 1..<maxnoofTNLAssociation> |  |  |  |  | EACH |  | ignore |  |
| >>TNL Association Transport Layer Address |  | M |  |  |  | CP Transport Layer Information 9.3.2.4 | Transport Layer Address of the gNB-CU. |  | - |  |  |  |
| >>TNL Association Transport Layer Address gNB-DU |  | O |  |  |  | CP Transport Layer Information 9.3.2.4 | Transport Layer Address of the gNB-DU. |  | YES |  | reject |  |
| gNB-CU TNL Association To Update List |  |  |  | 0..1 |  |  |  |  | YES |  | ignore |  |
| >gNB-CU TNL Association To Update Item IEs |  |  |  | 1..<maxnoofTNLAssociations> |  |  |  |  | EACH |  | ignore |  |
| >>TNL Association Transport Layer Address |  | M |  |  |  | CP Transport Layer Information 9.3.2.4 | Transport Layer Address of the gNB-CU. |  | - |  |  |  |
| >>TNL Association Usage |  | O |  |  |  | ENUMERATED (ue, non-ue, both, ...) | Indicates whether the TNL association is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.472 [22]. |  | - |  |  |  |
| Cells to be barred List |  |  |  | 0..1 |  |  | List of cells to be barred. |  | YES |  | ignore |  |
| >Cells to be barred List Item |  |  |  | 1.. <maxCellingNBDU> |  |  |  |  | EACH |  | ignore |  |
| >>NR CGI |  | M |  |  |  | 9.3.1.12 |  |  | - |  |  |  |
| >>Cell Barred |  | M |  |  |  | ENUMERATED (barred, not-barred, ...) |  |  | - |  |  |  |
| >>IAB Barred |  | O |  |  |  | ENUMERATED (barred, not-barred, ...) | Corresponds to information provided in the iab-Support contained in the PLMN-IdentityInfo IE or contained in the NPN-IdentityInfo IE as defined in TS 38.331 [8]. The codepoint value “barred” indicates that the iab-Support is not sent in SIB1, and the codepoint value “not-barred” indicates that the iab-Support is sent in SIB1. |  | - |  |  |  |
| >>Mobile IAB Barred |  | O |  |  |  | ENUMERATED (barred, not-barred, ...) | Corresponds to information provided in the mobileIAB-Support contained in the PLMN-IdentityInfo IE or contained in the NPN-IdentityInfo IE as defined in TS 38.331 [8]. The codepoint value “barred” indicates that the mobileIAB-Support is not sent in SIB1, and the codepoint value “not-barred” indicates that the mobileIAB-Support is sent in SIB1. |  | - |  |  |  |
| Protected E-UTRA Resources List |  |  |  | 0..1 |  |  | List of Protected E-UTRA Resources. |  | YES |  | reject |  |
| >Protected E-UTRA Resources List Item |  |  |  | 1.. <maxCellineNB> |  |  |  |  | EACH |  | reject |  |
| >>Spectrum Sharing Group ID |  | M |  |  |  | INTEGER (1.. maxCellineNB) | Indicates the E-UTRA cells involved in resource coordination with the NR cells affiliated with the same Spectrum Sharing Group ID. |  | - |  |  |  |
| >>E-UTRA Cells List |  |  |  | 1 |  |  | List of applicable E-UTRA cells. |  | - |  |  |  |
| >>>E-UTRA Cells List Item |  |  |  | 1 .. <maxCellineNB> |  |  |  |  | - |  |  |  |
| >>>>EUTRA Cell ID |  | M |  |  |  | BIT STRING (SIZE(28)) | Indicates the E-UTRAN Cell Identifier IE contained in the ECGI as defined in subclause 9.2.14 in TS 36.423 [9]. |  | - |  |  |  |
| >>>>Served E-UTRA Cell Information |  | M |  |  |  | 9.3.1.64 |  |  | - |  |  |  |
| Neighbour Cell Information List |  |  |  | 0..1 |  |  |  |  | YES |  | ignore |  |
| >Neighbour Cell Information List Item |  |  |  | 1 .. <maxCellingNBDU> |  |  |  |  | EACH |  | ignore |  |
| >>NR CGI |  | M |  |  |  | 9.3.1.12 |  |  | - |  |  |  |
| >>Intended TDD DL-UL Configuration |  | O |  |  |  | 9.3.1.89 |  |  | - |  |  |  |
| >>SBFD Configuration |  | O |  |  |  | FFS (pending on RAN2 progress) | FFS |  | - |  |  |  |
| >>SSB Resource Configuration |  |  |  |  |  | OCTET STRING | Includes the MeasTiming contained in the MeasurementTimingConfiguration message as defined in 38.331 [10]. |  | - |  | - |  |
| >>NZP CSI-RS ResourcesConfiguration |  | O |  |  |  | 9.3.1.y |  |  | - |  | - |  |
| >>SRS-Resource-List |  |  | O |  |  |  |  |  | 9.3.1.z |  |  |  |  |  | - |  |  | - |  |
| Transport Layer Address Info |  | O |  |  |  | 9.3.2.5 |  |  | YES |  | ignore |  |
| Uplink BH Non-UP Traffic Mapping |  | O |  |  |  | 9.3.1.103 |  |  | YES |  | reject |  |
| BAP Address |  | O |  |  |  | 9.3.1.111 | Indicates a BAP address assigned to the IAB-donor-DU. |  | YES |  | ignore |  |
| CCO Assistance Information |  | O |  |  |  | 9.3.1.211 | Indicates CCO Assistance Information for cells and beams served by the gNB-DU of the same NG-RAN node or for cells and beams not served by the gNB-DU. |  | YES |  | ignore |  |
| Cells for SON List |  | O |  |  |  | 9.3.1.214 |  |  | YES |  | ignore |  |
| gNB-CU Name |  | O |  |  |  | PrintableString(SIZE(1..150,...)) | Human readable name of the gNB-CU. |  | YES |  | ignore |  |
| Extended gNB-CU Name |  | O |  |  |  | 9.3.1.206 |  |  | YES |  | ignore |  |
| Cells Allowed to be Deactivated List |  |  |  | 0..1 |  |  |  |  | YES |  | ignore |  |
| >Cells Allowed to be Deactivated List Item |  |  |  | 1 .. <maxCellingNBDU> |  |  |  |  | EACH |  | ignore |  |
| >>NR CGI |  | M |  |  |  | 9.3.1.12 |  |  | - |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

=============================Next change==============================

#### 9.3.1.z	SRS-Resource-List

This IE contains a list of SRS-Resource of UEs in the current cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SRS-Resource-List |  | 1 |  |  |
| >SRS-Resource-Item |  | 1..<maxNrofSRS-Resources> |  |  |
| >>SRS-Resource | M |  | OCTET STRING | Includes the SRS-Resource IE as defined in TS 38.331 |

| Range bound | Explanation |
| --- | --- |
| maxNrofSRS-Resources | Maximum Nr. of SRS-Resource. Value is 64 |

=============================Next change==============================

### 9.2.y	CLI Indication Message

#### 9.2.y.1	 CLI INDICATION

This message is sent by gNB-DU to report the results of the CLI measurements or sent by gNB-CU to forward the results of the CLI measurements, or to request a neighbor gNB to send UE-to-UE CLI measurement assistance information..

Direction: gNB-DU  gNB-CU and gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-to-gNB-CLI Measurement Result List | O |  |  |  | YES | ignore |
| >gNB-to-gNB-CLI Measurement Result Item |  | 1.. < maxCellingNBDU > |  |  | YES | ignore |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | – |  |
| >>SSB index | O |  | INTEGER (0..63) | Strongest DL SSB beam information |  |  |
| >>CRI | O |  | INTEGER (1..64) | Strongest DL NZP CSI-RS beam information |  |  |
| >>CLI Mitigation Indication | O |  | ENUMERATED (true, …) | Indicates to request CLI mitigation |  |  |
| UE-to-UE-CLI- Measurement Result | O |  |  |  | YES | Ignore |
| >MeasResultCLI-r16 | M |  | As defined in TS38.331 |  | - |  |
| >CLI Mitigation Indication | O |  | ENUMERATED (true, …) | Indicates to request CLI mitigation |  |  |
| UE-to-UE-CLI- MeasurementAssistanceRequest | O |  | ENUMERATED(true, …) | Indication to neighbor gNB to send SRS-RSRP related info to assist with SRS measurements |  |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |

===========================End of changes=============================

6 Background Information

## 6.1 SRS-Config (from TS 38.331)

SRS-Config ::= SEQUENCE {

srs-ResourceSetToReleaseList SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSetId OPTIONAL, -- Need N

srs-ResourceSetToAddModList  SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSet OPTIONAL, -- Need N

srs-ResourceToReleaseList    SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-ResourceId OPTIONAL, -- Need N

srs-ResourceToAddModList     SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-Resource OPTIONAL, -- Need N

tpc-Accumulation             ENUMERATED {disabled} OPTIONAL, -- Need S

...,

[[

srs-RequestDCI-1-2-r16       INTEGER (1..2) OPTIONAL, -- Need S

srs-RequestDCI-0-2-r16       INTEGER (1..2) OPTIONAL, -- Need S

srs-ResourceSetToAddModListDCI-0-2-r16  SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSet OPTIONAL, -- Need N

srs-ResourceSetToReleaseListDCI-0-2-r16 SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSetId OPTIONAL, -- Need N

srs-PosResourceSetToReleaseList-r16     SEQUENCE (SIZE(1..maxNrofSRS-PosResourceSets-r16)) OF SRS-PosResourceSetId-r16

OPTIONAL, -- Need N

srs-PosResourceSetToAddModList-r16 SEQUENCE (SIZE(1..maxNrofSRS-PosResourceSets-r16)) OF SRS-PosResourceSet-r16 OPTIONAL,--Need N

srs-PosResourceToReleaseList-r16 SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF SRS-PosResourceId-r16 OPTIONAL,-- Need N

srs-PosResourceToAddModList-r16  SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF SRS-PosResource-r16 OPTIONAL -- Need N

]],

[[

dci-TriggeringPosResourceSetLink-r18 ENUMERATED { enabled } ]]

OPTIONAL -- Need R

}

## 6.2 SRS-ResourceSet (from TS 38.331)

SRS-ResourceSet ::= SEQUENCE {

srs-ResourceSetId SRS-ResourceSetId,

srs-ResourceIdList SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-ResourceId OPTIONAL, -- Cond Setup

resourceType CHOICE {

aperiodic SEQUENCE {

aperiodicSRS-ResourceTrigger INTEGER (1..maxNrofSRS-TriggerStates-1),

csi-RS NZP-CSI-RS-ResourceId OPTIONAL, -- Cond NonCodebook

slotOffset INTEGER (1..32) OPTIONAL, -- Need S

...,

[[

aperiodicSRS-ResourceTriggerList SEQUENCE (SIZE(1..maxNrofSRS-TriggerStates-2))

OF INTEGER (1..maxNrofSRS-TriggerStates-1) OPTIONAL -- Need M

]]

},

semi-persistent SEQUENCE {

associatedCSI-RS NZP-CSI-RS-ResourceId OPTIONAL, -- Cond NonCodebook

...

},

periodic SEQUENCE {

associatedCSI-RS NZP-CSI-RS-ResourceId OPTIONAL, -- Cond NonCodebook

...

}

},

usage ENUMERATED {beamManagement, codebook, nonCodebook, antennaSwitching},

alpha Alpha OPTIONAL, -- Need S

p0 INTEGER (-202..24) OPTIONAL, -- Cond Setup

pathlossReferenceRS PathlossReferenceRS-Config OPTIONAL, -- Need M

srs-PowerControlAdjustmentStates ENUMERATED { sameAsFci2, separateClosedLoop} OPTIONAL, -- Need S

...,

[[

pathlossReferenceRSList-r16 SetupRelease { PathlossReferenceRSList-r16} OPTIONAL -- Need M

]],

[[

usagePDC-r17 ENUMERATED {true} OPTIONAL, -- Need R

availableSlotOffsetList-r17 SEQUENCE (SIZE(1..4)) OF AvailableSlotOffset-r17 OPTIONAL, -- Need R

followUnifiedTCI-StateSRS-r17 ENUMERATED {enabled} OPTIONAL -- Need R

]],

[[

applyIndicatedTCI-State-r18 ENUMERATED {first, second} OPTIONAL -- Cond FollowUTCI

]]

}

## 6.3 SRS-Resource (from TS 38.331)

SRS-Resource ::= SEQUENCE {

srs-ResourceId SRS-ResourceId,

nrofSRS-Ports ENUMERATED {port1, ports2, ports4},

ptrs-PortIndex ENUMERATED {n0, n1 } OPTIONAL, -- Need R

transmissionComb CHOICE {

n2 SEQUENCE {

combOffset-n2 INTEGER (0..1),

cyclicShift-n2 INTEGER (0..7)

},

n4 SEQUENCE {

combOffset-n4 INTEGER (0..3),

cyclicShift-n4 INTEGER (0..11)

}

},

resourceMapping SEQUENCE {

startPosition INTEGER (0..5),

nrofSymbols ENUMERATED {n1, n2, n4},

repetitionFactor ENUMERATED {n1, n2, n4}

},

freqDomainPosition INTEGER (0..67),

freqDomainShift INTEGER (0..268),

freqHopping SEQUENCE {

c-SRS INTEGER (0..63),

b-SRS INTEGER (0..3),

b-hop INTEGER (0..3)

},

groupOrSequenceHopping ENUMERATED { neither, groupHopping, sequenceHopping },

resourceType CHOICE {

aperiodic SEQUENCE {

...

},

semi-persistent SEQUENCE {

periodicityAndOffset-sp SRS-PeriodicityAndOffset,

...

},

periodic SEQUENCE {

periodicityAndOffset-p SRS-PeriodicityAndOffset,

...

}

},

sequenceId INTEGER (0..1023),

spatialRelationInfo SRS-SpatialRelationInfo OPTIONAL, -- Need R

...,

[[

resourceMapping-r16 SEQUENCE {

startPosition-r16 INTEGER (0..13),

nrofSymbols-r16 ENUMERATED {n1, n2, n4},

repetitionFactor-r16 ENUMERATED {n1, n2, n4}

} OPTIONAL -- Need R

]],

[[

spatialRelationInfo-PDC-r17 SetupRelease { SpatialRelationInfo-PDC-r17 } OPTIONAL, -- Need M

resourceMapping-r17 SEQUENCE {

startPosition-r17 INTEGER (0..13),

nrofSymbols-r17 ENUMERATED {n1, n2, n4, n8, n10, n12, n14},

repetitionFactor-r17 ENUMERATED {n1, n2, n4, n5, n6, n7, n8, n10, n12, n14}

} OPTIONAL, -- Need R

partialFreqSounding-r17 SEQUENCE {

startRBIndexFScaling-r17 CHOICE{

startRBIndexAndFreqScalingFactor2-r17 INTEGER (0..1),

startRBIndexAndFreqScalingFactor4-r17 INTEGER (0..3)

},

enableStartRBHopping-r17 ENUMERATED {enable} OPTIONAL -- Need R

} OPTIONAL, -- Need R

transmissionComb-n8-r17 SEQUENCE {

combOffset-n8-r17 INTEGER (0..7),

cyclicShift-n8-r17 INTEGER (0..5)

} OPTIONAL, -- Need R

srs-TCI-State-r17 CHOICE {

srs-UL-TCI-State TCI-UL-StateId-r17,

srs-DLorJointTCI-State TCI-StateId

} OPTIONAL -- Need R

]],

[[

repetitionFactor-v1730 ENUMERATED {n3} OPTIONAL, -- Need R

srs-DLorJointTCI-State-v1730 SEQUENCE {

cellAndBWP-r17 ServingCellAndBWP-Id-r17

} OPTIONAL -- Cond DLorJointTCI-SRS

]],

[[

nrofSRS-Ports-n8-r18 ENUMERATED {ports8, ports8tdm} OPTIONAL, -- Need R

combOffsetHopping-r18 SEQUENCE {

hoppingId-r18 INTEGER (0..1023) OPTIONAL, -- Need R

hoppingSubset-r18 CHOICE {

transmissionComb-n4 BIT STRING (SIZE (4)),

transmissionComb-n8 BIT STRING (SIZE (8))

} OPTIONAL, -- Need R

hoppingWithRepetition-r18 ENUMERATED {symbol, repetition} OPTIONAL -- Need R

} OPTIONAL, -- Need R

cyclicShiftHopping-r18 SEQUENCE {

hoppingId-r18 INTEGER (0..1023) OPTIONAL, -- Need R

hoppingSubset-r18 CHOICE {

transmissionComb-n2 BIT STRING (SIZE (8)),

transmissionComb-n4 BIT STRING (SIZE (12)),

transmissionComb-n8 BIT STRING (SIZE (6))

} OPTIONAL, -- Need R

hoppingFinerGranularity-r18 ENUMERATED {enable} OPTIONAL -- Need R

} OPTIONAL -- Need R

]]

}

6.4 SRS-RSRP-MeasResourceSet information element (from R2-2503862)

-- ASN1START

-- TAG-SRS-RSRP-MEASRESOURCESET-START

SRS-RSRP-MeasResourceSet-r19 ::=      SEQUENCE {

aperiodicTriggeringOffset-r19     INTEGER (0..31)            OPTIONAL,   -- Need R

srs-RSRP-MeasResourceSetId-r19    SRS-RSRP-MeasResourceSetId OPTIONAL,   -- Need R

srs-RSRP-MeasResourceIdList-r19   SEQUENCE (SIZE (1.. maxNrofSRS-RSRPMeasResourcesPerSet-r19) ) OF SRS-RSRP-MeasResourceId

OPTIONAL,   -- Need R

...

}

-- TAG-SRS-RSRP-MEASRESOURCESET-STOP

-- ASN1STOP

There is a typo / mistake in the definition of SRS-RSRP-MeasResourceSet IE in R2-2503862.  Specifically srs-RSRP-MeasResourceIdList should be a sequence of SRS-RSRP-MeasResourceId, not of SRS-RSRP-MeasResourceSetId.

6.5 SRS-RSRP-MeasResourceSetId information element (from R2-2503862)

-- ASN1START

-- TAG-SRS-RSRP-MEASRESOURCESETID-START

SRS-RSRP-MeasResourceSetId-r19 ::=      INTEGER(0..maxNrofSRS-RSRP-MeasResourceSets-r19)

-- TAG-SRS-RSRP-MEASRESOURCESETID-STOP

-- ASN1STOP

6.6 SRS-RSRP-MeasResource information element (from R2-2503862)

-- ASN1START

-- TAG-SRS-RSRP-MEASRESOURCE-START

SRS-RSRP-MeasResource-r19 ::=         SEQUENCE {

srs-RSRP-MeasResourceId-r19              SRS-RSRP-MeasResourceId-r19   OPTIONAL,   -- Need R

srs-Resource-r19                         SRS-Resource                  OPTIONAL,   -- Need R

qcl-InfoPeriodicSRS-RSRP-MeasResource    TCI-StateId                   OPTIONAL,   -- Need R

...

}

-- TAG-SRS-RSRP-MEASRESOURCE-STOP

-- ASN1STOP

It should be noted that in the definition of SRS-RSRP-MeasResource above they actually wrote SRS-RSRP-MeasResourceSet.  Some form of typo, but needs to be corrected.

6.7 SRS-RSRP-MeasResourceId information element (from R2-2503862)

-- ASN1START

-- TAG-SRS-RSRP-MEASRESOURCEID-START

SRS-RSRP-MeasResourceId-r19 ::=      INTEGER(0..maxNrofSRS-RSRP-MeasResources-1-r19)

-- TAG-SRS-RSRP-MEASRESOURCEID-STOP

-- ASN1STOP

6.8 Value definitions

maxNrofSRS-RSRP-MeasResources-19                 INTEGER ::= FFS     -- Maximum number of SRS RSRP Measurement Resources.

maxNrofSRS-RSRP-MeasResources-1-19               INTEGER ::= FFS     -- Maximum number of SRS RSRP Measurement Resources minus 1.

maxNrofSRS-RSRP-MeasResourceSets-r19             INTEGER ::= FFS     -- Maximum number of SRS RSRP Measurement Resource Sets

maxNrofSRS-RSRP-MeasResourceSetsPerConfig-r19    INTEGER ::= FFS     -- Maximum number of SRS RSRP Measurement Resource Sets Per Configuration

maxNrofSRS-RSRP-MeasResourcesPerSet-r19          INTEGER ::= FFS     -- Maximum number of SRS RSRP Measurement Resources Per Resource Set