3GPP TSG-RAN WG3 Meeting #129														       R3-255268

Bengaluru, India, 25th – 29th August 2025

Agenda Item:	13.2

Source:	Ericsson

Title:	Completion of Inter-CU LTM

Document for:	Discussion, Agreement

# Introduction

At the past RAN3 meetings, good progress was achieved for inter-CU LTM. In this contribution, we provide further discussion on how to support inter-CU LTM for standalone and NR-DC.

TPs corresponding to our proposals can be found in R3-255269 for XnAP, F1AP, and stage-2.

# Discussion

## Inter-CU LTM in single connectivity

For inter-CU LTM in single connectivity, we consider the following agreements and open points captured at the previous RAN3 meetings:

Turn the WA into agreement: Use a single UE association (e.g. identified by a pair of {Source NG-RAN node UE XnAP ID IE and Target NG-RAN node UE XnAP ID IE}) for multiple LTM handover request to the same candidate gNB.

FFS on the Rel-19 Set ID(s) assignment among CUs, down select from Option1 and Option2:

Option 1: Source gNB sends the Rel-19 Set ID(s) or Rel-19 set ID range assigned to the candidate gNB in the HANDOVER REQUEST message, then candidate gNB assigns Rel-19 set ID(s) to its own candidate cells and feedback via HANDOVER REQUEST ACKNOWLEDGE message.

Option 2: Candidate gNB provides Rel-19 set ID per candidate cell in HANDOVER REQUEST ACKNOWLEDGE message, then source gNB may update the Rel-19 set ID to ensure that the Rel-19 set IDs under different candidate gNB-CU are different.

FFS on the procedure to be used for source gNB to transfer Rel-19 set ID per candidate cell to the candidate gNB.

### Single UE association for multiple LTM handover requests

At RAN3#127-bis it was agreed to: “Use a single UE association (e.g. identified by a pair of {Source NG-RAN node UE XnAP ID IE and Target NG-RAN node UE XnAP ID IE}) for multiple LTM handover request to the same candidate gNB.”. It remains to be clarified how to identify the same UE from multiple Handover preparation procedure.

The current baseline indicates the following for the Handover Preparation procedure:

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover or LTM, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

The current text shows similarities between conditional handover and LTM (in both cases parallel transactions are possible), however reusing the sentence “Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.” is not accurate for LTM.

For the LTM case, we need to ensure that to identify a specific UE both source UE AP IDs and target UE AP IDs are the same (and not only the source UE AP ID is the same). This is due to subsequent LTM, where the new candidate gNBs (after LTM execution) need to recognize the same UE for which they have established a UE association towards the previously serving gNB. A similar need did not exist for conditional handover.

- For subsequent LTM the new candidate gNBs (after LTM execution) needs to recognize the same UE for which they established a UE association towards the previously serving gNB. Similar need does not exist for conditional handover.

To reduce the specification impact and clarifies the description for LTM, we propose to update the general section of the Handover Preparation procedure as follows:

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover or LTM, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same. If the procedure concerns an LTM when the candidate cells are configured within the same target NG-RAN node, parallel transactions are allowed using the same pair of source NG-RAN node UE XnAP ID and target NG-RAN node UE XnAP ID.

- In the general description for the Handover Preparation indicates that if this concerns LTM, and the configured candidate cells are in the same target NG-RAN node, parallel transactions are allowed using the same pair of source NG-RAN node UE XnAP ID and target NG-RAN node UE XnAP ID.

### Support for subsequent inter-CU LTM

In this section we discuss open issues concerning the support for subsequent inter-CU LTM.

One remaining issue is how to ensure that the new serving gNB can identify the UE context. Related agreements are:

(RAN3#126): “To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.”.

(RAN3#127-bis): For subsequent LTM, the candidate gNB uses the old target UE XnAP ID (the target UE AP ID it previously allocated between it and the old serving gNB) to identify the UE context when receiving the LTM Configuration Update message from the new serving gNB. FFS on how to deliver the old target UE XnAP ID(s) to the new serving gNB.

During LTM preparation, the first serving gNB establishes a UE association towards the candidate gNBs. After the LTM cell switch, the new serving gNB establishes UE associations between itself and the other candidate gNBs. There will be then new pairs of UE XnAP IDs. The figure below shows an example to visualize how it is possible to “follow” the UE as it moves across the gNBs. The source gNB is gNB1, gNB2, gNB3 and gNB4 are candidate gNBs.

During the initial LTM preparation, one UE association is established between gNB1 and gNB2, another between gNB1 and gNB3, and a third one between gNB1 and gNB4. A pair of values “source NG-RAN node UE XnAP ID, target NG-RAN node UE XnAP ID” identifies one UE association. After the initial LTM preparation, we have 3 UE associations:

- (A) between gNB1 and gNB2: {(gNB1 to gNB2) UE XnAP ID, (gNB2 to gNB1) UE XnAP ID}

- (B) between gNB1 and gNB3: {(gNB1 to gNB3) UE XnAP ID, (gNB3 to gNB1) UE XnAP ID}

- (C) between gNB1 and gNB4: {(gNB1 to gNB4) UE XnAP ID, (gNB4 to gNB1) UE XnAP ID}

For example, the “(gNB1 to gNB2) UE XnAP ID” is the identity assigned by gNB1 for a UE1, which gNB2 can use to when signaling something that concerns UE1 to gNB1. In principle, a node can assign the same “UE XnAP IDs” to identify the same UE over different UE associations, e.g.:  (A) = (1,2), (B) = (1, 3), (C) = (1, 4). In the unlikely event where every node selects the same ID for the UE, (A), (B) and (C) are the same, for example: (A) = (B) = (C) = (1,1).

- The “UE XnAP ID” assigned by one node to refer to a certain UE in two different UE associations towards different targets can be the same.

After a first LTM cell switch, for example from gNB1 to gNB2, the previously established UE associations can be kept, and the new serving gNB2 will setup new UE associations, one between itself and gNB3 and another between itself and gNB4. The new UE associations will have new identifiers, for example:

- (D) between gNB2 and gNB3: {(gNB2 to gNB3) UE XnAP ID, (gNB3 to gNB2) UE XnAP ID}

- (E) between gNB2 and gNB4: {(gNB2 to gNB4) UE XnAP ID, (gNB4 to gNB2) UE XnAP ID}

If a node assigns the same ID to identify the same UE over different UE associations, then it could be, for example: (A) = (1,2), (B) = (1, 3), (C) = (1, 4), (D) = (2, 3), (E) = (2, 4). In the worst case, all IDs (A), (B), (C), (D), (E) are equal.

To “follow” the UE, one of the candidates, for example gNB3, needs to understand that the newly established UE association (D) points to the same UE referenced by the UE association (B). To let gNB3 understand that the UE referenced by (D) and (B) is the same, either:

- before the LTM cell switch execution, the old serving gNB, gNB1, sends (B) to gNB2 (in the LTM CONFIGURATION UPDATE),

or

- at LTM Cell Switch, the old serving gNB, gNB1, sends (B) to the new serving gNB, gNB2, (in the CELL SWITCH NOTIFICATION), and then gNB2 sends (B) to gNB3 (in the LTM CONFIGURATION UPDATE).

During the RAN3#128 meeting, two main options were discussed for transferring the old target UE XnAP ID(s) to the candidate gNB(s). While a single ID can uniquely identify a UE, there is a risk that the sending node may provide an incorrect value, leading to potential errors when the receiving node(s) interprets it.

To minimize the likelihood of error handling, we propose including both the old source UE XnAP ID and the old target UE XnAP ID as a pair in the subsequent LTM. This ensures unambiguous UE identification and reduces the chance of misinterpretation.

As discussed, the “UE XnAP ID” assigned by one node for one UE can be the same over different UE associations. Therefore, using the “UE XnAP ID” alone to uniquely identify the same UE when the UE moves across different nodes, might not be enough. To be sure that there is no ambiguity, we can combine the “UE XnAP ID” with the gNB ID. For example, if new serving gNB is gNB2, and sends to gNB3 the information: (B + gNB IDs), and now gNB3 can identify that the UE pointed by the UE association (B) between gNB1 and gNB3.

Using more general terms, to support subsequent LTM, and allow a candidate to recognize with no ambiguity a UE for which LTM preparation was done, as the UE moves across different gNBs, we think that after an LTM cell switch the new serving gNB can send to one candidate gNB the following information:

- “old source NG-RAN node UE XnAP ID”: the “NG-RAN node UE XnAP ID” assigned by the gNB that served the UE before LTM cell switch (gNB1)”,

- “old source NG-RAN node ID”: the ID of the old serving gNB (e.g., gNB1 ID),

- “old target NG-RAN node UE XnAP ID”: the “NG-RAN node UE XnAP ID” assigned by the candidate gNB, before the LTM execution

- “old target NG-RAN node ID”: the ID of the candidate gNB, (e.g. gNB3 ID).

- After an LTM Cell Switch, the new serving gNB sends in an LTM CONFIGURATION UPDATE message the following information to allow the candidate gNB to identify the UE as it moves across candidate gNBs: old source NG-RAN node UE XnAP ID, old source NG-RAN ID, old target NG-RAN node UE XnAP ID, old target NG-RAN node ID.

If the event where more than one node assigns the same “XnAP ID” to the same UE is acceptable, then information to signal can be reduced to the combination: “old source NG-RAN node UE XnAP ID” “old target NG-RAN node UE XnAP ID”.

- After an LTM Cell Switch, the new serving gNB sends in an LTM CONFIGURATION UPDATE message the following information to allow the candidate gNB to identify the UE as it moves across candidate gNBs: old source NG-RAN node UE XnAP ID, old target NG-RAN node UE XnAP ID.

At RAN3#127-bis it was further agreed that “Data Forwarding Information IE is a list including multiple data forwarding addresses from each candidate gNB(s).” The granularity of each item in the list remains to be decided, e.g., should this information be per-node, per-PDU session, per DRB?

The sync up among the candidates is performed with the LTM Configuration Update, we propose to keep a consistent approach between the preparation phase (Handover Preparation) and the sync up phase (LTM Configuration Update) and to resolve the FFS indicating that the source gNB sends to the candidate gNB the Data Forwarding Info from target NG-RAN node IE for each PDU Session.

- The source gNB sends the Data Forwarding Information as per-PDU session level to the candidate gNBs in the LTM CONFIGURATION UPDATE message.

### Rel-19 Set ID

RAN2 has introduced a “Rel-19 Set ID” (ltm-NoSecurityChangeID) cell level parameter, used by the UE to understand whether it should perform a vertical key derivation at LTM cell switch execution. According to the RAN2 LS received in R3-253009, if the network wants to trigger security key update in case of LTM, the network can configure different Rel-19 Set IDs for different cells, including serving and/or intra-CU candidate cells. The topic was discussed at the last RAN3 meeting, and the following open points were captured:

FFS on the Rel-19 Set ID(s) assignment among CUs, down select from Option1 and Option2:

Option 1: Source gNB sends the Rel-19 Set ID(s) or Rel-19 set ID range assigned to the candidate gNB in the HANDOVER REQUEST message, then candidate gNB assigns Rel-19 set ID(s) to its own candidate cells and feedback via HANDOVER REQUEST ACKNOWLEDGE message.

Option 2: Candidate gNB provides Rel-19 set ID per candidate cell in HANDOVER REQUEST ACKNOWLEDGE message, then source gNB may update the Rel-19 set ID to ensure that the Rel-19 set IDs under different candidate gNB-CU are different.

FFS on the procedure to be used for source gNB to transfer Rel-19 set ID per candidate cell to the candidate gNB.

First, we think that the same Rel-19 Set ID is assigned to cells of different candidate gNBs. This is because a vertical key derivation is needed when inter-CU LTM is executed. Because of that, it is beneficial to have a single entity that assigns the potential Rel-19 Set IDs that can be used across all possible LTM candidate cells. However, we want to allow a candidate gNB, controlling two (or more) LTM candidate cells, to assign, if it wants, the same Rel-19 Set ID to all candidates under its control, or assign different Rel-19 Set IDs to each candidate cell, or something in between (for example, the signaling should support the case where one candidate gNB controlling three candidate cell, can assign one Rel-19 Set ID to two cells, and another Rel-19 Set ID to the third cell).

Let’s consider this example: gNB1 is the source gNB, and controls only one (source) cell, Cell A, which is also an LTM candidate cell, gNB2 is a candidate gNB and controls 3 LTM candidate cells, Cell B, Cell C, Cell D. The gNB3 is another candidate gNB and controls 2 LTM candidate cells, Cell E and Cell F.

We think that a candidate gNB should be in control on whether different cells should have different Rel-19 Set ID for the cells. That is, it should be the gNB2 (and not the gNB1) to choose whether to assign different Rel-19 Set ID for Cell B and Cell D, while Cell B and Cell C can have the same Rel-19 Set ID. Similarly, it should be gNB3 to choose whether to assign different Rel-19 Set ID for Cell E and Cell F. An example of assigned Rel-19 Set ID can be:

gNB1, Cell A: Rel-19 Set ID=1

gNB2, Cell B: Rel-19 Set ID=2, 			Cell C: Rel-19 Set ID=2		Cell D: Rel-19 Set ID = 3

gNB3, Cell E: Rel-19 Set ID=4			Cell F: Rel-19 Set ID=5

- A candidate gNB chooses whether to assign the same or different Rel-19 Set ID for the candidate cells under its control.

Analysing the options on the table, our understanding of Option 1 is the following: the source gNB, in the HANDOVER REQUEST, sends a list of Rel-19 Set IDs which the candidate gNB can use for the cell being prepared. The candidate gNB chooses one Rel-19 Set ID from the list, assigns that value to the cell being prepared, and sends the value to the source gNB in the HANDOVER REQUEST ACKNOWLEDGE.

If Option 1 is selected “as is”, there will be a security problem if the source gNB contacts different candidate gNBs, sending to each one of them the same list of Rel-19 Set IDs. Consider again the same example where gNB1 is the source gNB, and gNB2 and gNB3 are the candidate gNBs. Since there are 5 candidate cells in total, the gNB1 sends the same list of Rel-19 Set IDs to both gNB2 and gNB3:

gNB1 sends Rel-19 Set ID = 2, 3, 4, 5, 6 to gNB2

gNB1 sends Rel-19 Set ID = 2, 3, 4, 5, 6 to gNB3

gNB2 selects Rel-19 Set ID=2 for Cell B and Cell C, and Rel-19 Set ID=3 for Cell D

gNB3 does not know the Rel-19 Set IDs already “taken” by gNB2 and selects Rel-19 Set ID=2 for Cell E, and Rel-19 Set ID=3 for Cell C

The situation becomes:

gNB1, Cell A: Rel-19 Set ID=1

gNB2, Cell B: Rel-19 Set ID=2, 			Cell C: Rel-19 Set ID=2		Cell D: Rel-19 Set ID = 3

gNB3, Cell E: Rel-19 Set ID=2			Cell F: Rel-19 Set ID=3

With this setting, there will be a security problem, for example if the UE moves from Cell C (in gNB2) to Cell E (in gNB3), because it will look as if no security key change is needed, and this is not true. A possible solution is that the source gNB resolves the conflicts reassigning new Rel-19 Set IDs and then sends the consolidated Rel-19 Set IDs to all candidate gNBs. This can be done with a second round of signaling (e.g., with the LTM Configuration Update).

Another way is that, from the beginning, the lists of Rel-19 Set IDs sent to different candidate gNBs do not overlap. In the “worst case” (in terms of the amount of Rel-19 Set IDs to be assigned), we can imagine that each candidate gNB wants to perform security key change for an LTM cell switch between any pair of candidate cells under its control. In this case, the source gNB, needs to reassign practically all the Rel-19 Set IDs. If, instead the source gNB sends separate lists of Rel-19 Set IDs to the candidates, i.e., the lists sent to different candidate gNBs do not have any element in common, each candidate can assign the Rel-19 Set IDs and there will be no conflict.

For example:

- In the HANDOVER REQUEST messages from gNB1 to gNB2, gNB1 sends as proposed Rel-19 Set IDs the values 2, 3, 4

- In the HANDOVER REQUEST messages from gNB1 to gNB3, gNB1 sends as proposed Rel-19 Set IDs the values 5, 6

Each candidate gNB is free to choose the Rel-19 Set ID according to its wish to use or not use security key change for LTM cell switches among its own cells, and the Rel-19 Set IDs one candidate gNB can assign do not overlap with the Rel-19 Set IDs that another candidate gNB can assign.

For example, if gNB2 wants security key change for LTM between Cell B to Cell D, or between Cell C to Cell D, but no security key change for LTM between Cell B to Cell C, the gNB2 assigns the same Rel-19 Set ID to Cell B and Cell C (e.g., value 2), and a different Rel-19 Set ID to Cell D (e.g., value 3). Similar situation is valid for gNB3.

Instead of the HANDOVER REQUEST, the source gNB can send all the proposed Rel-19 Set IDs for a certain candidate gNB in one message, using the LTM CONFIGURATION UPDATE instead. If the lists of Rel-19 Set IDs sent to different candidate gNBs do not overlap, we can still avoid the conflicts, and instead of using multiple HANDOVER REQUEST, only one message is used. The candidate gNB assigns one Rel-19 Set ID for each candidate cell and returns the information to the source gNB in the LTM CONFIGURATION UPDATE ACKNOWLEDGE.

To properly support subsequent LTM, regardeless if the “multiple Handover Preparations” approach or the “single LTM Configuration Update” approach is used, the candidate gNBs need synchronized.

With the “multiple Handover Preparations” approach, the source gNB, after it has collected all the Rel-19 Set IDs assigned by the candidates, can send an LTM CONFIGURATION UPDATE with the list of Rel-19 Set IDs.

With the “single LTM Configuration Update” approach, at LTM Cell Switch the old serving gNB can send the list of Rel-19 Set IDs to the new serving gNB in the CELL SWITCH NOTIFICATION, and then the new serving gNB sends an LTM CONFIGURATION UPDATE to inform all the candidate gNBs. This approach is shown in the figure below.

- The source gNB sends to a candidate gNB a list of proposed Rel-19 Set IDs with as many elements as the candidate cells owned by the candidate gNB. The lists of Rel-19 Set IDs sent to one candidate gNB does not overlap with the list of Rel-19 Set IDs sent to any other candidate gNB. The candidate gNB assigns one Rel-19 Set ID to one candidate cell and feedback the information to the source gNB.

- After the LTM cell switch, the old source gNB sends to the new serving gNB the list of Rel-19 Set IDs of the other candidate gNBs. The new serving gNB uses the LTM CONFIGURATION UPDATE to synchronize the candidate gNBs (i.e., sends the list of Rel-19 Set IDs assigned by the other candidate gNBs).

- Clarify in TS 38.300 that the source gNB sends separate lists of Rel-19 Set IDs to the candidate gNBs.

The Rel-19 Set ID selected by the source CU or a candidate CU for a certain cell should be sent to the underlying DUs. For this we propose that the CU can send, to the source DU or the candidate DU a UE CONTEXT MODIFICATION message containing a list of Rel-19 Set ID, one for each cell (either source cell, or candidate cell).

- The CU sends to the source DU and to candidate DU a list of Rel-19 Set IDs in the UE CONTEXT MODIFICATION REQUEST message.

### Partial Xn connectivity among candidate gNBs

We want now to raise an issue that can occur, which impacts how to achieve a proper sync up of the configurations among the candidate gNBs for subsequent LTM. We think that a deployment scenario as in the figure below is possible, where there is NOT a full mesh Xn connectivity among all the gNBs involved in the LTM. The source gNB is gNB A and gNBs B, C, D, E are candidate gNBs for the first LTM Cell Switch. There is no Xn connectivity between gNB B and gNB E, nor between gNB D and gNB C. An obvious reason for the absence of the Xn connectivity can be that there is no coverage continuity between gNB B and gNB E (nor between gNB D and gNB C).

Figure 1 - Subsequent LTM when no full mesh Xn connectivity is present among the gNBs.

If the UE moves from gNB A to gNB B, the gNB E will not be used for the very next LTM Cell Switch, i.e., the gNB B will never send an LTM Cell Switch command to move the UE from gNB B to gNB E, so, for example, what would be the purpose of sending the Data Forwarding Info of gNB E from gNB A to gNB B? Also, if gNB A sends to gNB B the Data Forwarding Info of gNB E, how can gNB B understand it?

As per RAN3#126 agreement, after an inter-CU LTM Cell Switch, the new source gNB can use the LTM Configuration Update to establish the UE association between itself and the other candidate gNBs. In the example, when the UE moves from gNB A to gNB B, the gNB B can establish a new UE association between itself and gNB C and gNB D (and keep the UE association towards gNB A). Even if the UE continues to keep the same LTM configuration provided by gNB A, at the network side, there is no need to keep the previously established UE associations between gNB A and gNB E, nor between gNB E and gNB C. In fact, keeping this UE association is a waste of resources in gNB E. We would like to discuss this issue, and hopefully find a solution for that.

- RAN3 to discuss how to sync up configuration among candidate gNBs in case of partial Xn connectivity among the candidate gNBs.

### Reference to LTM Candidate in F1 and Xn

One open issue from last meeting is the following: “FFS on whether the TCI State/QCL-info List needs to be included in CSI-RS COORDINATION procedure.”. At the last RAN3 meeting, some companies (e.g., in R3-253324, R3-253198) mentioned the following agreement taken at RAN2#129-bis:

Agreements on L1 event-triggered MR 1. Introduce new MAC CE to activate/deactivate the semi-persistent CSI-RS resource. 2. The new MAC CE includes: A/D indication, Target configuration id or {SP CSI-RS resource set id and/or candidate id}, TCI state id.

Based on the above, and the fact that candidate CU/DU decides the TCI states, it was proposed to transfer the TCI states of the activated/deactivated SP CSI-RS resources in the CSI-RS Coordination procedures in F1 and Xn. The TCI state and QCL information is part of the RRC LTM-TCI-Info IE included in the LTM-Candidate IE, and in F1 it is currently signalled in the TCI States Configurations List IE, which is part of the Early sync information contained in the UE CONTEXT SETUP RESPONSE and UE CONTEXT MODIFICATION REQUEST messages. On the other hand, the transfer of this information is optional (the parent IE, e.g., Early Sync Information, is optionally present), therefore it is not certain that the source DU can receive the LTM-TCI-Info information from the candidate DU. Furthermore, we note that other information is added in Rel-19, which is also included in the RRC LTM-Candidate IE, (e.g., ltm-CSI-ReportConfigToAddModList, ltm-CSI-IM-ResourceSetToAddModList, etc.), and the source DU needs to send it information to the UE. Therefore, to address the FFS, and allow the transfer from candidate DU to source DU of the new information added in Rel-19, we propose to add a new IE (e.g., LTM Candidate IE) in the UE CONTEXT SETUP RESPONSE and UE CONTEXT MODIFICATION REQUEST messages to transfer the RRC LTM-Candidate IE.

- Add a new LTM Candidate IE in the UE CONTEXT SETUP RESPONSE and UE CONTEXT MODIFICATION REQUEST messages which refers to the LTM-Candidate IE defined in TS 38.331.

Regarding the Xn baseline for Rel-19, the LTM Information Response IE currently refers to the LTM-CandidateConfig. On the other hand, the RRC LTM-Candidate-r18 IE contains both the LTM-TCI-Info-r18 IE and the LTM-CandidateConfig IE. If we modify the current reference to point at the LTM-Candidate IE, without introducing another IE, we can transfer the mapping between SSB and TCI state, and the mapping between CSI-RS and TCI-state, and at the same time we align the F1 and Xn specification design.

- Change the semantics description of the LTM Information Response IE to refer to the LTM-Candidate IE defined in TS 38.331.

Another open point to address is the following: “FFS on whether to include the SP CSI-RS and SSB mapping in the HANDOVER REQUEST ACKNOWLEDGE message and UE Context Modification Request message.”. In our understanding the FFS concerns two aspects, i.e. the mapping between SSB and TCI state, and the mapping between CSI-RS and TCI state. The two mappings are contained in the LTM-TCI-Info-r18 IE. Hence, if the previous proposals are agreed, also this FFS is resolved.

### CSI Resource Configuration

In Rel-19, RAN3 discussed the procedure to request a CSI-RS resource configuration, and an indicator was introduced for this purpose. However, the case over Xn differs from the Rel-18 intra-CU LTM scenario. The commonality is that the CSI resource configuration specifies the list of reference signals to be measured, which may be either SSB or CSI-RS.

For inter-CU LTM, the simplest approach is to unify the handling of SSB and CSI-RS, since the candidate network node decides which reference signal is used. This unification also simplifies the signalling design when referencing RRC messages. Instead of requesting a “CSI-RS resource configuration,” the request should be for a “CSI resource configuration,” after which the candidate network node provides the configuration, for either SSB RS or CSI-RS measurement. Accordingly, it is proposed to rename the IE to “Request for CSI Resource Configuration”, with the candidate network node providing the CSI Resource Configuration covering both options.

In addition, alignment with the F1 interface should be addressed. Since SSB Information is mandatory in F1AP messages, it is proposed to keep the same design as Xn and to add semantic descriptions to the existing IE, i.e., SSB Information IE, for Rel-19.

CSI Resource Configuration over Xn:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CSI Resource Configuration To AddModList | O |  | OCTET STRING | Contains the ltm-CSI-ResourceConfigToAddModList as defined in TS 38.331 [10]. |
| CSI Resource Configuration To Release List | O |  | OCTET STRING | Includes the ltm-CSI-ResourceConfigToReleaseList contained in the LTM-Config IE as defined in TS 38.331 [10]. |

- Change the name of “Request for CSI-RS Resource Configuration” IE to “Request for CSI Resource Configuration” to cover both SSB and CSI-RS over Xn.

- Align F1AP with similar changes, and add semantics description to the mandatory “SSB Information” IE indicating that it is only applicable in Rel-18.

### CSI-RS resource type

Regarding the following FFS: “FFS on whether gNB-DU/gNB provides the report type (periodic or semi-persistent) of the CSI-RS resources in both F1AP and XnAP.”, first of all we observe that, based on the proposals and the discussion of the last RAN3 meeting, the formulation is not correct and it should rather be: “FFS on whether gNB-DU/gNB provides the resource report type (periodic or semi-persistent) of the CSI-RS resources in both F1AP and XnAP.” This would reflect e.g., the following proposal in R3-253599:

Proposal 7: When a candidate gNB-DU/gNB provides CSI-RS resources, it indicates the resource type (periodic or semi-persistent) of the CSI-RS resources in both F1AP and XnAP.

As shown below, the resource type of the CSI-RS resources is already included in the ltm-CSI-ResourceConfigToAddModList IE, therefore we think that the FFS can be closed with no signalling impact.

LTM-CSI-ResourceConfig-r18 ::=      SEQUENCE {

ltm-CSI-ResourceConfigId-r18        LTM-CSI-ResourceConfigId-r18,

ltm-SSB-ResourceSet-r18         LTM-SSB-ResourceSet-r18,

...,

[[

ltm-NZP-CSI-RS-ResourceSet-r19  LTM-NZP-CSI-RS-ResourceSet-r19                     OPTIONAL, -- Need R

ltm-CSI-IM-ResourceSet-r19      LTM-CSI-IM-ResourceSet-r19                         OPTIONAL, -- Need R

resourceType-r19                ENUMERATED {periodic, semiPersistent}              OPTIONAL  -- Cond CSI-RS

]]

}

LTM-SSB-ResourceSet-r18 ::=     SEQUENCE {

ltm-SSB-ResourceList-r18        SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF SSB-Index,

ltm-CandidateIdList-r18             SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF LTM-CandidateId-r18,

...

}

LTM-NZP-CSI-RS-ResourceSet-r19 ::=     SEQUENCE {

ltm-CSI-RS-ResourceList-r19         SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF NZP-CSI-RS-ResourceId,

ltm-CandidateIdList-r19             SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF LTM-CandidateId-r18,

...

}

### Stage 2 updates

In the current baseline for TS 38.300, the old “Figure 9.2.3.5.2-1. Signalling procedure for LTM” is replaced by a flow chart where we can identify the following issues, related to the use of functional entities only present in a split gNB:

- There is an actor named “Target gNB-DU from Source gNB”

- The step 3.a is for “LTM configuration preparation for Intra-CU candidates”

- The step 6.a is for “LTM configuration update for Intra-CU candidates”

We appreciate that the intention of the new signalling flow, specifically in relation to the bullets above, is to highlight that intra-CU LTM and inter-CU LTM can coexist. However, the TS 38.300 is not meant to present functional entities of a split gNB, nor signaling exchange between them. To address this and make sure that stage 2 clearly indicates that for LTM both configuration of both intra-CU candidates and inter-CU candidates is possible, we propose to:

- Revert the change for deleting the old signaling procedure, and indicate that it refers to intra-gNB LTM (e.g., use the name: “Signalling procedure for intra-gNB LTM”)

- Rename the new signaling procedure (e.g., “Signalling procedure for inter-gNB LTM”)

- For the new signaling procedure:

  - remove the actor named “Target gNB-DU from Source gNB”

  - Add one note, after step 2, to clarify that LTM configuration preparation can be done for intra-gNB candidates.

  - Add one node, after step 5, to clarify that LTM configuration update can happen for intra-gNB candidates.

- Apply the following modifications:

  - Keep the old (previously removed) signaling procedure and name it “Signaling procedure for intra-gNB LTM”.

  - Rename the new signaling procedure “Signaling procedure for inter-gNB LTM” and modify it as follows:

    - Remove the actor named “Target gNB-DU from Source gNB”

    - Add one note after step 2, to indicate that LTM configuration preparation for intra-gNB candidate(s) is possible

    - Add one note after step 5, to indicate that LTM configuration update for intra-gNB candidate is possible.

At the last meeting, the following was agreed: “Remove Note in TS 38.300 BL CR “Editor’s Note: step 6 and 7 are optional.”. In the figure illustrating the signaling procedure for inter-gNB LTM, steps 6 and 7 correspond to the LTM Configuration Update procedure executed during the preparation phase. To properly reflect the agreement in the graphics, the two arrows should be made dashed lines, instead of continuous lines.

- Update the signaling procedure for inter-gNB LTM in TS 38.300, using dashed lines for steps 6 and 7.

Before the LTM execution, the CSI-RS Coordination procedure may be executed, e.g., to activate/deactivate SP CSI-RS resources. To capture this in stage 2, we propose to add two optional steps, 11a and 11b, to include the CSI-RS COORDINATION REQUEST and CSI-RS COORDINATION RESPONSE messages.

Similarly, in the stage 2 signaling procedure for inter-gNB-DU LTM in 38.401, we propose to add dedicated steps (steps 12a – 12d) to show the DU-CU CSI-RS Coordination and CU-DU CSI-RS Coordination procedures.

- In the signaling procedure for inter-gNB LTM in TS 38.300, add steps 11a and 11b to show the CSI-RS Coordination procedure.

- In the signaling procedure for inter-gNB-DU LTM in TS 38.401, add steps 12a-12d to show the DU-CU CSI-RS Coordination and CU-DU CSI-RS Coordination procedures.

## Inter-CU LTM in NR-DC

For inter-CU LTM in NR-DC, we consider the following agreements and open points captured at the previous RAN3 meetings:

Update the text description of CELL SWITCH NOTIFICATION message to capture the RAN3#127bis agreement: Include the target PSCell ID and corresponding TCI State ID(s) in the Cell Switch Notification message, and reusing the existing IEs.

The SCG reference configuration is provided by an implicit way in the CG-Config RRC container in the SN Change Required message. FFS on the SN Addition Request Acknowledge message from the source SN/candidate SN to the MN.

WA: Different candidate PSCells in the same SN can have different Rel-19 set IDs. FFS on the solutions. Try to reuse the same solution for inter-CU LTM.

If the source SN has the SCG reference configuration, the source SN will provide the SCG reference configuration in the SN Change Required message and thus the MN will not request source SN to provide the SCG reference configuration.

Normal data forwarding can be initiated after the source SN decides to trigger the LTM cell switch for the UE. The exact timing of its initiation is left to implementation.

Enhance XN-U ADDRESS INDICATION message and define IE to cover Inter-CU MCG LTM case. FFS on Inter-SN LTM case.

MN uses SN modification request message to notify the Source SN that UE has successfully accessed to the target SN. FFS whether Handover Success is used from the target-SN to the MN to notify that UE has successfully accessed to the target SN.

The source SN generates the common LTM CSI resource configuration for inter-CU SCG LTM and then provides the common LTM CSI resource configuration to the MN via the SN Modification Request ACK message.

For the SSB based L1 measurement, the common CSI resource configuration may be included in the SN Change Required message and the SN Addition Request message.

The CSI Resource Configuration IE is the common CSI resource configuration, which refers to the ltm-CSI-ResourceConfig-r18 IE in the RRC spec.

The L1 Configuration IE is the L1 RS configuration per candidate PSCell, which refers to the ltm-SSB-Config-r18 IE in the RRC spec for the SSB based L1 measurement, or the ltm-NZP-CSI-RS-ResourceConfigToAddModList-r19 IE associated with the ltm-NZP-CSI-RS-ResourceSetToAddModList-r19 IE in the RRC spec for the CSI-RS based L1 measurement.

FFS on whether LTM modification/cancel related procedures are needed.

### Rel-19 Set ID

Regarding the support for security key change at LTM cell switch when the UE is in NR-DC, at the last meeting a WA was taken with one FFS: “WA: Different candidate PSCells in the same SN can have different Rel-19 set IDs. FFS on the solutions. Try to reuse the same solution for inter-CU LTM.”. Given that the network can configure different Rel-19 Set IDs for different cells, this principle applies to the NR-DC case as well. In our view, it is then straightforward to confirm the above WA. Regarding the open issue, we think the same solution described in section 2.3 can be used. This means that:

- For MN initiated inter-CU MCG LTM, the source MN sends to each candidate MN disjoint lists of proposed Rel-19 Set IDs (in the HANDOVER REQUEST message), and the candidate MN sends to the source MN the Rel-19 Set ID assigned to the candidate cell (in the HANDOVER REQUEST ACKNOWLEDGE message).

- For SN initiated inter-CU SCG LTM, the source SN sends to each candidate SN (via the MN) disjoint lists of proposed Rel-19 Set IDs (in the S-NODE CHANGE REQUIRED and S-NODE ADDITION REQUEST messages). The candidate SN sends to the source SN (via the MN) the Rel-19 Set ID assigned to the candidate cell (in the S-NODE ADDITION REQUEST ACKNOWLEDGE and S-NODE CHANGE CONFIRM messages).

- Different candidate PSCells in the same SN can have different Rel-19 set IDs.

- For inter-CU MCG LTM, the source MN sends to each candidate MN disjoint lists of proposed Rel-19 Set IDs (in the HANDOVER REQUEST message), and the candidate MN sends to the source MN the Rel-19 Set ID assigned to the candidate cell (in the HANDOVER REQUEST ACKNOWLEDGE message).

- For SN initiated inter-CU SCG LTM, the source SN sends to each candidate SN (via the MN) disjoint lists of proposed Rel-19 Set IDs (in the S-NODE CHANGE REQUIRED and S-NODE ADDITION REQUEST messages). The candidate SN sends to the source SN (via the MN) the Rel-19 Set ID assigned to the candidate cell (in the S-NODE ADDITION REQUEST ACKNOWLEDGE and S-NODE CHANGE CONFIRM messages).

### Data Forwarding

Another FFS from the last RAN3 meeting is the following: “Enhance XN-U ADDRESS INDICATION message and define IE to cover Inter-CU MCG LTM case. FFS on Inter-SN LTM case.”. At the moment, we don’t see any reason for different indications to distinguish whether the data forwarding is for inter-CU MCG LTM or SN-initiated inter-CU SCG LTM, and the LTM DC Data Forwarding Indicator agreed at the last RAN3 meeting is sufficient. As a clarification, we can update the semantics of the IE and the procedure text, to include the SN-initiated Inter-CU SCG LTM.

- Update the procedure text for XN-U Address Indication and the semantics description for the LTM DC Data Forwarding Indicator to clarify that the IE is used also in case of SN-Initiated Inter-CU SCG LTM.

### Cell Switch Notification

At RAN3#127, it was agreed to reuse the Cell Switch Notification message from source SN to target SN via MN, and further details were FFS: “The Cell Switch Notification message can be reused from the source SN to the target SN via the MN. The detailed IE can be further discussed.”

For Inter-CU MCG LTM with SN addition and for SN initiated inter-CU SCG LTM (without MCG change), the MN needs to send to the new SN the security configuration that SN can use (SN security key and the list of possible sk-counters the UE can apply).

- For SN initiated inter-CU SCG LTM, the MN sends to the target SN the SN security keys and a list of sk-counters in the CELL SWITCH NOTIFICATION message.

- For Inter-CU MCG LTM with SN addition, the MN sends to the target SN the SN security keys in the CELL SWITCH NOTIFICATION message.

Concerning the Cell Switch Notification, the following was also agreed at the last meeting: “Update the text description of CELL SWITCH NOTIFICATION message to capture the RAN3#127bis agreement: Include the target PSCell ID and corresponding TCI State ID(s) in the Cell Switch Notification message, and reusing the existing IEs.”. Moreover, in case of SN-initiated inter-CU SCG LTM, as already captured in stage 2 baseline (37.340) the Cell Switch Notification is reused to inform the new SN of the initiation of the cell switch command to the UE. To reflect the agreements above and the new behaviour, we propose to update the Cell Switch Command in the current baseline accordingly.

- Update Cell Switch Notification definition and content to include the target PSCell ID and corresponding TCI state ID(s), and to reflect the use of the message in dual connectivity scenarios.

### Informing MN of successful access of target SN

The following FFS is pending from the last meeting: “MN uses SN modification request message to notify the Source SN that UE has successfully accessed to the target SN. FFS whether Handover Success is used from the target-SN to the MN to notify that UE has successfully accessed to the target SN.”

We observe that in the HANDOVER SUCCESS message the presence of the Requested Target Cell ID IE is mandatory, which is the “Target cell indicated in the corresponding Handover Preparation procedure” (see below). The Handover Preparation procedure is not used in case of SN-Initiated inter-CU SCG LTM, therefore we don’t think it is appropriate to use the HANDOVER SUCCESS message in this case. In SN-Initiated inter-CU SCG LTM, the MN receives from the UE the RRCReconfigurationComplete message, which tells the MN that the UE successfully accessed the PSCell. Therefore, we don’t think the target SN needs to notify the MN that UE has successfully accessed the target SN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node. | YES | reject |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node. | YES | reject |
| Requested Target Cell ID | M |  | Target Cell Global ID 9.2.3.25 | Target cell indicated in the corresponding Handover Preparation procedure | YES | reject |
| Accessed PSCell ID | O |  | NR CGI 9.2.2.7 |  | YES | ignore |

- For SN initiated inter-CU SCG LTM, there is no need for the target SN to notify the MN that UE has successfully accessed the target SN.

### LTM modification and cancel

One open issue from the last RAN3 meeting relates to whether LTM modification/cancel related procedures are needed for NR-DC: “FFS on whether LTM modification/cancel related procedures are needed.”.

In the SoD for NR-DC two cases were identified in relation the above FFS, namely:

Case 1: candidate SN can decide to update or release some of the prepared candidate PSCells.

Case 2: source SN decides to update or release some of the suggested candidate PSCell

Case 2-1: the source SN decides to add or release a candidate PSCell prepared at the source SN

Case 2-2: the source SN decides to update the suggested candidate PSCells based on the latest L3 measurement results, or to update the upper limited number of PSCells of the candidate SN.

For Case 1, we think that a candidate SN can cancel an already prepared LTM handover towards a certain candidate PSCell reusing the LTM Cancel procedure introduced for single connectivity. In this case the target SN indicates its intention to the source SN via the MN.

For Case 2, we note that for intra-CU LTM and inter-CU LTM in single connectivity, there is no support to dynamically update the list of candidate cells, and we don’t see any reason for introducing such a feature in dual connectivity, also in consideration of the fact that the WI is to be closed.

We propose to update the LTM Cancel procedure, to indicate that candidate SN can cancel an already prepared PSCell.

- For SN initiated inter-CU SCG LTM, the target SN can cancel an already prepared candidate PSCell sending an LTM Cancel to the MN. The MN forwards the information to the source SN with the LTM Cancel procedure.

- Agree on the TPs in R3-255269 that capture the above proposals.

# Conclusions and Proposals

In this contribution we made observations and proposals for different LTM related topics, as listed below.

Observation 1	For subsequent LTM the new candidate gNBs (after LTM execution) needs to recognize the same UE for which they established a UE association towards the previously serving gNB. Similar need does not exist for conditional handover.

Observation 2	The “UE XnAP ID” assigned by one node to refer to a certain UE in two different UE associations towards different targets can be the same.

Proposal 1	In the general description for the Handover Preparation indicates that if this concerns LTM, and the configured candidate cells are in the same target NG-RAN node, parallel transactions are allowed using the same pair of source NG-RAN node UE XnAP ID and target NG-RAN node UE XnAP ID.

Proposal 2	After an LTM Cell Switch, the new serving gNB sends in an LTM CONFIGURATION UPDATE message the following information to allow the candidate gNB to identify the UE as it moves across candidate gNBs: old source NG-RAN node UE XnAP ID, old source NG-RAN ID, old target NG-RAN node UE XnAP ID, old target NG-RAN node ID.

Proposal 3	After an LTM Cell Switch, the new serving gNB sends in an LTM CONFIGURATION UPDATE message the following information to allow the candidate gNB to identify the UE as it moves across candidate gNBs: old source NG-RAN node UE XnAP ID, old target NG-RAN node UE XnAP ID.

Proposal 4	The source gNB sends the Data Forwarding Information as per-PDU session level to the candidate gNBs in the LTM CONFIGURATION UPDATE message.

Proposal 5	A candidate gNB chooses whether to assign the same or different Rel-19 Set ID for the candidate cells under its control.

Proposal 6	The source gNB sends to a candidate gNB a list of proposed Rel-19 Set IDs with as many elements as the candidate cells owned by the candidate gNB. The lists of Rel-19 Set IDs sent to one candidate gNB does not overlap with the list of Rel-19 Set IDs sent to any other candidate gNB. The candidate gNB assigns one Rel-19 Set ID to one candidate cell and feedback the information to the source gNB.

Proposal 7	After the LTM cell switch, the old source gNB sends to the new serving gNB the list of Rel-19 Set IDs of the other candidate gNBs. The new serving gNB uses the LTM CONFIGURATION UPDATE to synchronize the candidate gNBs (i.e., sends the list of Rel-19 Set IDs assigned by the other candidate gNBs).

Proposal 8	Clarify in TS 38.300 that the source gNB sends separate lists of Rel-19 Set IDs to the candidate gNBs.

Proposal 9	The CU sends to the source DU and to candidate DU a list of Rel-19 Set IDs in the UE CONTEXT MODIFICATION REQUEST message.

Proposal 10	RAN3 to discuss how to sync up configuration among candidate gNBs in case of partial Xn connectivity among the candidate gNBs.

Proposal 11	Add a new LTM Candidate IE in the UE CONTEXT SETUP RESPONSE and UE CONTEXT MODIFICATION REQUEST messages which refers to the LTM-Candidate IE defined in TS 38.331.

Proposal 12	Change the semantics description of the LTM Information Response IE to refer to the LTM-Candidate IE defined in TS 38.331.

Proposal 13	Change the name of “Request for CSI-RS Resource Configuration” IE to “Request for CSI Resource Configuration” to cover both SSB and CSI-RS over Xn.

Proposal 14	Align F1AP with similar changes, and add semantics description to the mandatory “SSB Information” IE indicating that it is only applicable in Rel-18.

Proposal 15	Apply the following modifications:

a.	Keep the old (previously removed) signaling procedure and name it “Signaling procedure for intra-gNB LTM”.

b.	Rename the new signaling procedure “Signaling procedure for inter-gNB LTM” and modify it as follows:

i.	Remove the actor named “Target gNB-DU from Source gNB”

ii.	Add one note after step 2, to indicate that LTM configuration preparation for intra-gNB candidate(s) is possible

iii.	Add one note after step 5, to indicate that LTM configuration update for intra-gNB candidate is possible.

Proposal 16	Update the signaling procedure for inter-gNB LTM in TS 38.300, using dashed lines for steps 6 and 7.

Proposal 17	In the signaling procedure for inter-gNB LTM in TS 38.300, add steps 11a and 11b to show the CSI-RS Coordination procedure.

Proposal 18	In the signaling procedure for inter-gNB-DU LTM in TS 38.401, add steps 12a-12d to show the DU-CU CSI-RS Coordination and CU-DU CSI-RS Coordination procedures.

Proposal 19	Different candidate PSCells in the same SN can have different Rel-19 set IDs.

Proposal 20	For inter-CU MCG LTM, the source MN sends to each candidate MN disjoint lists of proposed Rel-19 Set IDs (in the HANDOVER REQUEST message), and the candidate MN sends to the source MN the Rel-19 Set ID assigned to the candidate cell (in the HANDOVER REQUEST ACKNOWLEDGE message).

Proposal 21	For SN initiated inter-CU SCG LTM, the source SN sends to each candidate SN (via the MN) disjoint lists of proposed Rel-19 Set IDs (in the S-NODE CHANGE REQUIRED and S-NODE ADDITION REQUEST messages). The candidate SN sends to the source SN (via the MN) the Rel-19 Set ID assigned to the candidate cell (in the S-NODE ADDITION REQUEST ACKNOWLEDGE and S-NODE CHANGE CONFIRM messages).

Proposal 22	Update the procedure text for XN-U Address Indication and the semantics description for the LTM DC Data Forwarding Indicator to clarify that the IE is used also in case of SN-Initiated Inter-CU SCG LTM.

Proposal 23	For SN initiated inter-CU SCG LTM, the MN sends to the target SN the SN security keys and a list of sk-counters in the CELL SWITCH NOTIFICATION message.

Proposal 24	For Inter-CU MCG LTM with SN addition, the MN sends to the target SN the SN security keys in the CELL SWITCH NOTIFICATION message.

Proposal 25	Update Cell Switch Notification definition and content to include the target PSCell ID and corresponding TCI state ID(s), and to reflect the use of the message in dual connectivity scenarios.

Proposal 26	For SN initiated inter-CU SCG LTM, there is no need for the target SN to notify the MN that UE has successfully accessed the target SN.

Proposal 27	For SN initiated inter-CU SCG LTM, the target SN can cancel an already prepared candidate PSCell sending an LTM Cancel to the MN. The MN forwards the information to the source SN with the LTM Cancel procedure.

Proposal 28	Agree on the TPs in R3-255269 that capture the above proposals.