3GPP TSG-RAN WG3 Meeting #129	R3-255271

Bengaluru, India, 25 – 29 August 2025

Agenda Item:		11.3

Source:		Ofinno

Title:	AI/ML enabled Coverage and Capacity Optimization: Cause value in the Neighbour Future Coverage Modification Notification

Document for:		Discussion and decision

# Introduction

In the RAN3 #128 meeting, Opton2 was agreed: gNB-CU directly forwards the received further CCO state of neighbor cells to gNB-DU.

However, there is cause value missing in the Neighbour Future Coverage Modification Notification.

This contribution and the accompanying TP to BL CR for 38.473 propose to resolve this issue.

# Discussion

In the RAN3 #128 meeting, Option 2 was agreed: gNB-CU directly forwards the received further CCO state of neighbor cells to gNB-DU.

This is implemented in the BL CR to 38.473 as follows:

#### 9.3.1.C	Neighbour Future Coverage Modification Notification

This IE includes a list of cells and/or SS/PBCH block indexes with the corresponding future coverage configuration selected by one or more neighbour node(s).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Neighbour Future Coverage Modification Notification List |  | 1 |  |  |
| >Neighbour Future Coverage Modification Notification Item |  | 1..< maxNeighbourCellingReport > |  |  |
| >>NR CGI | M |  | 9.3.1.12 |  |
| >>Neighbour Future Cell Coverage State | M |  | INTEGER (0..63, ...) | Value ‘0’ indicates that the cell will be inactive. Other values indicate that the cell will be active and also indicate the future coverage configuration of the concerned cell. |
| >>Neighbour Future SSB Modification Notification List |  | 0..1 |  |  |
| >>>Neighbour Future SSB Modification Notification Item |  | 1..<maxnoofSSBAreas> |  |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  |
| >>>>Neighbour Future SSB Coverage State | M |  | INTEGER (0..15, ...) | Value ‘0’ indicates that the SSB beam will be inactive. Other values indicate that the SSB beams will be active and also indicate the future coverage configuration of the concerned SSB beams. |
| >>Time for Neighbour Future Coverage Modification | O |  | INTEGER (1..60, ...) | Indicates the time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the gNB-DU relative to the time of receiving this information, in seconds. |

| Range bound | Explanation |
| --- | --- |
| maxNeighbourCellingReport | Maximum no. neighbour cells for which the future neighbour state can be reported. Value is 512. |
| maxnoofSSBAreas | Maximum numbers of SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

The agreed Option 2 is as follows:

gNB-CU directly forwards the received further CCO state of neighbor cells to gNB-DU.

A gNB-CU receives the further CCO state of neighbor cells in the following format:

| Future Coverage Modification List |  | 0..1 |  | List of cells whose coverage will be modified. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >Future Coverage Modification Item |  | 1..<maxnoofCellsinNG-RAN node> |  |  | – |  |
| >>Global NG-RAN Cell Identity | M |  | 9.2.2.27 | NG-RAN Cell Global Identifier of the cell whose coverage will be modified. | – |  |
| >>Future Cell Coverage State | M |  | INTEGER (0..63, ...) | Value ‘0’ indicates that the cell will be inactive. Other values indicates that the cell will be active and also indicates the future coverage configuration of the concerned cell. | – |  |
| >>Future SSB Coverage Modification List |  | 0..1 |  | List of SSB beams whose coverage will be modified. | – |  |
| >>>Future SSB Coverage Modification Item |  | 1..<maxnoofSSBAreas> |  |  | – |  |
| >>>>SSB index | M |  | INTEGER (0..63) | Identifier of the SSB beam whose coverage will be modified. | – |  |
| >>>>Future SSB Coverage State | M |  | INTEGER (0..15, ...) | Value ‘0’ indicates that the SSB beam will be inactive. Other values indicate that the SSB beams will be active and also Indicates the future coverage configuration of the concerned SSB beams. | – |  |
| >>Predicted Coverage Modification Cause | O |  | ENUMERATED (coverage, cell edge capacity, ...) | Indicates the reason for the predicted coverage modification in NG-RAN node1. | – |  |
| >>Time for Future Coverage State | O |  | INTEGER (1..60, …) | Indicates the time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the NG-RAN node1 relative to the time of receiving this information, in seconds. | – |  |

The structures are identical except the Predicted Coverage Modification Cause (highlighted in red) of the Neighbour Future Coverage Modification is not forwarded from the neighbour gNB to the gNB-DU.

During offline discussions, it was mentioned that the cause value in the Future Coverage Modification Notification (highlighted in red) may be reused:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Future Coverage Modification Notification List |  | 1 |  |  |
| >Future Coverage Modification Notification Item |  | 1..<maxCellingNBDU> |  |  |
| >>NR CGI | M |  | 9.3.1.12 |  |
| >>Future Cell Coverage State | M |  | INTEGER (0..63, ...) | Value ‘0’ indicates that the cell will be inactive. Other values indicate that the cell will be active and also indicate the future coverage configuration of the concerned cell. |
| >>Future SSB Modification Notification List |  | 0..1 |  |  |
| >>>Future SSB Modification Notification Item |  | 1..<maxnoofSSBAreas> |  |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  |
| >>>>Future SSB Coverage State | M |  | INTEGER (0..15, ...) | Value ‘0’ indicates that the SSB beam will be inactive. Other values indicate that the SSB beams will be active and also indicate the future coverage configuration of the concerned SSB beams. |
| >>Time for Future Coverage Modification | O |  | INTEGER (1..60, ...) | Indicates the time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the gNB-DU relative to the time of receiving this information, in seconds. |
| >>Future Coverage Modification Cause | O |  | ENUMERATED(coverage, cell edge capacity, …) |  |

We believe that such solution may be confusing.

First, currently there is no description in the BL CR for 38.473 connecting the Future Coverage Modification Cause in the Future Coverage Modification Notification to the Neighbour Future Coverage Modification Notification.

Second, how gNB-CU should interpret the Future Coverage Modification Cause if other elements in the Future Coverage Modification Notification are present and the Neighbour Future Coverage Modification Notification is present. Will the cause value relate to own cells and beams or to neighbour cells and beams or to both?

Third, the source of the Neighbour Future Coverage Modification Notification is neighbour gNB. gNB-CU just forwards the Neighbour Future Coverage Modification based on the agreed Option 2. The source of the Future Coverage Modification Notification is gNB-CU. This means that the gNB-CU can change the Future Coverage Modification Cause, which contradicts with the agreed Option 2.

We think that much cleaner solution is to add Cause for Neighbour Future Coverage Modification to the Neighbour Future Coverage Modification Notification. This will be the same value as in the Predicted Coverage Modification Cause in the Future Coverage Modification that the gNB-CU receives from a neighbour gNB.

Proposal 1: Add Cause for Neighbour Future Coverage Modification to the Neighbour Future Coverage Modification Notification.

Accompanying TP to BL CR for 38.473 proposes implementation of the Proposal 1.

# Proposals

Proposal 1: Add Cause for Neighbour Future Coverage Modification to the Neighbour Future Coverage Modification Notification.

# Reference

[1] R3-255001, RAN3#129 Meeting Agenda

[2] R3-255002, RAN3#128 Meeting Report

[3] R3-255046, (BL CR to TS 38.473) Support of enhancements on AI/ML for NG-RAN