3GPP TSG-RAN WG3 Meeting #129	R3-255282

Bengaluru, India, 25 – 29 August 2025

Agenda Item:		13.2

Source:		Ofinno

Title:	(TP for TS 38.423) Cell Switch Notification for LTM DC Scenario

Document for:		Agreement

# Introduction

In the past meetings, we have agreed that the Cell Switch Notification message can be applied to LTM DC scenario. However, the stage 3 for Cell Switch Notification message applied to dual connectivity scenario has not been reflected in the BLCR TS 38.423. This contribution contains the text proposal for this purpose.

# Text Proposal to BLCR of TS 38.423 - Cell Switch Notification for LTM DC Scenario

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 8.y.1	Cell Switch Notification

Editor’s note: Details need to be continued.

#### 8.y.1.1	General

The purpose of the Cell Switch Notification procedure is to enable the source NG-RAN node to inform the target NG-RAN node about the initiation of the cell switch command to the UE. This procedure is also used to transfer the selected TCI state from the source NG-RAN node to the target NG-RAN node. The procedure uses UE-associated signalling.

For dual connectivity, the Cell Switch Notification procedure is to enable the source S-NG-RAN node to inform the target S-NG-RAN node, via the M-NG-RAN node, about the initiation of the cell switch command to the UE as specified in TS 37.340 [8]. This procedure is also used to transfer the selected TCI state from the source S-NG-RAN node to the target S-NG-RAN node, via the M-NG-RAN node. The procedure uses UE-associated signalling.

#### 8.y.1.2	Successful Operation

Figure 8.y.1.2-1: Cell Switch Notification procedure. Successful operation.

Figure 8.y.1.2-2: Cell Switch Notification procedure for dual connectivity. Successful operation.

Handover

The source NG-RAN node initiates the procedure by sending a CELL SWITCH NOTIFICATION message.

Upon reception of the CELL SWITCH NOTIFICATION message, the target NG-RAN node shall, if supported, consider that a cell switch command was sent to the UE towards one cell of the target NG-RAN node indicated by the included Target Cell Global ID IE.

Dual Connectivity

The source S-NG-RAN node initiates the procedure by sending a CELL SWITCH NOTIFICATION message to the M-NG-RAN node. The M-NG-RAN node forwards the CELL SWITCH NOTIFICATION to the target S-NG-RAN node.

Upon reception of the CELL SWITCH NOTIFICATION message, the target S-NG-RAN node shall, if supported, consider that a cell switch command was sent to the UE towards one PSCell of the target S-NG-RAN node indicated by the included Target Cell Global ID IE.

<<<<<<<<<<<<<<<<<<<< End of first Change >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Second Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.1.x1 CELL SWITCH NOTIFICATION

This message is sent by the source NG-RAN node to inform the target NG-RAN node about the initiation of the cell switch command to the UE.

This message is also sent by the source S-NG-RAN node to the M-NG-RAN node or from the M-NG-RAN node to the target S-NG-RAN node to inform the target S-NG-RAN node about the initiation of the cell switch command to the UE.

Direction: source NG-RAN node  target NG-RAN node.

Direction: M-NG-RAN node  target S-NG-RAN node or source S-NG-RAN node  M-NG-RAN node (Dual Connectivity).

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node, at the source S-NG-RAN node, or at the M-NG-RAN node. | YES | reject |
| Target NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node, at the M-NG-RAN node, or at the target S-NG-RAN node. | YES | reject |
| Target Cell Global ID | M |  | NR CGI 9.2.2.7 | Or target PSCell ID. | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Corresponds to the TCI-StateId IE as defined in TS 38.331 [10]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Corresponds to the TCI-UL-StateId IE as defined in TS 38.331 [10]. |  |  |

<<<<<<<<<<<<<<<<<<<< End of Second Change >>>>>>>>>>>>>>>>>>>>