3GPP TSG-RAN WG3 Meeting #129	R3-255283

Bengaluru, India, 25 – 29 August 2025

Agenda Item:		13.2

Source:		Ofinno

Title:	Open Issue on Access Success for Inter-SN SCG LTM

Document for:		Discussion and Decision

# Introduction

In last RAN3 #128 meeting, the following progress has been achieved on the agenda item 13.2, i.e., support for Inter-CU LTM NR-DC:

RAN3#128:

The suggested PSCell list should be explicitly included in the SN Change Required message.

The maximum number of PSCells that can be prepared by each candidate SN is included in the SN Change Required message.

The LTM Configuration ID Mapping List IE may be included in the SN Change Required message and the SN Addition Request message.

The LTM Security Configuration List IE including a list of pair of {security key, sk-counter} is included in the SN Addition Request message and the SN Modification Request message to support subsequent inter-CU SCG LTM.

The SN Security Key IE included in the SN Addition Request message should be ignored if the procedure is triggered for the LTM.

WA: Different candidate PSCells in the same SN can have different Rel-19 set IDs. FFS on the solutions. Try to reuse the same solution for inter-CU LTM.

If the source SN has the SCG reference configuration, the source SN will provide the SCG reference configuration in the SN Change Required message and thus the MN will not request source SN to provide the SCG reference configuration.

Early data forwarding is supported in inter-CU LTM in DC.

Normal data forwarding can be initiated after the source SN decides to trigger the LTM cell switch for the UE. The exact timing of its initiation is left to implementation.

Enhance XN-U ADDRESS INDICATION message and define IE to cover Inter-CU MCG LTM case. FFS on Inter-SN LTM case.

MN uses SN modification request message to notify the Source SN that UE has successfully accessed to the target SN. FFS whether Handover Success is used from the target-SN to the MN to notify that UE has successfully accessed to the target SN.

The source SN generates the common LTM CSI resource configuration for inter-CU SCG LTM and then provides the common LTM CSI resource configuration to the MN via the SN Modification Request ACK message.

For the SSB based L1 measurement, the common CSI resource configuration may be included in the SN Change Required message and the SN Addition Request message.

The CSI Resource Configuration IE is the common CSI resource configuration, which refers to the ltm-CSI-ResourceConfig-r18 IE in the RRC spec.

The L1 Configuration IE is the L1 RS configuration per candidate PSCell, which refers to the ltm-SSB-Config-r18 IE in the RRC spec for the SSB based L1 measurement, or the ltm-NZP-CSI-RS-ResourceConfigToAddModList-r19 IE associated with the ltm-NZP-CSI-RS-ResourceSetToAddModList-r19 IE in the RRC spec for the CSI-RS based L1 measurement.

For the avoidance of simultaneous configurations of inter-CU MCG LTM and inter-CU SCG LTM, no RAN3 impact is foreseen.

FFS on whether LTM modification/cancel related procedures are needed.

The SP CSI-RS activation/(de)activation for the inter-CU LTM in DC is deprioritized in rel-19.

This contribution discusses an open issue on access success for SN initiated Inter-SN SCG LTM.

# Discussion

Fig. 1. SN initiated inter-SN SCG LTM

In last RAN3 meeting, a baseline CR for Inter-SN SCG LTM procedure was endorsed [2]. In the procedure description, there is an open issue related to Step 23 to 29.

23. Whether notify message is needed from target SN to MN is FFS

“23~29. Editor’s Note: Details for related procedures and descriptions are up to RAN3 discussion.”

Fig. 2 (SN Modification – SN-initiated without MN involvement and SRB3 is not used to configure intra-SN SCG LTM (Figure 10.3.2-6 in TS 37.340)

Fig. 2 shows the SN initiated intra-SN SCG LTM procedure, wherein the following text describes from step 8 to step 10:

- 8. The UE sends an ULInformationTransferMRDC message to the MN which includes an embedded RRCReconfigurationComplete message to the target cell.

- 9. The RRCReconfigurationComplete message is forwarded to the SN embedded in RRC Transfer message.

- 10. The UE performs the random access procedure towards the target cell, if the UE does not have valid TA of the target cell.

It can be seen from Fig. 2 that the random-access procedure happens after the RRCReconfigurationComplete message is sent and forwarded to the SN. If the MN determines the inter-SN SCG LTM procedure is successful based on receiving the RRCReconfigurationComplete message from UE, it is not sufficient. At the timing it is not clear if the random-access procedure is successful or not. If the random-access procedure fails, triggering the SN Modification procedure would be meaningless. Therefore, a notify message is necessary from target SN to MN. HANDOVER SUCCESS message can be reused instead of defining a new message.

Proposal 1: For SN-initiated inter-SN LTM procedure, a notify message is necessary from target SN to MN. HANDOVER SUCCESS message can be reused.

Proposal 2: To align with Rel-18 SN initiated intra-SN SCG LTM procedure and SN initiated subsequent CPAC in TS 37.340, change the order of step 20 and step 21/step 22 for SN initiated inter-SN SCG LTM procedure in BLCR.

On the other hand, step 23 and step 24 are currently mandatory in Fig. 1, which is not correct. The source SN may not be a candidate for a subsequent LTM. Thus, the source SN should be released. Instead of sending a SN modification request message, SN release request message should be sent to the source SN.

The following agreement that we made last meeting should be reconsidered.

- MN uses SN modification request message to notify the Source SN that UE has successfully accessed to the target SN. FFS whether Handover Success is used from the target-SN to the MN to notify that UE has successfully accessed to the target SN.

Proposal 3: Revise the agreement as follows:

- Option 1: reuse HANDOVER SUCCESS message to notify the Source SN that UE has successfully accessed to the target SN.

- Option 2: MN uses SN modification request message or SN release request message to notify the Source SN that UE has successfully accessed to the target SN.

Proposal 4: To revise step 23 / step 24 based on option 1 or 2 for SN initiated inter-SN SCG LTM procedure in BLCR.

# Proposals

Proposal 1: For SN-initiated inter-SN LTM procedure, a notify message is necessary from target SN to MN. HANDOVER SUCCESS message can be reused.

Proposal 2: To align with Rel-18 SN initiated intra-SN SCG LTM procedure and SN initiated subsequent CPAC in TS 37.340, change the order of step 20 and step 21/step 22 for SN initiated inter-SN SCG LTM procedure in BLCR.

Proposal 3: Revise the agreement as follows:

- Option 1: reuse HANDOVER SUCCESS message to notify the Source SN that UE has successfully accessed to the target SN.

- Option 2: MN uses SN modification request message or SN release request message to notify the Source SN that UE has successfully accessed to the target SN.

Proposal 4: To revise step 23 / step 24 based on option 1 or 2 for SN initiated inter-SN SCG LTM procedure in BLCR.

# Reference

[1] R3-253786/R3-253787, Summary of offline discussion on Mobility Enh

[2] R3-254006, (BL CR to 37.340) Stage 2 for inter-CU LTM in NR-DC

[3] R3-254009, (BL CR to 38.423) Xn support for inter-CU LTM

[4] R3-254010, (BL CR to 38.423 for DC) Xn support for inter-CU LTM in DC