3GPP TSG-RAN WG3 #129	          R3-255294

# Bengaluru, India, 25 – 29 August 2025

Agenda item:	15.2

Source:	Nokia, Nokia Shanghai Bell, Huawei, Ericsson, LG Electronics, Samsung

Title:	(TP to BL CR for TS 36.300) Correction on S1 Removal

Document for:	Discussion and Decision

# 1	Introduction

This contribution proposes following changes to BL CR for TS 36.300:

- add the missing Removal function in Section 19.2.1.8	S1-interface management functions

- Delete the “instance” from the “interface instance”, and correct the typo in “NG REMOVAL RESPONSE message”.

# 2	Conclusion

It is proposed to RAN3 to agree the TP in the Annex.

# TP to BL CR for TS 36.300

-----------------Start of the Changes-------------------

#### 19.2.1.0	General

The S1 interface provides the following functions:

-	E-RAB Service Management function:

-	Setup, Modify, Release.

-	Mobility Functions for UEs in ECM-CONNECTED:

-	Intra-LTE Handover;

-	Inter-3GPP-RAT Handover.

-	S1 Paging function:

-	NAS Signalling Transport function;

-	LPPa Signalling Transport function;

-	S1-interface management functions:

-	Error indication;

-	Reset;

-	Removal.

-	Network Sharing Function;

-	Roaming and Access Restriction Support function;

-	NAS Node Selection Function;

-	Initial Context Setup Function;

-	UE Context Modification Function;

-	UE Context Resume Function;

-	MME Load balancing Function;

-	Location Reporting Function;

-	PWS (which includes ETWS and CMAS) Message Transmission Function;

-	Overload function;

-	RAN Information Management Function;

-	Configuration Transfer Function;

-	S1 CDMA2000 Tunnelling function;

-	Trace function;

-	UE Radio Capability Match;

-	Retrieve UE Information Function;

-	UE Information Transfer Function;

-	Report of Secondary RAT data volumes Function;

-	UE Radio Capability ID Mapping Function.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 19.2.1.8	S1-interface management functions

The S1-interface management functions provide

-	means to ensure a defined start of S1-interface operation (reset);

-	means to handle different versions of application part implementations and protocol errors (error indication);

-	means to remove the interface between eNB and MME in a controlled manner (removal).

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 19.2.2.7.x	S1 Removal procedure

The purpose of the S1 Removal procedure is to remove the interface between the eNB and the MME in a controlled manner. The S1 Removal procedure is triggered by the eNB.

Figure 19.2.2.7.x-1: S1 Removal procedure

-	The eNB initiates the S1 Removal procedure by sending the S1 REMOVAL REQUEST message to the MME.

-	In the successful case the MME responds with the S1 REMOVAL RESPONSE message. After receiving the S1 REMOVAL RESPONSE message, the eNB shall initiate removal of the TNL association towards the MME and may remove all resources associated with that interface. The MME may then remove all resources associated with the eNB.

-	If the MME cannot accept to remove the interface with eNB, it shall respond with a S1 REMOVAL FAILURE message with an appropriate cause value.

-----------------End of the Changes-------------------