3GPP TSG-RAN WG3 Meeting #129	R3-255299

# Bengaluru, India, 25 – 29 August 2025

Agenda item:	22.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP to BL CR for TS 38.473) Discussion on the support of multi-hop Layer-2 UE-to-Network relay

Document for:	Discussion and Decision

# 1	Introduction

Last meeting agreed following aspects requiring further discussion in RAN3.

WA: RAN3 supports that the remote UE local ID uniquely identifies a multi-hop remote UE within the last relay UE.

FFS whether the same PC5 RLC channel ID can be allocated toward the parent UE and child UE.

FFS whether upon the reception of the RRCSetupRequest message, the gNB-DU needs to know that which Relay UE is the First Relay UE of the U2N Remote UE to configure lower layer configuration of Remote UE’s SRB1.

FFS whether to enhance the F1 signaling to support the multiplexing of PC5 Relay RLC channel is needed can be further discussed in next meeting.

This contribution discusses the above issues.

# 2	Discussion

For WA: RAN3 supports that the remote UE local ID uniquely identifies a multi-hop remote UE within the last relay UE, since RAN2 already agreed to reuse the single-hop relay mechanism (including the SRAP) for multi-hop relay. There is no technical issue with current WA. So we propose to turn the WA to an agreement.

Proposal 1: turn the following WA to an agreement

WA: RAN3 supports that the remote UE local ID uniquely identifies a multi-hop remote UE within the last relay UE.

For the 3rd FFS

FFS whether to enhance the F1 signaling to support the multiplexing of PC5 Relay RLC channel is needed can be further discussed in next meeting.

RAN2 agreed the RBs of U2N Remote-UEs can be multiplexed to the same PC5 RLC CH. The gNB-DU need to be aware of this multiplexing, e.g. when a new U2N Remote-UE is “added to”, or “removed from” an existing PC5 RLC CH. So the UE CONTEXT MODIFICATION REQUEST message needs to be enhanced to inform the gNB-DU when add/remove an U2N Remote UE for an existing PC5 RLC CH.

Proposal 2a: To support multiplexing of PC5 RLC CH, the F1AP UE CONTEXT MODIFICATION REQUEST message needs to be enhanced to support the add/remove an U2N Remote UE for an existing PC5 RLC CH.

The contribution ([1]) from Rapporteur can be used to finalize the F1AP impact. We suggest some modification to the behaviour text, shown in Annex section.

Proposal 2b: RAN3 consider the F1AP TP in Annex – TP to BL CR for TS 38.473.

For the 1st FFS

FFS whether the same PC5 RLC channel ID can be allocated toward the parent UE and child UE.

This FFS seems caused by the potential issue that the PC5 RLC CH ID itself is not able to uniquely identify the peer UE.  In Current F1AP,

- The PC5 RLC Channel to be Setup Item IEs includes PC5 RLC Channel ID, Remote UE Local ID, and Peer UE ID.

- The PC5 RLC Channel to be Modified Item IEs includes PC5 RLC Channel ID, Remote UE Local ID, and potentially Remote UE Local ID to Add/Delete (per above Proposal 2).

- The PC5 RLC Channel to be Released Item IEs includes PC5 RLC Channel ID and Remote UE Local ID.

During the PC5 RLC CH setup, the gNB-DU know the PC5 RLC CH ID is associated with the 1st Remote UE. Later, in case the multiplexing is supported, the gNB-DU knows the PC5 RLC CH ID is associated with other Remote UEs. By using the PC5 RLC Channel ID and Remote UE local ID, the gNB-DU is still able to identify the PC5 RLC CH to be modified/released. There is no need to only use the PC5 RLC CH ID to uniquely identify the peer UE. There is no issue to use same PC5 RLC CH ID towards the parent UE and child UE. So the FFS can be addressed by the existing F1AP, and no RAN3 impact.

Proposal 3: via existing F1AP, there is no issue to use same PC5 RLC CH ID towards the parent UE and child UE.

For the 2nd FFS

FFS whether upon the reception of the RRCSetupRequest message, the gNB-DU needs to know that which Relay UE is the First Relay UE of the U2N Remote UE to configure lower layer configuration of Remote UE’s SRB1.

The call flow for Remote UE initial access for Multi-hop Layer-2 UE-to-Network Relay is shown as below.

The gNB-DU receives the RRCSetupRequest message of the U2N Remote-UE in Step 14. Before Step 14, the PC5 RLC CH/Uu RLC CH need to be ready, e.g. either setup new one, or modify existing one. In case new PC5 RLC CH is established, the gNB-DU knows the first relay UE serving the U2N Remote UE in Step 6. In case the existing PC5 RLC CH is reused (i.e. multiplexing scenario), the gNB-DU also need to know the U2N Remote UEs sharing the PC5 RLC CH in Step 6. (This is related to the 3rd FFS discussed above). So in all cases, the gNB-DU can know the first relay UE. So there is no issue the gNB-DU can know which Relay UE is the First Relay UE of the U2N Remote UE upon the reception of RRCSetupRequest message.

Proposal 4: the gNB-DU can know which Relay UE is the First Relay UE of the U2N Remote UE upon the reception of RRCSetupRequest message.

# 3	other corrections to BL CR for TS 38.473

The BL CR for TS 38.473 has some Editor’s notes that can be removed. In addition, the terminology “U2N Relay UE” has already be defined in RAN2 BL CR. There is no need to repeat the definition in RAN3 spec. It is suggested to just reference to RAN2 spec, similar to other definitions.

Proposal 5: update the F1AP BL CR to remove the ENs, and reference to TS 38.300 for the definition for “U2N Relay UE”

TP based on ([1]) can be found in Annex – TP to BL CR for TS 38.473

# 4	Conclusion

In this contribution, we discussed the remaining issues to support multi-hop UE-to-Network Relay. Our proposals are:

Proposal 1: turn the following WA to an agreement

WA: RAN3 supports that the remote UE local ID uniquely identifies a multi-hop remote UE within the last relay UE.

Proposal 2a: To support multiplexing of PC5 RLC CH, the F1AP UE CONTEXT MODIFICATION REQUEST message needs to be enhanced to support the add/remove an U2N Remote UE for an existing PC5 RLC CH.

Proposal 2b: RAN3 consider the F1AP TP in Annex – TP to BL CR for TS 38.473.

Proposal 3: via existing F1AP, there is no issue to use same PC5 RLC CH ID towards the parent UE and child UE.

Proposal 4: the gNB-DU can know which Relay UE is the First Relay UE of the U2N Remote UE upon the reception of RRCSetupRequest message.

Proposal 5: update the F1AP BL CR to remove the ENs, and reference to TS 38.300 for the definition for “U2N Relay UE”

TP based on ([1]) can be found in Annex – TP to BL CR for TS 38.473

# References

- R3-253147, (TP for TS 38.401 and 38.473) Support of Multi-hop relay

- R3-255111, (BL CR to 38.401) Support of Multi-hop relay

- R3-255113, (BL CR to 38.473) Support of Multi-hop relay

# Annex – TP to BL CR for TS 38.473

# 3	Definitions and abbreviations

## 3.1	Definitions

elementary procedure: F1AP consists of Elementary Procedures (EPs). An Elementary Procedure is a unit of interaction between gNB-CU and gNB-DU. These Elementary Procedures are defined separately and are intended to be used to build up complete sequences in a flexible manner. If the independence between some EPs is restricted, it is described under the relevant EP description. Unless otherwise stated by the restrictions, the EPs may be invoked independently of each other as standalone procedures, which can be active in parallel. The usage of several F1AP EPs together is specified in stage 2 specifications (e.g., TS 38.470 [2]).

An EP consists of an initiating message and possibly a response message. Two kinds of EPs are used:

-	Class 1: Elementary Procedures with response (success and/or failure).

-	Class 2: Elementary Procedures without response.

For Class 1 EPs, the types of responses can be as follows:

Successful:

-	A signalling message explicitly indicates that the elementary procedure successfully completed with the receipt of the response.

Unsuccessful:

-	A signalling message explicitly indicates that the EP failed.

-	On time supervision expiry (i.e., absence of expected response).

Successful and Unsuccessful:

-	One signalling message reports both successful and unsuccessful outcome for the different included requests. The response message used is the one defined for successful outcome.

Class 2 EPs are considered always successful.

BH RLC channel: as defined in TS 38.300 [6].

CG-SDT-CS-RNTI: as defined in TS 38.300 [6].

Child UE: as defined in TS 38.300 [6].

Complete candidate configuration: one type of a candidate configuration as defined in TS 38.331 [8].

Conditional handover: as defined in TS 38.300 [6].

Conditional PSCell Addition: as defined in TS 37.340 [7].

Conditional PSCell Change: as defined in TS 37.340 [7].

DAPS Handover: as defined in TS 38.300 [6].

EN-DC operation: Used in this specification when the F1AP is applied for gNB-CU and gNB-DU in E-UTRAN.

F1-terminating IAB-donor: as defined in TS 38.401 [4].

First U2N Relay UE: as defined in TS 38.300 [6].

gNB: as defined in TS 38.300 [6].

gNB-CU: as defined in TS 38.401 [4].

gNB-CU UE F1AP ID: as defined in TS 38.401 [4].

gNB-DU: as defined in TS 38.401 [4].

gNB-DU UE F1AP ID: as defined in TS 38.401 [4].

en-gNB: as defined in TS 37.340 [7].

IAB-DU: as defined in TS 38.300 [6].

IAB-MT: as defined in TS 38.300 [6].

IAB-donor: as defined in TS 38.300 [6].

IAB-donor-CU: as defined in TS 38.401 [4].

IAB-donor-DU: as defined in TS 38.401 [4].

IAB-node: as defined in TS 38.300 [6].

Intermediate U2N Relay UE: as defined in TS 38.300 [6].

Last U2N Relay UE: as defined in TS 38.300 [6].

MBS-associated signalling: When F1AP messages associated to one MBS session uses the MBS-associated logical F1-connection for association of the message to the MBS session in gNB-DU and gNB-CU.

MBS-associated logical F1-connection: The MBS-associated logical F1-connection uses the identities GNB-CU MBS F1AP ID and GNB-DU MBS F1AP ID according to the definition in TS 38.401 [4]. For a received MBS-associated F1AP message the gNB-CU identifies the associated MBS session based on the GNB-CU MBS F1AP ID IE and the gNB-DU identifies the associated MBS session based on the GNB-DU MBS F1AP ID IE.

MBS Session context in a gNB-DU: as defined in TS 38.401 [4].

MBS session resource: as defined in TS 38.401 [4].

Mobile IAB-DU: as defined in TS 38.300 [6].

Mobile IAB-MT: as defined in TS 38.300 [6].

Mobile IAB-node: as defined in TS 38.300 [6].

MP Relay UE: as defined in TS 38.300 [6].

MP Remote UE: as defined in TS 38.300 [6].

Multi-path: as defined in TS 38.300 [6].

Multicast F1-U Context: as defined in TS 38.401 [4].

Other SI: as defined in TS 38.300 [6].

Parent UE: as defined in TS 38.300 [6].

PC5 Relay RLC channel: as defined in TS 38.300 [6].

Public network integrated NPN: as defined in TS 23.501 [21].

RRC-terminating IAB-donor: as defined in TS 38.401 [4].

SRAP: Sidelink relay adaptation protocol, as defined in TS 38.300 [6].

Stand-alone Non-Public Network: as defined in TS 23.501 [21].

UE-associated logical F1-connection: The UE-associated logical F1-connection uses the identities GNB-CU UE F1AP ID and GNB-DU UE F1AP ID according to the definition in TS 38.401 [4]. For a received UE associated F1AP message the gNB-CU identifies the associated UE based on the GNB-CU UE F1AP ID IE and the gNB-DU identifies the associated UE based on the GNB-DU UE F1AP ID IE. The UE-associated logical F1-connection may exist before the F1 UE context is setup in gNB-DU.

UE-associated signalling: When F1AP messages associated to one UE uses the UE-associated logical F1-connection for association of the message to the UE in gNB-DU and gNB-CU.

U2N Relay UE: as defined in TS 38.300 [6].

U2N Remote UE: a UE that communicates with the network via a U2N Relay UE.

U2U Relay UE: as defined in TS 38.300 [6].

U2U Remote UE: as defined in TS 38.300 [6].

Uu Relay RLC channel: as defined in TS 38.300 [6].

[snip]

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

[Unchanged text skipped]

If the musim-CandidateBandList IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it for temporary capability restriction.

If the DL LBT Failure Information Request IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider that the gNB-CU requests collection of DL LBT failure information for the analysis of the MRO events of the UE specified in TS 38.300 [6], , and act as specified in TS 38.401 [4].

If the Ranging and Sidelink Positioning Service Information IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, update its service information for the UE accordingly. If the Ranging and Sidelink Positioning Authorized IE within the Ranging and Sidelink Positioning Service Information IE is set to "not authorized", the gNB-DU shall, if supported, initiate actions to ensure that the UE is no longer accessing the Ranging and Sidelink Positioning service.

If the Remote UE Local ID To Add IE is contained within the PC5 RLC Channel To Be Modified List IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, store the information and consider the PC5 RLC Channel is also used for the indicated U2N Remote UE. If the Remote UE Local ID To Delete IE is contained within the PC5 RLC Channel To Be Modified List IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, update the information and consider the PC5 RLC Channel is not used for the indicated U2N Remote UE.

Interaction with UE Inactivity Notification procedure

If the SDT Volume Threshold IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use the information during an SDT transaction to inform the gNB-CU via the UE INACTIVITY NOTIFICATION message as specified in TS 38.401 [4].

Interaction with UE Context Setup or UE Context Modification (gNB-CU initiated) procedures

If the UE CONTEXT MODIFICATION REQUEST message is sent for a UE context set up for S-CPAC and contains the Transmission Action Indicator IE set to "stop", the gNB-DU shall, if supported, reset the UE context for the included SpCell ID IE, prepare for subsequent CPAC. The gNB-DU shall include the SpCell ID IE as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message.