3GPP TSG-RAN WG3 Meeting #129	R3-255300

# Bengaluru, India, 25 – 29 August 2025

Agenda item:	22.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP to BL CR for TS 38.401) update for service continuity

Document for:	Discussion and Decision

# 1	Introduction

This contribution discusses the updates to Stage-2 TP for service continuity.

# 2	Discussion

For Scenario C/D, Last RAN2 meeting agreed:

Agreements:

For scenario C, the legacy path-switching command and procedure are reused for the Remote UE and the target first Relay UE. Upon receiving the path-switching command, the Remote UE performs the legacy UE behavior toward the target first Relay UE. The (first)intermediate/last Relay UE belonging to the target path should be in the RRC-connected state.

For scenario D, the legacy path-switching command and procedure are reused for the Remote UE and the first Relay UE in the target path. Upon receiving the path-switching command, the Remote UE performs legacy UE behavior toward the first Relay UE on the target path. The (first)intermediate/last Relay UE belonging to the target path should be in the RRC-connected state.

RAN2 agreed to reuse the existing procedure to support the service continuity. The main difference is the RRC state of all L2 U2N Relay UEs of the target path are in the RRC_CONNECTED when target path is multi-hop, vs. the target U2N Relay UE can be in any RRC state when target path is single-hop.  RAN3 Stage-2 TP need to be updated.

Proposal 1: Update TS 38.401 TP to clarify the existing procedure can be reused for multi-hop and the RRC state of all L2 U2N Relay UEs of the target path are in the RRC_CONNECTED.

Other RAN2 agreements:

Agreements:

The target last relay UE can be the same as the source single-hop U2N relay UE, but the target first/intermediate relay UE cannot be the source single-hop U2N relay UE.  No spec impact specific to this decision is expected (rely on the baseline procedure).

The gNB is expected to avoid triggering mobility to a path where the first/intermediate relay UE is the same as the source relay UE.

The RAN2 agreements have no impact to RAN3 specification.

# 3	Conclusion

In this contribution, we discussed the Stage-2 Update for Service Continuity. Our proposals are:

Proposal 1: Update TS 38.401 TP to clarify the existing procedure can be reused for multi-hop and the RRC state of all L2 U2N Relay UEs of the target path are in the RRC_CONNECTED.

# TP to BL CR for TS 38.401

-----------------Start of Change-------------------

### 8.19.4	Service Continuity for L2 U2N relay

#### 8.19.4.1	Inter-gNB-DU switch from direct to indirect path

The signalling flow for U2N Remote UE switch from direct to indirect path with gNB-DU change is shown in Figure 8.19.4.1-1. If the target path is multi-hop, it is assumed that all L2 U2N Relay UEs along the target path are in the RRC_CONNECTED. However, if the target path is a single-hop, the target U2N Relay UE can be in any RRC state.

Figure 8.19.4.1-1: U2N Remote UE Direct-to-indirect Path Switch with gNB-DU change procedure

1.	The Uu measurement configuration and measurement report signalling is performed between U2N Remote UE and gNB-CU to evaluate both relay link measurement and Uu link measurement. The U2N Remote UE may report one or multiple candidate U2N Relay UE(s) and Uu measurement results after it measures/discovers the candidate U2N Relay UE(s).

2.	The gNB-CU decides to switch the U2N Remote UE to a target U2N Relay UE under a different gNB-DU (i.e., target gNB-DU).

3.	The reconfiguration to target U2N Relay UE is performed among U2N Relay UE, the target gNB-DU and gNB-CU, if the U2N Relay UE is in RRC_CONNECTED state. The gNB-CU sends an RRCReconfiguration message to the target U2N Relay UE. If the target Relay UE is in RRC_IDLE/INACTIVE state, this step is skipped and the configuration to the target U2N Relay UE is performed in Step 10.

4.	gNB-CU sends the UE CONTEXT SETUP REQUEST message for the U2N Remote UE to the target gNB-DU, which contains the path switch configuration at least.

5.	gNB-DU responds with the UE CONTEXT SETUP RESPONSE message to gNB-CU.

6.	gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message by including the RRCReconfiguration message to the source gNB-DU. The contents in the RRCReconfiguration message may include at least path switch configuration, PC5 Relay RLC channel configuration for relaying traffic, bearer mapping configuration and the associated radio bearer(s).

7.	The source gNB-DU sends the RRCReconfiguration message to the U2N Remote UE. The U2N Remote UE stops UP and CP transmission over Uu after reception of RRCReconfiguration message from the gNB.

8.	The source gNB-DU sends the UE CONTEXT MODIFICATION RESPONSE message to the gNB-CU.

9.	The U2N Remote UE establishes PC5 connection with target U2N Relay UE.

10.	The U2N Remote UE completes the path switch procedure by sending the RRCReconfigurationComplete message to the target gNB-DU via the target U2N Relay UE. In case the U2N relay UE is in RRC_IDLE/ INACTIVE state, when the U2N relay UE receives the RRCReconfigurationComplete message, the reception of the RRCReconfigurationComplete message will first trigger RRC setup/resume procedure for the U2N relay UE to enter RRC_CONNECTED state.

11.	The target gNB-DU sends the UL RRC MESSAGE TRANSFER message to gNB-CU by including the RRCReconfigurationComplete message.

12.	The gNB-CU sends an UE CONTEXT RELEASE COMMAND message to the source gNB-DU.

13.	The source gNB-DU releases the UE context and responds the gNB-CU with an UE CONTEXT RELEASE COMPLETE message.

#### 8.19.4.2	Intra-gNB-DU switch from direct to indirect path

The signalling flow for U2N Remote UE switch from direct to indirect path without gNB-DU change is shown in Figure 8.19.4.2-1. If the target path is multi-hop, it is assumed that all L2 U2N Relay UEs along the target path are in the RRC_CONNECTED. However, if the target path is a single-hop, the target U2N Relay UE can be in any RRC state.

Figure 8.19.4.2-1: U2N Remote UE Direct-to-indirect Path Switch without gNB-DU change procedure

1.	The Uu measurement configuration and measurement report signalling is performed between U2N Remote UE and gNB-CU to evaluate both relay link measurement and Uu link measurement. The U2N Remote UE may report one or multiple candidate U2N Relay UE(s) and Uu measurement results after it measures/discovers the candidate U2N Relay UE(s).

2.	The gNB-CU decides to switch the U2N Remote UE to a target U2N Relay UE under the same gNB-DU.

3.	The reconfiguration to target U2N Relay UE is performed among U2N Relay UE, gNB-DU and gNB-CU if U2N Relay UE is in RRC_CONNECTED state. The gNB-CU sends an RRCReconfiguration message to the target U2N Relay UE. If the target Relay UE is in RRC_IDLE/INACTIVE state, this step is skipped and the configuration to the target U2N Relay UE is performed in Step 9.

4.	gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message for the U2N Remote UE to gNB-DU, which contains the path switch configuration at least. The F1-U packets of the U2N Remote UE can be continuously transmitted via previous tunnels if there is no tunnel update in this step.

5.	gNB-DU responds with the UE CONTEXT MODIFICATION RESPONSE message to gNB-CU.

6.	gNB-CU sends the DL RRC MESSAGE TRANSFER message by including the RRCReconfiguration message to gNB-DU. The contents in the RRCReconfiguration message may include at least path switch configuration, PC5 Relay RLC channel configuration for relaying traffic, bearer mapping configuration and the associated radio bearer(s).

7.	gNB-DU sends the RRCReconfiguration message to the U2N Remote UE. The U2N Remote UE stops UP and CP transmission over Uu after reception of RRCReconfiguration message from the gNB.

8.	The U2N Remote UE establishes PC5 connection with target U2N Relay UE.

9.	The U2N Remote UE completes the path switch procedure by sending the RRCReconfigurationComplete message to the gNB-DU via the target U2N Relay UE. In case the U2N Relay UE is in RRC_IDLE/INACTIVE state when receiving the RRCReconfigurationComplete message, the reception of the RRCReconfigurationComplete message will first trigger RRC setup/resume procedure for the U2N relay UE to enter RRC_CONNECTED state.

10.	The gNB-DU sends the UL RRC MESSAGE TRANSFER message to gNB-CU by including the RRCReconfigurationComplete message.

-----------------End of Change-------------------