3GPP TSG-RAN3 WG3 Meeting #129	R3-255314

Bengaluru, India, 25 – 29 August 2025

Agenda Item:	16.2

Source: 	Qualcomm Incorporated

Title: 	Open issues on Inventory and Command procedures

Document for:	Discussion/Decision

# Background

In this paper, we provide our views on some of the open issues to support Inventory and Command procedures for Topology 1 of A-IoT.

# Discussion

## Command Type

It is FFS to introduce Command Type (write, read, disable) in Command Request Transfer.

It was agreed in the last RAN3 meeting to include the “Expected Command Response size” in Command Assistance Information IE.

With the knowledge of Expected Command Response size, the gNB serving the BS Reader can do appropriate resource allocation. For example, gNB may schedule more resources (or a larger TB) in case of Read Response and smaller number of resources (or a smaller TB) for Write Response (which is just an ACK or NAK).

We therefore think that there is no need for AIOTF to explicitly indicate the Command Type (read, write or disable) in the Command Request and resource allocation can be done based on Expected Command Response Size. Also, the Command is an end-to-end protected information (via A-IoT NAS) and there is no need to expose this to the gNB.

With the knowledge of Expected Command Response size, the gNB serving the BS Reader can implictly know the Command Type and do appropriate resource allocation for Command Responses (e.g., larger TB for Read Response or smaller TB for Write Response).

A-IoT NAS Command Request is an end-to-end protected message and it is not secure to expose details about the Command Request (e.g., Command Type) to the gNB.

There is no need to explicitly include the Command Type (read, write, disable) in Command Assistance Information

## Inventory “all devices”

It seems the current version of SA2 TS doesn’t include the inventory "all devices” scenario (i.e., including neither Device ID or filter criteria) in Inventory Request from AIOTF to gNB.

The AIoT Identification Information to be provided to NG-RAN can include Filtering Information, as defined in clause 5.8, or a single AIoT Device Identifier. ==> This doesn't mention the scenario of omitting both

7.2.2       Naiotf_AIoT_Inventory service operation

Service operation name: Naiotf_AIoT_Inventory.

Description: The NF consumer requests an inventory operation for one or multiple AIoT Device(s).

Inputs, Required:

1)   AF ID.

2)   At least one of the following parameters are included:

-     Target Area information for the inventory operation.

-     Information about the target AIoT Device(s):

-    either the AIoT Device ID(s) or the filtering information (see clause 5.8) for multiple target AIoT Devices.

RAN3 has a placeholder for the “inventory all devices” in the running BL CR assuming SA2 will support this and RAN2 also made Paging ID optional considering this scenario. The RAN WID also mentions this "no ID" scenario. Some companies also raised some security concerns with supporting this “inventory all” use case.

So, it’s better if RAN3 send an LS to SA2/SA3 to check this misalignment with SA2 TS and whether “inventory all” scenario is supported or not.

Latest version of SA2 TS doesn’t include the “inventory all” use case i.e., the scenario where neither the A-IoT device ID nor the filtering information is present.

There were some security concerns raised in SA3 on supporting this “inventory all” use case

Send an LS to SA2 and SA3 to check whether the “inventory all” scenario is to be supported and highlight the missing support in SA2 TS.

# Summary

Based on the discussion above, we have following observations and proposals:

2.1 Command Type

Observation 1:	With the knowledge of Expected Command Response size, the gNB serving the BS Reader can implictly know the Command Type and do appropriate resource allocation for Command Responses (e.g., larger TB for Read Response or smaller TB for Write Response).

Observation 2:	A-IoT NAS Command Request is an end-to-end protected message and it is not secure to expose details about the Command Request (e.g., Command Type) to the gNB.

Proposal 1:	There is no need to explicitly include the Command Type (read, write, disable) in Command Assistance Information

2.2 Inventory “all devices”

Observation 3:	Latest version of SA2 TS doesn’t include the “inventory all” use case i.e., the scenario where neither the A-IoT device ID nor the filtering information is present.

Observation 4:	There were some security concerns raised in SA3 on supporting this “inventory all” use case

Proposal 2:	Send an LS to SA2 and SA3 to check whether the “inventory all” scenario is to be supported and highlight the missing support in SA2 TS.