3GPP TSG-RAN3 WG3 Meeting #129	R3-255315

Bengaluru, India, 25 – 29 August 2025

Agenda Item:	10.2

Source: 	Qualcomm Incorporated

Title: 	MRO enhancements for LTM

Document for:	Discussion/Decision

# Background

In this paper, we continue to discuss MRO enhancements for LTM based on the open issues and agreements last meeting.

# Discussion

## Background

In case of a RACH-less cell switch failure followed by successful reconnection/re-establishment/recovery in a different beam in the same target cell, a network-based solution was agreed.

And the following was captured in the BL CR to TS 38.401 for the network-based solution:

The target gNB-DU identifies that the UE successfully performs a RACH-based access with reconnecting/re-establishing/recovering to the same target cell but a beam different from the target beam for the LTM cell switch execution.

The target gNB-DU may send the reconnected/re-established/recovery beam information and the gNB-CU UE F1AP ID to the source gNB-DU via the gNB-CU.

The source gNB-DU performs analysis to detect RACH-less LTM failure caused by wrong beam selection.

Also, the following was agreed and captured in 38.473:

Target gNB-DU indicates the reconnect/re-establish/recovery beam information and gNB-CU UE F1AP ID to gNB-CU via F1: DU-CU ACCESS AND MOBILITY INDICATION

gNB-CU indicates the reconnect/re-establish/recovery beam information and Source gNB-DU UE F1AP ID to source gNB-DU via F1: ACCESS AND MOBILITY INDICATION

There are 3 sub-cases captured in the above text in 38.401 and this needs further clarification.

## Reconnection scenario

Post a RACH-less LTM cell switch failure, the UE may go to RRC_IDLE in case UE doesn’t find any suitable cell for attempting LTM recovery/re-establishment for a certain time.

Later upon finding a suitable cell, the UE may attempt connection setup (“reconnection”) from RRC_IDLE and in this case as well, the (new) gNB-DU in which UE attempts reconnection will not be able to identify the UE.

Hence, the target gNB-DU will not be able to include the gNB-CU UE F1AP ID when sending the recovery beam info in F1: DU-CU ACCESS AND MOBILITY INDICATION.

Such a scenario where UE never finds any suitable cell post LTM cell switch failure (therefore it transitions to RRC_IDLE) and finding the same target cell upon reconnection is quite rare.

Also, in case the UE goes to RRC_IDLE due to re-establishment failure and later performs re-connection, UE would have encountered two failures – (1) LTM cell switch failure and (2) re-establishment failure.

Even existing MRO scenarios don’t consider this double failure scenario (e.g., L3 HO failure followed by re-establishment failure is not considered). Also, the current RLF Report only includes details of the latest RLF and not multiple RLFs. Hence it is proposed to not consider this reconnection scenario in MRO for LTM.

Post a RACH-less LTM cell switch failure, a UE may transition to RRC_IDLE upon re-establishment/recovery failure or upon not finding any suitable cell for a long time.

The chances of UE reconnecting in the same target cell (in a different beam) after UE transitions to RRC_IDLE are quite rare.

No specific solutions for double failure scenarios (e.g., CHO execution failure followed by CHO recovery failure or reestablishment failure) was specified as part of MRO for L3 HO.

There is no need to consider the “reconnection scenario” (i.e., RACH-less LTM cell switch failure followed by successful reconnection from RRC_IDLE in the same target cell but in a different beam) for MRO for LTM.

Remove any mention of “reconnected beam” from stage-2 and stage-3 BL CRs.

## Recovery scenario

Suppose a RACH-less LTM cell switch fails, and the UE is configured to attempt LTM recovery. UE tries to find another suitable cell and in case the selected cell is also an LTM candidate, the UE performs RACH-based LTM recovery.

The following was agreed in RAN2#127:

For recovery of RLF / reconfiguration with sync failure via LTM, the UE performs CBRA to the selected PCell

When UE performs RACH-based LTM recovery to a candidate LTM cell, UE uses CBRA (and not CFRA)

Based on the above agreement, UE performs RACH-based LTM recovery to a candidate LTM cell via CBRA (and not via CFRA). In case of CBRA, the target gNB-DU can’t uniquely identify the UE (e.g., based on the preamble UE uses for sending Msg1). Hence target gNB-DU won’t not be able to include the gNB-CU UE F1AP ID when sending the recovery beam info in F1: DU-CU ACCESS AND MOBILITY INDICATION.

When RACH-based LTM recovery via CBRA is performed, the target gNB-DU can’t uniquely identify the UE (e.g., based on the preamble UE uses for sending Msg1). Hence target gNB-DU won’t not be able to include the gNB-CU UE F1AP ID when sending the recovery beam info in F1: DU-CU ACCESS AND MOBILITY INDICATION

Therefore, we propose to not deprioritize the “LTM recovery” scenario as well as the current network-based solution wouldn’t work.

Deprioritize the “LTM recovery scenario” as well (i.e., RACH-less LTM cell switch failure followed by successful LTM recovery in the same target cell but in a different beam) and remove it from stage-2/stage-3 BL CRs.

## Reestablishment scenario

Suppose a RACH-less LTM cell switch fails, and the UE is not configured to attempt LTM recovery, the UE attempts to find a suitable cell and performs re-establishment procedure. Before re-establishment, UE releases spCellConfig (which may include the RACH-ConfigDedicated). Hence, UE can only performs CBRA during re-establishment.

In case of re-establishment:

- Target gNB-DU forwards the RRCReestablishmentRequest (which includes the old C-RNTI and old PCI) received from UE to the gNB-CU via INITIAL UL RRC MESSAGE TRANSFER over F1AP, including the new gNB-DU UE F1AP ID.

- gNB-CU identifies this UE (based on the old C-RNTI) and sends the RRCReestablishment along with the previously established gNB-CU UE F1AP ID in the pair (gNB-CU UE F1AP ID, new gNB-DU UE F1AP ID)

- Target gNB-DU can then send the re-established beam ID and the received gNB-CU UE F1AP in DU-CU ACCESS AND MOBILITY INDICATION after RRCReestablishmentComplete

In case of re-establishment, the gNB-CU can identify the UE (based on the source C-RNTI included in RRCReestablishmentRequest) and can allocate the same gNB-CU UE F1AP ID when UE re-establishes via a new target gNB-DU. Thereby, the target gNB-DU can identify the UE and include the gNB-CU UE F1AP ID along with the re-established beam information.

Confirm to support the “re-established beam” scenario i.e., RACH-less LTM cell switch failure followed by successful RRC reestablishment in the same target cell but via a different beam.

# Summary

2.1	Background

2.2	Reconnection scenario

Observation 1.	Post a RACH-less LTM cell switch failure, a UE may transition to RRC_IDLE upon re-establishment/recovery failure or upon not finding any suitable cell for a long time.

Observation 2.	The chances of UE reconnecting in the same target cell (in a different beam) after UE transitions to RRC_IDLE are quite rare.

Observation 3.	No specific solutions for double failure scenarios (e.g., CHO execution failure followed by CHO recovery failure or reestablishment failure) was specified as part of MRO for L3 HO.

Proposal 1:	There is no need to consider the “reconnection scenario” (i.e., RACH-less LTM cell switch failure followed by successful reconnection from RRC_IDLE in the same target cell but in a different beam) for MRO for LTM.

Proposal 2:	Remove any mention of “reconnected beam” from stage-2 and stage-3 BL CRs.

2.3	Recovery scenario

Observation 4.	When UE performs RACH-based LTM recovery to a candidate LTM cell, UE uses CBRA (and not CFRA)

Observation 5.	When RACH-based LTM recovery via CBRA is performed, the target gNB-DU can’t uniquely identify the UE (e.g., based on the preamble UE uses for sending Msg1). Hence target gNB-DU won’t not be able to include the gNB-CU UE F1AP ID when sending the recovery beam info in F1: DU-CU ACCESS AND MOBILITY INDICATION

Proposal 3:	Deprioritize the “LTM recovery scenario” as well (i.e., RACH-less LTM cell switch failure followed by successful LTM recovery in the same target cell but in a different beam) and remove it from stage-2/stage-3 BL CRs.

2.4	Reestablishment scenario

Observation 6.	In case of re-establishment, the gNB-CU can identify the UE (based on the source C-RNTI included in RRCReestablishmentRequest) and can allocate the same gNB-CU UE F1AP ID when UE re-establishes via a new target gNB-DU. Thereby, the target gNB-DU can identify the UE and include the gNB-CU UE F1AP ID along with the re-established beam information.

Proposal 4:	Confirm to support the “re-established beam” scenario i.e., RACH-less LTM cell switch failure followed by successful RRC reestablishment in the same target cell but via a different beam.