3GPP TSG-RAN3 WG3 Meeting #129	R3-255316

Bengaluru, India, 25 – 29 August 2025

Agenda item:	10.3.1

Source:	Qualcomm Incorporated

Title:	MDT enhancements for NTN

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss remaining issues for SON/MDT enhancements for NTN based on agreements and open issues in the previous meetings.

# Discussion

  - Where to include the Geographical Area Scope in Logged MDT Configuration over NGAP?

For NTN, RAN3 agreed to introduce a geographical area scope for logged MDT.

However, there is an open issue on where to include this geographical area in MDT Configuration-NR IE over NGAP, and the following options were briefly discussed last meeting:

- Option 1: Inside the Area Scope of MDT IE as a new CHOICE

- Option 2: Outside the Area Scope of MDT IE

Here is our analysis:

|  | Impacts to RAN3 | Impacts to RAN2 |
| --- | --- | --- |
| Option 1: Inside the Area Scope of MDT IE as a new CHOICE | Simpler from RAN3 perspective as we don’t need to consider co-existence with existing legacy area scopes | Simpler from UE perspective as UE doesn’t need to check 2 area scopes (i.e., the legacy area scope and geographical area scope) |
| Option 2: Outside the Area Scope of MDT IE | Complex from RAN3 perspective e.g., e.g., RAN3 needs to consider how this geographical area scope co-exists with cell based/TA based/PLMN wide/TAI based/PNI-NPN based/SNPN based area scopes | Complex from UE perspective (as UE need to check 2 area scopes i.e., the legacy area scope and geographical area scope) |

RAN2 is also discussing the UE behaviour when there is no UE location available e.g., whether the UE should pause geographical area based logged MDT or “fall back” to legacy area scope (and for this both legacy area scope and geographical area scope needs to be configured together). In our view, UE can simply pause geographical area based logged MDT if there is no UE location available and there is no need to configure the legacy area scope as “fall back”.

Regardless of RAN2’s decision on the UE behaviour when no UE location is available, we think RAN3 can decide where to include the geographical area scope considering the analysis above. And based on that, we propose to go ahead with Option 1 considering its simplicity.

Including the geographical area scope outside the Area Scope of MDT would mean more complexity as

  - RAN3 would need to discuss the co-existence of this new geographical area scope with existing area scopes (e.g., cell based, TA based, PLMN wide, TAI based, PNI-NPN based, SNPN based)

  - UE need to check 2 area scopes i.e., the legacy area scope and geographical area scope

Add the geographical area scope inside the Area Scope of MDT within MDT Configuration-NR as a new CHOICE (i.e., there is no need to add the geographical area scope outside the Area Scope of MDT)

  - Location-based event trigger for logged MDT

NTN supports location-based measurements in RRC_IDLE and RRC_INACTIVE. Based on this, the UE will measure in RRC_IDLE and RRC_INACTIVE if it has moved a certain distance larger than a threshold and if the radio signal strength is above a certain threshold.

Current event triggers in logged MDT are only for Out-of-coverage or when signal strength is below a threshold. We propose to introduce a location-based event trigger for logged MDT.

Introduce a new “location-based” event trigger to collect logged MDT only when the UE has moved a certain distance w.r.t. a reference location and if the signal strength is above a certain threshold.

# Conclusion

Observation 1.	Including the geographical area scope outside the Area Scope of MDT would mean more complexity as

 RAN3 would need to discuss the co-existence of this new geographical area scope with existing area scopes (e.g., cell based, TA based, PLMN wide, TAI based, PNI-NPN based, SNPN based)

 UE need to check 2 area scopes i.e., the legacy area scope and geographical area scope

Proposal 1:	Add the geographical area scope inside the Area Scope of MDT within MDT Configuration-NR as a new CHOICE (i.e., there is no need to add the geographical area scope outside the Area Scope of MDT)

Proposal 2:	Introduce a new “location-based” event trigger to collect logged MDT only when the UE has moved a certain distance w.r.t. a reference location and if the signal strength is above a certain threshold.