3GPP TSG-RAN3 Meeting #129	R3-255318

Bengaluru, India, Aug 25 - 29, 2025

Agenda Item:	11.4

Source: 	Qualcomm Incorporated

Title: 	Discussion on UE Performance Reporting for Split Architecture

Document for:	Discussion/Decision

# Background

In RAN3#128 meeting, the UE performance metrics issues were all addressed with good progress. UE throughput was agreed to be at RLC layer and UE Packet loss and Packet Delay was agreed to be in PDCP layer. The remaining open issues on Packet Loss and F1 Delay Monitoring is addressed in this paper.

RAN3#128 Split architecture support for Rel-18 use cases Introduce new IE to include DL and UL Packet Loss Rate at PDCP Level in R19 refer to SA5 spec DL/UL UE Throughput Measurement will not be supported in split architecture in R19. Introduce the start and stop indication over F1 for Delay measurement. Introduce Data Collection ID in UE associated messages over E1 Assign new BL CR for TS 38.425 Nokia Turn WA to agreement: Using the Legacy procedure, i.e., resource status initiation procedure & resource status report procedure to request and report measured EC over F1 interface.

# Discussion

## UE Packet Loss

SA5 provided clarification on UE Packet Loss definition in their LS response as below -

•	UL PDCP SDU Loss Rate – TS 28.552 clause 5.1.3.1.1

•	DL PDCP SDU Drop Rate in gNB-CU-UP – TS 28.552 clause 5.1.3.2.1

28.552 defines Packet Loss as a function of NRCell. It is not available at per UE level.

TS 28.552 Clause 5.1.3.1.1 c) This measurement is obtained as: 1000000* Number of missing UL PDCP sequence numbers, representing packets that are not delivered to higher layers, of a data radio bearer, divided by Total number of UL PDCP sequence numbers (also including missing sequence numbers) of a bearer, starting from the sequence number of the first packet delivered by UE PDCP to gNB-CU-UP until the sequence number of the last packet. If transmission of a packet might continue in another cell, it shall not be included in this count. Separate counters are optionally maintained for mapped 5QI (or QCI for EN-DC) and per supported S-NSSAI. d) Each measurement is an integer value representing the loss rate multiplied by 1E6. The number of measurements is equal to one. If the optional QoS and S-NSSAI level measurements are performed, the measurements are equal to the number of mapped 5QIs and the number of supported S-NSSAIs. e) The measurement name has the form DRB.PacketLossRateUl and optionally DRB.PacketLossRateUl.QOS where QOS identifies the target quality of service class, and DRB.PacketLossRateUl.SNSSAI where SNSSAI identifies the S-NSSAI.

For UE performance metrics as AI/ML feedback, UE Packet Loss is needed at per UE aggregation level. RAN3 can wait for SA5 to respond on the UE Packet Loss at per UE level. For Rel19, RAN3 can agree to remove the Editor’s Notes and correct the semantics based on response from SA5.

Packet Loss definition in TS28.552 in Clause 5.1.3.1.1 and 5.1.3.2.1 is not available at per UE level.

RAN3 to remove Editor’s Notes for Packet Loss in TS 37.483 and correct the semantics of Packet Loss in Release 20 based on response from SA5.

## UE Packet Delay Monitoring

In the last RAN3 meeting, there was a discussion whether to introduce a new delay monitoring IE over F1 interface. Since the QoS Monitoring framework is reused for UE packet delay for AI/ML performance monitoring, the delay monitoring is same for QoS monitoring and UE performance monitoring. As the UE packet delay may be requested for certain periodicities, it is better to propagate the same over F1 interface.

Periodicity of UE Performance reporting over Xn interface should be propagated over F1 and E1 interface for UE throughput, UE Packet Loss and UE Packet delay.

Introduce a new Packet Delay monitoring IE over F1 interface for UE performance monitoring with periodicity same as Reporting Periodicity for Data Collection.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	Packet Loss definition in TS28.552 in Clause 5.1.3.1.1 and 5.1.3.2.1 is not available at per UE level.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	RAN3 to remove Editor’s Notes for Packet Loss in TS 37.483 and correct the semantics of Packet Loss in Release 20 based on response from SA5.

Proposal 2:	Periodicity of UE Performance reporting over Xn interface should be propagated over F1 and E1 interface for UE throughput, UE Packet Loss and UE Packet delay.

Proposal 3:	Introduce a new Packet Delay monitoring IE over F1 interface for UE performance monitoring with periodicity same as Reporting Periodicity for Data Collection.