3GPP TSG-RAN WG3 Meeting #129	R3-255319

Bangalore, India, August 25 - 29, 2025

Title:	Support Cell Level Measurements signaling from CU to DU for L3 measurement based R18 LTM

Agenda Item:	9.2

Source:	Qualcomm Inc, Apple Inc, NTT DOCOMO, AT&T, Boost Mobile Networks, JIO Platforms, Charter Communications, Rakuten, CEWiT, Vivo, Tejas Networks, Xiaomi.

Document for:	Discussion/Agreement

# 1	Introduction

In RAN3#125bis, RAN3 received an LS [1] from RAN2 about support for L3 measurement based LTM, and is given shown below:

1. Overall Description: In the late phase of Rel-18 LTM definition, it was decided, that LTM can be triggered based on Layer 3 measurements by the UE. Consequently, Layer 3 measurements are delivered via RRC to the CU and the CU needs to request the DU to send MAC CE to trigger LTM execution. During RAN2 discussion, it was recognised, that there is no message over F1 which would be sent from CU to DU requesting to trigger MAC CE for the case that the LTM mobility is based on L3 measurements. It was also recognised that such a message would be needed from Release 18 onwards. 2. Actions: RAN2 kindly asks RAN3 to introduce F1 support for the case where the CU needs to request the DU to send MAC CE to trigger LTM execution based on UE Layer 3 measurements.

During RAN3#125bis online discussion, some companies proposed CU to decide about triggering of L3 measurements based LTM and CU will inform DU to trigger MAC-CE based cell switch command. But some companies argued that it is enough for CU to provide L3 measurements to DU and then DU will make decisions about when to trigger Early TA Acquisition and MAC-CE based cell switch command etc.

Couple of companies argued that RAN2 LS does NOT talk about any Early TA Acquisition for L3 measurements based LTM and it is not applicable. Which is not aligned with many other companies understanding and as a result another LS R3-245710 was sent from RAN3 to RAN2 asking for clarification about whether Early TA Synchronization is applicable for L3 measurement based LTM or not.

RAN WG3 discussed the introduction of F1 support for the case where LTM is based on UE layer 3 measurements and acknowledge that this is not supported in Rel-18.

During the discussion, some companies raised the concern whether the early synchronization should be supported or not for L3 measurements based LTM.

RAN2 sent another reply LS R2-2409373 to RAN3 and given below.

RAN WG2 would like to thank RAN WG3 for their LS in R2-2409382. RAN WG2 understands that early synchronisation is also applicable to L3 measurements based LTM. It is also RAN WG2 understanding that for L3 measurements based LTM, the CU decides early synchronisation according to the results of L3 measurements and indicates that decision with the corresponding beam information to a DU and then the DU triggers early synchronisation.

Note that it is not RAN2 realm to make decision about whether CU or DU to make decisions about whether L3 measurements based LTM decisions like when to trigger early TA synchronization, MAC-CE based TCI state and MAC-CE based cell switch command. It is purely RAN3 to make such decision by taking LTM performance requirements into account and specify necessary F1 enhancements needed.

In RAN3#126, this issue was discussed and chair noted “Continue the discussion and decide the solution in next meeting.“

In Jan-2025, there was an offline email discussion among multiple companies to discuss issues of CU and DU based solution . Qualcomm has raised many issues with CU based solution.

In RAN3#127, RAN3 discussed without making any progress and chair decided not to support any solution in spite of 8 companies objected for not specifying any solution in not good practice.RAN3 sent reply LS to RAN2, cc: RAN4, RAN.

RAN WG3 has discussed the solutions, however does not reach a consensus to specify the support for L3 measurement based LTM for CU/DU distributed architecture. As a result, F1 support for L3 measurement based LTM will not be specified in Release 18

RAN#107 discussed the issue and provided following guidance to RAN3 in [16].

RAN kindly requests RAN3 to agree by RAN#108, a single standard solution as Rel-18 correction, to support the L3 measurement based LTM over the F1 interface (see WI Code above).

RAN3#127bis Agreements:

CU triggers the new procedure with the L3 measurement information and trigger indicator towards DU for early synchronization and cell switch.

DU takes it into account if supported.

FFS on the presence and details of the L3 measurement information and trigger indicator.

SSB Index list is per candidate Cell.

FFS: It has to be clarified if CU-DU Mobility Initiation message includes CSI-RS Index List.

To be continued...

R3-252412 : Taking this CR as the baseline, the FFSes and the measurements structure to be continued…

RAN3#128 Agreements:

After long controversial discussions for multiple meetings, finally RAN3#128 made following agreements to support both CU and DU based solution as CHOICE structure and corresponding stage-2 and F1-AP stage-3 CRs were agreed.

Define the below two kind of information as Choice IE structure from CU to DU:

- Mobility Triggering Indication -> CU based solution

- Mobility Assistance Information including L3 measurement details -> DU based solution

Mobility Assistance Information including measurement of the serving cell and measurement list of candidate cells including:

- SSB beam level measurement quantities (RSRP, RSRQ and SINR)

Cell level measurement to be further checked in future meeting.

Following are agreed CRs.

| R3-253869 | F1 support of L3 measurements-based LTM | CR1558r3, TS 38.473 v18.5.0, Rel-18, Cat. F |
| --- | --- | --- |
| R3-253870 | F1 support for L3 measurements-based LTM | CR0158r2, TS 38.470 v18.4.0, Rel-18, Cat. F |
| R3-253946 | Stage 2 updates for support of L3 measurements based LTM | CR0428r5, TS 38.401 v18.5.0, Rel-18, Cat. F |

However, some companies who are promoted only CU based solution blocked both online and offline discussions to prevent CU from passing Cell Level Measureemnts received from UE to DU for L3 measurement based R18 LTM feature.

Following are Chair Notes:

QC: The cell level measurements along with beam level measurements can be used for L3 measurement based LTM decision in DU.

Cell level measurement to be further checked in future meeting.

In this contribution, we discuss what are key changes needed for F1-AP CR to enable CU to pass Cell Level Measurements received from UE via RRC signalling to DU to support flexible implementation of L3 Measurement based LTM feature in R18.

Please note that in order to keep the document concise and all the technical points discussed in RAN3#127bis Tdoc R3-251761 still holds good are not repeated again and is considered as reference for this discussion in addition to RAN3#128  agreements.

# 2	Discussion

In Rel-18, the main motivation for introducing LTM is to reduce the handover interruption latency and ability to switch cells/beams quickly by DU based on L1 filtered measurements received from UE.

During initial development phase, some vendors claimed that they may not be able to support L1 measurements processing and some UEs may not be able to support L1 measurements reporting for all scenarios. Thus, to enable initial deployments, L3 measurements based R18 LTM was also agreed to be supported in RAN4 and RAN2.

It is important to note that L3 measurements based LTM was agreed in RAN4 primarily for FR1 only and is not intended for FR2 case.

Observation 1 : L3 measurement based R18 LTM is supported only for FR1.

Overtime, it is possible that multiple gNB vendors supporting both L1 and L3 measurement based LTM configuration and different UEs based on their capability are expected to support L1 and/or L3 measurement based LTM for FR1. In such mixed deployment scenario, it is desirable to have one entity in gNB  should control LTM HO triggering for both L1 and L3 measurements based LTM. As compromise, during RAN3#128 agreed to support both CU and DU based solutions as CHOICE structure in F1-AP signalling procedure.

The spirit of this compromise is depending on specific implementation of either CU or DU based solutions, all necessary UE reported L3 filtered measurements needs to be fully available at both CU and DU to give flexibility for implementation.

Observation 2 : The spirit of RAN3#128 compromise is to support signalling enhancements for both CU and DU based solutions as CHOICE structure in F1-AP signalling to enable all L3 measurements made available to both CU and DU to support full flexibility of different implementations.

Let us first try to understand what is difference between L1 measurement reports and L3 measurement reports from UE. Following Figure from TS 38.300 depicts UE processing of L1 and L3 measurements.

In case of L3 based measurements, UE does L3 based filtering which is long term average of reporting radio metrics (i.e., RSRP, RSRQ and SINR) and reports using RRC measurement reports. For L1 based measurements, UE does L1 based filtering based on UE implementation (which is quicker than L3 filtering but it is not one short L1 measurement) and reports to gNB via either PUCCH or PUSCH in UCI. R18 Layer 1 LTM measurements are based on SSB only and UE reports L1-RSRP which is directly received by DU.

Between L1-RSRP and L3 RSRP, the only difference is what filter is being used by UE and these metric values meaning remains same. Example : RSRP of -90 dBm value is same whether it is L1 or L3 filtered and receiving node does not have to perform any additional decoding.

Following standard table gives, how to interpret and it is clear from the table that RSRP Index value from 16 to 113 represent same value of RSRP measurements for both L1 and L3 based measurements.

Based on implementation-based filtering coefficient, DU can use different values based on whether it is L1-RSRP or L3-RSRP value. How DU makes decisions based on L3 measurement reports received from CU is upto DU implementation matter and is not standards issue.

L3-RSRP

L1-RSRP

Observation 3 : The difference between L1 vs L3 measurements is only about usage of L1 filter  vs L3 filter by the UE. At receiving RAN node, there is no additional de-filtering needed and meaning of absolute value of L1-RSRP Vs L3-RSRP remains same. DU will understand radio measurement very well and DU will handle processing of L3 measurements based on implementation.

In L3 measurements based LTM, CU will configure UE with Measurement Configuration including whether to report  optional L3 filtered beam level measurements (SSB and/or CSI-RS beam based), necessary L3 filters, measurement quantities (RSRP, RSRQ, SINR), measurement objects, measurement type (periodic, event based). But UE will always reports L3 filtered Cell Level Measurements in RRC Measurement Report to CU. Note that CU itself will not compute any Cell Level Measurements and is calculated by the UE based on procedures defined in RRC for UE behavior.

In agreed F1-AP CR, CU will provide all L3 measurement reports to DU for all configured candidate cells and serving PCell at both cell level and SSB level to allow better pre-sync and CSC decisions by taking both serving PCell and candidate Pcell measurements into account.

In R18 LTM L1 measurements, it is allowed for UE report both candidate and serving cell beam level measurements based on configuration. In R19 L1 LTM measurements, max of 4 beams per cell and max of 4 cells can be reported. One cell report can be configured as serving cell L1 report.

Observation 4 : when UE reports L3 based measurements via RRC signalling to CU, UE will compute L3 filtered Cell level and Beam level measurement quantities. It is mandatory for UEs to report Cell level measurements and optional to report beam based measurements based on RRC configuration.

During RAN3#128 offline discussion, unfortunately some companies used following arguments to block the discussion and prevented CU from reporting L3 cell level measurements to DU via F1-AP signalling message.

- Since CU RRC provides L3 filters, it shall not be given to DU

- CU is the only network to process and apply cell level filters

- It is violation of CU-DU split functionality to share cell level measurements to DU.

- L1 measurements does not provide cell level measurements but only beam level measurements.

It is important to note that all cells and lower layers are hosted and operated by DU (not the CU). CU only provides cell management like activate/deactivate cells and UE configuration management functionality. In fact, LTM is purely L1/L2 controlled mobility functionality but still as compromise it was agreed to support CU triggered L3 measurements based LTM (inspite of multiple technical limitations of CU triggered methos). CU-DU split argument itself is not correct because CU shall not control DU L1/L2 functionality but still CU solution was agreed as compromise. Technically there is no issue from passing L3 cell level measurements to DU.

UE L1 based measurements does not provide cell level measurements to DU, but this does not prevent CU from passing L3 based cell level measurements received from UE to DU.

Now the question is what is need for DU to know L3 cell level measurements for LTM?

The main motivation is when multiple candidate cells beam level measurements are received by DU from CU (via F1-AP signalling), DU implementation based algorithm can make use of L3 cell level measurements computed by UE (not computed by CU) to filter candidate cells first. Based on filtered candidate cells, DU can make decision which beams within filtered candidate cells can be used for triggering LTM mobility procedure. This avoids DU from calculating cell level measurements based on beam level measurements of each cell and avoids some unnecessary DU processing. Also all beam level measurements used by UE for calculating cell measurements may not reported by UE and not available at DU. L3 filter used by UE for calculating cell level measurements is not available at DU unless CU provides the same L3 filter to DU, which is not provided in any of existing F1-AP siganling.

Observation 5 :  When multiple candidate cells beam level measurements are received by DU from CU (via F1-AP signalling), DU implementation based algorithm can make use of L3 cell level measurements computed by UE (not computed by CU) to filter candidate cells first.

Observation 6 :  Based on filtered candidate cells, DU can make decision which beams within filtered candidate cells can be used for triggering LTM mobility procedure. This avoids DU from calculating cell level measurements based on beam level measurements of each cell and avoids some unnecessary DU processing.

There are no real technical issues for providing L3 cell level measurements from CU to DU. All the L3 measurements reported by UEs shall be made available to both CU and DU to give same parity of implementation flexibility and 3GPP WG shall not put artificial restrictions to DU based solution.

Observation 7 : All the L3 measurements reported by UEs shall be made available to both CU and DU to give same parity of implementation flexibility and 3GPP WG shall NOT put artificial restrictions to DU based solution.

Thus, we propose to enhance F1-AP CR to provide L3 cell level measurements from CU to DU.

Proposal 1: For L3 measurement based R18 LTM, in F1-AP CU-DU MOBILITY INITIATION REQUEST message CU provide L3 measurements to DU shall also include L3 Cell Level measurement information for both serving and candidate cells.

Proposal 2: F1-AP signalling message changes are available in R3-25xxxx.

# 3	Conclusion

In this contribution, we discussed how to support L3 measurement based R18 LTM  and following are key observations and proposals for discussion.

Observation 1 : L3 measurement based R18 LTM is supported only for FR1.

Observation 2 : The spirit of RAN3#128 compromise is to support signalling enhancements for both CU and DU based solutions as CHOICE structure in F1-AP signalling to enable all L3 measurements made available to both CU and DU to support full flexibility of different implementations.

Observation 3 : The difference between L1 vs L3 measurements is only about usage of L1 filter  vs L3 filter by the UE. At receiving RAN node, there is no additional de-filtering needed and meaning of absolute value of L1-RSRP Vs L3-RSRP remains same. DU will understand radio measurement very well and DU will handle processing of L3 measurements based on implementation.

Observation 4 : when UE reports L3 based measurements via RRC signalling to CU, UE will compute L3 filtered Cell level and Beam level measurement quantities. It is mandatory for UEs to report Cell level measurements and optional to report beam based measurements based on RRC configuration.

Observation 5 :  When multiple candidate cells beam level measurements are received by DU from CU (via F1-AP signalling), DU implementation based algorithm can make use of L3 cell level measurements computed by UE (not computed by CU) to filter candidate cells first.

Observation 6 :  Based on filtered candidate cells, DU can make decision which beams within filtered candidate cells can be used for triggering LTM mobility procedure. This avoids DU from calculating cell level measurements based on beam level measurements of each cell and avoids some unnecessary DU processing.

Observation 7 : All the L3 measurements reported by UEs shall be made available to both CU and DU to give same parity of implementation flexibility and 3GPP WG shall NOT put artificial restrictions to DU based solution

Thus, based on above observations, we propose

Proposal 1: For L3 measurement based R18 LTM, in F1-AP CU-DU MOBILITY INITIATION REQUEST message CU providing L3 measurements to DU shall also include L3 Cell Level measurement information for both serving and candidate cells.

Proposal 2: F1-AP signalling message changes are available in R3-255322.