3GPP TSG-RAN WG3 Meeting #129	R3-255326

Bengaluru, India, 25th ~29th Aug, 2025

Title:	[TPs to TS 38.300 38.413 BL CRs] A-IoT Security Aspects

Agenda Item:	16.2

Source:	Huawei, China Telecom, China Unicom

Document for:	other

# Introduction

In this contribution, we discuss RAN3 impacts based on SA3 progress.

# 2	Discussion

In [1], SA3 inform RAN2 about their progress and also ask RAN2 to confirm the feasibility of including additional parameters and their maximum size:

1. Overall Description: SA3 is currently working on the authentication procedure. For this procedure, SA3 is assuming the ability to include an additional parameter, a 128bit random number in the paging request message. There may be other security parameters required. Therefore, SA3 would like to request RAN2 to confirm the feasibility of including additional security parameters and their maximum size. 2. Actions: To RAN2 ACTION: RAN2 is kindly requested to take the above information into account and provide feedback.

The following can be found in TS 33.369[2]:

### 5.2.2 Authentication procedure The authentication procedure is aligned with inventory procedure and command procedure in 6.2.2 and 6.2.3 of TS 23.369[2]. Figure 5.2.1-1: Authentication procedure … 2. AIOTF shall send inventory request message including RANDAIOT_n to NG-RAN. Editor’s Note: The inclusion of RANDAIOT_n in Paging Request and the size of RANDAIOT_n needs RAN confirmation. 3. NG-RAN shall send the paging request message including RANDAIOT_n to the AIoT device. Editor’s Note: Whether replay attack is possible is FFS. 4. Upon receiving the paging request message, AIoT device shall derive RESAIOT using KAIoT and RANDAIOT_n for network authenticating AIoT Device. Editor’s Note: How RESAIOT is derived and whether it is derived from KAIoT or intermediate key is FFS. Editor’s Note: Where the authentication credentials are processed in AIOT device is FFS. 5. AIoT device sends D2R message to the NG-RAN, including RESAIOT and RANDAIOT_d from device. Editor’s Note: The security requirements of generating RANDAIOT_d are FFS. Editor’s Note: Whether RANDAIOT_d is required for inventory procedure is FFS. 6. NG-RAN sends Inventory report message to AIOTF, including the RESAIOT and RANDAIOT_d. …

In step 2 and step 3, it is understood that, the RANDAIOT_n is sent from the AIOTF to the gNB in NGAP: INVENTORY REQUEST message, i.e., within the Inventory Request Transfer IE, and then the gNB include it in the Paging message over AIoT radio.

It is noticed that, in TR 33.713 [3], SA3 concluded that “For network layer security, the AIoTF acts as the security termination point on the network side” as general principle. Therefore, in step 5 and 6, the device sends RESAIOT and RANDAIOT_d to the AIOTF via the gNB, and the RESAIOT and RANDAIOT_d are intended to be handled by the AIOTF, i.e., carried by NAS and cannot be interpreted by the gNB, there is no RAN3 impact foreseen for these two steps.

Proposal 1: Introduce a new IE to carry RANDAIOT_n BIT STRING (SIZE (128, ...)) within the Inventory Request Transfer IE, in order to sent it in Paging message over the A-IoT radio.

Proposal 2: No RAN3 impact is foreseen to support the security aspects in Inventory Response/Report.

Then in Section 5.3.2 of TS 33.369, i.e., Security procedure on the information protection during command procedure, the Command Request and Response messages are protected A-IoT NAS messages:

### 5.3.1 General This clause describes the security procedures for the information protection in command message. The protection of information is provided as part of the AIoT NAS protocol between AIoT device and AIOTF. The AIOTF acts as the security termination point for AIoT information protection. ### 5.3.2 Security procedure on information protection during command procedure Figure 5.3.2-1: Security procedure on the information protection during command procedure 1. The command procedure is initiated as specified in step 1-6 of clause 6.2.3 of TS 23.369 [2]. 2. The procedure as described in clause 5.2.2 shall be performed. The device and AIOTF acquire the KAIOTF key to be used for command protection. Editor’s Note: The derivation and retrieval of the KAIOTF key are FFS. Editor’s Note: How to prevent key stream reuse is FFS. 3. The AIoTF shall construct a NAS Command Request and protect the message based on the KAIoTF, the confidentiality and integrity algorithms for the AIoT device. The AIoTF shall send the Command Request containing the protected NAS Command Request to NG-RAN. Editor’s Note: the selection of confidentiality and integrity algorithms by the network is FFS 4. The NG-RAN shall send a R2D message containing the protected NAS Command Request as specified in as specified in TS 38.300 [3] and TS 38.391 [6] . 5. The device shall verify the integrity of the command message. If the verification of integrity is successful, the AIoT device shall decipher it in case it is confidentiality protected. The AIoT device shall construct a NAS Command Response and protect the message based on the KAIoTF key using the same algorithms. 6. The AIoT device shall send a D2R message containing the protected NAS Command Response to the NG-RAN as specified in as specified in TS 38.300 [3] and TS 38.391 [6]. 7. The NG-RAN shall forward the Command Response containing the protected NAS Command Response to the AIoTF. 8-9. The AIoTF shall verify the integrity of the command message. If the verification of integrity is successful, the AIoT device shall decipher it in case it is confidentiality protected. Then, the AIOTF shall continue the procedure as specified in clause 6.2.3 of TS 23.369 [2].

Proposal 3: No RAN3 impact is foreseen to support the security aspects in Command Request/Response.

# 3	Conclusion

In this contribution, we discuss the A-IoT security aspects, and get the following proposals:

Proposal 1: Introduce a new IE to carry RANDAIOT_n BIT STRING (SIZE (128, ...)) within the Inventory Request Transfer IE, in order to sent it in Paging message over the A-IoT radio.

Proposal 2: No RAN3 impact is foreseen to support the security aspects in Inventory Response/Report.

Proposal 3: No RAN3 impact is foreseen to support the security aspects in Command Request/Response.

The TPs for Proposal 1 is provided in section 5 and 6.

# 4	Reference

[1]	S3-252392 (to RAN2, Cc SA2) on security parameter in paging request message

[2]	3GPP TS 33.369 V0.2.0 (2025-05) Security aspects of Ambient IoT service (Release 19)

[3]	3GPP TR 33.713 Study on Security Aspect of Ambient IoT Services in 5G (Release 19)

# 5	Text Proposal to TS 38.300 BL CR

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 16.xx.x3	Inventory procedure

Figure 16.xx.x3-1 depicts the basic communication between the gNB and the A-IoT CN node for the Inventory procedure.

Figure 16.xx.x3-1: Inventory procedure

1.	The A-IoT CN node initiates the Inventory procedure over NG-C by sending the Inventory Request message to the gNB. The Inventory Request message includes a Correlation ID and A-IOTF Identifier to identify the inventory session. The Inventory Request message also includes an A-IoT Device Identification Requested corresponding to the A-IoT device(s) which are targeted for the inventory. The A-IoT Device Identification Requested may indicate inventory for a single device, a group of devices or for all devices.

The Inventory Request message also includes a RANDAIOT_n to be provided to the device(s) in step 4 for security purpose.

The Inventory Request message also includes Requested Service Area Information (list of A-IoT Area IDs and/or reader list), the Expected D2R message size and may include Inventory Assistance Information e.g. Approximate number of Target A-IoT devices.

Editor’s Note 1:	Whether the Expected D2R message size is mandatory is FFS.

The Inventory Request message may also include a follow-on command indicator to indicate that at least for one of the targeted A-IoT device(s) a Command procedure will follow.

Editor’s Note 2:	Specification of further parameters of the Inventory Request message (e.g., other assistance information from 5GC) needs further work.

2.	The gNB allocates and co-ordinates the usage of A-IoT radio resources.

<<<<<<<<<<<<<<<<<<<< End of the Change >>>>>>>>>>>>>>>>>>>>

# 6	Text Proposal to TS 38.413 BL CR

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

//skip unchanged part

[58]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[z]	3GPP TS 23.369: "Architecture support for Ambient power-enabled Internet of Things; Stage 2".

[x]	3GPP TS 33.369: “Security aspects of Ambient IoT service”

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>

#### 9.3.x.1	Inventory Request Transfer

This IE provides the inventory request related information from the AIOTF to the NG-RAN node.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| A-IoT Device Identification Requested | M |  | 9.3.3.xx5 |  | YES | reject |
| Requested Service Area Information | M |  | 9.3.3.xx3 |  | YES | reject |
| Inventory Assistance Information | O |  | 9.3.3.xx4 |  | YES | reject |
| Follow on Command Indication | O |  | ENUMERATED (true, ...) | This IE indicates that there is command(s) to be transmitted. | YES | reject |
| Expected D2R Message Size | M[FFS] |  | FFS | Unit: FFS. This IE indicates the size of the A-IoT NAS PDU(s) carried in Inventory Report. | YES | reject |
| A-IoT RAND-n | M |  | BIT STRING (SIZE (128, ...)) | Contains the RANDAIOT_n as defined in TS 33.369 [x]. | YES | reject |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

id-AdditionalULForwardingUPTNLInformation,

id-AdditionalDLQosFlowPerTNLInformation,

id-AdditionalDLUPTNLInformationForHOList,

id-AdditionalNGU-UP-TNLInformation,

id-AdditionalRedundantDL-NGU-UP-TNLInformation,

id-AdditionalRedundantDLQosFlowPerTNLInformation,

id-AdditionalRedundantNGU-UP-TNLInformation,

id-AdditionalRedundantUL-NGU-UP-TNLInformation,

id-AdditionalUL-NGU-UP-TNLInformation,

id-AIOTFIdentifier,

id-AIoT-CorrelationIdentifier,

id-AIoT-DeviceIdentificationRequested,

id-AIoT-RequestedServiceAreaInformation,

id-AIoT-InventoryAssistanceInformation,

id-AIoT-InventoryExpectedD2RMessageSize,

id-AIoT-FollowonCommandIndication,

id-RAN-AIOT-Device-NGAP-ID,

id-AIoT-CommandAssistanceInformation,

id-AIoT-NASPDU,

id-AIoT-RAND-n,

id-AlternativeQoSParaSetList,

id-AssistanceInformationQoE-Meas,

id-AdditionalCancelledlocationReportingReferenceIDList,

id-BurstArrivalTimeDownlink,

//skip unchanged part

AIoT-NASPDU  ::= OCTET STRING

AIoT-RAND-n ::= BIT STRING (SIZE(128, ...))

AIoT-ReaderReportList ::= SEQUENCE (SIZE(1..maxnoofReaders)) OF AIoT-ReaderReportItem

AIoT-ReaderReportItem ::= SEQUENCE {

readerIndex					AIoT-ReaderIndex,

deviceReportList			AIoT-DeviceReportList,

iE-Extensions		ProtocolExtensionContainer { { AIoT-ReaderReportItem-ExtIEs} } OPTIONAL,

...

}

AIoT-ReaderReportItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AIoT-ReaderIndex ::= INTEGER (1..65535, ...)

AIoT-RequestedReaderList ::= SEQUENCE (SIZE(1..maxnoofReaders)) OF AIoT-RequestedReaderItem

AIoT-RequestedReaderItem ::= SEQUENCE {

globalgNB-ID				GlobalGNB-ID,

readerIndex					AIoT-ReaderIndex,

iE-Extensions		ProtocolExtensionContainer { { AIoT-RequestedReaderItem-ExtIEs} } OPTIONAL,

...

}

AIoT-RequestedReaderItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

//skip unchanged part

InventoryRequestTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { InventoryRequestTransferIEs} },

...

}

InventoryRequestTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AIoT-CorrelationIdentifier				CRITICALITY reject	TYPE AIoT-CorrelationIdentifier							PRESENCE	mandatory	}|

{ ID id-AIoT-DeviceIdentificationRequested		CRITICALITY reject	TYPE AIoT-DeviceIdentificationRequested				PRESENCE	mandatory	}|

{ ID id-AIoT-RequestedServiceAreaInformation	CRITICALITY reject	TYPE AIoT-RequestedServiceAreaInformation			PRESENCE mandatory	}|

{ ID id-AIoT-InventoryExpectedD2RMessageSize	CRITICALITY reject	TYPE AIoT-InventoryExpectedD2RMessageSize			PRESENCE mandatory[FFS]	}|

{ ID id-AIoT-InventoryAssistanceInformation		CRITICALITY reject	TYPE AIoT-InventoryAssistanceInformation				PRESENCE	optional		}|

{ ID id-AIoT-FollowonCommandIndication			CRITICALITY reject	TYPE AIoT-FollowonCommandIndication					PRESENCE	optional		}|

{ ID id-AIoT-RAND-n								CRITICALITY reject	TYPE AIoT-RAND-n											PRESENCE mandatory	},

...

}

<<<<<<<<<<<<<<<<<<<< Last Change >>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM NGAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-AMFConfigurationUpdate					ProcedureCode ::= 0

id-AMFStatusIndication						ProcedureCode ::= 1

//skip unchanged part

id-ExtendedBackupAMFName    							ProtocolIE-ID ::= 442

id-ExtendedOldAMF			    						ProtocolIE-ID ::= 443

id-InventoryRequestTransfer							ProtocolIE-ID ::= 993 --to be allocated

id-InventoryResponseTransfer						ProtocolIE-ID ::= 994 --to be allocated

id-InventoryFailureTransfer							ProtocolIE-ID ::= 995 --to be allocated

id-InventoryReportTransfer							ProtocolIE-ID ::= 996 --to be allocated

id-CommandRequestTransfer							ProtocolIE-ID ::= 997 --to be allocated

id-CommandResponseTransfer							ProtocolIE-ID ::= 998 --to be allocated

id-CommandFailureTransfer							ProtocolIE-ID ::= 999 --to be allocated

id-AIOTSessionReleaseCommandTransfer					ProtocolIE-ID ::= 1000 --to be allocated

id-AIOTSessionReleaseCompleteTransfer					ProtocolIE-ID ::= 1001 --to be allocated

id-AIOTSessionReleaseRequestTransfer				ProtocolIE-ID ::= 1002 --to be allocated

id-AIoT-CorrelationIdentifier							ProtocolIE-ID ::= 880 --to be allocated

id-AIOTFIdentifier									ProtocolIE-ID ::= 881 --to be allocated

id-AIoT-DeviceIdentificationRequested					ProtocolIE-ID ::= 882 --to be allocated

id-AIoT-RequestedServiceAreaInformation					ProtocolIE-ID ::= 883 --to be allocated

id-AIoT-InventoryAssistanceInformation					ProtocolIE-ID ::= 884 --to be allocated

id-AIoT-FollowonCommandIndication						ProtocolIE-ID ::= 885 --to be allocated

id-RAN-AIOT-Device-NGAP-ID								ProtocolIE-ID ::= 886 --to be allocated

id-AIoT-CommandAssistanceInformation					ProtocolIE-ID ::= 887 --to be allocated

id-AIoT-NASPDU										ProtocolIE-ID ::= 888 --to be allocated

id-AIoT-InventoryExpectedD2RMessageSize					ProtocolIE-ID ::= 889 --to be allocated

id-AIoT-Support											ProtocolIE-ID ::= 890 --to be allocated

id-AIoT-RAND-n											ProtocolIE-ID ::= 891 --to be allocated

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>