3GPP TSG-RAN3 Meeting #129	R3-255330

Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

Agenda item:	16.2

Source:	Xiaomi

Title:	Discussion on open issues of A-IoT

Document for:	Discussion

1	Introduction

In this contribution, we would like to discuss the following open issues on the support of A-IoT.

### - A-IoT Session management

- A-IoT inventory and command related signalling

- NG connection management

2	Discussion

### A-IoT Session management

RAN3 128 agreements

In the case of Inventory only scenario, introduce Inventory Complete indication to inform the AIoT CN about the complete of the triggered Inventory session. FFS on the detail about Inventory Complete indication via new procedure or introducing indication IE in current Inventory Report message.

In the case of Command after Inventory scenario, whether need to introduce the complete indication needs to be further checked.

Inventory complete indication

### RAN3 has agreed to introduce an inventory complete indication for the inventory-only use case. For the combined inventory and command use case, we believe this indication is also necessary.

### RAN3 has also agreed that the gNB can signal inventory completion to the AIoT CN, although the signaling design remains FFS. From our perspective, the existing inventory report procedure can be leveraged, with the inventory complete indication added to the Inventory Report Transfer IE. Since the message may or may not contain an actual report, we propose setting the Reader Report List IE to a range of“0..1”.

### Proposal 1: Introduce inventory complete indication for both inventory-only and inventory+command use cases.

### Proposal 2: Add the inventory complete indication to the Inventory Report Transfer IE.

### Proposal 3: Modify the Reader Report List IE range to“0..1”.

Per device context release

### In the inventory and command use case, the gNB shall maintain device-specific context (e.g., RAN NGAP Device ID, AS ID) to support per-device communication. If no communication is expected for a particular device, its context may be released.

### We consider the command procedure to be device-associated, and therefore, the context release should be handled at the individual device level. The following conditions may trigger per-device context release:

### - No further command are pending for the specific device

- - A-IoT radio link failure occurs with the specific device

- The following call flow is an example to clarify the scenario.

- It is possible some of the command procedures fail over A-IoT radio interface, and it may take time to finish all the command procedures for all devices, thus, there’s no need to keep the context of the failed device in case the there will no further communication.

Proposal 4, Include NGAP Device ID List in the A-IoT Session Release Command message to support per-device context release.

### A-IoT inventory and command related signalling

Encoding of A-IoT device identification

RAN3 128 agreements

Define the A-IoT Device Identification Requested IE as CHOICE type, with three branches:

-	single device inventory:		OCTECT STRING	refer to Device Identifier

-	group devices inventory:		FFS on the encoding

-	all devices inventory:		NULL

Define the Expected D2R Message Size IE in Inventory Request Transfer IE. FFS on the detail encoding.

For group device identification, according to CT4’s design in [1], the device permanent ID includes the following IDs and some of them are not byte-aligned.

1.ID type : Fixed 8 bits (byte-aligned)

2.PLMN (Optional):

Encoded as BCD (4 bits/digit)

5 digits = 20 bits (non-aligned)

6 digits = 24 bits (byte-aligned)

3.NID (Optional):

Hex digits (4 bits/digit)

11 digits = 44 bits (non-aligned)

4.3rd party ID (Optional): Fixed 24 bits (byte-aligned)

5.ID info : Fixed 96 or 128 bits (both byte-aligned)

The following are some different composition scenarios for the A-IoT device permanent ID:

| Scenario | Composition | Total Bits | Byte-aligned? |
| --- | --- | --- | --- |
| Min | ID type + ID info (96b) | 104 bits | ✅ Aligned (13B) |
| Max | All fields + PLMN 6d + ID info 128b | 228 bits | ❌ Non-aligned (28.5B, 4b remainder) |
| Mixed | ID type + PLMN 5d + NID + ID info 128b | 200 bits | ✅ Aligned |

It is observed that the A-IoT Device Permanent Identifier is not always byte-aligned, primarily due to the optional 44-bit NID component, which itself is not byte-aligned. The total length of the Device Identifier can vary, with a maximum of 228 bits and a minimum of 104 bits. So it’s suggested to use BIT STRING with a future-proof size.

Observation 1, The A-IoT Device Permanent Identifier is not consistently byte-aligned due to the optional 44-bit NID.

Proposal 5, Use BIT STRING (SIZE(104..512, ...)) for encoding the single Device Identifier.

Regarding group device identification, it should be based on the filtering information defined by SA2 and CT4. This filtering information is used to identify or filter multiple A-IoT devices and is composed of one or more components of the A-IoT Device Permanent Identifier—such as ID Type, PLMN Identifier, NID, third-party identifier, and Identification Information. As such, the filtering information is expected to be shorter than the full Device Identifier.

The AIoT Identification Information to be provided to NG-RAN can include Filtering Information, as defined in clause 5.8, or a single AIoT Device Identifier.

According to CT4 design, only ID type (8 bit) is mandatory, so it’s suggested use BIT STING SIZE(8..512, ...) as the encoding of the filtering information.

Proposal 6, Group identification information can be encoded as BIT STING SIZE(8..512, ...) and refer to the filtering information defined in SA2.

Security information

There is an LS from SA3 to RAN2 on the security parameters and according to TS 33.369 [3], a RANDAIOT_n needs to be transferred from AIOTF to RAN, and then to the A-IoT device. The A-IoT device may provided the RESAIOT and RANDAIOT_d to the RAN, which are in the A-IoT NAS container from reader perspective.

2. AIOTF shall send inventory request message including RANDAIOT_n to NG-RAN. Editor’s Note: The inclusion of RANDAIOT_n in Paging Request and the size of RANDAIOT_n needs RAN confirmation. 3. NG-RAN shall send the paging request message including RANDAIOT_n to the AIoT device. Editor’s Note: Whether replay attack is possible is FFS. 4. Upon receiving the paging request message, AIoT device shall derive RESAIOT using KAIoT and RANDAIOT_n for network authenticating AIoT Device. Editor’s Note: How RESAIOT is derived and whether it is derived from KAIoT or intermediate key is FFS. Editor’s Note: Where the authentication credentials are processed in AIOT device is FFS. 5. AIoT device sends D2R message to the NG-RAN, including RESAIOT and RANDAIOT_d from device. Editor’s Note: The security requirements of generating RANDAIOT_d are FFS. Editor’s Note: Whether RANDAIOT_d is required for inventory procedure is FFS.

From RAN3 perspective, the RANDAIOT_n need to be added in the inventory request message.

Proposal 7, Introduce an optional security information including RANDAIOT_n in the inventory request transfer IE.

Encoding of D2R message size in inventory response

The size of the D2R message in the Inventory Request Transfer depends on two key factors: the Device Identifier size and the security information included in the device's response.

- Device Identifier Size: This is influenced by the CT1 design. While it is not yet confirmed whether the identifier is byte-aligned, assuming byte alignment may result in higher resource requirements.

- Security Parameters Size: SA3 has discussed the inclusion of a 128-bit RAND, and additional security parameters may be introduced in the future.

Given these considerations and the need for future extensibility, it is proposed that the D2R Message Size in the Inventory Request Transfer IE be encoded as an INTEGER with SIZE(104..1024, ...).

Proposal 8, Define the D2R Message Size in Inventory Request Transfer IE as a mandatory INTEGER (SIZE 104..1024, ...) to accommodate varying device ID lengths and security requirements.

Time interval

There remains an FFS regarding the inclusion of a time interval as assistance information in the Inventory Request.According to the SA2 specification [2], if provided by the AIOTF, the time interval is used by the gNB to aggregate reports from multiple A-IoT devices. This mechanism supports efficient inventory reporting and device grouping.

It is FFS on the Time Interval as assistance information in the Inventory Request message. Clarify with SA2 on the usage of this information in NG-RAN node.

An AIoT service operation may involve many AIoT Devices. The NG-RAN may perform AIoT service operation result aggregation with a specific Correlation ID based on the aggregation assistance information received from the AIOTF for a service operation request as specified in clause 5.4. The AIOTF determines the aggregation assistance information based on the request from the AF or local configuration, which includes: - Time interval: the fixed time interval for which NG-RAN collects multiple AIoT Devices’ operation responses before reporting the aggregated AIoT response to the AIOTF. The reporting based on time interval may potentially happen multiple times until the NG-RAN completes the request operation. If the AF has provided a time interval, then the AIOTF should signal a time interval to the NG-RAN that is equal or shorter than the time interval received from the AF. NOTE: Based on local configuration, the AIOTF can reject the AF request, e.g. if the AF provided time interval is shorter than a locally configured minimum interval. If the AIOTF does not provide aggregation assistance information, the aggregation process in the NG-RAN may be determined by implementation. The AIOTF may also aggregate the results of a requested service operation before sending them to the NEF or trusted AF.

To align with SA2, the time interval can be introduced as assistance information for inventory report aggregation.

Proposal 9, Introduce time interval as assistance information for inventory report aggregation.

Command type

The following items remain FFS regarding the awareness of the command type.

It is FFS to introduce Command Type (write, read, disable) in Command Request Transfer.

From our perspective, if the command type is primarily used for resource allocation, then the expected D2R message size can serve that purpose effectively. As a result, there appears to be no strong justification for explicitly signaling the command type.

Proposal 10, No need to introduce command type in the command request transfer IE.

### NG connection management

### Progress has been made on NG connection management in previous meetings, and the WA for the NG setup request should now be agreed.

### For the NG Setup Response message, in the case of direct connectivity, the gNB may be aware of this configuration through OAM. Therefore, it can ignore the IEs that are not applicable.

WA: Define the A-IoT Support IE in the NG Setup Request as ENUMERATED (A-IoT only, A-IoT and NR Uu, …) Whether to have A-IoT Support Indicator in the NG SETUP RESPONSE message?

Proposal 11, Turns the WA: Define the A-IoT Support IE in the NG Setup Request as ENUMERATED (A-IoT only, A-IoT and NR Uu, …) to an agreement .

Proposal 12, gNB can ignore the IEs not applicable for A-IoT by OAM configuration.

Based on the above discussion, TPs to BL CR TS 38.300 and BL CR TS38.413 are provided to reflect the above proposals in the [4] and [5].

3	Conclusion

In this contribution, we discussed the issue and potential solutions to support AIoT in topology 1 and have the following observations and proposals.

Inventory complete indication

### Proposal 1: Introduce inventory complete indication for both inventory-only and inventory+command use cases.

### Proposal 2: Add the inventory complete indication to the Inventory Report Transfer IE.

### Proposal 3: Modify the Reader Report List IE range to“0..1”.

### Per device context release

Proposal 4, Include NGAP Device ID List in the AIOT Session Release Command message to support per-device context release.

Encoding of A-IoT device identification

Observation 1, The A-IoT Device Permanent Identifier is not consistently byte-aligned due to the optional 44-bit NID.

Proposal 5, Use BIT STRING (SIZE(104..512, ...)) for encoding the single Device Identifier.

Proposal 6, Group identification information can be encoded as BIT STING SIZE(8..512, ...) and refer to the filtering information defined in SA2.

Security information

Proposal 7, Introduce an optional security information including RANDAIOT_n in the inventory request transfer IE.

Encoding of D2R message size in inventory response

Proposal 8, Define the D2R Message Size in Inventory Request Transfer IE as a mandatory INTEGER (SIZE 104..1024, ...) to accommodate varying device ID lengths and security requirements.

Time interval

Proposal 9, Introduce time interval as assistance information for inventory report aggregation.

Command type

Proposal 10, No need to introduce command type in the command request transfer IE.

NG connection management

Proposal 11, Turns the WA: Define the A-IoT Support IE in the NG Setup Request as ENUMERATED (A-IoT only, A-IoT and NR Uu, …) to an agreement .

Proposal 12, gNB can ignore the IEs not applicable for A-IoT by OAM configuration.

4	Reference

[1]	C4-252464, Definition of AIoT Device Permanent Identifier,CT4

[2] 3GPP TS 23.369 V19.0.0, Architecture support for Ambient power-enabled Internet of Things.

[3] 3GPP TS 33.369 V0.2.0, Security aspects of Ambient IoT service

[4] R3-255331, (TP to BL CR TS 38.300) Support of A-IoT, Xiaomi

[5] R3-255332, (TP to BL CR TS 38.413) Support of A-IoT, Xiaomi