3GPP TSG-RAN3 Meeting #129	R3-255331

Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

Agenda item:	16.2

Source:	Xiaomi

Title:	(TP to BL CR TS 38.300) Support of A-IoT

Document for:	Discussion

1	Introduction

In this contribution, a TP to TS 38.300 is provided to solve the leftover issues and clean-up, including the following:

- re-org the information provided in the inventory request message

- introduce security information in the inventory request message

- add “command procedure failures over A-IoT radio” as one command failure case over NG

- add device level context release information in A-IoT session release command.

2	Annex TP for TS 38.300

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Change starts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

<<<<<<<<<<skip the unchanged parts>>>>>>>>>>>

[65]	3GPP TS 38.351: "NR; Sidelink Relay Adaptation Protocol (SRAP) Specification".

[xx]	3GPP TS 23.369: " Architecture support for Ambient power-enabled Internet of Things; Stage 2".

[yy]	3GPP TS 33.369: “Security aspects of Ambient IoT service”

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

16.xx.x3	Inventory procedure

Figure 16.xx.x3-1 depicts the basic communication between the gNB and the A-IoT CN node for the Inventory procedure.

Figure 16.xx.x3-1: Inventory procedure

1.	The A-IoT CN node initiates the Inventory procedure over NG-C by sending the Inventory Request message to the gNB. The Inventory Request message includes a Correlation ID and A-IOTF Identifier to identify the inventory session. The Inventory Request message also includes an A-IoT Device Identification Requested corresponding to the A-IoT device(s) which are targeted for the inventory. The A-IoT Device Identification Requested includes the device identification information for a single device, a group of devices or all devices. The Inventory Request message also includes Inventory Assistance Information for resource allocation and report aggregation purpose.

The Inventory Request message also includes an Requested Service Area Information (list of A-IoT Area IDs and/or reader list), the gNB may select the readers according to the Requested Service Area Information.

The Inventory Request message may also include a follow-on command indicator to indicate that at least for one of the targeted A-IoT device(s) a Command procedure will follow.

The Inventory Request message may also include a security information for authentication as specified in TS 33.369 [yy].

2.	The gNB allocates and co-ordinates the usage of A-IoT radio resources.

NOTE X, the gNB ensures that no parallel service requests occur in overlapping reader scenario at A-IoT radio by gNB implementation

3.	The gNB confirms the request from the A-IoT CN node by replying with the Inventory Response message. The Inventory Response message includes the AIOTF Identifier and the Correlation ID received in the Inventory Request message.

NOTE 2:	If the gNB is not able to perform the Inventory procedure, it rejects the request and sends an Inventory Failure message to the A-IoT CN node.

4.	The gNB performs the Inventory procedure towards the A-IoT device(s) over the A-IoT radio interface.

5./6.	Upon receiving the inventory result from the A-IoT device(s), the gNB sends the Inventory Report message to the A-IoT CN node. If the Inventory procedure concerns multiple A-IoT devices, multiple Inventory Report messages may be sent to the A-IoT CN node.

The Inventory Report message includes the AIOTF Identifier and the Correlation ID received in the Inventory Request message and further includes the A-IoT NAS PDU(s) carrying the inventoried A-IoT device ID(s).

If a follow-on Command indicator was received at step 1, the Inventory Report message also includes a RAN A-IoT device NGAP ID for each A-IoT device which was inventoried. In this case, the gNB needs to maintain the device context (including AS ID and the associated RAN device NGAP ID ) for each inventoried A-IoT device until the session release.

NOTE 3:	If the Inventory procedure concerns multiple A-IoT devices, an Inventory Report may include reports from multiple A-IoT devices.

Upon completion of the inventory procedure the gNB may include an Inventory Complete Indication in the Inventory Report message.

16.xx.x4	Command procedure

Figure 16.xx.x4-1 depicts the basic communication between the gNB and the A-IoT CN node for the Command procedure.

Figure 16.xx.x4-1: Command procedure

The Command procedure addresses only one A-IoT device.

1.	Prior to the Command procedure, the Inventory procedure is performed involving the concerned device and potentially other A-IoT devices.

NOTE 4: the step 2 may happen in parallel of the Inventory procedure involving other A-IoT devices or in parallel to other command procedures targeting other A-IoT devices within the same session.

2.	The A-IoT CN node initiates the Command procedure over NG-C by sending the Command Request message to the gNB in order to send a command to a single A-IoT device. The Command Request message includes the same Correlation ID and AIOTF Identifier as the ones used in its corresponding inventory procedure.

The Command Request also includes the RAN A-IoT device NGAP ID and the A-IoT NAS PDU corresponding to the targeted A-IoT device and may include some command assistance information. This command assistance information may contain the estimate of expected D2R message size.

3.	The gNB allocates and co-ordinates the usage of A-IoT radio resources and performs the command procedure over the A-IoT radio interface.

4.	The gNB replies to the Command Request with the Command Response message including the AIOTF Identifier  the Correlation ID, the RAN A-IoT device NGAP ID received in the Command Request, and the A-IoT NAS PDU received from the device.

NOTE 5:	If the gNB is not able to perform the Command procedure or the Command procedure fails over the A-IoT radio interface, it sends a Command Failure message to the A-IoT CN node.

### 16.xx.x5	A-IoT CN initiated A-IoT Session Release procedure

Figure 16.xx.x5-1: A-IoT CN initiated A-IoT Session Release procedure

1.	The A-IoT CN node initiates the A-IoT Session Release procedure over NG-C by sending the A-IoT Session Release Command message to the gNB. The A-IoT Session Release Command message includes the Correlation ID, the AIOTF Identifier and optional NGAP Device ID List to identify the session or the device context to be released.

2.	The gNB releases the session context, the associated NG resources and the A-IoT radio resources.

3.	The gNB confirms the release to the A-IoT CN node by replying with the A-IoT Session Release Complete message. The A-IoT Session Release Complete message includes the AIOTF Identifier and the Correlation ID received in the A-IoT Session Release Command message.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Change ends>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>