3GPP TSG-RAN3 Meeting #129	R3-255335

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

Agenda item:	14.3

Source:	Xiaomi

Title:	(Draft LS out) Support of NG suspend/resume

Document for:	Other

1	Introduction

RAN3 agrees to introduce a suspend/resume indicator from the on-board gNB to the AMF (this is also applied to IoT NTN), allowing the core network to suspend/resume the NG connection.

In this contribution, we provides a draft LS to SA2, cc CT4 to ask them support the NG suspend/resume.

The potential SA2 impacts would be:

- the AMF/MME behavior by receiving the suspend/resume indicator (SA2 impacts)

- how AMF notify SMF then to the UPF and how MME notify SGW to suspend/resume user plane data transmission. (SA2 impacts, maybe CT4 as well)

2	Draft LS out

Title:	LS on support of NG/S1 transmission suspend/resume

Response to:

Release:	Release 19

Work Item:	NR_NTN_Ph3-Core, IoT_NTN_Ph3-Core

Source:	to be RAN3

To:	SA2

Cc:	CT4

Contact person:	Lisi Li

lilisi@xiaomi.com

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	agreed TPs to be added

1	Overall description

RAN3 has discussed the NG/S1 interruption handling for regenerative payload scenarios.

To minimize data loss during a hard feeder link switch, RAN3 agrees to introduce a suspend/resume indicator from the on-board gNB to the AMF. This indicator enables the core network to temporarily suspend and subsequently resume downlink signaling and data transmission as needed.

Additionally, RAN3 considers that this indicator could also be applied when the on-board gNB moves out of the AMF/AMF set serving area, allowing the AMF to retain node-level context and resume the connection once the on-board gNB re-enters the serving area.

The same mechanism described above can also be applied to IoT NTN scenarios.

RAN3 considers that this may have impacts on SA2.

2	Actions

To SA2

ACTION: 	RAN3 respectfully asks SA2 to take the above into account.

3	Dates of next TSG RAN WG 3 meetings

TSG-RAN3 Meeting #129-bis	13-17 Oct 2025			Prague, CZ

TSG-RAN3 Meeting #130	17-21 Nov 2025			Dallas , US