3GPP TSG-RAN3 Meeting #129	R3-255338

Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

Agenda item:	20.2

Source:	Xiaomi

Title:	(Draft reply LS out) Logged data handling during handover

Document for:	other

1	Introduction

This is an LS on Logged Data Handling During Handover in [1] from RAN2 to RAN3.

Regarding the handling of logged data collected for NW-side AI/ML model training at handover RAN2 has made the following agreements: At RAN2#127bis: UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection. FFS of details of the message The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest. FFS trigger/definition of availability indication. and FFS how data availability indication is sent to the network. At RAN2#129: - UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE). - As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT. At RAN2#130: - (RRC-18) 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration with synch is introduced. In case of HO, the source sends the 1-bit indication to target cell in HandoverPreparationInformation message. This 1-bit indication is included in HO command by target cell (if the target cell wants to keep the data). We should have single UE behaviour, when it receives the indication it keeps it, otherwise it removes it. Notify RAN3 - The UE should report the CGI of the serving cell whenever feasible. If CGI is unavailable, the UE shall log PCI-ARFCN as a fallback. RAN2 would like to clarify that the data collection configuration could originate in the source gNB or from OAM. RAN2 would also like to clarify that the purpose of data availability indication in RRCReconfigurationComplete message sent during handover is for the target gNB to be able to fetch the collected data from the UE. RAN2 respectfully asks RAN3 to take the agreement into account, and provide concern, if any, on RAN2 agreement. 2. Actions: To RAN3 ACTION: RAN2 respectfully asks RAN3 to take the agreement into account, and provide concern, if any, on RAN2 agreement.

According to above information, there may be two parts need RAN3 confirm.

- logged data retention during handover

- logged data forwarding after handover.

2	Discussion

Logged data retention during handover

RAN2 agreement In case of HO, the source sends the 1-bit indication to target cell in HandoverPreparationInformation message. This 1-bit indication is included in HO command by target cell (if the target cell wants to keep the data)

According to the RAN2 agreement, a 1-bit indication has been included in the HandoverPreparationInformation to allow the target cell to signal to the UE whether to retain data during the handover command. The impacted signaling aspects have been defined by RAN2.

RAN3 may consider whether procedural text is needed to describe the target gNB behavior. However, based on the current RAN3 specifications, there is no procedural text defined for RRC context information, suggesting that additional procedure text for this purpose may not be necessary.

Observation 1, There are no concerns or specification impacts in RAN3 regarding support for the data retention indication.

Logged data forwarding during handover

RAN2 would like to clarify that the data collection configuration could originate in the source gNB or from OAM. RAN2 would also like to clarify that the purpose of data availability indication in RRCReconfigurationComplete message sent during handover is for the target gNB to be able to fetch the collected data from the UE.

From the RAN2 perspective, data collection is initiated by either the source gNB or OAM, which can be regarded as the data collection termination entity. If the target gNB receives collected data reported by the UE, how to forward it to the source gNB or OAM can be left to gNB implementation, especially considering this is the final RAN3 meeting for Rel-19 topics.

Observation 2, How the target gNB forwards the collected data from UE to the source gNB or OAM can be left to gNB implementation.

Proposal 1, RAN3 reply to RAN2 that there’s no concerns from RAN3 perspective and the collected data forwarding can be left to gNB implementation in Rel-19.

An draft reply LS is provided in the Annex.

3	Conclusion

In this contribution, we discuss the RAN2 LS on logged data handling during handover and have the following observations and proposal:

Observation 1, There are no concerns or specification impacts in RAN3 regarding support for the data retention indication.

Observation 2, How the target gNB forwards the collected data from UE to the source gNB or OAM can be left to gNB implementation.

Proposal 1, RAN3 reply to RAN2 that there’s no concerns from RAN3 perspective and the collected data forwarding can be left to gNB implementation in Rel-19.

4	Reference

[1] R3-255016, Logged Data Handling During Handover, RAN2(Nokia)

5	Annex (Draft LS reply)

Title:	Reply LS on Logged Data Handling During Handover

Response to:	R3-255016 (R2-2504950)

Release:	Release 19

Work Item:	NR_AIML_air-Core

Source:	RAN3

To:	RAN2

Cc:

Contact person:	Lisi Li

lilisi@xiaomi.com

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

RAN3 thanks RAN2 LS on Logged Data Handling During Handover.

There is no concerns from RAN3 to support the logged data handling during handover, the collected data forwarding can be left to gNB implementation in Rel-19.

2	Actions

To RAN2

ACTION: 	RAN3 respectfully requests RAN2 to take above replies into account.

3	Dates of next TSG RAN WG 3 meetings

TSG-RAN3 Meeting #129-bis	13-17 Oct 2025			Prague, CZ

TSG-RAN3 Meeting #130	17-21 Nov 2025			Dallas , US