3GPP TSG-RAN WG3 Meeting #129	R3-255345

Bangaluru, IND, 25th – 29th August, 2025

Agenda item:	15.2

Source: 	Samsung

Title: 	Remaining issue on support of regenerative payload for IoT NTN

Document for:	Discussion and decision

Introduction

RAN3#127 meeting discussed the support of regenerative payload for IoT NTN, and the following common understanding is achieved,

No support on the multiple SCTP association for IoT NTN.

In this contribution, we continue to discuss the support of regenerative payload for IoT NTN.

Discussion

The remaining open issue to discuss is based on the progress of NR NTN.

According to the progress of NR NTN achieved last meeting, companies agreed to capture an indicator in NGAP RAN CONFIGURATION UPDATE message in the latest BL CR to 38.413 and 38.300 as follows,

BL CR to 38.413 in RAN CONFIGURATION UPDATE message:

| Downlink NG Transmission Control | O |  | ENUMERATED (suspend, resume, …) | Indicates the CN to suspend or resume the downlink NG Transmission | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

Editor’s Note: FFS whether this IE is needed.

BL CR to 38.300:

For regenerative payload, an indication to suspend and resume data and signaling procedures over NG interface may be sent to AMF from gNB in the RAN Configuration Update procedure when the hard feeder link switch over occurs.

Editor’s Note: FFS whether this text is needed.

It can be observed that, the BL CRs capture the suspend/resume indicator mainly for the purpose of avoiding DL transmission loss in case of hard feeder link switch over, which in our understanding is a valid case to introduce the suspend/resume indicator in RAN CONFIGURATION UPDATE message.

For IoT NTN, when the hard feeder link switch over happens, the similar issue is also possibly to happen over S1-C and S1-U, so we think it is also needed to introduce a suspend/resume indicator in S1AP ENB CONFIGURATION UPDATE message to avoid DL packet loss in case of hard feeder link switchover.

In addition, similarly, whether a suspend/resume indicator is needed for S1 Suspend/Resume can also be discussed, and in our understanding we can also adopt a suspend/resume indicator in ENB CONFIGURATION UPDATE message to support S1 Suspend/Resume. The detailed reason can be found in our contribution submitted to NR NTN WI.

Briefly speaking, the benefit of introducing suspend/resume indicator to support S1 Suspend/Resume is clear: The TA supported list in S1 SETUP REQUEST message is mandatory while optional in ENB CONFIGURATION UPDATE message, which means S1 SETUP REQUEST message has to send TA supported list; under such case, the signalling overhead can be saved on TA supported list if the eNB sends ENB CONFIGURATION UPDATE message with one resume indicator.

As a result, from saving signalling overhead point of view, it is beneficial to introduce one suspend/resume indicator in ENB CONFIGURATION UPDATE message.

Proposal 1: Agree to introduce a suspend/resume indicator in ENB CONFIGURATION UPDATE message, for the following purposes,

- gNB on-board leaves/enters the AMF service area

- Avoid DL packet loss in case of hard FLSO.

If we want to adopt such indicator for multiple purposes, the IE name should be general enough.

Conclusion

In this paper, we provide primary discussion on support of regenerative payload for IoT NTN. The following proposals are provided,

Proposal 1: Agree to introduce a suspend/resume indicator in ENB CONFIGURATION UPDATE message, for the following purposes,

- gNB on-board leaves/enters the AMF service area

- Avoid DL packet loss in case of hard FLSO.

References