3GPP TSG-RAN WG3 Meeting #129	R3-255347

Bangaluru, IND, 25th – 29th August, 2025

Agenda item:	16.2

Source: 	Samsung

Title: 	(TP to BL CR for 38.300 and 38.413) Discussion on inventory complete and session release

Document for:	Discussion and decision

Introduction

Last meeting discussed the inventory complete and session release, and the following agreements and open issues are captured,

Introduce new CN triggered Class 1 Session Release procedure, which includes A-IoT SESSION RELEASE REQUEST message and A-IoT SESSION RELEASE RESPONSE message.

In the case of Inventory only scenario, introduce Inventory Complete indication to inform the AIoT CN about the complete of the triggered Inventory session. FFS on the detail about Inventory Complete indication via new procedure or introducing indication IE in current Inventory Report message.

In the case of Command after Inventory scenario, whether need to introduce the complete indication needs to be further checked.

Introduce a class2 procedure to allow NG-RAN node to trigger Session Release procedure towards AIoT CN.

In this contribution, we further discuss these leftover issues.

Discussion

Last meeting discussed the inventory complete and session release, and there are still several open issues left.

Firstly we’d like to start with the following open issue,

In the case of Command after Inventory scenario, whether need to introduce the complete indication needs to be further checked.

Recall that in current 23.369, SA2 has introduced a Time Interval used for Service Result aggregation as Inventory Assistance Information. If the complete indication is not introduced for Command after Inventory scenario, the AIOTF will still expect the periodic Inventory Report containing the inventoried devices during the Time Interval, but the AIOTF actually will not receive any Inventory Report containing the inventoried devices at all, which will cause mismatch between AIOTF and gNB. As a result, we see benefit to introduce the inventory complete indication in the case of Command after Inventory scenario also.

Proposal 1: Introduce the Inventory Complete Indication also in case of Command after Inventory scenario.

The next open issue is,

In the case of Inventory only scenario, introduce Inventory Complete indication to inform the AIoT CN about the complete of the triggered Inventory session. FFS on the detail about Inventory Complete indication via new procedure or introducing indication IE in current Inventory Report message.

According to the discussion last meeting, basically there are three options on the table,

- Option1: The Inventory Complete Indication is included in current Inventory Report message, and update the presence of Reader Report List from mandatory to optional.

- Option2: The Inventory Complete Indication is included in the SESSION RELEASE REQUEST message, and such message is used not only for indicating inventory complete, but also for requesting the AIoT session release. So the message name should be updated to AIOT SESSION NOTIFY message.

- Option3: The Inventory Complete Indication is included in a new Class2 Inventory Complete procedure.

The motivation to propose Option3 is because the Reader Report List is mandatorily provided in Inventory Report Transfer IE in current BLCR to 38.413, and it is also possible that the Reader sends the Inventory Complete Indication only without any Inventoried Device information.

We acknowledge the case mentioned above may happen, but Option1 will also work after updating the presence of Reader Report List from mandatory to optional. As a result, we do not see clear benefit of Option3, let alone Option3 will introduce new additional Class2 procedure which can be avoided. Therefore, compared to Option3, Option1 is better.

For the comparison between Option1 and Option2, if gNB is able to identify the inventory complete as soon as the last device is inventoried, Option1 is able to signal the Reader Report List as well as Inventory Complete Indication in one INVENTORY REPORT message; while Option2 has to send two messages. So in some situation Option1 is better, and we do not see other clear benefit of Option2 compared to Option1.

As a summary, we prefer Option1.

Proposal 2: The Inventory Complete Indication is included in current Inventory Report message, and update the presence of Reader Report List from mandatory to optional.

The last open issue is whether the AIOT CN will always has the final say on the Session Release, which means that the gNB will not consider the session is released until the Class1 AIoT Session Release procedure is performed.

In our understanding, for Command after Inventory procedure, it is fine that the AIOT CN has the final say because only the AIOT CN understands whether the Command procedure is completed or not.

However, for Inventory only procedure, the gNB itself can decide when the inventory procedure is completed, and the gNB is able to send Inventory Complete Indication after determining the inventory procedure is completed. After receiving the Inventory Complete Indication for inventory only procedure, the AIOTF has no other choice but to release the session context. So we do not see the real need for the AIOTF to initiate another round of Class1 AIoT Session Release procedure to perform the session release again.

As a result, we support the solution that for Inventory only procedure, the gNB can release the AIoT session context after sending the Inventory Complete Indication, and the AIOTF can also release the AIOT session context after receiving the Inventory Complete Indication without triggering AIoT Session Release procedure.

It should be noted that in current NGAP spec, similar mechanism has already been adopted for e.g. PDU Session Resource Notify procedure, wherein the NG-RAN node is able to release QoS flows using QoS Flow Released List in PDU Session Resource Notify Transfer IE, or release the PDU session resource using PDU Session Resource Notify Released Transfer IE, without another round of PDU Session Resource Release procedure initiated by AMF.

In addition, no spec has ever mentioned that the AMF needs to store any AIoT Session Context for indirect deployment; and in our understanding, the only thing the AMF needs to store is the association between AIOTF identifier and a specific AIOTF, but such association information is irrelevant to any specific AIoT Session Context.

Proposal 3: For Inventory only procedure, the gNB can release the AIoT session context after sending the Inventory Complete Indication, and the AIOTF can also release the AIOT session context after receiving the Inventory Complete Indication without triggering AIoT Session Release procedure.

Proposal 4: Agree TPs for 38.300 and 38.413 as in Annex.

Conclusion

In this paper, we provide further discussion on inventory complete and session release. The following proposals are provided,

Proposal 1: Introduce the Inventory Complete Indication also in case of Command after Inventory scenario.

Proposal 2: The Inventory Complete Indication is included in current Inventory Report message, and update the presence of Reader Report List from mandatory to optional.

Proposal 3: For Inventory only procedure, the gNB can release the AIoT session context after sending the Inventory Complete Indication, and the AIOTF can also release the AIOT session context after receiving the Inventory Complete Indication without triggering AIoT Session Release procedure.

Proposal 4: Agree TPs for 38.300 and 38.413 as in Annex.

Annex A: TP to BL CR for TS 38.300

16.xx.x3	Inventory procedure

Figure 16.xx.x3-1 depicts the basic communication between the gNB and the A-IoT CN node for the Inventory procedure.

Figure 16.xx.x3-1: Inventory procedure

1.	The A-IoT CN node initiates the Inventory procedure over NG-C by sending the Inventory Request message to the gNB. The Inventory Request message includes a Correlation ID and A-IOTF Identifier to identify the inventory session. The Inventory Request message also includes an A-IoT Device Identification Requested corresponding to the A-IoT device(s) which are targeted for the inventory. The A-IoT Device Identification Requested may indicate inventory for a single device, a group of devices or for all devices.

The Inventory Request message also includes Requested Service Area Information (list of A-IoT Area IDs and/or reader list), the Expected D2R message size and may include Inventory Assistance Information e.g. Approximate number of Target A-IoT devices, Time Interval.

The Inventory Request message may also include a follow-on command indicator to indicate that at least for one of the targeted A-IoT device(s) a Command procedure will follow.

2.	The gNB allocates and co-ordinates the usage of A-IoT radio resources.

3.	The gNB confirms the request from the A-IoT CN node by replying with the Inventory Response message. The Inventory Response message includes the AIOTF Identifier and the Correlation ID received in the Inventory Request message.

NOTE 2:	If the gNB is not able to perform the Inventory procedure, it rejects the request and sends an Inventory Failure message to the A-IoT CN node.

4.	The gNB performs the Inventory procedure towards the A-IoT device(s) over the A-IoT radio interface.

5./6.	Upon receiving the inventory result from the A-IoT device(s), the gNB sends the Inventory Report message to the A-IoT CN node. If the Inventory procedure concerns multiple A-IoT devices, multiple Inventory Report messages may be sent to the A-IoT CN node.

The Inventory Report message includes the AIOTF Identifier and the Correlation ID received in the Inventory Request message and further includes the A-IoT NAS PDU(s) carrying the inventoried A-IoT device ID(s).

If a follow-on Command indicator was received at step 1, the Inventory Report message also includes a RAN A-IoT device NGAP ID for each A-IoT device which was inventoried.

NOTE 3:	If the Inventory procedure concerns multiple A-IoT devices, an Inventory Report may include reports from multiple A-IoT devices.

7.	Upon completion of the inventory procedure the gNB may send an Inventory Complete Indication in Inventory Report message. If a follow-on Command indicator was not received at step 1, the gNB shall release all the context and resources related to the A-IoT session after this step.

<< Skip Unchanged >>

### 16.xx.x6	gNB initiated A-IoT Session Release procedure

Figure 16.xx.x6-1: gNB initiated A-IoT Session Release procedure

1.	The gNB initiates the A-IoT Session Release Request procedure over NG-C by sending the A-IoT Session Release Request message to the A-IoT CN node in order to request the release of an A-IoT session. The A-IoT Session Release Request message includes the Correlation ID and the AIOTF Identifier to identify the session requested to be released.

2.	The A-IoT CN node accepts the request and triggers the CN initiated A-IoT Session Release procedure.

3.	The CN initiated A-IoT Session Release procedure is performed as described in section 16.xx.x5.

Annex B: TP to BL CR for TS 38.413

8.xx.5	A-IoT Session Release Request

8.xx.5.1	General

The purpose of the A-IoT Session Release Request procedure is to enable the NG-RAN node to request the A-IoT CN node to release the A-IoT session. This procedure applies only if the NG-RAN node is a gNB. The procedure uses non-UE associated signalling.

Editor’s Note:	The name of the procedure, the name of the messages and the purpose of the procedure are FFS.

8.xx.5.2	Successful Operation

Figure 8.xx.5.2-1: A-IoT Session Release Request

The NG-RAN node initiates the procedure by sending an AIOT SESSION RELEASE REQUEST message to the A-IoT CN node.

Interactions with the A-IoT Session Release procedure:

Upon reception of the AIOT SESSION RELEASE REQEUST message, the A-IoT CN node should initiate the A-IoT Session Release procedure for the A-IoT session denoted by the AIOTF Identifier IE and the Correlation ID contained in the AIOT SESSION RELEASE REQUEST message.

8.xx.5.3	Abnormal Conditions

Void.

<< Skip Unchanged >>

9.3.x.4	Inventory Report Transfer

This IE provides the inventory report related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  |
| Global gNB ID | M |  | 9.3.1.6 |  |
| Reader Report List |  | 0..1 |  |  |
| >Reader Report Item |  | 1..< maxnoofReaders> |  |  |
| >>Reader Index | M |  | 9.3.1.aa1 |  |
| >>Device Report List |  | 1 |  |  |
| >>>Device Report Item |  | 1..<maxnoofDevices> |  |  |
| >>>>A-IoT NAS PDU | M |  | OCTET STRING |  |
| >>>>RAN A-IoT Device NGAP ID | O |  | 9.3.3.yy1 |  |
| Inventory Complete Indication | O |  | ENUMERATED (true, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofReaders | Maximum no. of Readers served by a gNB. Value is 65535. |
| maxnoofDevices | Maximum no. of Devices within a reader. Value is 65535. |

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

A2X-PC5-QoS-Parameters,

AerialUEsubscriptionInformation,

AIoT-CorrelationIdentifier,

AIOTFIdentifier,

AIOTSessionReleaseCommandTransfer,

AIOTSessionReleaseCompleteTransfer,

AIOTSessionReleaseRequestTransfer,

AIoT-Support,

AllowedNSSAI,

<< Skip Unchanged >>

InventoryReportTransfer ::= SEQUENCE {

correlationIdentifier		AIoT-CorrelationIdentifier,

globalgNB-ID				GlobalGNB-ID,

readerReportList			AIoT-ReaderReportList													OPTIONAL,

inventoryCompleteIndication	ENUMERATED {true, ...}													OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { InventoryReportTransfer-ExtIEs} }		OPTIONAL,

...

}

InventoryReportTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}