3GPP TSG-RAN WG3 Meeting #129	R3-255355

Bengaluru, India, 25th – 29th August, 2025

Title: 	(TP for AI/ML BLCR to TS 38.401) Capturing agreements in Stage 2 RAN specifications

Source: 	Huawei, Jio Platforms, Deutsche Telekom, FiberCop, InterDigital, 		CATT

Agenda item:	11.2

Document Type:	Discussion and decision

# 1. Introduction

RAN3 is discussing the objectives of the Rel-19 WI “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”[1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [2] and whose outcome is summarized in TR 38.743 [3].

In the last meeting RAN3 agreed a Stage 2 TP for TS 38.401 in [8], which became the BLCR for that specification, covering the agreements related to the OAM requirements for Energy Cost reporting in split architecture. This contribution provides a TP for the AI/ML BLCR to TS 38.401 in [9] capturing additional RAN3 agreements which are relevant for Stage 2 description of Rel-19 AI/ML for NG-RAN.

# 2. Discussion

## 2.1 RAN3 agreements relevant for the AI/ML BLCR to TS 38.401

***** RAN3#125bis ***** [4] AI/ML-based Coverage and Capacity Optimization WA: Reusing the CU configuration update procedure to transfer the predicted CCO issue and reusing the DU configuration update procedure to transfer the Future Coverage State over F1. Introducing separate time information for future coverage state and predicted CCO issue. Rel-18 leftovers: Split architecture support for Rel-18 use cases Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 interface to transfer measured EC from DU to CU. ***** RAN3#126 ***** [5] AI/ML-based Coverage and Capacity Optimization Change WA to agreement: Transferring the predicted CCO issue via the CU configuration update procedure and transferring the Future Coverage State via the DU configuration update procedure over F1. Introducing new IE for Future Coverage Modification Notification over F1. Introducing new IE for predicted CCO assistance information over F1. Rel-18 leftovers: Split architecture support for Rel-18 use cases In split architecture, the Energy Cost of gNB is the sum of the Energy Cost of its gNB-DUs. Mapping rule is only configured at gNB-DU and shall be unified for all the gNB-DUs within a defined area. ***** RAN3#127bis ***** [6] Rel-18 leftovers: Split architecture support for Rel-18 use cases Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over E1 interface to transfer UL/DL UE throughput and UL/DL packet delay from CU-UP to CU-CP upon request. Change the agreement: Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 interface to transfer measured EC from DU to CU. To the following WA: WA: Using the Legacy procedure, i.e., resource status initiation procedure & resource status report procedure to request and report measured EC over F1 interface. ***** RAN3#128 ***** [7] Rel-18 leftovers: Split architecture support for Rel-18 use cases Introduce new IE to include DL and UL Packet Loss Rate at PDCP Level in R19 refer to SA5 spec DL/UL UE Throughput Measurement will not be supported in split architecture in R19. Turn WA to agreement: Using the Legacy procedure, i.e., resource status initiation procedure & resource status report procedure to request and report measured EC over F1 interface.

It is proposed to agree the TP to AI/ML BLCR to TS 38.401 in Annex 1.

Proposal: RAN3 to agree the TP to AI/ML BLCR to TS 38.401 in Annex 1.

# 3. Conclusion

We make the following proposal:

Proposal: RAN3 to agree the TP to AI/ML BLCR to TS 38.401 in Annex 1.

# 4. Reference

- RP-250326, “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#107

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- 3GPP RAN3#125b Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM.doc)

- 3GPP RAN3#126 Chairman’s notes (RAN3_126_agenda_20241122_EOM1.doc)

- 3GPP RAN3#127bis Chairman’s notes (RAN3_127bis_agenda_20250415_EOM1.doc)

- 3GPP RAN3#128 Chairman’s notes (RAN3_128_agenda_20250523_1530_EOM.doc)

- R3-253949, (TP for BLCR to TS 38.401) OAM Requirements for Energy Saving for split architecture, CMCC, Samsung, Huawei, NEC, CATT, ZTE, Nokia, Ericsson, Deutsche Telekom, FiberCop, Jio Platforms

- R3-255043, (BL CR to 38.401) Enhancement of AI/ML Energy Saving for NGRAN split architecture, CMCC, Samsung, Huawei, NEC, CATT, ZTE, Nokia, Ericsson, Deutsche Telekom, FiberCop, Jio Platforms

# 5. Annex 1 – TP for AI/ML BLCR to TS 38.401 (based on R3-255043)

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

# 7.11	Support of AI/ML for NG-RAN

### 7.11.1	General

The support of AI/ML for NG-RAN is specified in TS 38.300 [2].

In case of CU-DU split architecture, the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

### 7.11.2	Data Collection and Reporting

The following information can be configured to be reported by the gNB-DU to the gNB-CU(-CP):

-	Energy Cost (EC);

For a split gNB, the EC of the gNB is the sum of the EC index values of its gNB-DUs.

The following information can be configured to be reported per DRB per UE by the gNB-CU-UP to the gNB-CU-CP:

-	Average Packet Delay;

-	Average Packet Loss DL;

-	Average Packet Loss UL.

The collection and reporting of information by the gNB-DU are configured through the Resource Status Reporting Initiation procedure, while the actual reporting is performed through the Resource Status Reporting procedure.

The collection and reporting of information by the gNB-CU-UP are configured through the Data Collection Reporting Initiation procedure, while the actual reporting is performed through the Data Collection Reporting procedure.

Predicted CCO issue with the list of cells/SSBs predicted to be affected and the associated timing information are transferred to the gNB-DU via the gNB-CU Configuration Update procedure, while the future coverage states of cells/SSBs predicted to be affected with the associated timing information are transferred to the gNB-CU(-CP) via the gNB-DU Configuration Update procedure for, e.g., resolution of predicted CCO issues.

### 7.11.3	OAM requirements

OAM configures the following on gNB-DU:

-	the minimum and maximum energy consumption values corresponding to the minimum and maximum EC index values respectively, based on an implementation-specific mapping rule, which is unified within a defined area.

-	the recommended time interval within which a gNB-DU selects an implementation-specific time window for averaging of the measurements of the gNB-DU's consumed energy.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>