3GPP TSG-RAN WG3 Meeting #129	R3-255358

Bangalore, India, 25th – 29th August, 2025

Title: 	(TP for AI/ML BLCR to TS 38.423) Per-UE per-slice UE performance in AI/ML-based Network Slicing

Source: 	Huawei, Ofinno, Jio Platforms, FiberCop, BT

Agenda item:	11.4

Document Type:	Discussion and Decision

# 1. Introduction

This is a TP for the AI/ML BLCR to TS 38.423 [1] reflecting proposals in our paper [2].

# 2. Reference

- R3-255044, (BL CR to 38.423) Support of enhancements on AI/ML for NG-RAN, ZTE Corporation, Qualcomm, China Unicom, Ericsson, Samsung, Nokia, Lenovo, CATT, Huawei, NEC, CMCC, China Telecom, Telecom Italia, Deutsche Telekom, Verizon Wireless, LGE, Jio Platforms (JPL), Rakuten, Orange, FiberCop, InterDigital, Ofinno

- R3-255357, Further discussion on per-UE per-slice UE performance in AI/ML-based Network Slicing, Huawei, Jio Platforms, FiberCop, BT

# 3. TP for BLCR to TS 38.423 for AI/ML-based Network Slicing (based on R3-255044)

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.x4	Slice Based UE Performance

This IE represents UE performance metrics per S-NSSAI.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Average Slice UE Throughput DL | O |  | Bit Rate 9.2.3.4 | Corresponds to Average UE Throughput DL per S-NSSAI as specified in TS 28.558 [XX] clause 6.3.1.4.1. |
| Average Slice UE Throughput UL | O |  | Bit Rate 9.2.3.4 | Corresponds to Average UE Throughput UL per S-NSSAI as specified in TS 28.558 [XX] clause 6.3.1.4.2. |
| Average Slice Packet Delay | O |  | Average Packet Delay 9.2.3.187 | Corresponds to the Average Packet Delay per S-NSSAI as specified in TS 28.558 [XX], clause 6.3.1.1. |
| Average Slice Packet Loss DL | O |  | INTEGER (0..10000000, ...) | Ratio of lost packets per number of packets sent, expressed in units of 10-8 percent. Corresponds to DL Packet Loss Rate on Uu per S-NSSAI as specified in TS 28.558 [XX] clause 6.3.1.2.1. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

**** skip unmodified parts ****

#### -- A

AveragePacketLoss ::= INTEGER (0..10000000, ...)

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### -- S

**** skip unmodified parts ****

SliceUEPerformance ::= SEQUENCE {

pLMNIdentity								PLMN-Identity,

s-NSSAIUEPerformance-List					S-NSSAIUEPerformance-List,

iE-Extensions								ProtocolExtensionContainer { { SliceUEPerformance-ExtIEs} } OPTIONAL,

...

}

SliceUEPerformance-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

S-NSSAIUEPerformance-List ::= SEQUENCE (SIZE(1..maxnoofSliceItems)) OF S-NSSAIUEPerformance-List-Item

S-NSSAIUEPerformance-List-Item	::= SEQUENCE {

sNSSAI								S-NSSAI,

sliceBasedUEPerformance				SliceBasedUEPerformance,

iE-Extensions						ProtocolExtensionContainer { { S-NSSAIUEPerformance -List-Item-ExtIEs} }	OPTIONAL,

...

}

S-NSSAIUEPerformance-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SliceBasedUEPerformance	::= SEQUENCE {

averageSliceUEThroughputDL			BitRate,		OPTIONAL,

averageSliceUEThroughputUL			BitRate,		OPTIONAL,

averageSliceUEPacketDelay			AveragePacketDelay,					OPTIONAL,

averageSlicePacketLossDL			AveragePacketLoss,								OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { SliceBasedUEPerformance-ExtIEs} } OPTIONAL,

...

}

SliceBasedUEPerformance-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>