3GPP TSG-RAN WG3 Meeting #129	R3-255359

Bengaluru, India, 25th – 29th August, 2025

Title: 	Discussion on timing aspects for AI/ML-based Coverage and Capacity Optimization

Source: 	Huawei, Jio Platforms, Deutsche Telekom, Orange, BT, Vodafone

Agenda item:	11.3

Document Type:	Discussion and Decision

# 1. Introduction

In the last meeting RAN3 kept on discussing the objectives of the Rel-19 WI “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [2] and whose outcome is summarized in TR 38.743 [3].

One of the objectives of the WI is to specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based CCO (Coverage and Capacity Optimization).

This contribution analyzes further the aspect related to timing information over Xn for AI/ML-based CCO, and provides proposals for discussion based on the outcome of the last RAN3 meeting.

# 2. Discussion

In the last RAN3 meeting there were further discussions on whether the time for predicted CCO issue should be transferred over the Xn interface towards the neighbour node, along with the already agreed pieces of information (that is, predicted coverage modification cause, future coverage states for the list of cells/SSBs predicted to be affected and time for future coverage states). However, there was no conclusion on such open aspect, which remains captured in the meeting minutes as follows [4]:

Timing information for predicted CCO issue over Xn is not needed?

The timing information associated to the predicted CCO issue (i.e., Time for Predicted CCO Issue IE in the F1AP BLCR [5]) is transferred from the gNB-CU to the gNB-DU so that the gNB-DU is made aware of the fact that the new CCO state to cope with the predicted CCO issue is to be applied in the future and not immediately because the CCO issue did not occur yet. Conversely, the timing information of the future CCO state (i.e., Time for Future Coverage Modification IE in the F1AP BLCR [5]) is sent from the gNB-DU to the gNB-CU so that the gNB-CU will know when the future CCO state will be activated, hence preventing (or limiting at an early stage) the occurrence of the CCO issue.

It should be straightforward to state that the time information of the future CCO state is lower than (or equal to) the time for predicted CCO issue, as it is natural to assume that once a predicted CCO issue is known to occur at a certain future time instant, the gNB-DU should derive a future CCO state to be activated before the occurrence of the predicted CCO issue, in order to avoid the CCO issue itself.

Observation 1: A reasonable implementation of the AI/ML-based CCO assumes that the Time for Future Coverage Modification is lower than the Time for Predicted CCO Issue.

Concerning the XnAP signalling, RAN3 agreed to transfer the timing information of the future CCO state (i.e., Time for Future Coverage Modification IE in the XnAP BLCR [6]) over Xn, within the NG-RAN CONFIGURATION UPDATE message. However, it needs to be further discussed whether also the time for predicted CCO issue  needs to be transferred over Xn as well.

Let us assume that we have a split gNB1 (gNB-CU1 and gNB-DU1) where the CCO issue is predicted (in gNB-CU1), and a neighbor split gNB2 (gNB-CU2 and gNB-DU2) that is also impacted by this predicted CCO issue.

In order for gNB(-DU)2 to take proper counter-measures following the future CCO state derived at gNB-DU1 side and yet to be applied, RAN3 agreed in the last meeting to introduce a new Neighbour Future Coverage Modification Notification IE in the gNB-CU CONFIGURATION UPDATE message sent by gNB-CU2 which carries the future coverage states (and corresponding timing information) derived by the gNB(-DU)1 for its cells and beams predicted to be affected by the predicted CCO issue in gNB-CU1, and sent by the gNB-CU1 within the NG-RAN CONFIGURATION UPDATE message over Xn. Therefore, at gNB-CU2 side, the only missing information to actually allow the gNB-DU2 to derive the matching future coverage state is the time for the predicted CCO issue: in fact, the predicted CCO issue as well as the future coverage states and related timing information of gNB-DU1’s cells/SSBs predicted to be affected by the issue are sent over Xn to gNB-CU2 and hence made available at gNB-CU2 side.

While the future coverage states and related timing information of gNB-DU1’s cells/SSBs predicted to be affected by the issue will be signalled to gNB-DU2 via the new Neighbour Future Coverage Modification Notification IE, the predicted CCO issue will be delivered to gNB-DU2 via the Predicted CCO Assistance Information IE which, however, also includes the time for the predicted CCO issue that cannot be set to a proper value by the gNB-CU2 as the latter didn’t receive this information over Xn.

Observation 2: The time for the predicted CCO issue is the only missing information at gNB-CU2 side to be signalled to gNB-DU2 in order for gNB-DU2 to actually derive the matching future coverage state of its cells/SSBs.

Therefore, in our view, the gNB-CU2 needs to receive over Xn the Time for Predicted CCO Issue indicating when the CCO issue is predicted to occur, so that gNB-DU2 can determine its future CCO state (matching the future CCO state of gNB-DU1) knowing that the future CCO state in gNB-DU1 will be activated at a time equal to Time for Future Coverage Modification t1 and the CCO issue in gNB-CU1 is predicted to occur at a time equal to Time for Predicted CCO Issue t0, as depicted in Figure 1.

Figure 1 - Time relation between the Time for Future Coverage Modification (from gNB-DU1) and Time for Predicted CCO Issue (from gNB-CU1).

Without the information on the point in time where the CCO issue in gNB-CU1 is predicted to occur (i.e., Time for Predicted CCO Issue t0 in Figure 1) the case as shown in Figure 2 may happen.

Figure 2 – Scenario where the Time for Predicted CCO Issue is not known at gNB-DU2.

Specifically, the gNB-DU2 may determine a matching future coverage state to be activated at a point in time (i.e., Time for Future Cell Coverage State t2 in Figure 2) that is after the point in time in the future where the CCO issue is predicted to occur (i.e., Time for Predicted CCO Issue t0 in Figure 2), simply because the gNB-DU2 knows that there will be a CCO issue in the future but it does not know when. This means that the whole AI/ML optimization for CCO is basically useless, since likely the CCO issue actually occurred and it was not possible to timely adapt the coverage states of affected cells/SSBs in gNB-DU1 and gNB-DU2 to avoid the issue.

Observation 3: Without knowing when a predicted CCO issue is expected to occur, the gNB-DU2 may derive a matching future coverage state to be activated after the point where the CCO issue is predicted to occur.

We acknowledge that, in principle, it would be better that the matching future coverage state in gNB-DU2 is activated at the same point in time where the future coverage state in gNB-DU1 is activated. But we also think that this poses a strict synchronization requirement (and hence it adds complexity to the whole mechanism) that provides negligible benefits. In fact, it should be discussed whether the negative impact on performance (network and UE) deriving from having the matching future coverage state in gNB-DU2 being activated after the activation time of the future coverage state in gNB-DU1 (but before the predicted CCO issue time, to avoid the previous scenario) is significant considering that the mismatch between the activation times would be really short in time, and very likely to be solved well before the occurrence of the predicted CCO issue, i.e., before the predicted CCO issue time.

In our view, having the future coverage states activated at the same time should be more a nice-to-have rather than a strict design requirement, as what it matters is that, at the time the issue is predicted to occur, the involved DUs have rearranged their own coverage configurations properly (to avoid the actual occurrence of the CCO issue).

Observation 4: Having “misaligned” activation of future coverage states at gNB-DU1 and at gNB-DU2, if occurs, is a temporary situation that will be solved in a short timing horizon, therefore the impact on UE and network performance is negligible.

In the last meeting, in contribution [7], it was stated that by delaying the application of matching coverage configuration at gNB-DU2, the gNB-CU1 will not know when gNB-DU2 has applied the matching coverage state. As a consequence, the mechanism of collecting and reporting of feedback information to gNB-CU1 that may be required to validate the outcome of the CCO issue prediction (selection of Future CCO state) will be negatively impacted as gNB-CU1 cannot determine the time of change of coverage configuration at gNB-DU2. In our view, this statement is based on assumptions that still need confirmation in RAN3: firstly, it is yet unclear what we want to validate by means of feedback information, that is, whether it is the predicted CCO issue or the future coverage state, and contribution [7] assumes that it is the future CCO state that is validated by such feedback information. Secondly, it seems to us that gNB-DU2 is forced to apply the matching coverage state for its cells/SSBs at the same time instant at which the future coverage state is activated by the gNB-DU1 which, to us, is an unnecessary implementation constraint for gNB-DU2 because what eventually matters is that the future coverage states at gNB-DU1 and gNB-DU2 are already activated when reaching the point in time where the CCO issue is expected to occur.

Observation 5: Whether the mechanism for collecting feedback information at gNB-CU2 is impacted by a delayed application of the matching coverage configuration at gNB-DU2 is not clear, as it is based on assumptions that still needs confirmation in RAN3.

Still on collection of UE performance feedback metrics, the gNB-CU2 would actually benefit from knowing the Time for Predicted CCO Issue in order to understand the point in time immediately after which it needs to start collecting metrics to be used as feedback information. It is our understanding that neighbor nodes need to collect and report feedback information to be transferred back to the gNB-CU1 immediately after the predicted CCO issue was expected to occur. We will discuss feedback information more in detail a separate contribution in [8].

Observation 6: The Time for Predicted CCO Issue is needed to be known at gNB-CU2 in order for gNB-CU2 to understand the point in time immediately after which it needs to start collecting feedback information.

In the last meeting, contribution [9] highlighted another reason for which the knowledge of the Time for Predicted CCO Issue at the gNB-CU2 side is beneficial: if multiple predicted CCO issues from multiple gNB-CUs are provided to the same gNB-CU2 (acting as neighbour node), then receiving the time information related to each predicted CCO issue from each gNB-CU over Xn can be used by the gNB-CU2 to understand which predicted CCO issue has precedence over others, i.e., when it is more appropriate to apply a certain future CCO state (to cope with the predicted CCO issue in the “most near” future) or another.

Observation 7: By knowing the Time for Predicted CCO Issue, the gNB-CU2 can cope with the most imminent predicted CCO issue (with gNB-DU2 taking corresponding actions) in case multiple predicted CCO issues from multiple gNB-CUs are received simultaneously.

Based on the above observations, we propose the following:

Proposal: RAN3 to agree that, for the timing information, the gNB(-CU) which predicted the CCO issue sends to a neighbour gNB(-CU) over Xn also the Time for Predicted CCO Issue.

A TP to AI/ML BLCR to TS 38.423 reflecting the above proposal can be found in [10].

# 3. Conclusion

Based on the above discussion, we have the following observations and proposal:

Observation 1: A reasonable implementation of the AI/ML-based CCO assumes that the Time for Future Coverage Modification is lower than the Time for Predicted CCO Issue.

Observation 2: The time for the predicted CCO issue is the only missing information at gNB-CU2 side to be signalled to gNB-DU2 in order for gNB-DU2 to actually derive the matching future coverage state of its cells/SSBs.

Observation 3: Without knowing when a predicted CCO issue is expected to occur, the gNB-DU2 may derive a matching future coverage state to be activated after the point where the CCO issue is predicted to occur.

Observation 4: Having “misaligned” activation of future coverage states at gNB-DU1 and at gNB-DU2, if occurs, is a temporary situation that will be solved in a short timing horizon, therefore the impact on UE and network performance is negligible.

Observation 5: Whether the mechanism for collecting feedback information at gNB-CU2 is impacted by a delayed application of the matching coverage configuration at gNB-DU2 is not clear, as it is based on assumptions that still needs confirmation in RAN3.

Observation 6: The Time for Predicted CCO Issue is needed to be known at gNB-CU2 in order for gNB-CU2 to understand the point in time immediately after which it needs to start collecting feedback information.

Observation 7: By knowing the Time for Predicted CCO Issue, the gNB-CU2 can cope with the most imminent predicted CCO issue (with gNB-DU2 taking corresponding actions) in case multiple predicted CCO issues from multiple gNB-CUs are received simultaneously.

Proposal: RAN3 to agree that, for the timing information, the gNB(-CU) which predicted the CCO issue sends to a neighbour gNB(-CU) over Xn also the Time for Predicted CCO Issue.

A TP to AI/ML BLCR to TS 38.423 reflecting the above proposal can be found in [10].

# 4. Reference

- RP-251245, “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#108

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- 3GPP RAN3#128 Chairman’s notes (RAN3_128_agenda_20250023_1530_EOM.doc)

- R3-255046, (BL CR to 38.473) Support of enhancements on AI/ML for NG-RAN, Ericsson (Rapporteur), ZTE, NEC, Nokia, Huawei, Ofinno

- R3-255044, (BL CR to 38.423) Support of enhancements on AI/ML for NG-RAN, ZTE Corporation, Qualcomm, China Unicom, Ericsson, Samsung, Nokia, Lenovo, CATT, Huawei, NEC, CMCC, China Telecom, Telecom Italia, Deutsche Telekom, Verizon Wireless, LGE, Jio Platforms (JPL), Rakuten, Orange, FiberCop, InterDigital, Ofinno

- R3-253531, (TP for BLCR to TS 38.473) Discussion on open points for AI/ML-based CCO, Nokia

- R3-255361, Discussion on feedback information for AI/ML-based Coverage and Capacity Optimization, Huawei, Jio Platforms, Deutsche Telekom, Orange, Vodafone

- R3-253426, AIML enabled CCO - Multiple CCO issues, Ericsson, Jio Platforms, Deutsche Telekom, InterDigital

- R3-255360, (TP for AI/ML BLCR to TS 38.423) Timing information for predicted CCO issue over Xn, Huawei, Jio Platforms, Deutsche Telekom, Orange, BT, Vodafone