3GPP TSG-RAN WG3 Meeting #129	R3-255363

Bengaluru, India, 25th – 29th August, 2025

Title:	Discussion on prediction update/cancel for AI/ML-based Coverage and Capacity Optimization

Agenda Item:	11.3

Source:	Huawei, Jio Platforms, Vodafone, Deutsche Telekom, Orange

Document for:	Discussion

# 1	Introduction

In the last meeting RAN3 kept on discussing the objectives of the Rel-19 WI “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [2] and whose outcome is summarized in TR 38.743 [3].

One of the objectives of the WI is to specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split and split architectures) for AI/ML-based Coverage and Capacity Optimization (CCO).

In the paper we discuss how to specify a mechanism to cancel/update a previously provided CCO-related prediction in case a detected CCO issue occurs or another CCO-related prediction is made available.

# 2	Discussion

## 2.1 Background

In the last RAN3 meeting [4] the following was agreed:

Specify mechanisms (F1, Xn) to update/cancel a prediction, no new IE is needed to identify the previous prediction (affected cells and beams will identify the previously signalled issue). Addition of new code-point in predicted coverage modification cause (Xn) / predicted CCO issue (F1), with details FFS.

In this paper we make an attempt to list the scenarios which may require a modification of a previously provided CCO-related prediction (e.g., predicted CCO issue) due to the occurrence of a detected CCO issue or because another CCO-related prediction is provided.

In the latest version of the F1AP BLCR for Rel-19 RAN AI/ML in [5] the Predicted CCO Assistance Information IE is defined as follows:

#### 9.3.1.A Predicted CCO Assistance Information This IE provides predicted assistance information for the future Coverage and Capacity Optimisation (CCO) actions for predicted CCO issues. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Predicted CCO issue; O; ENUMERATED (coverage, cell edge capacity, ...); Indicates the type of predicted CCO issue.. Predicted Affected Cells and Beams; O; Affected Cells and Beams 9.3.1.212. Time for Predicted CCO Issue; O; INTEGER (1..60, ...); Indicates the time when the predicted CCO issue will happen from the time of receiving this information, in seconds..

In our understanding, and based on the content of the above Predicted CCO Assistance Information IE, it should be possible to modify a prediction in terms of either predicted CCO issue type, and/or list of cells/SSBs predicted to be affected, and/or time for predicted CCO issue.

Observation 1: It should be possible to modify a prediction in terms of either predicted CCO issue type, and/or list of cells/SSBs predicted to be affected, and/or time for predicted CCO issue.

We will now discuss in the next section the scenarios of interest for a CCO-related prediction modification.

## 2.2 Scenarios for prediction modification

Scenario A – a detected CCO issue occurs for at least one cell/SSB predicted to be affected by the previously provided predicted CCO assistance information

Let’s consider a predicted CCO assistance information (i.e., predicted CCO issue type, list of predicted affected cells/SSBs and time for predicted CCO issue) that has already been provided by the gNB-CU hosting the AI/ML model to the gNB-DU(s) controlling the cells/SSBs predicted to be affected. Let’s now assume that an actual CCO issue is detected by the gNB-CU which affects at least one cell/SSB among the cells/SSBs which were predicted to be affected by the predicted CCO issue. As a general principle, a detected issue should be prioritized with respect to a predicted CCO issue since it is occurring at the present time and hence it should be resolved as soon as possible to avoid further negative impacts on UE and network performance.

Observation 2: A detected CCO issue affecting at least one cell/SSB among the cells/SSBs predicted to be affected by the predicted CCO issue should be prioritized with respect to the predicted CCO issue and solved as soon as possible.

It should be acknowledged that addressing a detected CCO issue requires a re-arrangement of the network situation (CCO state changes for impacted cells/SSB were applied to solve the detected issue), which always implies that the previous predicted CCO assistance information becomes invalid. Therefore, after resolving the detected CCO issue, the AI/ML model in the CU should output new predicted CCO assistance information, and we foresee the following two cases:

- Case A-1: after the detected CCO issue has been resolved, the model in gNB-CU predicts that there will not be any CCO issue for all the cells/SSBs previously predicted to be affected, due to the re-arrangement of the network situation needed to solve the detected CCO issue. Therefore, the previous predicted CCO assistance information provided over F1 to gNB-DU (and over Xn to the neighbor gNB) needs to be cancelled;

- Case A-2: after the detected CCO issue has been resolved, the model in gNB-CU predicts that there will be a CCO issue for some of (or all) the cells/SSBs previously predicted to be affected, which means that the re-arrangement of the network situation following the resolution detected CCO issue resulted in new predicted CCO assistance information which will still affect some (or all) cells/SSBs previously predicted to be affected. Therefore, the previous prediction needs to be replaced.

Proposal 1: RAN3 to agree that if a CCO issue is detected, and if it impacts at least one cell/SSB among the cells/SSBs which were predicted to be affected by the predicted CCO issue previously signalled, the detected CCO issue resolution takes precedence over the predicted CCO issue and it is solved immediately.

Proposal 2: RAN3 to agree that, if the AI/ML model in the gNB-CU provides new predicted CCO assistance information after the detected CCO issue resolution, and if there will not be any issue impacting the cells/SSBs which were predicted to be affected by the previous prediction, the previous predicted CCO assistance information needs to be cancelled. Otherwise, the previous predicted CCO assistance information needs to be replaced.

Scenario B – a detected CCO issue occurs for none of the cells/SSBs signalled in the predicted CCO assistance information

Similarly as before, let’s now assume that an actual CCO issue is detected which affects other cells/SSBs not predicted to be affected by the previous prediction.

Also in this scenario, as a general principle, a detected issue should be prioritized with respect to a predicted CCO issue since the detected issue is occurring at the present time and hence it should be resolved as soon as possible. But the main difference with respect to the previous scenario is that the new network situation following the resolution of the detected CCO issue may imply that the previous predicted CCO assistance information becomes invalid. Therefore, after resolving the detected CCO issue, the AI/ML model in the CU should output new predicted CCO assistance information, and we foresee the following two cases:

- Case B-1: after the detected CCO issue has been resolved, the model in gNB-CU predicts that there will be a CCO issue for all the cells/SSBs previously predicted to be affected, and if the CCO issue type and time for predicted CCO issue do not change compared to the previous prediction, the previous predicted CCO assistance information does not need to be replaced, and can be kept valid in both gNB-DU and neighbor gNB;

- Case B-2: after the detected CCO issue has been resolved, the model in gNB-CU predicts that there will be a CCO issue for some of the cells/SSBs previously predicted to be affected, hence the previous predicted CCO assistance information needs to be replaced since the list of predicted affected cells/SSBs in the new predicted CCO assistance information differs from the prediction previously signalled.

Proposal 3: RAN3 to agree that, following the resolution of a detected CCO issue affecting none of the cells/SSBs which were predicted to be affected, in case a new predicted CCO assistance information is made available by the AI/ML model, and if the time for the predicted issue, affected cells/SSBs and CCO issue type are same as those in the previous predicted CCO assistance information, such previous prediction does not need to be replaced. Otherwise, the previous predicted CCO assistance information needs to be replaced.

Scenario C - another CCO issue is predicted by the model

Let’s consider a predicted CCO assistance information (i.e., predicted CCO issue type, list of predicted affected cells/SSBs and time for predicted CCO issue) that has already been provided by the gNB-CU hosting the AI/ML model to the gNB-DU(s) controlling the cells/SSBs predicted to be affected. Let’s now assume that a new predicted CCO assistance information is provided by the model and, depending on content of the new predicted CCO assistance information, the following cases can be considered:

- Case C-1: the new predicted CCO issue (same or different issue type than the one on the previous prediction), impacting the same cells/SSBs as the previous prediction, is expected to occur at a shorter time for predicted CCO issue than the previously signalled predicted CCO issue; as a general principle, the latest predicted CCO assistance information should replace the previous one, since it is based on the most recent network status, and hence it is expected to be more reliable. Any network change based on the new predicted CCO assistance information would likely make the previous prediction not valid anymore;

- Case C-2: the new predicted CCO issue (same or different issue type), impacting the same cells/SSBs as the previous one, is expected to occur at a longer time for predicted CCO issue that the previously signalled predicted CCO issue; as a general principle, the latest predicted CCO assistance information should replace the previous one, since it is based on the most recent network status, which could have made the previous prediction likely not to occur;

- Case C-3: the new predicted CCO assistance information is generated impacting a different set of cells/SSBs which includes some of the cells/SSBs of the previous prediction. In our understanding, the new prediction should be treated as an update of the previous one (as the list of cells/SSBs predicted to be affected is changed), hence implying that the previous prediction should be replaced;

- Case C-4: the new predicted CCO assistance information is generated, impacting a different set of cells/SSBs which does not include any of the cells/SSBs of the previous prediction. In our view, the new prediction should be treated as a separate prediction (i.e., distinct from the previous one), hence implying that the previous prediction should be kept.

Proposal 4: RAN3 to agree that, in case new predicted CCO assistance information is made available for the same cells/SSBs predicted to be affected, but either the CCO issue type or the time for predicted CCO issue (or both) changes, then the previous prediction needs to be replaced.

Proposal 5: RAN3 to agree that, in case new predicted CCO assistance information is made available and it impacts a different set of cells/SSBs which includes some of the cells/SSBs of the previous prediction, the new prediction should be treated as an update of the previous one, hence the previous prediction should be replaced.

Proposal 6: RAN3 to agree that, in case new predicted CCO assistance information is made available and it impacts a different set of cells/SSBs which does not include any of the cells/SSBs of the previous prediction, the new prediction should be treated as a separate prediction, hence the previous prediction should be kept.

Below in Table 1 a summary of all the cases and scenarios discussed in this paper, along with the suggested way to treat (i.e., keep or replace) the predicted CCO assistance information previously signalled over F1 and Xn.

Table 1 - Summary of cases and scenarios for updating a predicted CCO assistance information previously signalled.

|  | Detected CCO issue occurs for… |  | New predicted CCO assistance information is generated for… |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | At least one cell/SSB in predicted CCO assistance information is affected | None of the cells/SSBs in predicted CCO assistance information is affected | Same cells/SSBs but shorter time for predicted CCO issue | Same cells/SSBs but higher time for predicted CCO issue | Different set of cells/SSBs which includes some of the cells/SSBs of the previous prediction | Different set of cells/SSBs which does not include any of the cells/SSBs of the previous prediction |
| What to do with the predicted CCO assistance information previously signalled? | - Case A-1: cancel - Case A-2: replace | - Case B-1: keep - Case B-2: replace | Case C-1: replace | Case C-2: replace | Case C-3: replace | Case C-4: keep |

## How to implement the mechanism to replace a prediction

In order to replace CCO-related prediction information (i.e., cancel the previous CCO-related prediction information and provide a new one) we need to:

- identify the previous CCO-related prediction information provided over F1 to the gNB-DU and over Xn to the neighbor gNB,

- cancel the CCO-related prediction information previously provided, and

- signal the new CCO-related prediction information over F1 to the gNB-DU and over Xn to the neighbor gNB.

Let’s consider the F1 interface first.

For 1., the RAN3 agreement from the last meeting is taken into account, according to which the list of cells/SSBs predicted to be affected is used to identify the previously signalled predicted CCO assistance information; for 2., instead, a new codepoint ‘cancel’ could be added in the Predicted CCO issue IE within the F1AP Predicted CCO Assistance Information IE, as per the FFS from the last meeting. And finally, to provide the new prediction replacing the previous one, a new F1AP gNB-CU CONFIGURATION UPDATE message is sent to the gNB-DU carrying the new predicted CCO assistance information.

Proposal 7: RAN3 to agree to introduce the new codepoint ‘cancel’ in the F1AP Predicted CCO issue IE to cancel a previously signalled predicted CCO assistance information.

Let’s now consider the Xn interface; by checking the content related to future coverage modification information pertaining to cells/SSBs predicted to be affected and provided to the neighbor gNB within the NG-RAN NODE CONFIGURATION UPDATE message, it should be noted that the Predicted Coverage Modification Cause (reflecting the CCO issue predicted at the local node) is signalled simultaneously with the future CCO states pertaining to those cells/SSBs, along with the associated activation time, as part of the Future Coverage Modification List IE. Therefore, when there is the need to cancel information previously provided to the neighbor gNB (refer to Step 2. Above), we think that the whole Future Coverage Modification List IE stored in the neighbor gNB needs to be cancelled.

Observation 3: When there is the need to cancel information previously provided to the neighbor gNB the whole Future Coverage Modification List IE (which includes the predicted coverage modification cause, the future CCO states for predicted affected cells and beams, and the activation times of future CCO states) needs to be cancelled.

Moreover, the current signalling design to transfer the Future Coverage Modification List IE allows to transfer over Xn multiple future CCO states for the same cell subject to different coverage modification causes and/or times for future CCO state. This is one of the main differences compared to the legacy CCO, where a cell cannot be assigned with more than one CCO state, and the timing variable is not present. Therefore, in our view it is not sufficient to simply rely on the list of cells and SSBs to identify the information to be cancelled, but the predicted coverage modification cause and time for future CCO state should be considered as well.

Observation 4: To identify the information to be cancelled at the neighbor node side, the list of cells and SSBs predicted to be affected in the local node, the predicted coverage modification cause and the time for future CCO state should be considered jointly.

Therefor, we think that at least the following two signalling options could be considered to implement a mechanism to cancel information already provided to the neighbor over Xn:

- Option 1: introduce a new Future Coverage Modification To Be Cancelled List IE within the NG-RAN NODE CONFIGURATION UPDATE message which includes the same cells and SSBs, the same predicted coverage modification cause and the same time for future CCO state as in the previously signalled Future Coverage Modification List IE, so that the Future Coverage Modification List IE previously provided (and which needs to be cancelled) could be identified; or

- Option 2: relying on the existing Future Coverage Modification List IE within the NG-RAN NODE CONFIGURATION UPDATE message where, in the Predicted Coverage Modification Cause IE, a new codepoint “cancel” is introduced. When “cancel” is present, the neighbor node clears any previously received information (Future Coverage Modification List IE) identified by the same cells and SSB beams, and time for future coverage state as previously signalled.

Proposal 8: RAN3 to agree to either Option 1 or Option 2 to allow to cancel a previously signalled Future Coverage Modification List IE over Xn.

TPs to F1AP and XnAP BLCRs for Rel-19 RAN AI/ML implementing the above Proposal 7 and both options in Proposal 8 are provided in [6] and [7], respectively.

# 3. Conclusion

Based on the above discussion, we have the following observation and proposals:

Observation 1: It should be possible to modify a prediction in terms of either predicted CCO issue type, and/or list of cells/SSBs predicted to be affected, and/or time for predicted CCO issue.

Observation 2: A detected CCO issue affecting at least one cell/SSB among the cells/SSBs predicted to be affected by the predicted CCO issue should be prioritized with respect to the predicted CCO issue and solved as soon as possible.

Proposal 1: RAN3 to agree that if a CCO issue is detected, and if it impacts at least one cell/SSB among the cells/SSBs which were predicted to be affected by the predicted CCO issue previously signalled, the detected CCO issue resolution takes precedence over the predicted CCO issue and it is solved immediately.

Proposal 2: RAN3 to agree that, if the AI/ML model in the gNB-CU provides new predicted CCO assistance information after the detected CCO issue resolution, and if there will not be any issue impacting the cells/SSBs which were predicted to be affected by the previous prediction, the previous predicted CCO assistance information needs to be cancelled. Otherwise, the previous predicted CCO assistance information needs to be replaced.

Proposal 3: RAN3 to agree that, following the resolution of a detected CCO issue affecting none of the cells/SSBs which were predicted to be affected, in case a new predicted CCO assistance information is made available by the AI/ML model, and if the time for the predicted issue, affected cells/SSBs and CCO issue type are same as those in the previous predicted CCO assistance information, such previous prediction does not need to be replaced. Otherwise, the previous predicted CCO assistance information needs to be replaced.

Proposal 4: RAN3 to agree that, in case new predicted CCO assistance information is made available for the same cells/SSBs predicted to be affected, but either the CCO issue type or the time for predicted CCO issue (or both) changes, then the previous prediction needs to be replaced.

Proposal 5: RAN3 to agree that, in case new predicted CCO assistance information is made available and it impacts a different set of cells/SSBs which includes some of the cells/SSBs of the previous prediction, the new prediction should be treated as an update of the previous one, hence the previous prediction should be replaced.

Proposal 6: RAN3 to agree that, in case new predicted CCO assistance information is made available and it impacts a different set of cells/SSBs which does not include any of the cells/SSBs of the previous prediction, the new prediction should be treated as a separate prediction, hence the previous prediction should be kept.

Proposal 7: RAN3 to agree to introduce the new codepoint ‘cancel’ in the F1AP Predicted CCO issue IE to cancel a previously signalled predicted CCO assistance information.

Observation 3: When there is the need to cancel information previously provided to the neighbor gNB the whole Future Coverage Modification List IE (which includes the predicted coverage modification cause, the future CCO states for predicted affected cells and beams, and the activation times of future CCO states) needs to be cancelled.

Observation 4: To identify the information to be cancelled at the neighbor node side, the list of cells and SSBs predicted to be affected in the local node, the predicted coverage modification cause and the time for future CCO state should be considered jointly.

Proposal 8: RAN3 to agree to either Option 1 or Option 2 to allow to cancel a previously signalled Future Coverage Modification List IE over Xn.

TPs to F1AP and XnAP BLCRs for Rel-19 RAN AI/ML implementing the above Proposal 7 and Proposal 8 are provided in [6] and [7], respectively.

# 4. Reference

- RP-251245, “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#108

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- 3GPP RAN3#128 Chairman’s notes (RAN3_128_agenda_20250523_1530_EOM.doc)

- R3-255046, (BL CR to 38.473) Support of enhancements on AI/ML for NG-RAN, Ericsson (Rapporteur), ZTE, NEC, Nokia, Huawei, Ofinno

- R3-255364, (TP for AI/ML BLCR to TS 38.473) Replacing a predicted CCO assistance information previously signalled, Huawei, Jio Platforms, Vodafone, Deutsche Telekom, Orange

- R3-255365, (TP for AI/ML BLCR to TS 38.423) Replacing a predicted CCO assistance information previously signalled, Huawei, Jio Platforms, Vodafone, Deutsche Telekom, Orange