3GPP TSG-RAN WG3 Meeting #129	R3-255369

Bengaluru, India, 25th – 29th August, 2025

Title: 	[DRAFT] LS reply to LS on energy saving for split gNB

Source: 	Huawei

Agenda item:	11.4

Document Type:	Discussion and decision

# 1. Introduction

This contribution provides a draft Reply LS to SA5 on energy saving for split gNB reflecting the analysis in [1].

# 2. Reference

- R3-255368, Remaining open issues on split architecture support for Rel-18 “AI/ML for NG-RAN” use cases, Huawei

# 3. Draft Reply LS to SA5 on gNB-DU’s EC reporting over F1

3GPP TSG-RAN3 Meeting #129	R3-25xxxx

Bengaluru, India, 25th – 29th August, 2025

Title:	[Draft] LS reply to LS on energy saving for split gNB

Response to:	R3-251529 (S5-251087: LS reply to LS on energy saving for split gNB)

Release:	Rel-19

Work Item:	NR_AIML_NGRAN_enh-Core

Source:	Huawei [to be RAN3]

To:	SA5

Cc:

Contact person:	Damiano Rapone

damiano.rapone@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN3 thanks SA5 for their reply LS on energy saving for split gNB. RAN3 discussed the topic further, and captured the following in the Rel-19 TS 38.401 BLCR:

### 7.11.2 OAM requirements OAM configures the following on gNB-DU: - the minimum and maximum energy consumption values corresponding to the minimum and maximum EC index values respectively, based on an implementation-specific mapping rule, which is unified within a defined area. - the recommended time interval within which a gNB-DU selects an implementation-specific time window for averaging of the measurements of the gNB-DU's consumed energy.

RAN3 would like to ask SA5 to check the above text and introduce the needed changes (if any) in relevant SA5 specifications to allow the gNB-DU to be provided by the OAM with the requested information to enable the reporting of the gNB-DU’s measured Energy Cost index over F1 (from gNB-DU to gNB-CU).

# 2	Actions

To SA5

ACTION: 	RAN3 respectfully asks SA5 to take the above into account and, if needed, update 					their relevant specifications accordingly to enable the gNB-DU to report its measured 				Energy Cost index over F1.

# 3	Dates of next TSG RAN WG3 meetings

RAN3#129bis		13 October – 19 October 2025			Prague, CZ

RAN3#130			17 November – 21 November			Dallas, US