3GPP TSG RAN WG3#129	R3-255379

Bengaluru, India, 25 – 29 August, 2025

Agenda item:	17.2

Source: 	China Telecom

Title: 	(TP to 38.473) On support on-demand SSB SCell operation

Document for:	Discussion and decision

- Introduction

In last meeting, we discuss on the possible specification impact on F1 interface for case#1, but no consensus was reached as follows:

- The CU reuses the existing IE (the Cells Allowed to be Deactivated List IE) to indicate the allowed cells for on-demand SSB operation to the DU. The details of stage2/3 impact can be further discussed.

In this paper, we provided our view on this issue and hope to make some progress in the meeting.

- Whether reusing the existing IE to indicate the allowed cells for on-demand SSB operation

In the CU-DU splitting architecture, the CU configures the UE to use carrier aggregation (CA) to improve the traffic throughput, as it has better knowledge of the transmission requirements and mobility state of the UE than the DU. Since the CU possesses the load status of each SCell, it should be responsible for deciding which SCells need to perform the on-demand SSB operation. Therefore, the CU should send the cell list to DU to indicate which cells should switch to on-demand SSB operation mode.

Proposal 1: The CU should send the cell list to DU, indicating the cells that can operate in on-demand SSB transmission mode.

During the last meeting, the method for sending the cell list from CU to DU was also discussed. Some companies propose introducing new IEs to convey the on-demand SSB cell list. The benefit of this approach is that the new IE clearly indicates this as a Rel-18 new feature, ensuring dedicated meaning and usage, while avoiding conflicts with existing IEs. However other companies suggest reusing the existing IE for minimizing the specification impact. While this reduces modification efforts, it introduces ambiguity between the traditional and the new meaning of the IE.

We prefer introducing a new IE to indicate the list of on-demand SSB SCells. However, to facilitate progress, we are also open to reusing an existing IE. If an existing IE is reused, the DU should be responsible for interpreting its usage. For instance, if we reuse the IE “Cells Allowed to be Deactivated List” – currently used to indicate the deactivated cells – to instead indicate the on-demand SSB cells, the DU must determine whether to deactivate the indicated cells or configure them for on-demand SSB mode operation when receiving this IE within GNB-CU CONFIGURATION UPDATE from the CU.

Additionally, the TP for TS 38.473 is required.

Proposal 2: The IE “Cells Allowed to be Deactivated List” in TS38.473 could be reused to indicate the list of on-demand SSB SCells, and a TP for TS 38.473 is needed.

- Conclusion

Based on the above discussion regarding support for on-demand SSB operation, we have the following proposals:

Proposal 1: The CU should send the cell list to DU, indicating the cells that can operate in on-demand SSB transmission mode.

Proposal 2: The IE “Cells Allowed to be Deactivated List” in TS38.473 could be reused to indicate the list of on-demand SSB SCells, and a TP for TS 38.473 is needed.

- Reference

[1] RP-242354, New WID: Enhancements of network energy savings for NR, Ericsson.

- TP for TS 38.473

<<<<<<<<<<<<<<<<<<<< Change Begins >>>>>>>>>>>>>>>>>>>>

8.2.5	gNB-CU Configuration Update

8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

The gNB-CU initiates the procedure by sending a GNB-CU CONFIGURATION UPDATE message including the appropriate updated configuration data to the gNB-DU. The gNB-DU responds with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

The updated configuration data shall be stored in the respective node and used as long as there is an operational TNL association or until any further update is performed.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall activate the cell indicated by NR CGI IE and reconfigure the physical cell identity for which the NR PCI IE is included.

If the SSBs within the cell to be Activated List IE is included in the Cells to be Activated List Item IE within the gNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, only activate those SSB beams indicated by the SSB Index IE.

If at least one requested SSB beam in the SSBs within the cell to be Activated List IE is activated, the gNB-DU includes the Cells with SSBs Activated List IE in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message. The gNB-CU shall consider that the SSB beams indicated by the SSBs activated List IE as activated.

If Cells to be Deactivated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall deactivate the cell indicated by NR CGI IE.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message and the indicated cells are already activated, the gNB-DU shall update the cell information received in Cells to be Activated List Item IE.

If Cells to be Activated List Item IE is included in the GNB-CU CONFIGURATION UPDATE message, and the information for the cell indicated by the NR CGI IE includes the IAB Info IAB-donor-CU IE, the gNB-DU shall, if supported, apply the IAB STC Info IE therein to the indicated cell.

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams or to be OD-SSB SCell within the indicated cells for network energy saving purpose.

If the gNB-CU System Information IE is contained in the gNB-CU CONFIGURATION UPDATE message, the gNB-DU shall include the Dedicated SI Delivery Needed UE List IE in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message for UEs that are unable to receive system information from broadcast.

If Dedicated SI Delivery Needed UE List IE is contained in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-CU should take it into account when informing the UE of the updated system information via the dedicated RRC message.

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>