3GPP TSG RAN WG3#129	R3-255380

Bengaluru, India, 25 – 29 August, 2025

Agenda item:	12.3

Source: 	China Telecom

Title: 	On remaining issues for NR Femto

Document for:	Discussion and decision

- Introduction

In RAN3#127, the agreement was made as follows:

  - Agree to capture security aspects confirmed by SA3 in a TP to the BLCR to TS38.300

  - Agree that terminology alignment is needed between the “NR Femto Node” and NR Femto” terms.

  - Agree to adopt the term NR Femto Node and reflect that in a revision of R3-253450.

  - RAN3 to replace the term “requested S-NSSAI” with “requested NSSAI” in the BL CR to TS 38.413.

And the TPs to 38.300 and 38.413 i.e. R3-253912, R3-253913, R3-253848 were agreed. An issue was left as follows:

  - To be further discussed: whether to send a specific Femto indication in the NG: Initial UE message, from NR Femto to enable control of sending Allowed PNI NPN List or not.

The other issue mentioned in [5] is whether the Femto GW can find the correct target femto node.

In this paper, we discuss the remaining issues and present our proposals.

- Discussion

2.1 Whether the AMF needs to be aware of Femto connections

In the last meeting, companies raise an issue that AMF should be aware of Femto connections because an Femto node may be untrusted according to Section 5.8 of TS 33.545, as follows.

5.8 CAG ID verification The NR Femto GW shall verify that all UE associated messages from Femtos operating in closed access mode can be mapped to a specific CAG ID and that this CAG ID is allowed for the identity of the originating Femto. In the absence of a NR Femto GW, the AMF shall perform this CAG ID verification.

In practical networks, a Femto can connect to the 5G Core (5GC) in two ways: via an operator’s dedicated network or via a public network, as illustrated in Fig. 1.

- When a Femto connects to the 5GC through a dedicated operator-managed network (e.g., Femto#A and gNB#A in Fig. 1), it is typically trusted at the same level as a gNB. In this case, the AMF does not need to distinguish between a gNB connection and a Femto connection, as they are treated identically.

- If a Femto node connects to the 5GC via a public IP network (such as the internet), it must first establish a connection through a Femto Gateway (GW). The Femto node then exchanges signaling with the 5GC via this GW, meaning it cannot directly connect to the 5GC over an untrusted network. In this scenario, “The NR Femto GW shall verify that all UE associated messages from Femtos operating in closed access mode can be mapped to a specific CAG ID and that this CAG ID is allowed for the identity of the originating Femto.” [5] and AMF does not need to explicitly identify or manage the Femto node itself.

Typically, when a subscriber or enterprise customer purchases an operator’s Femto service, the operator technician will deploy and configure the Femto device. If the customer’s network is untrusted, the technician configures Femto device to connect to the Femto GW to ensure the connection security.

Once the Femto device is deployed by operator and registered in the network, for ensuring the security of the device connection, it can’t be allowed to relocated by customers. If a customer needs to move a Femto node to a new location, they must submit a relocation request to the operator, an operator technician will then unregister the device from its current place and reinstall and register the device at the new location.

So, according to current operating procedure of Femto, the security of Femto connection has been guaranteed by operator deploying procedure, we think the AMF may not need to be aware of the Femto node connection.

Allowing customers to self-install Femto nodes could pose security risks. Currently, operators do not offer this type of Femto service.

Proposal 1: AMF may not need to be aware of the Femto node connection.

Figure 1 The connections between Femtos and the 5GC

2.2 Whether AMF can find the correct target Femto GW

In traditional NG-based HO, the source gNB will send the target Global gNB ID and selected TAI in HANDOVER REQUIRED message to AMF, the target Global gNB ID and TAI can get from the cell identity information by measurement report of UE. Then the AMF selects the target gNB to send the Handover Request message based on the target Global gNB ID and sends the HANDOVER REQUEST message to the target gNB.

In Femto case, the procedure is different. The AMF can’t select the target Femto GW based on the target Global gNB ID because it points to a target Femto node, not a target Femto GW. Since the target Global gNB ID points to a Femto node rather than a Femto GW, the AMF cannot use it to select the target Femto GW. Per RAN3#127 agreement (i.e. “To avoid routing ambiguities, a TAI used in a NR Femto GW shall not be reused in another NR Femto GW”), the AMF should determine the target Femto GW based on the selected TAI and forwards the HANDOVER REQUEST message to it. As HANDOVER REQUEST message does not contain the target gNB ID, the target Femto GW should read the Target Cell ID (NR CGI) from the container to identify the target Femto node.

In an operator's network, the gNB ID length is standardized and consistent across the entire network. For instance, the NCGI length is 36 bits, if an operator allocates 28 bits for the gNB ID length, thus the remaining 8 bits are used for the Cell ID of each gNB. Therefore, a Femto GW can identify each Femto node uniquely by the NR CGI.

Observation 1: As the gNB ID length is standardized and consistent across the entire network, a Femto GW can identify and select the target Femto node based on the NR CGI value.

- Conclusion

Based on the above discussion on the remaining issues for NR Femto, we provide the following observations and proposals:

Proposal: AMF may not need to be aware of the Femto node connection.

Observation: As the gNB ID length is standardized and consistent across the entire network, a Femto GW can identify and select the target Femto node based on the NR CGI value.

- Reference

- R3-251524 LS on NR Femto node shared by PLMN and PNI-NPN, SA2.

- TS 38.331, NR; Radio Resource Control (RRC) protocol specification Rel-18, 3GPP.

- RP-242395, New WID on additional topological enhancements for NR, NTT DOCOMO, AT&T.

- RP-241925, TR 38.799 v2.1.0 NR; Study on additional topological enhancements for NR, NTT DOCOMO INC., AT&T.

- TS 33.545, Security aspects of NR Femto, Rel-19, 3GPP.

- R3-253346, (TP for Femto BL CR for TS 38.300/38.413) Discussion on remaining issues for NR Femto, Huawei.

- TS 38.300, NR; NR and NG-RAN Overall description; Stage-2 Rel-18, 3GPP.

- TS 38.413, NG-RAN; NG Application Protocol (NGAP) Rel-18, 3GPP.

- TS 36.300, E-UTRAN; Overall description; Stage 2 Rel-18, 3GPP.

- TS 36.413, E-UTRAN; S1 Application Protocol (S1AP) Rel-18, 3GPP.