3GPP TSG-RAN WG3 Meeting #129       	   				    R3-255383

Bengaluru, IN, 25th – 29th Aug, 2025

Agenda Item:		16.2

Source:			China Telecom

Title:	Remaining issues for AIoT

Document for:	 	Discussion & Decision

- Introduction

The Rel-19 Ambient IoT WID contains the following objective[1]:

- RAN3 scope:

  - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including:

    - Inventory and command operations

    - Device location reporting at reader ID granularity

Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

In this contribution, we discuss the architecture aspects and interface management procedures based on the progress achieved in last RAN3 meeting.

- Discussion

There were still some open issues from the last meeting.

WA: Define the A-IoT Support IE in the NG Setup Request as ENUMERATED (A-IoT only, A-IoT and NR Uu, …)

Whether to have A-IoT Support Indicator in the NG SETUP RESPONSE message?

Option 1: A-IoT Support Indication

Option 2: known via OAM, gNB can ignore the not applicable IEs in case of direct connectivity.

Option 3: reuse existing IE (e.g., AMF name) to indicate that the CN node is AIOTF

The purpose of introducing the A-IoT support indicator in the NG SETUP RESPONSE message is to instruct the gNB to ignore mandatory AMF specific IEs.

For Option1, if introduce a new AIOTF support indicator IE, the gNB can ignore the not appliable IEs. For Option2, rely on OAM pre-configuration, the gNB should be aware of direct/indirect connectivity and ignores non-applicable IEs. For Option3, reuse existing IE (e.g., AMF Name) to implicitly identify AIOTF nodes may cause interoperability issues.

All options are workable, considering that gNB can already know the connection type (direct/indirect) via OAM before NG Setup procedure, we are slightly prefer to use Option 2.

Proposal 1: Turn WA to Agreement: Define the A-IoT Support IE in the NG Setup Request as ENUMERATED (A-IoT only, A-IoT and NR Uu, …)

Proposal 2 If gNB is only support A-IoT and directly connect to AIOTF, the “AMF Name IE, Served GUAMI List IE, Relative AMF Capacity IE, PLMN Support List IE” contained in NG Setup Response message shall be ignored according to OAM configuration.

Reader Selection refers to SA2 TS 23.369 clause 5.3.3. Further check is needed.

WA: Upon only receiving the reader list in Inventory Request, the gNB selects the readers indicated by the reader list.

FFS: Upon receiving both the area and the reader list in Inventory Request, the gNB selects the readers within the indicated area and the readers within the reader list.

Based on TS 23.369, AIOTF can perform RAN reader selection. Therefore, it is proposed to turn the above WA and FFS into agreements

Proposal 3: Turn WA to Agreement: Upon only receiving the reader list in Inventory Request, the gNB selects the readers indicated by the reader list.

Proposal 4: Upon receiving both the area and the reader list in Inventory Request, the gNB selects the readers within the indicated area and the readers indicate by the reader list.

Introduce new CN triggered Class 1 Session Release procedure, which includes A-IoT SESSION RELEASE REQUEST message and A-IoT SESSION RELEASE RESPONSE message.

Introduce a class2 procedure to allow NG-RAN node to trigger Session Release procedure towards AIoT CN.

The name of the procedure, the name of the messages and the purpose of the procedure are FFS.

The procedure text regarding the interaction with the A-IoT Session Release procedure is FFS.

Both RAN and CN nodes can trigger A-IoT session release procedure. Similar to the UE Context Release procedure in NGAP, we can use the A-IOT SESSION RELEASE COMMAND and A-IOT SESSION RELEASE COMPLETE message name for the CN triggered procedure. Use A-IOT SESSION RELEASE REQUEST message name for gNB triggered procedure. The A-IoT session release procedure should be initiated upon reception of a A-IoT SESSION RELEASE REQUEST message. Critically, the gNB may initiate a new A-IoT session over the A-IoT radio interface without awaiting completion of an ongoing release procedure – whether the release was initiated by the gNB itself or triggered by the core network.

Proposal 5: The A-IoT session release procedure utilizes dedicated messages:

- A-IoT SESSION RELEASE COMMAND and A-IoT SESSION RELEASE COMPLETE(CN-initiated)

- A-IoT SESSION RELEASE REQUEST (gNB-initiated)

the gNB may initiate a new A-IoT session over the A-IoT radio interface without awaiting completion of an ongoing release procedure.

- Conclusions

In this paper, we have the following proposal:

Proposal 1: Turn WA to Agreement: Define the A-IoT Support IE in the NG Setup Request as ENUMERATED (A-IoT only, A-IoT and NR Uu, …)

Proposal 2 If gNB is only support A-IoT and directly connect to AIOTF, the “AMF Name IE, Served GUAMI List IE, Relative AMF Capacity IE, PLMN Support List IE” contained in NG Setup Response message shall be ignored according to OAM configuration.

Proposal 3: Turn WA to Agreement: Upon only receiving the reader list in Inventory Request, the gNB selects the readers indicated by the reader list.

Proposal 4: Upon receiving both the area and the reader list in Inventory Request, the gNB selects the readers within the indicated area and the readers indicate by the reader list.

Proposal 5: The A-IoT session release procedure utilizes dedicated messages:

- A-IoT SESSION RELEASE COMMAND and A-IoT SESSION RELEASE COMPLETE(CN-initiated)

- A-IoT SESSION RELEASE REQUEST (gNB-initiated)

the gNB may initiate a new A-IoT session over the A-IoT radio interface without awaiting completion of an ongoing release procedure.

- References

[1] 	RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR.