3GPP TSG-RAN WG3 Meeting #129	R3-255385

Bengaluru, India, 25th-29th August 2025

Title:	Discussion on AIML based Coverage and Capacity Optimization

Source:				China Telecom

Agenda item:		11.3

Document for:		Discussion and Decision

1. Introduction

In RAN3#128 meeting, the following agreements with several open issues were captured:

Adopt Opt2. (gNB-CU directly forwards the received further CCO state of neighbor cells to gNB-DU.) Timing information for predicted CCO issue over Xn is not needed? Specify mechanisms (F1, Xn) to update/cancel a prediction, no new IE is needed to identify the previous prediction (affected cells and beams will identify the previously signalled issue). Addition of new code-point in predicted coverage modification cause (Xn) / predicted CCO issue (F1), with details FFS.

In this contribution, we further discuss the timing information for predicted CCO issue over Xn and provide our view.

2. Discussion

RAN3 agreed to transfer the Time for Future Coverage Modification IE (i.e. time information for future CCO state) within the NG-RAN CONFIGURATION UPDATE message. However, whether to send time information for the coverage modification cause which reflects the predicted CCO issue over Xn needs further discussion.

From our side, providing time information for predicted CCO issues over Xn is essential for proactive network management. For example, consider a scenario where gNB1 predicts a capacity overload issue at 5 PM, with offloading to gNB2 starting at 4:30 PM. Knowing 5 PM, gNB2 can ensure it has sufficient resources ready at 5 PM to handle the offloaded users, especially if the offloading process takes time to complete and some strain might still occur at 5 PM if not fully mitigated. If gNB2 only knows the future CCO state will be applied at 4:30 PM, it might assume that the issue is fully handled by then, but if the offloading isn't complete and the issue still occurs at 5 PM, gNB2 might be unprepared for residual effects, risking degraded service. Moreover, time information for predicted CCO issues can be also used for future evaluation.

Proposal 1: RAN3 to agree that timing information for the predicted CCO issue can be transferred over Xn.

Proposal 2: Introduce the Time for coverage modification cause IE within the NG-RAN CONFIGURATION UPDATE message.

3. Conclusion

In this contribution, we further discuss AI/ML based Coverage and Capacity Optimization (CCO). We have the following proposal:

Proposal 1: RAN3 to agree that timing information for the predicted CCO issue can be transferred over Xn.

Proposal 2: Introduce the Time for coverage modification cause IE within the NG-RAN CONFIGURATION UPDATE message.

4. Reference

- 3GPP RAN3#125b Chair’s notes (RAN3_125bis_agenda_20241018_1430_EOM.doc)

- 3GPP RAN3#126 Chair’s notes (RAN3_126_agenda_20241122_EOM1.doc)

- 3GPP RAN3#127 Chair’s notes (RAN3_127_agenda_20250221_1300_EOM.doc)

- 3GPP RAN3#127bis Chair’s notes (RAN3_127bis_agenda_20250411_1400_EOM.doc)

- 3GPP RAN3#128 Chair’s notes (RAN3_128_agenda_20250523_1530_EOM.doc)

5. TP for AI/ML BLCR to TS38.423 (based on R3-255044)

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.4	NG-RAN NODE CONFIGURATION UPDATE

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to transfer updated information for an Xn-C interface instance.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| TAI Support List | O |  | 9.2.3.20 | List of supported TAs and associated characteristics. | GLOBAL | reject |
| <<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| Neighbour NG-RAN Node List |  | 0..<maxnoofNeighbourNG-RAN nodes> |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >Local NG-RAN Node Identifier | M |  | 9.2.2.101 |  | – |  |
| Local NG-RAN Node Identifier Removal | O |  | Local NG-RAN Node Identifier 9.2.2.101 |  | YES | ignore |
| Future Coverage Modification List |  | 0..1 |  | List of cells whose coverage will be modified. | YES | ignore |
| >Future Coverage Modification Item |  | 1..<maxnoofCellsinNG-RAN node> |  |  | – |  |
| >>Global NG-RAN Cell Identity | M |  | 9.2.2.27 | NG-RAN Cell Global Identifier of the cell whose coverage will be modified. | – |  |
| >>Future Cell Coverage State | M |  | INTEGER (0..63, ...) | Value ‘0’ indicates that the cell will be inactive. Other values indicates that the cell will be active and also indicates the future coverage configuration of the concerned cell. | – |  |
| >>Future SSB Coverage Modification List |  | 0..1 |  | List of SSB beams whose coverage will be modified. | – |  |
| >>>Future SSB Coverage Modification Item |  | 1..<maxnoofSSBAreas> |  |  | – |  |
| >>>>SSB index | M |  | INTEGER (0..63) | Identifier of the SSB beam whose coverage will be modified. | – |  |
| >>>>Future SSB Coverage State | M |  | INTEGER (0..15, ...) | Value ‘0’ indicates that the SSB beam will be inactive. Other values indicate that the SSB beams will be active and also Indicates the future coverage configuration of the concerned SSB beams. | – |  |
| >>Predicted Coverage Modification Cause | O |  | ENUMERATED (coverage, cell edge capacity, ...) | Indicates the reason for the predicted coverage modification in NG-RAN node1. | – |  |
| >>Time for Future Coverage State | O |  | INTEGER (1..60, …) | Indicates the time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the NG-RAN node1 relative to the time of receiving this information, in seconds. | – |  |
| >>Time for Predicted Coverage Modification Cause |  |  | INTEGER (1..60, …) | Indicates the time when the Predicted Coverage Modification Cause will happen in the NG-RAN node1 relative to the time of receiving this information, in seconds. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the NG RAN nodes. Value is 32. |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |
| maxnoofNeighbourNG-RAN nodes | Maximum no. of neighbour NG-RAN nodes. Value is 256. |

| Condition | Explanation |
| --- | --- |
| ifCellDeploymentStatusIndicatorPresent | This IE shall be present if the Cell Deployment Status Indicator IE is present. |

<<<<<<<<<<<<<<<<<<<< 2nd Change >>>>>>>>>>>>>>>>>>>>

#### -- F

Future-Coverage-Modification-List ::= SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF Future-Coverage-Modification-Item

Future-Coverage-Modification-Item ::= SEQUENCE {

globalNG-RANCell-ID						GlobalCell-ID,

future-cellCoverageState				INTEGER (0..63, ...),

future-SSB-Coverage-Modification-List	Future-SSB-Coverage-Modification-List		OPTIONAL,

predicted-Coverage-Modification-Cause	Predicted-CoverageModificationCause			OPTIONAL,

timeForFutureCoverageState				INTEGER (1..60, ...)						OPTIONAL,

timeForPredictedCoverageModificationCause	INTEGER	(1..60,...)						OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { Future-Coverage-Modification-Item-ExtIEs} }	OPTIONAL,

...

}

Future-Coverage-Modification-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Future-SSB-Coverage-Modification-List ::= SEQUENCE (SIZE (1..maxnoofSSBAreas)) OF Future-SSB-Coverage-Modification-Item

Future-SSB-Coverage-Modification-Item ::= SEQUENCE {

sSBIndex						INTEGER(0..63),

future-SSBCoverageState			INTEGER (0..15, ...),

iE-Extension					ProtocolExtensionContainer { { Future-SSB-Coverage-Modification-Item-ExtIEs} }	OPTIONAL,

...

}

Future-SSB-Coverage-Modification-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}