3GPP TSG-RAN WG3 Meeting #129	R3-255387

# Bangalore, India, 25th – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR | 1972 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on E-RAB management procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMob-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | A conflict can occur between an E-RAB management procedure and a handover preparation procedure. In this scenario, the E-RAB management procedure involves the eNB sending new or updated configurations to the UE. To prioritize timely handover to the best available cell, the eNB typically interrupts the E-RAB management procedure with a failure indication and proceeds with the handover preparation. This prevents any delays that might be caused by transmitting new or updated configurations to the UE. When an E-RAB management procedure collides with a conditional handover preparation, the same prioritization logic is applied: the eNB proceeds with the conditional handover and terminates the E-RAB management procedure. An E-RAB management procedure is often more critical and time-sensitive than a conditional handover. A conditional handover is not an immediate event; it's a preparatory action that might only be triggered later, based on specific conditions being met. Therefore, interrupting an E-RAB management procedure for a non-immediate conditional handover can lead to service interruptions for the UE. Besides, in case of the E-RAB Setup/Modify procedure during the Handover Preparation procedure for handover, the current text describes “the eNB shall indiate all the E-RABs fail with an appropriate cause value, e.g., ”S1 intra system Handover triggered”, “S1 inter system Handover triggered” or “X2 Handover triggered”.”. This description may be misleading. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - It is clarified that the eNB interrupts an E-RAB management procedure during an immedidate handover preparation. - In case of the E-RAB Setup/Modify procedure during the Handover Preparation procedure for immediate handover, the eNB indicates all the E-RABs failed to be set up/modified with an appropriate cause value. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). The impact can be considered isolated because the change only clarifies the E-RAB Setup procedure and the E-RAB Modify procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The eNB may interrupt an E-RAB management procedures during a Handover Preparation procedure for conditional handover. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1, 8.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 8.2	E-RAB Management procedures

### 8.2.1	E-RAB Setup

#### 8.2.1.1	General

The purpose of the E-RAB Setup procedure is to assign resources on Uu and S1 for one or several E-RABs and to setup corresponding Data Radio Bearers for a given UE. The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: E-RAB Setup procedure. Successful operation.

#### <Unrelated part omitted>

Interactions with Handover Preparation procedure:

If an immediate handover becomes necessary during E-RAB Setup, the eNB may interrupt the ongoing E-RAB Setup procedure and initiate the Handover Preparation procedure as follows:

1.	The eNB shall send the E-RAB SETUP RESPONSE message in which the eNB shall indicate, if necessary

-	all the E-RABs fail with an appropriate cause value, e.g., ”S1 intra system Handover triggered”, “S1 inter system Handover triggered” or “X2 Handover triggered”.

2.	The eNB shall trigger the handover procedure.

#### <Unrelated part omitted>

### 8.2.2	E-RAB Modify

#### 8.2.2.1	General

The purpose of the E-RAB Modify procedure is to enable modifications of already established E-RABs for a given UE. The procedure uses UE-associated signalling.

#### 8.2.2.2	Successful Operation

Figure 8.2.2.2-1: E-RAB Modify procedure. Successful operation.

#### <Unrelated part omitted>

Interactions with Handover Preparation procedure:

If an immediate handover becomes necessary during E-RAB modify, the eNB may interrupt the ongoing E-RAB Modify procedure and initiate the Handover Preparation procedure as follows:

1.	The eNB shall send the E-RAB MODIFY RESPONSE message in which the eNB shall indicate, if necessary

-	all the E-RABs fail to modify with an appropriate cause value, e.g., “S1 intra system Handover triggered”, “S1 inter system Handover triggered” or “X2 Handover triggered”.

2.	The eNB shall trigger the handover procedure.