3GPP TSG-RAN WG3 Meeting #129	R3-255392

# Bangalore, India, 25th – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1292 | rev | - | Current version: | 17.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on PDU session management procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | A conflict can occur between a PDU session management procedure and a handover preparation procedure. In this scenario, the PDU session management procedure involves the NG-RAN node sending new or updated configurations to the UE. To prioritize timely handover to the best available cell, the NG-RAN node typically interrupts the PDU session management procedure with a failure indication and proceeds with the handover preparation. This prevents any delays that might be caused by transmitting new or updated configurations to the UE. When a PDU session management procedure collides with a conditional handover preparation, the same prioritization logic is applied: the NG-RAN node proceeds with the conditional handover and terminates the PDU session management procedure. A PDU session management procedure is often more critical and time-sensitive than a conditional handover. A conditional handover is not an immediate event; it's a preparatory action that might only be triggered later, based on specific conditions being met. Therefore, interrupting a PDU session management procedure for a non-immediate conditional handover can lead to service interruptions for the UE. Besides, in case of the PDU Session Resource Modify procedure during the Handover Preparation procedure for immediate handover, the current text describes “the NG-RAN node shall indicate, if necessary, all the PDU sessions failed with an appropriate cause value.”. This description may be misleading. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - It is clarified that the NG-RAN node interrupts a PDU session management procedure during an immediate handover preparation. - In case of the PDU Session Resource Modify procedure during the Handover Preparation procedure for immediate handover, the NG-RAN node indicates all the PDU sessions failed to be modified with an appropriate cause value. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). The impact can be considered isolated because the change only clarifies the PDU Session Resource Setup procedure and the PDU Session Resource Modify procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NG-RAN node may interrupt a PDU session management procedure during a Handover Preparation procedure for conditional handover. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1, 8.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 8.2	PDU Session Management Procedures

### 8.2.1	PDU Session Resource Setup

#### 8.2.1.1	General

The purpose of the PDU Session Resource Setup procedure is to assign resources on Uu and NG-U for one or several PDU sessions and the corresponding QoS flows, and to setup corresponding DRBs for a given UE. The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: PDU session resource setup: successful operation

#### <Unrelated part omitted>

Interactions with Handover Preparation procedure:

If an immediate handover becomes necessary during the PDU Session Resource Setup procedure, the NG-RAN node may interrupt the ongoing PDU Session Resource Setup procedure and initiate the Handover Preparation procedure as follows:

1.	The NG-RAN node shall send the PDU SESSION RESOURCE SETUP RESPONSE message in which the NG-RAN node shall indicate, if necessary, all the PDU session resources which failed to be setup with an appropriate cause value, e.g. "NG intra-system handover triggered", "NG inter-system handover triggered" or "Xn handover triggered".

2.	The NG-RAN node shall trigger the handover procedure.

#### <Unrelated part omitted>

### 8.2.3	PDU Session Resource Modify

#### 8.2.3.1	General

The purpose of the PDU Session Resource Modify procedure is to enable configuration modifications of already established PDU session(s) for a given UE. It is also to enable the setup, modification and release of the QoS flow for already established PDU session(s). The procedure uses UE-associated signalling.

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: PDU session resource modify: successful operation

#### <Unrelated part omitted>

Interactions with Handover Preparation procedure:

If an immediate handover becomes necessary during the PDU Session Resource Modify procedure, the NG-RAN node may interrupt the ongoing PDU Session Resource Modify procedure and initiate the Handover Preparation procedure as follows:

1.	The NG-RAN node shall send the PDU SESSION RESOURCE MODIFY RESPONSE message in which the NG-RAN node shall indicate, if necessary, all the PDU sessions failed to be modified with an appropriate cause value, e.g. "NG intra-system handover triggered", "NG inter-system handover triggered" or "Xn handover triggered".

2.	The NG-RAN node shall trigger the handover procedure.