3GPP TSG-RAN WG3 Meeting #129	R3-255394

Bengaluru, India, August 25-29, 2025

Agenda item:	 10.2

Source:		Lenovo

Title:		(TP for SON BLCR for 38.423) Discussion on MRO for CHO with candidate SCG(s) and subsequent CPAC

Document for:		Discussion and Decision

1	Introduction

In RAN3#128, we agreed that:

MRO for CHO with Candidate SCG(s)

Reuse the existing “HO too early” and “HO to wrong cell” in case of too early CHO execution and CHO execution to wrong cell.

Handover Report message is not needed for case 7a.

In addition to SCGFailureInformation, information needed in the SCG Failure Indication message?

- CPC failure type (CPC failure type is needed in case of SCG Failure Indication is sent from the candidate to the source?)

- Suitable PCell ID

MRO for S-CPAC

The setting of the information on previousPSCellId and timeSCGFailure needs to be updated to support the failures due to the following S-CPAC.

Whether to enhance UHI in case of S-CPAC?

- SCG UHI should be updated to the new serving MN/target SN from source MN during S-CPAC procedure, i.e. Include SCG UHI in SN Reconfiguration Complete message from MN to the target SN

Inform the SN(s) about the outcome of mobility events for SN initiated S-CPAC using the SN RELEASE REQUEST or SN RELEASE CONFIRM.

In this paper, we would further discuss the left issues of MRO for CHO with candidate SCG(s) and subsequent CPAC.

2	Discussion

  - 2.1 MRO for CHO with candidate SCG(s)

For the FFS issue about whether CPC failure type is needed in the SCG Failure Indication message in addition to SCGFailureInformation, since SCGFailureInformation from the UE includes necessary information, after the respective MN receives the SCG Failure Indication message, it can perform failure type analysis and do corresponding optimisation, no matter the SCG Failure Indication is sent from the candidate MN to the source MN, or from the source MN to the candidate MN. Therefore, it is not needed to include CPC failure type in the SCG Failure Indication message.

Proposal 1: No need to include CPC failure type in the SCG Failure Indication message.

  - 2.2 MRO for subsequent CPAC

Last meeting agreed to inform the SN(s) about the outcome of mobility events for SN initiated S-CPAC using the SN RELEASE REQUEST or SN RELEASE CONFIRM. It is FFS on whether SCG UHI should be updated to the new serving MN/target SN from source MN during S-CPAC procedure, i.e. Include SCG UHI in SN Reconfiguration Complete message from MN to the target SN.

In R18, UHI mechanism for legacy CPAC procedure is specified as “When the target SN receives the SCG UHI from the MN via SN Addition Request message for CPC, the target SN updates the time UE stayed in cell of the latest PSCell entry (i.e. the source PSCell) when the UE successfully accesses to a candidate cell of the target SN. The updated value of the time UE stayed in the latest PSCell is equal to the value received from the MN via the SN Addition Request message plus the time from receiving SN Addition Request message from the MN to receiving SN Reconfiguration Complete from the MN”.

Similar as legacy, UHI mechanism for S-CPAC is beneficial to avoid PSCell change ping-pong issue (e.g. to avoid the latest/last target SN in the S-CPAC procedure triggers PSCell change procedure to the former target SN in the S-CPAC procedure). However, UHI mechanism for legacy CPAC procedure can’t totally work well for a subsequent CPAC procedure, since target SN of following subsequent CPC can’t get the latest and valid SCG UHI.

Take the target SN of first following subsequent CPC for an example, in preparation phase of the subsequent CPAC procedure, the target SN of first following subsequent CPC only gets UHI associated with the source SN of the initial CPAC (i.e. the initiating SN at S-CPAC preparation), but actually, before first following subsequent CPC phase, initial CPAC execution is successfully completed, but the target SN of first following subsequent CPC can’t know the UHI associated with the source SN of the first following subsequent CPC, after the first following subsequent CPC is successfully completed wherein we assume the whole S-CPAC procedure is terminated, there is a risk that the target SN of first following subsequent CPC which is now working as a new source SN may trigger a PSCell change procedure back to the source SN of the first following subsequent CPC. To solve the issue, the latest SCG UHI should be informed to the  target SN of first following subsequent CPC in SN Reconfiguration Complete message.

Therefore, SCG UHI should be updated to each target SN from MN during S-CPAC procedure, i.e. include SCG UHI in SN Reconfiguration Complete message.

Proposal 2: Include SCG UHI in SN Reconfiguration Complete message.

3	Conclusion

In this paper, the left issues of MRO for CHO with candidate SCG(s) and subsequent CPAC are discussed. We have the following proposals:

MRO for CHO with candidate SCG(s)

Proposal 1: No need to include CPC failure type in the SCG Failure Indication message.

MRO for subsequent CPAC

Proposal 2: Include SCG UHI in SN Reconfiguration Complete message.

Annex for TS38.423

<<<<<<<<<<<<<<<<<<<< Start of Change >>>>>>>>>>>>>>>>>>>>

### 8.3.2	S-NG-RAN node Reconfiguration Completion

#### 8.3.2.1	General

The purpose of the S-NG-RAN node Reconfiguration Completion procedure is to provide information to the S-NG-RAN node whether the requested configuration was successfully applied by the UE.

The procedure uses UE-associated signalling.

#### 8.3.2.2	Successful Operation

Figure 8.3.2.2-1: S-NG-RAN node Reconfiguration Complete procedure, successful operation.

The M-NG-RAN node initiates the procedure by sending the S-NODE RECONFIGURATION COMPLETE message to the S-NG-RAN node.

The S-NODE RECONFIGURATION COMPLETE message may contain information that

-	either the UE has successfully applied the configuration requested by the S-NG-RAN node. The M-NG-RAN node may also provide configuration information in the M-NG-RAN node to S-NG-RAN node Container IE.

-	or the configuration requested by the S-NG-RAN node has been rejected. The M-NG-RAN node shall provide information with sufficient precision in the included Cause IE to enable the S-NG-RAN node to know the reason for an unsuccessful reconfiguration. The M-NG-RAN node may also provide configuration information in the M-NG-RAN node to S-NG-RAN node Container IE.

Upon reception of the S-NODE RECONFIGURATION COMPLETE message the S-NG-RAN node shall stop the timer TXnDCoverall if TXnDCoverall is running.

If the SK-counter IE is included in the S-NODE RECONFIGURATION COMPLETE message, the S-NG-RAN node shall, if supported, use it to choose S-KSN as specified in TS 33.501 [28]

If the SCG UE History Information IE is included in the S-NODE RECONFIGURATION COMPLETE message, the S-NG-RAN node shall, if supported, use the information to update UE History Information with PSCell history.

#### 8.3.2.3	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.2.4	S-NODE RECONFIGURATION COMPLETE

This message is sent by the M-NG-RAN node to the S-NG-RAN node to indicate whether the configuration requested by the S-NG-RAN node was applied by the UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Response Information | M |  |  |  | YES | ignore |
| >CHOICE Response Type | M |  |  |  | – |  |
| >>Configuration successfully applied |  |  |  |  |  |  |
| >>>M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Includes the RRCReconfigurationComplete message as defined in subclause 6.2.2 of TS 38.331 [10] or the RRCConnectionReconfigurationComplete message as defined in subclause 6.2.2 of TS 36.331 [14]. | – |  |
| >>>SK-counter | O |  | INTEGER (0..65535) |  | YES | ignore |
| >>Configuration rejected by the M-NG-RAN node |  |  |  |  |  |  |
| >>>Cause | M |  | 9.2.3.2 |  | – |  |
| >>>M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in as defined in subclause 11.2.2 of TS 38.331 [10]. | – |  |
| SCG UE History Information | O |  |  | 9.2.3.151 | YES | ignore |