3GPP TSG-RAN WG3 Meeting #129	R3-255395

Bengaluru, India, August 25-29, 2025

Agenda item:	 10.3.1

Source:		Lenovo

Title:		Discussion on SON/MDT for NTN

Document for:		Discussion and Decision

1	Introduction

RAN3#127bis meeting agreed that:

No further discussion on Time-based CHO.

MRO for RACH-less Handover for NTN:

Divide the MRO for RACH-less handover case into two scenarios:

- RACH-less handover was attempted but failed;

- UE was configured RACH-less handover but did not attempt RACH-less handover because RSRP is below threshold

MDT enhancement for NTN:

Distance-based event trigger for logged MDT?

RAN3#128 meeting further agreed that:

To introduce a geographical area scope for NTN MDT including either reference location/radius or a polygon-based area for NG and Xn.

FFS on whether to add the geographical area scope inside or outside the existing area scope IE.

FFS on whether the geographical area scope applies to both logged and immediate MDT or only logged MDT.

In this paper, we would further discuss SON/MDT for NTN.

2	Discussion

MDT enhancement for NTN:

For the FFS on whether to introduce distance-based event trigger for logged MDT, the intention of distance-based event trigger is to avoid logging DL pilot strength measurements in the NTN cell. Currently, event L1 is supported for measurement based logged MDT, in which the UE performs logging of MDT measurements when the triggering condition (e.g. OOC or serving cell quality is lower than the configured threshold as configured in the event L1) is met, measurement based logged MDT can be reused for NTN. The concern is legacy event triggered logged MDT may be not triggered in NTN cell border, but this issue can be solved by configuring a suitable geographical area scope for NTN MDT. Additionally, introducing distance-based event trigger for logged MDT in NTN would cause complexity and RAN2 WG should also be involved (e.g. RAN2 needs to discuss UE capability, network configuration, UE logging behaviour, and etc), considering only one meeting is left in R19, it is unnecessary to introduce distance-based event trigger for logged MDT in NTN.

Proposal 1: No need to introduce distance-based event trigger for logged MDT in NTN.

Last RAN3#128 meeting agreed to introduce a geographical area scope for NTN MDT including either reference location/radius or a polygon-based area for NG and Xn, and it is FFS on whether to add the geographical area scope inside or outside the existing area scope IE. Since legacy Area Scope of MDT is a mandatory IE, and there is a possibility that the UE can't obtain its location, to allow the UE can still perform MDT logging as legacy for the case that a geographical area scope for NTN MDT is configured to the UE but the UE can’t perform corresponding logging, it is better to optionally add the geographical area scope outside the existing area scope IE.

Proposal 2: Introduce the geographical area scope outside the existing area scope IE.

MRO for RACH-less Handover for NTN:

Last RAN3#127bis meeting agreed to divide the MRO for RACH-less handover case into two scenarios:

- Case 1: RACH-less handover was attempted but failed;

- Case 2: UE was configured RACH-less handover but did not attempt RACH-less handover because RSRP is below threshold

For Case 1, it may happen due to low RSRP threshold for RACH-less HO or inappropriate HO related configuration (e.g. target cell is wrong, or RACH-less HO related configuration is not proper), since RSRP threshold for RACH-less HO is configured by network, it can know the RSRP threshold as well as whether SSB measurement is higher than the RSRP threshold or not, based on its stored/retrieved UE context and existing information (e.g. cell/SSB measurement results, last serving cell ID, failed cell ID and etc) included in RLF report. Further, network can determine the failure cause and do corresponding MRO.

For Case 2, UE did not attempt the configured RACH-less HO because measured RSRP is below RSRP threshold, similar as Case 1, network can know the RSRP threshold and whether SSB measurement is lower than the RSRP threshold.

Therefore, there is no need for the UE to report the RSRP threshold for RACH-less HO in RLF report.

Proposal 3: No need to include the RSRP threshold for RACH-less HO in RLF report.

When RACH-less handover was attempted but failed, fallback from RACH-less HO to RACH-based HO could be performed. If RA-InformationCommon is included in RLF report, it means that fallback from RACH-less HO to RACH-based HO is performed, then network may optimize RACH-less HO related configuration. So, an explicit indication regarding fallback from RACH-less HO to RACH-based HO is not needed in RLF report.

Proposal 4: An explicit indication regarding fallback from RACH-less HO to RACH-based HO is not needed in RLF report.

3	Conclusion

In this paper, SON/MDT enhancements for NTN are discussed. We have the following proposals:

Proposal 1: No need to introduce distance-based event trigger for logged MDT in NTN.

Proposal 2: Introduce the geographical area scope outside the existing area scope IE.

Proposal 3: No need to include the RSRP threshold for RACH-less HO in RLF report.

Proposal 4: An explicit indication regarding fallback from RACH-less HO to RACH-based HO is not needed in RLF report.