3GPP TSG-RAN WG3 Meeting #129	R3-255396

Bengaluru, India, August 25-29, 2025

Agenda item:	 11.3

Source:		Lenovo

Title:		Discussion on AI/ML based CCO

Document for:		Discussion and Decision

1	Introduction

RAN3#128 meeting agreed that:

Adopt Opt2: gNB-CU directly forwards the received further CCO state of neighbor cells to gNB-DU.

Timing information for predicted CCO issue over Xn is not needed?

Specify mechanisms (F1, Xn) to update/cancel a prediction, no new IE is needed to identify the previous prediction (affected cells and beams will identify the previously signalled issue).

Addition of new code-point in predicted coverage modification cause (Xn) / predicted CCO issue (F1), with details FFS.

In this contribution, we would further discuss the left issues related to AI/ML based CCO.

2	Discussion

In non-split architecture, it has agreed that:

- Step 0: gNB1 predicts the CCO issue/Coverage Modification Cause.

- Step 1: gNB1 generates the future coverage status based on the predicted CCO issue/Coverage Modification Cause and other information.

- Step 2: gNB1 sends the predicted CCO issue/Coverage Modification Cause and future coverage status to neighbour gNB2.

As captured in [1], gNB1 sends the Time for Future Coverage State to neighbour gNBs, which indicates the time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the gNB1 relative to the time of receiving the NG-RAN NODE CONFIGURATION UPDATE message. It is FFS on whether to send time information related to the predicted coverage modification cause over Xn. Regarding CU-DU split architecture, time information for Predicted CCO Issue/Coverage Modification Cause is a relative time from the time of receiving the gNB-CU Configuration Update message over F1, and time information for future CCO state is a relative time from the time of receiving the gNB-DU Configuration Update message over F1, the two time information are totally different. To make neighbour gNB2 clearly understand and use the information for deducing the CCO issue predicted in gNB1 and for configuring coverage state of its served cell(s), as well as to enable gNB2 to further predict its own CCO issue2 based on the predicted CCO issue1 and future coverage status received from gNB1, it is beneficial to introduce time information for predicted coverage modification cause over Xn.

Proposal 1: Introduce time information for predicted coverage modification cause over Xn.

In split-architecture, it has agreed that:

- Step 0: gNB-CU predicts the CCO issue.

- Step 1: gNB-CU sends the predicted CCO issue to gNB-DU.

- Step 2: gNB-DU generates the future coverage status based on the predicted CCO issue and other local information.

- Step 3: gNB-DU sends the future coverage status to gNB-CU.

- Step 4: gNB-CU sends the predicted CCO issue and future coverage status to neighbour gNB-CU.

Regarding the predicted CCO issue from gNB-CU to gNB-DU, the following two scenarios are agreed:

- gNB-CU1 predicts the CCO issue1 for gNB-DU1 and sends to its gNB-DU1;

- Based on the received predicted CCO issue1 and future CCO state from gNB-CU1, gNB-CU2 will deduce for gNB-DU2 the predicted CCO issue2 as well as affected cells and beams, and send it to its gNB-DU2.

For the issue on whether gNB-CU2 would send the received CCO issue1 predicted by gNB-CU1 to gNB-DU2, regarding gNB-CU2 would predict CCO issue2 for gNB-DU2 based on the predicted CCO issue1 and future coverage status received from gNB-CU1, it makes no sense of gNB-CU2 to redundantly send the CCO issue1 predicted by gNB-CU1 for gNB-DU1 to gNB-DU2.

Proposal 2: gNB-CU2 may not send the CCO issue predicted by gNB-CU1 to its gNB-DU2.

RAN3#128 meeting agreed that gNB-CU directly forwards the received further CCO state of neighbor cells to gNB-DU. With regarding to whether gNB-CU generates a recommend future CCO state and sends it to gNB-DU as assistance information, similar as legacy CCO mechanism, the gNB-DU may resolve the predicted CCO issue concerning own served cell by local action within the OAM configured limits, and the gNB-DU may also take into account information received for other cells when generating future CCO state. Therefore, it is unnecessary for the gNB-CU to generate a recommend future coverage state and send it to gNB-DU as assistance information.

Proposal 3: gNB-CU does not generate a recommend future coverage state and send it to gNB-DU as assistance information.

Since the future coverage state is a prediction or based on a predicted CCO issue, the prediction might be updated or might not be correct. Last RAN3 meeting agreed to update/cancel a prediction in Xn or F1 interface, and it is FFS on the details of addition of new code-point in predicted coverage modification cause (Xn) / predicted CCO issue (F1), regarding the intention of the new code-point is to update/cancel previous prediction related to coverage or cell edge capacity, new code-point such as “coverage update”, “coverage cancel”, “cell edge capacity update” or “cell edge capacity cancel” should be added in the Predicted Coverage Modification Cause IE over Xn interface, as well as Predicted CCO issue IE or Future Coverage Modification Cause IE over F1 interface.

Proposal 4: New code-point such as “coverage update”, “coverage cancel”, “cell edge capacity update” or “cell edge capacity cancel” should be added in the Predicted Coverage Modification Cause IE over Xn interface, as well as Predicted CCO issue IE or Future Coverage Modification Cause IE over F1 interface.

3	Conclusion

In this contribution, AI/ML based CCO are discussed. We have the following proposals:

Proposal 1: Introduce time information for predicted coverage modification cause over Xn.

Proposal 2: gNB-CU2 may not send the CCO issue predicted by gNB-CU1 to its gNB-DU2.

Proposal 3: gNB-CU does not generate a recommend future coverage state and send it to gNB-DU as assistance information.

Proposal 4: New code-point such as “coverage update”, “coverage cancel”, “cell edge capacity update” or “cell edge capacity cancel” should be added in the Predicted Coverage Modification Cause IE over Xn interface, as well as Predicted CCO issue IE or Future Coverage Modification Cause IE over F1 interface.

Reference

[1] R3- R3-255044, (BL CR to 38.423) Support of enhancements on AI/ML for NG-RAN