3GPP TSG-RAN WG3 Meeting #129	R3-255402

Bengaluru, India, 25th – 29th August, 2025

Agenda Item:	12.3

Source: 	Lenovo

Title: 	Discussion on remaining issues for NR Femto

Document for:	Discussion and decision

- Introduction

A new WID for additional topological enhancements [1] has been approved to be discussed in Rel-19, and following detail objectives are identified for 5G Femto fucus on RAN3.

The objectives of the 5G Femto work are as follows: - Specification to support NR Femto architecture with optional NR Femto GW for NG interface [RAN3]. - Specification to support access control for NR Femtos operating in open, hybrid and closed modes reusing existing CAG functionality [RAN3]. NOTE 10: For NR Femto access control, only stage 2 impact is expected on this objective. NOTE 11: Coordination with other WGs (e.g. SA2, SA3) when needed.

In this contribution, some remaining issues are discussed based on the latest BL CR.

- Discussion

Femto indication in NG

RAN3 #128: To be further discussed: whether to send a specific Femto indication in the NG: Initial UE message, from NR Femto to enable control of sending Allowed PNI NPN List or not.

This is a security issue related to the trusty of Femto node in the CN, which needs to be discussed at SA3. From RAN3 point of view, we had agreed to reuse current PNI-NPN functionalities for NR Femto, then the AMF just to follow the mechanism defined for NPN and send the Allowed PNI NPN List to the NR Femto node regardless of the Femto indication in the NG.

Proposal 1: No need to introduce a Femto indication in the NG unless triggered by SA3.

Transport layer address for NG-U

RAN3 #127: The Femto GW hosts the following function: - Selection of an IP version to be used for NG-U, if a NG-U UP transport layer information configuration contains two transport layer addresses of different versions. To be continued… RAN3 #127bis: If the SMF sends both IP versions in the Transport Layer Address IE, the NR Femto node selects the correct IP version. It is FFS whether the GW can perform IP Version selection in addition to the NR Femto. RAN3 #128: Whether NR Femto GW my perform IP version selection for NG-U transport?

For UE served by the NR Femto, the 5GC and NR Femto setup the PDU session resource via the UE-associated NGAP procedure, which is just relayed by the NR Femto GW, then the SMF is aware of the DL TNL address of the NR Femto for the NG-U transport.

In the LTE HeNB, the HeNB doesn’t support the IP version selection, and the HeNB GW can optional terminate the S1-U interface with the HeNB and with the S-GW, then the HeNB GW needs to support the IP version selection. However, based on the protocol stack for NG-U of NR Femto, the NR Femto GW just plays a role of IP router and not terminate NG-U from the 5GC. And in last meeting, it was agreed that NR Femto can select the correct IP version in case the SMF sends both IP versions. There is no need for the NR Femto GW to perform IP version selection.

Proposal 2: The NR Femto GW doesn’t need to perform IP version selection in addition to the NR Femto.

Handover

As in LTE HeNB, the source eNB configures the UE with proximity indication control and the UE sends an "entering" proximity indication when it determines it may be near a CSG member cell. Then the source eNB configures the UE with measurement configuration and further SI acquisition and reporting to acquire the relevant system information from the target HeNB.

7) The UE sends a measurement report including (E-)CGI, TAI, CSG ID and "member/non-member" indication. If the target cell is a shared CSG/hybrid cell, the measurement report also includes the subset of the broadcast PLMN identities that pass PLMN ID check and for which the CSG whitelist of the UE includes an entry comprising the cell's CSG ID and the respective PLMN identity.

However, in NR, each NG-RAN node informs the connected neighbour NG-RAN nodes of the list of supported CAG ID(s) per CAG cell in the Xn interface management procedures, e.g., Xn setup procedure. In addition, the cell type (CAG cell or NPN-only cell) can be also obtained implicitly via absent of the PLMN Identities and associated information in the PLMN-IdentityInfoList IE.

Therefore, there is no need to introduce proximity indication for NR Femto, and there is no need to exchange cell type via Xn interface.

Proposal 3: There is no need to introduce proximity indication for NR Femto, and there is no need to exchange cell type via Xn interface.

# Conclusion

This contribution aims to discuss remaining issues for NR Femto. And following observations and proposals are concluded.

Proposal 1: No need to introduce a Femto indication in the NG unless triggered by SA3.

Proposal 2: The NR Femto GW doesn’t need to perform IP version selection in addition to the NR Femto.

Proposal 3: There is no need to introduce proximity indication for NR Femto, and there is no need to exchange cell type via Xn interface.

# Reference

- RP-242395. New WID on additional topological enhancements for NR.

- Chairman notes of RAN3 127-128 meetings.