3GPP TSG-RAN WG3 #129	R3-255407

Bangaluru, India, 25th – 29th August 2025

Agenda Item:	20.2

Source:	Lenovo

Title: 		(TP to 38.455) Provision of associated ID in Case 1

Document for:	Discussion and Approval

# Introduction

In this paper, we further discuss issues related to AIML for gNB assisted positioning case 1.

# Discussion

In the previous RAN1 meeting, RAN1 agreed to support the transmission of associated ID to UE to ensure the consistency between the model training and the model inference.

RAN1 Agreement: Agreement For AI/ML based positioning Case 1, regarding Info #7 in the assistance information from legacy UE-based DL-TDOA, - If implicitly provided, the implicit indication of Info #7 is via associated ID. - For given TRP(s), same associated ID implies that geographical coordinates of the TRP(s) can be understood as consistent by the UE. - The associated ID is not expected to provide the real value of Info #7 (i.e., geographical coordinates of the TRP(s) are not disclosed). - an associated ID is configured per-cell (e.g., NCGI-r15) - UE does not expect to receive different values of associated ID for TRPs belonging to the same NCGI-r15 - Associated ID can be realized by an identifier of N bits (e.g., 8 bits)

It is also implemented in the LPP protocol that upon UE’s request LMF can transmit the associated ID to UE as part of NR-TRP-LocationInfo-Implicit IE when providing the assistance data to UE. Per definition, the associated ID is the identity associated with the coordinates of the indicated TRP and coordinates of all the antenna reference points of this TRP. The value of the nr-AIML-AssociatedID is changed if/when the coordinates of the TRP and/or coordinates of any of the antenna reference points of this TRP is changed.

#### NR-TRP-LocationInfo-Implicit The IE NR-TRP-LocationInfo-Implicit provides information to enable a target device to determine whether the coordinates of TRPs and coordinates of the antenna reference points for a set of TRPs are consistent between training and inference phases for NR DL AI/ML positioning. -- ASN1START NR-TRP-LocationInfo-Implicit-r19 ::= SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF NR-TRP-LocationInfo-Implicit-PerFreqLayer-r19 NR-TRP-LocationInfo-Implicit-PerFreqLayer-r19 ::= SEQUENCE { trp-LocationInfo-Implicit-List-r19 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF TRP-LocationInfo-Implicit-Element-r19, ... } TRP-LocationInfo-Implicit-Element-r19 ::= SEQUENCE { dl-PRS-ID-r19 INTEGER (0..255), nr-PhysCellID-r19 NR-PhysCellID-r16 OPTIONAL, -- Need ON nr-CellGlobalID-r19 NCGI-r15 OPTIONAL, -- Need ON nr-ARFCN-r19 ARFCN-ValueNR-r15 OPTIONAL, -- Need ON nr-AIML-AssociatedID-r19 NR-AIML-AssociatedID-r19, ... } NR-AIML-AssociatedID-r19 ::= SEQUENCE { nr-AIML-AssociatedID INTEGER (0..255), -- FFS ... } -- ASN1STOP NR-TRP-LocationInfo-Implicit field descriptions. dl-PRS-ID This field provides the DL-PRS ID of the DL-PRS configuration of the TRP.. nr-PhysCellID This field specifies the physical cell identity of the TRP.. nr-CellGlobalID This field specifies the NCGI, the globally unique identity of a cell in NR, of the TRP.. nr-ARFCN This field specifies the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID.. nr-AIML-AssociatedID This field provides an identity associated with the coordinates of the indicated TRP and coordinates of all the antenna reference points of this TRP. The value of the nr-AIML-AssociatedID is changed if/when the coordinates of the TRP and/or coordinates of any of the antenna reference points of this TRP is changed.. Editor's Note: The IE NR-TRP-LocationInfo-Implicit is FFS and depends on further RAN1 progress. In particular: - Must the "Associated ID" for the TRP coordinates be associated to a "cell" (e.g., NCGI)? If so, how should PRS-only TPs (which are not associated to a cell) being handled? - Is the "Associated ID" only for the TRP location coordinates, or also for the associated DL-PRS Resource Set/DL-PRS Resource ARPs?

In case 1, LMF can transmit associated ID to UE for the consistency of model training and model inference.

Then, considering the associated ID is strongly linked to the deployment of TRP, the next question is how LMF obtains the associated ID. There are two possibilities:

- Possibility 1: upon the request from LMF, gNB provides the associated ID of concerned TRP to LMF.

- Possibility 2: based on the TRP information received from gNB, LMF map it to an associated ID.

Comparing above two, we believe Possibility 1 is more aligned with the agreement from RAN1 that “The associated ID is not expected to provide the real value of Info #7” meaning the LMF may not determine the associated ID from real TRP coordinates.

For case 1, RAN3 supports the LMF to request the associated ID from gNB for concerned TRP(s) via NRPPa.

Such associated ID request and provision can be easily supported under the framework of TRP Information Request/Response procedure by introducing TRP Associated ID as one TRP Information Type. An exemplary TP is provided in the Annex.

RAN3 introduces TRP Associated ID as one TRP Information Type which can be requested and provided via the TRP INFORMATION REQUESTION/RESPONSE procedure.

# Conclusion

Based on the discussion above, we observe:

Observation 1	In case 1, LMF can transmit associated ID to UE for the consistency of model training and model inference.

Based on the discussion above, we propose:

Proposal 1	For case 1, RAN3 supports the LMF to request the associated ID from gNB for concerned TRP(s) via NRPPa.

Proposal 2	RAN3 introduces TRP Associated ID as one TRP Information Type which can be requested and provided via the TRP INFORMATION REQUESTION/RESPONSE procedure.

# Annex TP to 38.455

**************************************** CHANGE STARTS ****************************************

#### 9.1.1.14	TRP INFORMATION REQUEST

This message is sent by an LMF to request information for TRPs hosted by an NG-RAN node.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| TRP List |  | 0 ..1 |  |  | YES | ignore |
| >TRP Item |  | 1 .. <maxnoTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>PRS Bandwidth Aggregation Request Indication | O |  | ENUMERATED(true, …) |  | YES | ignore |
| TRP Information Type List |  | 1 |  |  | YES | reject |
| >TRP Information Type Item |  | 1 .. <maxnoTRPInfoTypes> |  |  | EACH | reject |
| >>TRP Information Type Item | M |  | ENUMERATED (nr pci, ng-ran cgi, nr arfcn, prs config, ssb config, sfn init time, spatial direction info, geo-coordinates, …, trp type, on-demand prs, trp tx teg, beam antenna info, mobile trp location info, common ta, associated id) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPs | Maximum no. of TRPs in a NG-RAN node. Value is 65535 |
| maxnoTRPInfoTypes | Maximum no of TRP information types that can be requested and reported with one message. Value is 64. |

**************************************** NEXT CHANGE ****************************************

### 9.2.25	TRP Information

The TRP Information IE contains information for one TRP within an NG-RAN node.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| TRP ID | M |  | 9.2.24 |  | - |  |
| TRP Information Type |  | 1 .. <maxnoTRPInfoTypes> |  |  | - |  |
| >CHOICE TRP Information Item | M |  |  |  | - |  |
| >>NR PCI |  |  | INTEGER (0..1007) | NR Physical Cell ID |  |  |
| >>NR CGI |  |  | 9.2.9 |  |  |  |
| >>NR ARFCN |  |  | INTEGER (0..3279165) |  |  |  |
| >>PRS Configuration |  |  | 9.2.44 |  |  |  |
| >>SSB Information |  |  | 9.2.54 |  |  |  |
| >>SFN Initialisation Time |  |  | Relative Time 1900 9.2.36 |  |  |  |
| >>Spatial Direction Information |  |  | 9.2.45 |  |  |  |
| >>Geographical Coordinates |  |  | 9.2.46 |  |  |  |
| >>TRP type |  |  | ENUMERATED (prs-only-tp, srs-only-rp, tp, rp, trp, …, mobile trp) | TS 38.305 [18] | YES | reject |
| >>On-demand PRS TRP Information |  |  | 9.2.65 |  | YES | reject |
| >>TRP Tx TEG Association |  |  | 9.2.79 |  | YES | reject |
| >>TRP Beam Antenna Information |  |  | 9.2.82 |  | YES | reject |
| >>Mobile TRP Location Information | M |  | 9.2.88 |  | YES | ignore |
| >>Common TA Parameters | M |  | 9.2.89 |  | YES | reject |
| >>TRP Associated ID | M |  | 9.2.xx |  | YES | reject |
| Mobile IAB-MT UE ID | C-ifMobileTRP |  | OCTET STRING | The UE ID of the IAB-MT associated with the mobile TRP. Includes the GPSI as defined in TS 29.571 [21] | YES | reject |

| Condition | Explanation |
| --- | --- |
| ifMobileTRP | This IE shall be present if the TRP type IE is set to the value "mobile trp" |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPInfoTypes | Maximum no of TRP information types that can be requested and reported with one message. Value is 64. |

**************************************** NEXT CHANGE ****************************************

### 9.2.xx	TRP Associated ID

The TRP Associated ID IE provides an identity associated with the coordinates of the indicated TRP and coordinates of all the antenna reference points of this TRP. The value changes if/when the coordinates of the TRP and/or coordinates of any of the antenna reference points of this TRP is changed.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| TRP Associated ID | M |  | INTEGER (0-255) |  | - |  |

**************************************** CHANGE ENDS ****************************************