3GPP TSG-RAN WG3 Meeting #129	R3-255409

Bengaluru, India, 25 – 29 August 2025

Agenda item:	21.3

Source: 	Lenovo

Title: 	(TP to XR BL CR for 38.425) On Unnecessary RLC Retransmission Avoidance

Document for:	Other

- 1. Introduction

In RAN3#128 meeting, the following agreements and open issues were made regarding RAN3 impact of Unnecessary RLC retransmission avoidance.

Reuse the existing PDCP discard indication in F1-U to indicate the gNB-DU to stop the transmission/retransmission of a RLC SDU or the segment of a RLC SDU. FFS on how to capture this agreement.

The contribution discusses the open issues listed above.

- 2. Discussion

For unnecessary RLC retransmission avoidance, it was agreed in last meeting:

Reuse the existing PDCP discard indication in F1-U to indicate the gNB-DU to stop the transmission/retransmission of a RLC SDU or the segment of a RLC SDU. FFS on how to capture this agreement.

The existing PDCP discard related IEs in the F1-U DL USER DATA frame is defined as:

The existing discard related IEs were introduced for PDCP duplication to avoid unnecessary transmission in the duplicated path if the PDCP PDUs has been delivered to UE successfully in the other path. However, the exiting ‘discard’ related IEs cannot be reused directly due to the following reasons:

-	For legacy, the existing "discard" indication applies only to RLC SDUs that are not currently being transmitted or retransmitted. It does not affect RLC PDUs that have already been submitted to lower layers.

-	For the new feature introduced in Rel-19, the "discard" indication has a further impact on ongoing transmissions or retransmissions. In RAN2, the UE behavior is defined as "stopping the transmission and retransmission of the SDU or the segment of the SDU.

Observation 1: The exiting PDCP discard related IEs the DL USER DATA frame are applies only to RLC SDUs that are not currently being transmitted or retransmitted, while in Rel-19 it has a further impact on the RLC PDUs that is under ongoing transmissions or retransmission.

In the existing PDCP discard there are two ways to indicate which PDU PDUs should be discarded. The first way is to use the DL discard NR PDCP PDU SN

To avoid unnecessary RLC retransmissions, the transmission / retransmission of an RLC SDU (or its segment) is stopped as soon as the corresponding PDCP discard timer expires.
When two discard-timers coexist—normal and “low-importance”—the following may happen:

- PDCP SDU #1 (with normal PSI) starts the normal discard timer.

- PDCP SDU #2 (with low importance PSI) starts the shorter low-importance discard timer.

If the low-importance PDCP discard timer for SDU #2 expires first while the normal PDCP discard timer for SDU #1 is still running, the gNB-CU will signal a DL discard indication for PDCP SN #2. Upon receiving this indication, the gNB-DU must stop transmitting or retransmitting only SDU #2; the ongoing transmission of SDU #1 must remain unaffected because its normal discard timer has not expired, and the discard indication is not intended for SDU #1.

However, according to current specification, if gNB-CU indicates the PDCP SDU#2 as ‘DL discard NR PDCP PDU SN’, the gNB-DU shall stop transmission or retransmission for both PDCP SDU#2 and PDCP SDU#1.

5.5.3.8 DL discard NR PDCP PDU SN Description: This parameter indicates the downlink NR discard PDCP PDU sequence number up to and including which all the NR PDCP PDUs should be discarded. Value range: {0..218-1}. Field length: 3 octets.

Observation 2: the DL discard NR PDCP PDU SN does not apply for that case that PDCP discard timer for low importance is configured.

The other way is to use the Discard Blocks. The Discard Blocks is to indicate to discard a number for NR PDCP PDU blocks to be is discarded, and each block includes a number of continuous PDCP SNs. The PDCP Discard Blocks is efficient in case of PDCP duplication, however, it is not efficient for avoiding unnecessary RLC retransmissions, in which the stop retransmission and retransmission is determined for per individual PDCP PDU.

Observation 3: the DL Discard Blocks are not efficient for avoiding unnecessary RLC retransmissions, in which the stop retransmission and retransmission is determined for per individual PDCP PDU.

According to the above observations and to avoid any potential backwards compatibility issue, it makes sense to introduce a new IE in the DL USER DATA frame specifically indicates the PDCP PDU associated with an RLC SDU, or a segment of the RLC SDU, for which the transmission and retransmission should be stopped.

Proposal:  Introduce a new IE in the DL USER DATA frame specifically indicates the PDCP PDU associated with an RLC SDU, or a segment of the RLC SDU, for which the transmission and retransmission should be stopped.

- 3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals.

Observation 1: The exiting PDCP discard related IEs the DL USER DATA frame are applies only to RLC SDUs that are not currently being transmitted or retransmitted, while in Rel-19 it has a further impact on the RLC PDUs that is under ongoing transmissions or retransmission.

Observation 2: the DL discard NR PDCP PDU SN does not apply for that case that PDCP discard timer for low importance is configured.

Observation 3: the DL Discard Blocks are not efficient for avoiding unnecessary RLC retransmissions, in which the stop retransmission and retransmission is determined for per individual PDCP PDU.

Proposal:  Introduce a new IE in the DL USER DATA frame specifically indicates the PDCP PDU associated with an RLC SDU, or a segment of the RLC SDU, for which the transmission and retransmission should be stopped.

- 5. Text Proposal

Text Proposal to TS 38.425

<<<<<<<<<<<<<<<change start>>>>>>>>>>>>>

- 5.2	NR user plane protocol layer services

NOTE 1:	In this section, NR user plane protocol layer services are also applicable to E-UTRA PDCP. With this understanding, each instance of NR PDCP can be replaced by E-UTRA PDCP.

The following functions are provided by the NR user plane protocol:

-	Provision of NR user plane specific sequence number information for user data transferred from the node hosting NR PDCP to the corresponding node for a specific data radio bearer.

-	Information of successful in sequence delivery of NR PDCP PDUs to the UE from the corresponding node for user data associated with a specific data radio bearer.

-	Information of NR PDCP PDUs that were not delivered to the UE or not transmitted to the lower layers.

-	Information of NR PDCP PDUs transmitted to the lower layers for user data associated with a specific data radio bearer.

-	Information of downlink NR PDCP PDUs to be discarded for user data associated with a specific data radio bearer;

-	Information of the currently desired buffer size at the corresponding node for transmitting to the UE user data associated with a specific data radio bearer.

-	Information of the currently desired data rate in bytes at the corresponding node for transmitting to the UE user data associated with a specific data radio bearer;

-	Information of successful in sequence delivery of NR PDCP PDUs to the UE from the corresponding node for retransmission user data associate with a specific data radio bearer;

-	Information of NR PDCP PDUs transmitted to the lower layers for retransmission user data associated with a specific data radio bearer.

-	Information of the specific events at the corresponding node.

-	Information on Radio Link Quality from the corresponding node for user data associated with a specific data radio bearer.

-	Information for QoS monitoring from the corresponding node for user data associated with a specific data radio bearer.

-	Information indicating to stop transmission and retransmission of an RLC SDU or a segment of a RLC SDU for user data associated with a specific data radio bearer.

<<<<<<<<<<<<<<<Next Change>>>>>>>>>>>>>

5.4.1	Transfer of Downlink User Data

5.4.1.1	Successful operation

The purpose of the Transfer of Downlink User Data procedure is to provide NR-U specific sequence number information at the transfer of user data carrying a DL NR PDCP PDU from the node hosting the NR PDCP entity to the corresponding node.

An NR user plane protocol instance making use of the Transfer of Downlink User Data procedure is associated to a single radio bearer only.

The node hosting the NR PDCP entity shall assign consecutive NR-U sequence numbers to each transferred NR-U packet. A retransmitted NR PDCP PDU shall be assigned a new NR-U sequence number.

The node hosting the NR PDCP entity indicates to the corresponding node whether this NR-U packet is a retransmission of NR PDCP PDU.

The node hosting the NR PDCP entity can indicate to the corresponding node to either discard all NR PDCP PDUs up to and including a defined DL discard NR PDCP PDU SN or discard one or a number of blocks of downlink NR PDCP PDUs.

If the Assistance Information Report Polling Flag is equal to 1, the corresponding node shall, if supported, send the ASSISTANCE INFORMATION DATA to the node hosting the NR PDCP entity.

The corresponding node shall detect whether an NR-U packet was lost and memorise the respective sequence number after it has declared the respective NR-U packet as being "lost".

The corresponding node shall transfer the remaining NR PDCP PDUs towards the UE and memorise the highest NR PDCP PDU sequence number of the NR PDCP PDU that was successfully delivered (as defined in TS 36.322 [6] and TS 38.322 [7]) in sequence towards the UE (in case RLC AM is used) and the highest NR PDCP PDU sequence number of the NR PDCP PDU that was transmitted to the lower layers.

The corresponding node shall send the DL DATA DELIVERY STATUS if the Report Polling Flag is set to 1 or when the NR PDCP PDU with the indicated DL report NR PDCP PDU SN has been successfully delivered, unless a situation of overload at the corresponding node is encountered. The DL DATA DELIVERY STATUS sent as a response to a specific DL report NR PDCP PDU SN shall be sent only when all PDCP PDU SNs up to this DL report NR PDCP PDU have been successfully delivered in-sequence.

If the Request OutOfSeq Report is set to 1, the corresponding node shall, if supported, include the NR PDCP PDU sequence number successfully delivered out of sequence in the DL DATA DELIVERY STATUS to the node hosting the NR PDCP entity.

NOTE:	The Transfer of Downlink User Data procedure and the associated feedback of lost NR-U packets assist the node hosting the NR PDCP entity in avoiding NR PDCP HFN de-synchronisation. If a deployment decides to not use the Transfer of Downlink User Data procedure, NR PDCP HFN synchronization should be ensured by other means.

If the User data existence flag is set to 1, the corresponding node assumes that the node hosting the NR PDCP entity has some user data for the concerned data radio bearer. The corresponding node decides whether and when to use DRX for the UE (i.e. the corresponding node may indicate the UE to use DRX even if the flag is set to 1 and the received DL USER DATA frame contains no user data).

The node hosting the NR PDCP entity can indicate the corresponding node to stop the transmission or retransmission of an RLC SDU or a segment of an RLC SDU which is with a defined DL Stop Retransmission NR PDCP PDU SN.

Figure 5.4.1.1-1: Successful Transfer of Downlink User Data

<<<<<<<<<<<<<<<Next Change>>>>>>>>>>>>>

5.5.2.1	DL USER DATA (PDU Type 0)

This frame format is defined e.g. to allow the corresponding node to detect lost NR-U packets and may be associated with the transfer of a Downlink PDCP PDU.

The following shows the respective DL USER DATA frame.

NOTE 1:	All information elements defined in Figure 5.5.2.1-1 are also applicable to E-UTRA PDCP. With this understanding, each instance of NR PDCP can be replaced by E-UTRA PDCP.

| Bits |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| PDU Type (=0) |  |  |  | DL Stop | DL Discard Blocks | DL Flush | Report polling | 1 |
| Spare |  |  | Request OutofSeq Report | Report Delivered | User data existence flag | Assistance Info. Report Polling Flag | Retransmission flag | 1 |
| NR-U Sequence Number |  |  |  |  |  |  |  | 3 |
| DL discard NR PDCP PDU SN |  |  |  |  |  |  |  | 0 or 3 |
| DL discard Number of blocks |  |  |  |  |  |  |  | 0 or 1 |
| DL discard NR PDCP PDU SN start (first block) |  |  |  |  |  |  |  | 0 or 3 |
| Discarded Block size (first block) |  |  |  |  |  |  |  | 0 or 1 |
| … |  |  |  |  |  |  |  |  |
| DL discard NR PDCP PDU SN start (last block) |  |  |  |  |  |  |  | 0 or 3 |
| Discarded Block size (last block) |  |  |  |  |  |  |  | 0 or 1 |
| DL report NR PDCP PDU SN |  |  |  |  |  |  |  | 0 or 3 |
| DL stop retransmission NR PDCP PDU SN |  |  |  |  |  |  |  | 0 or 3 |
| Padding |  |  |  |  |  |  |  | 0-3 |

Figure 5.5.2.1-1: DL USER DATA (PDU Type 0) Format

<<<<<<<<<<<<<<<Next Change>>>>>>>>>>>>>

5.5.3.x		DL Flush

Description: This parameter indicates the presence of DL stop retransmission NR PDCP PDU SN.

Value range: {0= DL stop retransmission	NR PDCP PDU SN not present, 1= DL stop retransmission NR PDCP PDU SN present}.

Field length: 1 bit.

<<<<<<<<<<<<<<<Next Change>>>>>>>>>>>>>

5.5.3.8	DL stop retransmission NR PDCP PDU SN

Description: This parameter indicates the downlink NR discard PDCP PDU sequence number for which transmission or retransmission of RLC SDU or segments of RLC SDU shall be stopped.

Value range: {0..218-1}.

Field length: 3 octets.