3GPP TSG-RAN WG3 #129	R3-255421

Bengaluru - India, 25 – 29 August 2025

Agenda Item:	13.2. Support for inter-CU LTM

Source:	NTT DOCOMO INC.

Title:	Discussion on inter-CU LTM

Document for:	Discussion, agreement

Introduction

At the previous meeting, there were following agreements for LTM in non-DC case.

[SP CSI-RS]

WA: RAN3 agrees that, for both F1AP and XnAP, the activation and deactivation of CSI-RS transmission in LTM candidate cells are performed at the level of individual CSI-RS Resource IDs.

Add description in Stage 2 TS 38.401 for describing that CU can request Candidate DU to provide CSI-RS configuration in UE CONTEXT SETUP REQUEST and UE CONTEXT MODIFICATION REQUEST message, and Candidate DU signals the CSI-RS configuration in UE CONTEXT SETUP RESPONSE and UE CONTEXT MODIFICATION RESPONSE message.

Reuse the CSI-RS coordination procedure over F1AP and XnAP for source gNB/gNB-DU to activate or deactivate the SP CSI-RS resource for CSI acquisition in candidate cell.

FFS on whether gNB-DU/gNB provides the report type (periodic or semi-persistent) of the CSI-RS resources in both F1AP and XnAP.

FFS on whether the TCI State/QCL-info List needs to be included in CSI-RS COORDINATION procedure.

FFS on whether to include the SP CSI-RS and SSB mapping in the HANDOVER REQUEST ACKKNOLEDGE message and UE Context Modification Request message.

FFS on whether to add a new IE for SP CSI-RS resource for CSI acquisition in the corresponding procedure of SP CSI-RS resource for L1 RSRP measurement in F1AP and XnAP.

[General]

Remove Note in TS 38.300 BL CR “Editor’s Note: step 6 and 7 are optional.”

In Xn interface, candidate gNB provides the LTM CFRA Resource Configuration of each candidate cell to source gNB for LTM cell switch command generation.

Source gNB generate the UE Based TA Measurement Configuration, and transfer it to all candidate gNB(s) via LTM Configuration Update message.

Update the TA INFORMATION TRANSFER message to provide a list of TA values of multiple candidate cells.

[Security]

For Inter-CU LTM, LTM CONFIGURATION UPDATE procedure is per node level basis with a list of cells, and security key is per cell.

Include the Rel-19 set IDs of source cell and each candidate cell(s) in UE Context Modification Request message. Introduce Rel-19 set IDs into LTM Security Information IE.

F FFS on the Rel-19 Set ID(s) assignment among CUs, down select from Option1 and Option2:

- Option 1: Source gNB sends the Rel-19 Set ID(s) or Rel-19 set ID range assigned to the candidate gNB in the HANDOVER REQUEST message, then candidate gNB assigns Rel-19 set ID(s) to its own candidate cells and feedback via HANDOVER REQUEST ACKNOWLEDGE message.

- Option 2: Candidate gNB provides Rel-19 set ID per candidate cell in HANDOVER REQUEST ACKNOWLEDGE message, then source gNB may update the Rel-19 set ID to ensure that the Rel-19 set IDs under different candidate gNB-CU are different.

FFS on the procedure to be used for source gNB to transfer Rel-19 set ID per candidate cell to the candidate gNB.

- Option 1: Via Cell Switch Notification message: ZTE, LGE

- Option 2: Via LTM Configuration Update message: CT, Lenovo, CATT, Ericsson

- Option 3: Via both message

Discussion

SP CSI-RS

At the RAN3#128 meeting, the following WAs were captured.

WA: RAN3 agrees that, for both F1AP and XnAP, the activation and deactivation of CSI-RS transmission in LTM candidate cells are performed at the level of individual CSI-RS Resource IDs.

According to LS [1] from RAN2, SP CSI-RS activation/deactivation is indicated per LTM-CSI-ResourceConfigId. However, according to following agreements in RAN1, RAN3 need to have different signalings for L1 measurement report and for CSI acquisition.

Agreement from RAN1#120

An LTM report configuration for L1-RSRP is associated with a single resource config that includes:

  - Alt.A: a single resource set containing CSI-RS resources corresponding to multiple candidate cells. i.e., the same design as that of SSB in Rel-18 LTM.

  - FFS: how to associate between the measurement CSI-RS resources and candidate cells

    - explicit or implicit signaling of candidate cells

Agreement from RAN1#120bis

Regarding CSI acquisition, for a candidate cell,

- A single CSI report configuration is configured

- Multiple CSI-RS resources for CMR can be associated with the CSI report configuration

  - The number of CSI-RS resources for CMR is subject to UE capability

Above agreements mean that, for L1 measurement report, there is no restriction on resource config and report config, thus multiple resource configs can be configured, while for CSI acquisition, there is restriction that single report config can be configured for each candidate cell, thus only one resource config can be configured for each candidate cell. This means that for L1 measurement report, NW have a flexibility to select which SP CSI-RS can be activated by a single LTM-CSI-ResourceConfig, but for CSI acquisition, NW does not have flexibility on how to include multiple CSI-RS corresponding a candidate cell into the LTM-CSI-ResourceConfig referred by LTM-CSI-ReportConfig for CSI acquisition.

For more flexibility on NW selection which CSI-RS to be activated/deactivated, we propose to have different granularity of target SP CSI-RS resource indication in Xn/F1 SP CSI-RS Coordination procedures. We also provide an example of F1AP stage3 implementation.

#### 9.2.2.x1	DU-CU CSI-RS COORDINATION REQUEST

This message is sent by the gNB-DU to request the gNB-CU e.g. to activate/deactivate the SP CSI-RS transmissions from specific cells. (Detail is FFS)

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  |
| CSI-RS to be Activated List |  | 0..1 |  |  |
| >CSI-RS Resource to be Activated item IEs |  | 1 .. <maxnoofCellList> |  |  |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  |
| >>SP CSI-RS Resource ID | M | INTEGER (0..191) | Corresponds to the NZP-CSI-RS-ResourceId IE, as defined in TS 38.331 [8]. |  |
| >CSI-RS Resource Set to be Activated item IE |  | 0.. 1 |  |  |
| >>SP CSI-RS Resource Config ID | M | INTEGER (0..111) | Corresponds to the LTM-CSI-ResourceConfigId IE, as defined in TS 38.331 [8]. |  |
| CSI-RS to be Deactivated List |  | 0..1 |  |  |
| >CSI-RS Resource to be Deactivated Item IEs |  | 1 .. <maxnoofCellList> |  |  |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  |
| >>SP CSI-RS Resource ID | M | INTEGER (0..191) | Corresponds to the NZP-CSI-RS-ResourceId IE, as defined in TS 38.331 [8]. |  |
| >CSI-RS Resource Set to be Dectivated item IE |  | 0.. 1 |  |  |
| >>SP CSI-RS Resource Config ID | M | INTEGER (0..111) | Corresponds to the LTM-CSI-ResourceConfigId IE, as defined in TS 38.331 [8]. |  |

Proposal 1:	For both F1AP and XnAP, the activation and deactivation of CSI-RS transmission for L1 measurement report in LTM candidate cells are performed at the level of LTM-CSI-ResourceConfig IDs. For CSI acquisition, the activation and deactivation are performed per NZP-CSI-RS-Resource.

In the RRC running CR, the resource type (periodic or semi-persistent) is indicated in the LTM-CSI-ResourceConfig. On the other hand, the CSI-RS Resource Configuration IE in the agreed F1AP TP [2] is a list of NZP-CSI-RS-Resource, which does not include the resource type. Hence, the source gNB, which receives CSI-RS Resource Configuration IE from the candidate DU, cannot configure the resource type appropriately for each CSI-RS resources when generating the LTM-CSI-ResourceConfig. Therefore, in the LTM preparation phase, the candidate DU/CU should indicate the resource type of each NZP-CSI-RS-Resource to the source CU via CSI-RS Resource Configuration IE in Xn/F1. Following messages should contain the information.

- Handover Request Acknowledge

- LTM Configuration Update Acknowledge

- UE Context Setup Response

- UE Context Modification Response

Proposal 2:	During LTM preparation phase, the candidate DUs/CUs should send to the source CU resource type corresponding to each NZP-CSI-RS-Resource within Xn/F1 CSI-RS Resource Configuration.

As described in [3], in order for source DU to determine to which resource to trigger SP CSI-RS activation/deactivation, it needs to know the SSB association toward each CSI-RS. However, the qcl-InfoPeriodicCSI-RS contained in the NZP-CSI-RS-Resource can only be used for periodic CSI-RS, therefore, new mapping information should be introduced in the Xn/F1 messages.

Furthermore, regarding SSB mapping, it is necessary to be notified to all candidate DUs which can be serving DUs in the subsequent cell switch. Xn/F1 already has CSI Resource Configuration IE, which includes LTM-CSI-ResourceConfig, so it includes resource type, but it does not include information about SSB association toward CSI-RSs.

From the above, in the LTM preparation phase, the candidate DU/CU needs to indicate to the source CU the SSB association toward each NZP-CSI-RS-Resource for the SP CSI-RS prepared by the candidate DU/CU  within Xn/F1 CSI-RS Resource Configuration IE. The message containing this is as follows.

- Handover Request Acknowledge

- LTM Configuration Update Acknowledge

- UE Context Setup Response

- UE Context Modification Response

The source CU should also inform the candidate DU/CU of the NZP-CSI-RS-ResourceId and SSB Index mapping for all LTM candidates via Xn/F1 CSI Resource Configuration IE. This mapping should be included in:

- Handover Request

- LTM Configuration Update

- UE Context Setup Request

- UE Context Modification Request

The CSI Resource Configuration IE contains multiple LTM-CSI-ResourceConfigs within the ltm-CSI-ResourceConfigToAddModList and it comprises multiple pairs of NZP-CSI-RS-ResourceId and LTM-CandidateId-r18. For all pair of them, SSB association should be indicated in above messages.

Proposal 3:	During the LTM preparation phase, the candidate DU/CU should send to the source CU the SSB mapping corresponding to each NZP-CSI-RS-Resource for SP CSI-RS within Xn/F1 CSI-RS Resource Configuration IE.

Proposal 4:	During the LTM preparation phase, the source CU should send the candidate DU/CU the SSB mapping corresponding to each NZP-CSI-RS-Resource for SP CSI-RS for all LTM candidates within Xn/F1 CSI Resource Configuration.

Security aspects

At the previous meeting, the following two options for Rel-19 Set IDs allocation during preparation phase were captured as FFS:

- Option 1: Source gNB sends the Rel-19 Set ID(s) or Rel-19 set ID range assigned to the candidate gNB in the HANDOVER REQUEST message, then candidate gNB assigns Rel-19 set ID(s) to its own candidate cells and feedback via HANDOVER REQUEST ACKNOWLEDGE message.

- Option 2: Candidate gNB provides Rel-19 set ID per candidate cell in HANDOVER REQUEST ACKNOWLEDGE message, then source gNB may update the Rel-19 set ID to ensure that the Rel-19 set IDs under different candidate gNB-CU are different.

The high-level procedure of Option 1/2 is as shown in the Fig 1/2. Here, candidate #0-#2 from S-gNB, candidate #3-#5 from C-gNB#1, and candidate #6-#7 from C-gNB#2 were considered. Both options are feasible, and the required round trip of inter-CU signaling is not different. However, the main difference is whether the RRCReconfiguration needs to be re-generated and sent again via acknowledgement message in the second round trip. In option1, the generation of the candidate configuration could be completed in HO request procedure. On the other hand, in option 2, since the R19 set ID is changed from the time of the HO request procedure, it is necessary to reassign this ID in the RRCReconfiguration for the UE,thus the RRC message should be regenerated. From the above, we prefer option 1 in order to avoid unnecessary regeneration of the RRC message.

Proposal 5:	Source gNB sends the Rel-19 Set ID(s) or Rel-19 set ID range assigned to the candidate gNB in the HANDOVER REQUEST message, then candidate gNB assigns Rel-19 set ID(s) to its own candidate cells and feedback via HANDOVER REQUEST ACKNOWLEDGE message.

Fig 1. high level procedures for R19 set IDs generation: option1.

Fig 2. high level procedures for R19 set IDs generation: option2.

When a candidate DU becomes a new serving DU, it needs to know whether security update is required for each candidate cell. If it is required, serving DU should wait until serving CU completes the Path Switch Request procedure and sends the new NCC value to it in the UE Context Modification Request message before sending the Cell Switch Command. If security key update is not required, the Cell Switch Command can be sent without waiting for NCC from the CU. For this reason, in the LTM preparation phase, all candidate DUs need to know the mapping between the R19 set ID and the candidates, thus the source CU should send the mapping to all candidate CUs in Xn LTM Configuration Update message.

Proposal 6:	During LTM preparation phase, the source CU sends LTM configuration update message to all candidate CUs with a mapping between the R19 set IDs and the candidate IDs.

Conclusions and proposals

Our proposals are summarized below.

Proposal 1:	For both F1AP and XnAP, the activation and deactivation of CSI-RS transmission for L1 measurement report in LTM candidate cells are performed at the level of LTM-CSI-ResourceConfig IDs. For CSI acquisition, the activation and deactivation are performed per NZP-CSI-RS-Resource.

Proposal 2:	During LTM preparation phase, the candidate DUs/CUs should send to the source CU resource type corresponding to each NZP-CSI-RS-Resource within Xn/F1 CSI-RS Resource Configuration.

Proposal 3:	During the LTM preparation phase, the candidate DU/CU should send to the source CU the SSB mapping corresponding to each NZP-CSI-RS-Resource for SP CSI-RS within Xn/F1 CSI-RS Resource Configuration IE.

Proposal 4:	During the LTM preparation phase, the source CU should send the candidate DU/CU the SSB mapping corresponding to each NZP-CSI-RS-Resource for SP CSI-RS for all LTM candidates within Xn/F1 CSI Resource Configuration.

Proposal 5:	Source gNB sends the Rel-19 Set ID(s) or Rel-19 set ID range assigned to the candidate gNB in the HANDOVER REQUEST message, then candidate gNB assigns Rel-19 set ID(s) to its own candidate cells and feedback via HANDOVER REQUEST ACKNOWLEDGE message.

Proposal 6:	During LTM preparation phase, the source CU sends LTM configuration update message to all candidate CUs with a mapping between the R19 set IDs and the candidate IDs.

References

- R3-255011, “LS on RAN2 agreements for SP CSI-RS activation/deactivation”, RAN2

- R3-253895, “(TP for LTM BLCR for TS38.473): Update on inter-CU LTM procedure”, Huawei

- R3-253599, “(TP for LTM BLCR for TS38.473, TS38.300): Further discussion on inter-CU LTM procedure”, Huawei