3GPP TSG-RAN WG3 Meeting #129															R3-255424

Bengaluru, India, 25th ~29th Aug, 2025

Title:	(TP for LTM BLCR for TS38.300):Inter-CU LTM

Agenda Item:	13.2

Source:	Huawei

Document for:	Other

1	Introduction

This contribution continues the discussion on the open issues in the inter-CU LTM topic.

2	Discussion

2.1 	R19 Set ID assignment

The R19 set ID assignment among candidate gNBs was discussed. The following FFs are left.

FFS on the Rel-19 Set ID(s) assignment among CUs, down select from Option1 and Option2:

Option 1: Source gNB sends the Rel-19 Set ID(s) or Rel-19 set ID range assigned to the candidate gNB in the HANDOVER REQUEST message, then candidate gNB assigns Rel-19 set ID(s) to its own candidate cells and feedback via HANDOVER REQUEST ACKNOWLEDGE message.

Option 2: Candidate gNB provides Rel-19 set ID per candidate cell in HANDOVER REQUEST ACKNOWLEDGE message, then source gNB may update the Rel-19 set ID to ensure that the Rel-19 set IDs under different candidate gNB-CU are different.

FFS on the procedure to be used for source gNB to transfer Rel-19 set ID per candidate cell to the candidate gNB.

The main requirement is to also support security key change for candidate cells within a single candidate gNB as stated in the incoming LS [5].

Technically speaking, both options mentioned above are feasible. The considered main factors for down-selection usually include signalling overhead, complexity, etc.

Source gNB assigning a range of R19 set ID seems more complex in practice, in case of range reassignment, range inconsecutive.  For example, in case of a candidate gNB with 3 candidate cells rejects 1 LTM preparation request for a candidate cell, the source gNB may reassign the unoccupied set ID value to a candidate cell in another candidate gNB. Furthermore, the source gNB may also reassign the released set ID to a new candidate cell in a gNB different from the gNB serving the  just canceled candidate cell. The signaling overhead and complexity will be high in those cases. Complexities can be seen also in subsequent LTM.

On the other hand, allowing candidate gNB to assign the R19 set ID for its candidate cells may face high collision possibility among the candidate cells from different gNBs if no set ID range is reserved beforehand. The high collision possibility may force the source gNB to always update the set ID assignment from candidate gNBs which increases the implementation complexity in source gNB and the signaling overhead to inform the candidate gNBs the updated set IDs.

Based on above analysis, a better option could be source gNB assigning the set ID for each candidate cell in the HANDOVER REQUEST message. And the candidate gNB may update or change the set ID to the one of its other candidate cells received earlier based on its security key change policy. If needed, the candidate gNB replies the updated set ID to the source gNB in the HANDOVER REQUEST ACKNOWLEDGE message.

Proposal 1: The source gNB assigns the R19 set ID for each candidate cell in the HANDOVER REQEUST message.

Proposal 2: The candidate gNB may update the R19 set ID to the one of its other candidate cells received earlier based on its security key change policy and reply the updated R19 set ID to the source gNB in the HANDOVER REQUEST ACKNOWLEDGE message.

The second FFS is to use which signaling to sync up the R19 Set IDs among candidate gNBs. The straight forward option is to reuse the LTM CONFIGURATION UPDATE message to keep a harmonized design as other LTM configurations.

Proposal 3: The source gNB transfers the R19 set IDs of other candidate cells to other candidate gNBs in LTM CONFIGURATION UPDATE message.

2.2	CSI-RS resource management

There are four FFSs from last meeting:

FFS on whether gNB-DU/gNB provides the report type (periodic or semi-persistent) of the CSI-RS resources in both F1AP and XnAP.

FFS on whether the TCI State/QCL-info List needs to be included in CSI-RS COORDINATION procedure.

FFS on whether to include the SP CSI-RS and SSB mapping in the HANDOVER REQUEST ACKKNOLEDGE message and UE Context Modification Request message.

FFS on whether to add a new IE for SP CSI-RS resource for CSI acquisition in the corresponding procedure of SP CSI-RS resource for L1 RSRP measurement in F1AP and XnAP.

1st FFS: CSI-RS resources type

When the candidate gNB-DU/gNB provides the CSI-RS resource configuration, it may provide both periodic CSI-RS resources and SP CSI-RS resources. The resource type should be indicated to the source gNB in both F1AP and XnAP, There are few reasons. The first reason is to enable the source gNB-CU not to group the periodic and SP CSI-RS resources into one single LTM-CSI-ResourceConfig, as the CSI-RS resource activation MAC CE uses the CSI-RS Resource Config ID. The second reason is to produce the LTM-CSI-ResourceConfig correctly, as the resource type has been introduced into the LTM-CSI-ResourceConfig IE as shown below.

#### – LTM-CSI-ResourceConfig The IE LTM-CSI-ResourceConfig defines a group of one or more CSI resources for one or more LTM candidate configurations. LTM-CSI-ResourceConfig information element -- ASN1START -- TAG-LTM-CSI-RESOURCECONFIG-START LTM-CSI-ResourceConfig-r18 ::= SEQUENCE { ltm-CSI-ResourceConfigId-r18 LTM-CSI-ResourceConfigId-r18, ltm-SSB-ResourceSet-r18 LTM-SSB-ResourceSet-r18, ..., [[ ltm-NZP-CSI-RS-ResourceSet-r19 LTM-NZP-CSI-RS-ResourceSet-r19 OPTIONAL, -- Need R ltm-CSI-IM-ResourceSet-r19 LTM-CSI-IM-ResourceSet-r19 OPTIONAL, -- Need R resourceType-r19 ENUMERATED {periodic, semiPersistent} OPTIONAL -- Cond CSI-RS ]] }

Considering that in current RAN3 F1AP BLCR, the CSI-RS resource configuration of a candidate cell references to the ltm-NZP-CSI-RS-ResourceToAddModList in RRC, while the resource type needs to be indicated per CSI-RS resource, the simple way for RAN3 specification encoding is to introduce two separate CSI-RS resource list for periodic and semi persistent respectively. The modification to the BLCR may look like:

#### 9.3.1.x1 CSI-RS Resource Configuration This IE contains the CSI-RS resource configuration used for LTM. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Periodic CSI-RS Resource Configuration To AddModList; O; OCTET STRING; Contains the ltm-NZP-CSI-RS-ResourceToAddModList as defined in TS 38.331 [8].. Semi Persistent CSI-RS Resource Configuration To AddModList; O; OCTET STRING; Contains the ltm-NZP-CSI-RS-ResourceToAddModList as defined in TS 38.331 [8]..

Note that there is no need to transfer the resource type to the source gNB-DU explicitly over F1, as it is already included in the CSI Resource configuration which will be sent to the source gNB -DU to generate the CSI report configuration used for the very first LTM by the source cell.

Proposal 4: During LTM preparation, The candidate gNB-DU provides the CSI-RS resources separately for periodic and semi persistent to the source gNB over F1AP and XnAP.

2nd FFS: TCI State/QCL-info in CSI-RS coordination

The TCI state associated to the CSI-RS resource to be activated or deactivated is already known by the candidate gNBs. Therefore, it seems no need to indicate the TCI state in the CSI-RS coordination procedure.

Proposal 5: There is no need to indicate the TCI state in the CSI-RS coordination procedure.

3rd FFS: Transfer SP CSI-RS and SSB mapping info

From function perspective, the source gNB-DU need to determine proper SP CSI-RS resources to activate based on the SSB measurement results. In order to do so, the source gNB-DU should be able to associate the SP CSI-RS resources with the SSB index. According to current RRC spec, the source gNB may identify the associated SSB Index by the TCI-StateId (qcl-InfoPeriodicCSI-RS) IE in the NZP-CSI-RS Resource IE. But it only works for periodic CSI-RS.

NZP-CSI-RS-Resource ::= SEQUENCE { nzp-CSI-RS-ResourceId NZP-CSI-RS-ResourceId, resourceMapping CSI-RS-ResourceMapping, powerControlOffset INTEGER (-8..15), powerControlOffsetSS ENUMERATED{db-3, db0, db3, db6} OPTIONAL, -- Need R scramblingID ScramblingId, periodicityAndOffset CSI-ResourcePeriodicityAndOffset OPTIONAL, -- Cond PeriodicOrSemiPersistent qcl-InfoPeriodicCSI-RS TCI-StateId OPTIONAL, -- Cond Periodic ..., ----omitted---- } qcl-InfoPeriodicCSI-RS For a target periodic CSI-RS, contains a reference to one TCI-State in TCI-States for providing the QCL source and QCL type. For periodic CSI-RS, the source can be SSB or another periodic-CSI-RS. Refers to the TCI-State or dl-OrJoint-TCI-State which has this value for tci-StateId and is defined in tci-StatesToAddModList or in dl-OrJointTCI-StateList in the PDSCH-Config included in the BWP-Downlink corresponding to the serving cell and to the DL BWP to which the resource belongs to (see TS 38.214 [19], clause 5.2.2.3.1). In case NZP-CSI-RS-Resources is received as part of an LTM-Candidate IE, it refers to the TCI state identifier in CandidateTCI-State and is defined in ltm-DL-OrJointTCI-StateToAddModList within the LTM-Candidate IE.. Periodic; The field is optionally present, Need M, for periodic NZP-CSI-RS-Resources (as indicated in CSI-ResourceConfig). The field is absent otherwise..

In consequence, the QCL Info (TCI state Id) should be carried together with the SP CSI-RS resource from the candidate gNB-DU to the source gNB-DU.

Proposal 6: For SP CSI-RS resources, the candidate gNB indicates the QCL Info (TCI state Id) to the source gNB in the HANDOVER REQUEST ACKKNOLEDGE message.

Proposal 7: On F1AP, the QCL Info (TCI state Id) is carried in the UE CONTEXT SETUP RESPONSE message from candidate gNB-DU to candidate gNB-CU and in the UE Context Modification Request message from source gNB-CU to source gNB-DU.

Proposal 7a: For candidate cells in source gNB-DU, the QCL Info (TCI state Id) is carried in the UE CONTEXT MODIFICATION RESPONSE message

Proposal 7b: In subsequent case, the QCL info of  SP CSI-RS resources should be informed to other candidate cells.

4th FFS: new IE for SP CSI-RS resource for CSI acquisition

In the incoming LS from RAN1 in [2], it states that semi-persistence CSI-RS resource is introduced for CSI acquisition.

Similarly to the semi-persistent CSI-RS resource supported for candidate cell L1-RSRP measurement for gNB scheduled reporting, RAN1 has also agreed to introduce semi-persistent CSI-RS resource for the purpose of CSI acquisition. Agreement For candidate cell CSI acquisition - In addition to periodic CSI-RS resource, semi-persistent CSI-RS resource is supported - Support of semi-persistent CSI-RS resource is subject to separate UE capability. - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2 As captured in the agreement, RAN1 sees the spec impact in RAN2 to define the MAC CE. Also, RAN1 foresees the RAN3 spec impact to enable this functionality for inter-DU/inter-CU case.

In the LTM-Candidate IE in the latest RRC running CR, there is only one CSI-RS resource related IE named as ltm-NZP-CSI-RS-ResourceToAddModList-r19 introduced for both CSI-RS based L1 measurement and CSI acquisition.

However, according to the latest RRC running CR, the CSI-RS resource for L1 measurement is indicated by the serving gNB in the ltm-CSI-ReportConfigToAddModList-r18 IE in CSI-MeasConfig in ServingCellConfig. While the CSI-RS resource for CSI acquisition is indicated by the candidate gNB in the ltm-CSI-ReportConfigToAddModList-r18 IE in LTM-Candidate which is required by RAN1 as per the incoming LS in [1].

Furthermore, the following field description for the repetition parameter in the NZP-CSI-ResourceSet hints that the CSI-RS resource for L1 measurement and the CSI-RS resource for CSI acquisition cannot be reused, as the value and presence of the repetition parameter is different for the two cases.

repetition Indicates whether repetition is on/off. If the field is set to off or if the field is absent, the UE may not assume that the NZP-CSI-RS resources within the resource set are transmitted with the same downlink spatial domain transmission filter (see TS 38.214 [19], clauses 5.2.2.3.1 and 5.1.6.1.2). It can only be configured for CSI-RS resource sets which are associated with CSI-ReportConfig with report of L1 RSRP, L1 SINR or "no report". If NZP-CSI-RS-ResourceSet if configured in LTM-Candidate, the field can only be configured as off, if present.

In consequence, defining a separate IE for CSI-RS resource for CSI acquisition is needed in LTM preparation phase meaning that the candidate gNB/candidate gNB-DU needs to provide the reserved CSI-RS resource for CSI acquisition separately.

Proposal 8: In LTM preparation phase, the candidate gNB/candidate gNB-DU provides the CSI-RS resource for CSI acquisition in a new IE.

One more question is whether an explicit request indication from the source gNB is needed or not. We believe that an explicit request indication from the source gNB is necessary. As the CSI acquisition measurement is optional for LTM. There should be one node to make the decision on whether to configure CSI acquisition measurement for the UE. Leaving the decision to the source gNB may have a harmonized design as L1 measurement.

Proposal 9: The source gNB requests the candidate gNB/gNB-DU to provide the CSI-RS resource for CSI acquisition by an explicit indicator.

Stage 3 design for CSI-RS resource in Handover Request and CSI-RS Coordination procedure

The reference IE for the CSI-RS resource configuration IE in current XnAP BLCR is incorrect. As the ltm-CSI-ResourceConfigToAddModList is the top level CSI-RS resource IE containing CSI-RS resources of all candidate cells. The correct reference should be the ltm-NZP-CSI-RS-ResourceToAddModList IE in LTM-Candidate.

#### 9.2.3.xx8 CSI Resource Configuration This IE contains the CSI resource configuration used for LTM. IE/Group Name; Presence; Range; IE type and reference; Semantics description. CSI Resource Configuration To AddModList; O; OCTET STRING; Contains the ltm-CSI-ResourceConfigToAddModList as defined in TS 38.331 [10].. CSI Resource Configuration To Release List; O; OCTET STRING; Includes the ltm-CSI-ResourceConfigToReleaseList contained in the LTM-Config IE as defined in TS 38.331 [10]..

Proposal 10: To correct the reference IE for the CSI-RS resource configuration IE in current XnAP BLCR to ltm-NZP-CSI-RS-ResourceToAddModList IE.

According to the latest MAC running CR, the LTM-CSI-ResourceConfigId is used to activate/deactivate the CSI-RS resource in the UE. The LTM-CSI-ResourceConfigId is the identifier of the LTM-CSI-ResourceConfig which contains the LTM-NZP-CSI-RS-ResourceSet-r19. The LTM-NZP-CSI-RS-ResourceSet-r19 contains the CSI-RS resources from multiple candidate cells that cannot be referenced by the CSI-RS coordination procedure in RAN3 directly. Instead, the NZP-CSI-RS-ResourceId is the correct one to be referenced.

Proposal 11: The reference IE for the SP CSI-RS resource to be activated/be deactivated in the CSI-RS coordination procedure is the NZP-CSI-RS-ResourceId.

Furthermore, there are two different implementations in the CSI-RS coordination procedure in F1 BLCR and Xn BLCR. One is to use two separate list CSI-RS resource to be Activated List and CSI-RS resource to be deactivated List. The other one is to use a single list and add a activation/deactivation indicator for each entry.

In general, both implementations are workable. From programming perspective, two separate list seems more efficient on message decoding, as the IE name itself can represent the intended action that the receiving node should perform. The two list style is also the popular style used in RAN3 and RAN2 protocols.

Proposal 12: To use two separate list CSI-RS resource to be Activated List and CSI-RS resource to be deactivated List to represent the CSI-RS resources in the CSI-RS coordination procedure.

According to the RAN1 incoming LS [1], RAN1 further introduced dedicated CSI-IM resources for interference measurement for CSI acquisition. In consequence, the CSI-IM resources may be optionally carried over Xn and F1 if CSI-RS resources for CSI-RS acquisition is configured during LTM preparation. And for SP CSI-IM resources, the CSI-RS coordination procedure also applies as descripted in section 6.1.3.12	SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE in the MAC running CR.

Proposal 13: The candidate gNB may optionally provide dedicated CSI-IM resources if CSI-RS resources for CSI-RS acquisition is configured during LTM preparation.

Proposal 14: To reuse the CSI-RS coordination procedure to activate/deactivate the SP CSI-IM resources.

Other new IEs in LTM-Candidate

Based on the incoming LS from RAN1 in [1], the following new IEs are introduced by RAN2 into the LTM-Candidate IE:

- ltm-NZP-CSI-RS-ResourceToAddModList (already captured in RAN3 BLCRs)

- ltm-NZP-CSI-RS-ResourceSetToAddModList-r19

This is the CSI-RS ResourceSet for the CSI-RS resources produced by a candidate call.

- ltm-CSI-ReportConfigr19

This is the CSI Report Configuration for CSI acquisition produced by a candidate call.

- ltm-CSI-IM-ResourceToAddModList-r19

This is the CSI-IM Resources for interference measurement produced by a candidate cell.

- ltm-CSI-IM-ResourceSetToAddModList-r19

This is the CSI-IM ResourcesSet for interference measurement produced by a candidate cell.

The RAN1 LS results in the following IEs to be transferred over Xn and F1AP:

Proposal 15: The following new IEs need to be transferred over Xn and F1AP as per RAN1 LS.

  - ltm-NZP-CSI-RS-ResourceSetToAddModList-r19

  - ltm-CSI-ReportConfigr19

  - ltm-CSI-IM-ResourceToAddModList-r19

  - ltm-CSI-IM-ResourceSetToAddModList-r19

#### – LTM-Candidate The IE LTM-Candidate concerns a LTM candidate configuration to add or modify. LTM-Candidate information element -- ASN1START -- TAG-LTM-CANDIDATE-START LTM-Candidate-r18 ::= SEQUENCE { ltm-CandidateId-r18 LTM-CandidateId-r18, ltm-CandidatePCI-r18 PhysCellId OPTIONAL, -- Need M ltm-SSB-Config-r18 LTM-SSB-Config-r18 OPTIONAL, -- Need M ltm-CandidateConfig-r18 OCTET STRING (CONTAINING RRCReconfiguration) OPTIONAL, -- Need M ltm-ConfigComplete-r18 ENUMERATED {true} OPTIONAL, -- Need R ltm-EarlyUL-SyncConfig-r18 OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18) OPTIONAL, -- Need R ltm-EarlyUL-SyncConfigSUL-r18 OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18) OPTIONAL, -- Need R ltm-TCI-Info-r18 LTM-TCI-Info-r18 OPTIONAL, -- Need M ltm-NoResetID-r18 INTEGER (1..maxNrofLTM-Configs-plus1-r18) OPTIONAL, -- Need M ltm-UE-MeasuredTA-ID-r18 INTEGER (1..maxNrofLTM-Configs-plus1-r18) OPTIONAL, -- Need M ..., [[ ltm-NoSecurityChangeID-r19 LTM-NoSecurityChangeId-r19 OPTIONAL, -- Need M ltm-ExecutionCondition-r19 SetupRelease {LTM-ExecutionConditionList-r19} OPTIONAL, -- Need M ltm-NZP-CSI-RS-ResourceToAddModList-r19 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource OPTIONAL, -- Need N ltm-NZP-CSI-RS-ResourceToReleaseList-r19 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-ResourceId OPTIONAL, -- Need N ltm-NZP-CSI-RS-ResourceSetToAddModList-r19 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet OPTIONAL, -- Need N ltm-NZP-CSI-RS-ResourceSetToReleaseList-r19 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId OPTIONAL, -- Need N ltm-CSI-ReportConfigr19 LTM-CSI-ReportConfig-r18 OPTIONAL, -- Need N ltm-CSI-IM-ResourceToAddModList-r19 SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-Resource OPTIONAL, -- Need N ltm-CSI-IM-ResourceToReleaseList-r19 SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-ResourceId OPTIONAL -- Need N ltm-CSI-IM-ResourceSetToAddModList-r19 SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSet OPTIONAL, -- Need N ltm-CSI-IM-ResourceSetToReleaseList-r19 SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSetId OPTIONAL -- Need N ]] }

2.3	Candidate cell SP CSI-RS deactivation after switch

In the incoming RAN2 LS  in [3], RAN2 made the following agreements.

- UE deactivates SP CSI-RS resource of candidate cells (other than the target cell) after cell switch. FFS on the target cell.

- Instead of candidate cell id and SP CSI-RS resource set id, LTM-CSI-ResourceConfigId is included into SP CSI-RS activation/deactivation MAC CE.

The first agreement may have RAN3 signaling impact. When the UE deactivates the SP CSI-RS resource of candidate cells after cell switch, the status of the SP CSI-RS resource should be synchronized to the corresponding candidate gNBs.

There are two options. One approach is to task the source gNB or source gNB-DU to invoke the CSI-RS coordination procedure towards candidate gNBs with the SP CSI-RS resources that need to be deactivated. The other one is to allow the new serving gNB to deactivate the SP CSI-RS resources in the candidate gNBs by LTM CONFIGURATION UPDATE message. The latter option seems have less spec impact.

Proposal 16: After cell switch, the new serving gNB deactivates the SP CSI-RS resources in the candidate gNBs by LTM CONFIGURATION UPDATE message.

2.4	UE association setup

The following agreements were made at previous RAN3 meetings.

RAN3 #125bis Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM, when and how to establish the UE association is FFS. RAN3 #126 To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch. RAN3 #127bis For subsequent LTM, the candidate gNB uses the old target UE XnAP ID (the target UE AP ID it previously allocated between it and the old serving gNB) to identify the UE context when receiving the LTM Configuration Update message from the new serving gNB. FFS on how to deliver the old target UE XnAP ID(s) to the new serving gNB.

RAN3 agreed that, in the LTM Configuration Update message, the old target UE XnAP ID that the candidate gNB allocated for the first source gNB should be included in the LTM Configuration Update message. As for how the serving gNB knows the old target UE XnAP ID, the previous source gNB can send the old target UE XnAP IDs to the new serving gNB via Cell Switch Notification message.

Proposal 17: In Cell Switch Notification message, the source gNB sends the previously obtained old target UE XnAP IDs to the new serving gNB.

Moreover, there are two possible understandings on how to interpret the “old target UE XnAP ID”. We think that it’s better to further clarify the exact meaning of this critical IE.

One understanding is that the old target UE XnAP ID is the target UE XnAP ID allocated by the candidate gNB at the initial LTM preparation which is used to identify the UE association between the initial source gNB and the candidate gNB. The other understanding is that it is the target UE XnAP ID allocated by the candidate gNB after last LTM cell switch, e.g., when receiving the LTM Configuration Update Request message from the new source gNB.

We think that the first understanding may be problematic if the initial source gNB is not a candidate gNB anymore. As the UE association between the candidate gNB and the initial source gNB may be released by the candidate gNB if it detects that the initial source gNB is released. Therefore, the “old target UE XnAP ID” should refer to the target UE XnAP ID allocated by the candidate gNB after last LTM cell switch. Below figure illustrates the issue in detail.

Proposal 18: To clarify that the “old target UE XnAP ID” is the target UE XnAP ID allocated by the candidate gNB after last LTM cell switch e.g., when receiving the LTM Configuration Update Request message from the new source gNB.

2.5	Early data forwarding data discard

RAN3 agreed to support early data forwarding for subsequent inter-CU LTM. There is one proposal to discuss how to deal with the data forwarded by early data forwarding procedure.

In legacy handover, the data forwarded to candidate gNB in early data forwarding will be released after handover procedure is completed, e.g. by reception of HANDCOVER CANCEL message. But for subsequent LTM, the data forwarded to the candidate gNB except target gNB in early data forwarding will be useless after the precedent LTM procedure is completed. And in the subsequent LTM procedure, the new source gNB may perform early data forwarding to the candidate too. For the candidate gNB, it is unclear whether and when it should flush the data received in the precedent LTM procedure. If the data received in precedent LTM procedure is not flushed, they will bring additional buffer requirement to the candidate gNB. Proposal 5: RAN3 to discuss whether and when a candidate gNB flush UE data that is received in early data forwarding after a LTM procedure.

The candidate gNB should discard the data after it determines that the UE is switched to another candidate gNB. One straightforward method is to use the LTM CONFIGURATION UPDATE message to let the candidate gNB know that the serving gNB is changed. After receiving the LTM CONFIGURATION UPDATE message, the candidate gNB determines that the UE has switched to another gNB, and it may discard the buffered data received from the original source gNB.

Proposal 19: After receiving the LTM CONFIGURATION UPDATE message, the candidate gNB discards the buffered data received from the original source gNB.

2.6	SA3 incoming LS

There is an incoming LS from SA3 in [4]. In which SA3 informs RAN3 the agreed CR for TS 33.501 for security procedure for inter-CU LTM.

The content in the attached SA3 stage 2 CR which has RAN3 impact is quoted as below.

#### 6.X.Y Security mechanism and procedures for L1/L2 Triggered Mobility ##### 6.X.Y.1 When DC is not configured […] If the target gNB receives UE's 5G security capabilities from the AMF (as detailed in clause 6.7.3.1 of the present document) and/or UE's UP security policy from the SMF (as detailed in clause 6.6.1 of the present document) in the Path-Switch Acknowledge message, then the target gNB may initiate an intra-gNB-CU handover to refresh KNG-RAN* or activate or de-activate the UP integrity/confidentiality as per the received policy from SMF or to indicate the selected algorithms appropriately. Further, the target gNB either releases or updates the LTM configuration (UE's 5G security capabilities and/or UE's UP security policy) of the candidate cells by LTM configuration Update message. If necessary, the serving gNB then updates the UE with the configuration aligned with LTM configuration in the network.

For inter-CU LTM, in case of security context mismatch, the target gNB directly releases the LTM resources seems overkilled. So that, SA3 also agrees that the target gNB can update the LTM configuration of the candidate gNBs.

To follow the same principle as in legacy Xn handover, if the candidate gNB receives the UE's 5G security capabilities from the new serving gNB in LTM CONFIGURATION UPDATE message, the candidate gNB shall select the algorithm with highest priority from these 5G security capabilities according to the locally configured prioritized list of algorithms, and update the UE's UP security policy with the received UE's UP security policy. If the algorithms selected by the candidate gNB and the UP security policies are different from the selected algorithms during the very first LTM preparation and the received UP security policy, the candidate gNB shall indicate the new selected algorithm and UP security activation status to the new serving gNB in the LTM CONFIGURATION UPDATE ACK message.

However, if the user plane security policy indicates "Required" or "Not needed", the candidate gNB shall not overrule the UP security policy provided by the SMF. If the candidate gNB cannot activate UP confidentiality and/or UP integrity protection when the received UP security policy is "Required", the candidate gNB shall reject the update of UP resources for the PDU Session by the LTM CONFIGURATION UPDATE FAILURE message.

For example, it is assumed that candidate gNB cannot activate UP security. In the preparation phase, the source gNB sends UP security policy as “preferred” to candidate gNB. The UP security activation status is “not activated”. During subsequent LTM, the UP security policy as “required” is provided by the SMF and transferred to candidate gNB by the  target gNB using LTM CONFIGURATION UPDATE message. However, the candidate gNB cannot activate UP confidentiality and/or UP integrity protection.  A normal processing is to reply the target gNB a LTM CONFIGURATION UPDATE FAILURE message with a proper cause value.

Proposal 20: if the candidate gNB cannot activate/deactivate UP security as required in the UP security policy during subsequent LTM, the candidate gNB reply the target gNB the LTM CONFIGURATION UPDATE FAILURE message with a cause value e.g., UP security policy applied failure.

3.	Conclusion

Based on the discussion in this contribution, it is proposed:

R19 Set ID assignment

Proposal 1: The source gNB assigns the R19 set ID for each candidate cell in the HANDOVER REQEUST message.

Proposal 2: The candidate gNB may update the R19 set ID to the one of its other candidate cells received earlier based on its security key change policy and reply the updated R19 set ID to the source gNB in the HANDOVER REQUEST ACKNOWLEDGE message.

Proposal 3: The source gNB transfers the R19 set IDs of other candidate cells to other candidate gNBs in LTM CONFIGURATION UPDATE message.

CSI-RS resource management

Proposal 4: During LTM preparation, The candidate gNB-DU provides the CSI-RS resources separately for periodic and semi persistent to the source gNB over F1AP and XnAP.

Proposal 5: There is no need to indicate the TCI state in the CSI-RS coordination procedure.

Proposal 6: For SP CSI-RS resources, the candidate gNB indicates the QCL Info (TCI state Id) to the source gNB in the HANDOVER REQUEST ACKKNOLEDGE message.

Proposal 7: On F1AP, the QCL Info (TCI state Id) is carried in the UE CONTEXT SETUP RESPONSE message from candidate gNB-DU to candidate gNB-CU and in the UE Context Modification Request message from source gNB-CU to source gNB-DU.

Proposal 7a: For candidate cells in source gNB-DU, the QCL Info (TCI state Id) is carried in the UE CONTEXT MODIFICATION RESPONSE message.

Proposal 7b: In subsequent case, the QCL info of  SP CSI-RS resources should be informed to other candidate cells.

Proposal 8: In LTM preparation phase, the candidate gNB/candidate gNB-DU provides the CSI-RS resource for CSI acquisition in a new IE.

Proposal 9: The source gNB requests the candidate gNB/gNB-DU to provide the CSI-RS resource for CSI acquisition by an explicit indicator.

Proposal 10: To correct the reference IE for the CSI-RS resource configuration IE in current XnAP BLCR to ltm-NZP-CSI-RS-ResourceToAddModList IE.

Proposal 11: The reference IE for the SP CSI-RS resource to be activated/be deactivated in the CSI-RS coordination procedure is the NZP-CSI-RS-ResourceId.

Proposal 12: To use two separate list CSI-RS resource to be Activated List and CSI-RS resource to be deactivated List to represent the CSI-RS resources in the CSI-RS coordination procedure.

Proposal 13: The candidate gNB may optionally provide dedicated CSI-IM resources if CSI-RS resources for CSI-RS acquisition is configured during LTM preparation.

Proposal 14: To reuse the CSI-RS coordination procedure to activate/deactivate the SP CSI-IM resources.

Proposal 15: The following new IEs need to be transferred over Xn and F1AP as per RAN1 LS.

  - ltm-NZP-CSI-RS-ResourceSetToAddModList-r19

  - ltm-CSI-ReportConfigr19

  - ltm-CSI-IM-ResourceToAddModList-r19

  - ltm-CSI-IM-ResourceSetToAddModList-r19

Candidate cell SP CSI-RS deactivation after switch

Proposal 16: After cell switch, the new serving gNB deactivates the SP CSI-RS resources in the candidate gNBs by LTM CONFIGURATION UPDATE message.

UE association setup

Proposal 17: In Cell Switch Notification message, the source gNB sends the previously obtained old target UE XnAP IDs to the new serving gNB.

Proposal 18: To clarify that the “old target UE XnAP ID” is the target UE XnAP ID allocated by the candidate gNB after last LTM cell switch e.g., when receiving the LTM Configuration Update Request message from the new source gNB.

Early data forwarding data discard

Proposal 19: After receiving the LTM CONFIGURATION UPDATE message, the candidate gNB discards the buffered data received from the original source gNB.

Security

Proposal 20: if the candidate gNB cannot activate/deactivate UP security as required in the UP security policy during subsequent LTM, the candidate gNB reply the target gNB the LTM CONFIGURATION UPDATE FAILURE message with a cause value e.g., UP security policy applied failure.

The stage 2 TP and stage 3 TP are provided in [6] and the annex.

4 	Reference

[1] R1-2503242 DRAFT LS on Rel-19 higher layers parameters list Post RAN1#121

[2] R1-2503079 LS on the support of semi-persistent CSI-RS resource for LTM CSI acquisition for candidate cells

[3] R2-2504727 LS on RAN2 agreements for SP CSI-RS activation/deactivation

[4] S3-252398 Reply LS on security handling for inter-CU LTM in non-DC cases

[5] R2-2503012 LS on RAN2 agreements for security key handling in inter-CU LTM

[6] R3-255425 TP for F1AP BLCR

Text Proposal for LTM BLCR for TS38.300

<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

The procedure for LTM is as follows:

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB containing measurements of neighbouring cells.

2.	The source gNB decides to configure LTM.

3.	The source gNB requests LTM for one or more candidate cells belonging to the source gNB and/or one or more candidate gNB(s). For inter-gNB LTM, the source gNB initiates a HANDOVER REQUEST message per candidate cell containing one candidate cell ID and may contain the CSI resource configuration for subsequent LTM. For both intra and inter-gNB LTM, the source gNB may request the candidate cell(s)/gNB(s) to provide the CSI resource configuration.

4.	Admission Control may be performed by the candidate cells(s)/gNB(s).

5.	The candidate prepares and provides the LTM configuration(s) to the source gNB. For inter-gNB LTM, the candidate gNB(s) respond(s) with HANDOVER REQUEST ACKNOWLEDGE message to the source gNB including the generated RRC configurations for the accepted candidate cell. For both intra and inter-gNB LTM, the candidate may also include additional information related to the CSI-RS resource configuration and early sync information upon request.

6.	The source gNB sends an LTM CONFIGURATION UPDATE message to the candidate gNB(s) to update the LTM configurations of candidate cell(s). The source gNB may include the common CSI resource configuration and the LTM configuration ID mapping list.

7.	The candidate gNB(s) sends the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the source gNB. The candidate gNB(s) may also provide the CSI report configuration.

NOTE :	Step 6 may also be triggered after step 14, or after step 17 by implementation for subsequent LTM.

8.	The source gNB sends an RRCReconfiguration message to the UE.

9.	The UE stores the LTM candidate configurations and sends an RRCReconfigurationComplete message to the source gNB.

NOTE :	The source gNB may initiate CSI-RS Coordination procedure to activate or deactivate CSI-RS resource(s) of some candidate cells(s).

9a	If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message to the candidate gNB(s).

10/11. Early DL and UL synchronization to some LTM candidate cell(s) may be performed. The source gNB may activate or deactivate the TCI states of the candidate LTM cells.  Depending on NW configuration, the UE may perform early UL synchronization with LTM candidate cell(s), by using UE-based TA measurement, if configured, and/or by transmitting a preamble towards the candidate cell, as triggered by the source gNB. With a NW triggered UL synchronization, a PDCCH order is received from the source cell to trigger CFRA to a candidate cell, the UE performs early TA acquisition by sending preamble towards the indicated candidate cell. In order to minimize the data interruption on the source cell due to CFRA towards the indicated candidate cell(s), the UE does not receive random access response from the network for the purpose of TA value acquisition. The candidate gNB(s) sends the TA INFORMATION TRANSFER message to the source gNB instead.

12.	 The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source gNB, if configured. L1 measurement should be performed as long as RRC reconfiguration (step 8) is applicable.

13.	The source gNB determines to initiate LTM. L3 measurement can also be used to determine this step.

14.	The source gNB decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration, a beam indicated with a TCI state or beams indicated with DL and UL TCI states, and a TA command for the target cell. In case of a security context change, the LTM cell switch command MAC CE also contains the NCC value. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID. In case of security context change, the UE generates and applies the new security keys based on the received NCC value.

NOTE :	Up to implementation, data forwarding and SN Status Transfer may be initiated once the source gNB triggers the inter-gNB LTM cell switch for the UE in Step 14.

15.	The source gNB sends the CELL SWITCH NOTIFICATION message to the target gNB to indicate the initiation of Cell Switch command to the UE.

16.	The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell as specified in clause 5.18.35 of TS 38.321[6].

17/18. The target gNB sends the HANDOVER SUCCESS message to the source gNB to inform that the UE has successfully accessed the target cell. In return, the source gNB sends the SN STATUS TRANSFER message following the principles described in step 7 of Intra-AMF/UPF Handover in clause 9.2.3.2.1.

NOTE :	Late data forwarding may be initiated as soon as the source gNB receives the HANDOVER SUCCESS message.

19.	The UE completes the LTM cell switch procedure by sending RRCReconfigurationComplete message to target cell. If the UE has performed a RA procedure in step 16 the UE considers that LTM cell switch execution is successfully completed when the random access procedure is successfully completed. For RACH-less LTM, the UE considers that LTM cell switch execution is successfully completed when the UE determines that the network has successfully received its first UL data

NOTE :	Steps 17/18 and 19 do not have to occur one after the other. The target gNB may send the HANDOVER SUCCESS message to the source gNB after receiving the RRCReconfigurationComplete message.

20. The new source gNB (i.e., the target gNB) sends the LTM CONFIGURATION UPDATE message to the candidate gNBs. This message includes the new security key(s) to be used with the UE.

21. The candidate gNB(s) responds to the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the new source gNB. If any, the candidate gNB(s) deactivate the activated SP CSI-RS resources for the UE.

22. The new source gNB may send the UE CONTEXT RELEASE message to inform the old source gNB to release radio and C-plane related resources associated to the UE context if no LTM candidate cell(s) exist in the old source gNB. Any ongoing data forwarding may continue.

The steps 10-22 can be performed multiple times for subsequent LTM cell switch executions using the LTM candidate configuration(s) provided in step 8.

The procedure over the air interface described in Figure 9.2.3.5.2-1 is applicable to both intra-gNB-DU LTM , inter-gNB-DU LTM and inter-gNB-CU LTM. The overall LTM procedures over F1-C interface are captured in TS 38.401[4].

<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>