3GPP TSG-RAN WG3 Meeting #129															R3-255428

Bengaluru, India, 25th ~29th Aug, 2025

Title:	Discussion on SONMDT for Intra-NTN mobility

Agenda Item:	10.3.1

Source:	Huawei, CMCC, Jio Platforms, China Unicom

Document for:	Discussion

1	Introduction

The following progress was made at last meeting for the geographical area in NTN:

To introduce a geographical area scope for NTN MDT including either reference location/radius or a polygon-based area for NG and Xn.

FFS on whether to add the geographical area scope inside or outside the existing area scope IE.

FFS on whether the geographical area scope applies to both logged and immediate MDT or only logged MDT.

This document continues the discussion by focusing on above two open issues.

2	Discussion

There are two FFSs from last meeting. The first one is whether to add the geographical area scope inside or outside the existing area scope.

Adding the geographical area scope inside the existing area scope means that only 1 area scope will be configured to the UE, and the UE only needs to evaluate one condition to trigger MDT measurements logging, i.e., whether its current location is inside the configured geographical area scope or not. While adding the geographical area scope outside the existing area scope will force the UE to evaluate two conditions to trigger MDT measurement logging which seems more complicated for UE implementation. From functional perspective, there seems no need and no any benefits to support the area scope combination for MDT in NTN.

On the other hand, adding the geographical area scope outside the existing area scope will cause misleading. One may misinterpret the mandatory legacy area scope as the one used in TN and think that the specification supports to configure the UE to continue logging in NTN to TN mobility case. While supporting MDT when the UE moves between NTN and TN is never discussed and should be out of the scope of the Rel-19 WI.

Based on above analysis, we propose:

Proposal 1: RAN3 to add the geographical area scope inside the existing area scope IE.

The second FFS is whether the geographical area scope applies to both logged and immediate MDT or only logged MDT.

One comment was raised at last meeting that whether the network can obtain the accurate UE location information for active UEs to evaluate the immediate MDT measurement collection condition defined by the geographical area.

The following specification texts are quoted here for reference.

TS 38.300 #### 16.14.2.2 Timing Advance and Frequency Pre-compensation For the serving cell, the network broadcast valid ephemeris information and Common TA parameters. The UE shall have valid GNSS position as well as ephemeris and Common TA before connecting to an NTN cell. To achieve synchronisation, before and during connection to an NTN cell, the UE shall compute the RTT between UE and the RP based on the GNSS position, the ephemeris, and the Common TA parameters (see clause 4.2 in TS 38.213 [38]), and autonomously pre-compensate the TTA for the RTT between the UE and the RP as illustrated in Figure 16.14.2.1-1 (see clause 4.3 of TS 38.211 [52]).

The second reference is the SID of the new study item in [1], Study on GNSS resilient NR-NTN operation.

RP-251863 # 3 Justification […] However, GNSS information in UE may be - temporarily unavailable - or available but degraded, leading to a reduction in GNSS position accuracy. These occurrences are becoming more and more common and result in a 3GPP NTN service that is not very robust. […] # 4 Objective ### 4.1 Objective of Core part WI The study will be carried assuming that - The GNSS information in UE with GNSS capability may be 1/ temporarily unavailable or 2/ available but with GNSS position accuracy degradation or 3/ available but with increased GNSS measurement period for power saving purpose - This study will initially focus on 1/ (where GNSS information is temporarily unavailable) - Note: This study does not include any enhancement on positioning for the above scenario Study GNSS resilient NR-NTN operation [RAN1, RAN4] - Assess impact on initial access and connected mode procedures for NR-NTN, including potential implications if any on RAN4 RRM specifications - Aim to minimize physical layer procedures impact - Avoid physical layer channel/signal changes - Backward compatible issues with legacy NTN capable UEs will be assessed and should be prevented - Applicable to NGSO and GSO constellations, NTN operating in below and in above 10 GHz frequency bands, transparent and regenerative satellites - Check in RAN#112 (June, 2026) to decide whether to proceed with normative work, extend the study item, or postpone normative work to Release 21

It can be learned that in Rel-19 and earlier releases, the UE shall have valid GNSS position before connecting to NTN. In Rel-20, RAN1 and RAN4 will study the impact on initial access and connected mode procedures for NR-NTN for the case of GNSS position temporarily unavailable.

Considering that the standardization work of Rel-19 SONMDT WI is taking the Rel-18 NTN function as a basis, we could conclude that an available GNSS position is a prerequisite for active UEs. Furthermore, as per TS38.331, obtaining UE’s GNSS location has been supported by includeCommonLocationInfo IE in RRC protocol since Rel-16.

obtainCommonLocation Requests the UE to attempt to have detailed location information available using GNSS. NR configures the field if includeCommonLocationInfo is configured for one or more measurements.

Proposal 2: RAN3 to agree that the geographical area scope applies to both logged MDT and immediate MDT in NTN.

At the end, a LS needs to be sent to RAN2 and SA5 to inform the outcome to them.

Proposal 3: RAN3 to send a LS to RAN2 and SA5 to inform the outcome.

3	Conclusion

This document proposes:

Proposal 1: RAN3 to add the geographical area scope inside the existing area scope IE.

Proposal 2: RAN3 to agree that the geographical area scope applies to both logged MDT and immediate MDT in NTN.

Proposal 3: RAN3 to send a LS to RAN2 and SA5 to inform the outcome.

The corresponding TPs are provided in [2] ~ [4].

The draft LS is provided in the annex.

4 	Reference

[1] RP-251863	Study on GNSS resilient NR-NTN operation

[2] R3-255429	(TP for MDT BLCR for TS38.413): Logged MDT enhancement for NTN

[3] R3-255430	(TP for MDT BLCR for TS38.423): Logged MDT enhancement for NTN

[3] R3-255431	(TP for MDT BLCR for TS37.320): Logged MDT enhancement for NTN

Annex – DRAFT LS OUT

3GPP TSG-RAN WG3 Meeting #128	R3-25xxxx

Malta, MT, 19 – 23 May, 2025

Title:	[DRAFT] LS on SONMDT for NTN

Response to:

Release:	Rel-19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	Huawei [will be RAN3]

To:	RAN2, SA5

Cc:

Contact person: 	Zhang Hongzhuo

zhanghongzhuo@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

- Overall description

RAN3 finishes the standardization work for MDT in NTN and agrees to introduce a geographical area as the new area scope for both immediate MDT and logged MDT in NTN.

The agreed TPs are attached for reference.

# 2	Actions

To RAN2 and SA5

ACTION: 	RAN3 kindly ask RAN2 and SA5 to take the above outcome into account and update their specification accordingly.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#129	25 – 29 August  2025 	Bangalore, India