3GPP TSG-RAN WG3 Meeting #129	R3-255432

Bengaluru, India, 25th – 29th August, 2025

Title: 	Ping-Pong Mitigation in CU-DU Coordination for LTM Mobility

Source: 	Jio Platforms

Agenda item:	10.2

Document Type:	Discussion and agreement

# 1. Introduction

During RAN3#128 discussions under Agenda Item 10.2, multiple down-selected options were identified to enhance mobility robustness optimization (MRO) in the context of LTM scenarios involving UHI and ping-pong handover phenomena as captured in the meeting minutes:

UHI and ping-pong issue:

CU informs relevant information to the target DU via CU-DU Cell Switch Notification.

Down selection of following solutions:

1a: CU provide full UHI list with L1/L3 type to the DU (maximum item number is 16).

1b: CU provide full UHI list (except PScell) with L1/L3 type to the DU (maximum item number is 16).

2a: CU provide last consecutive L1 entries to the DU.

2b: CU provide FFS number of last consecutive L1 entries to the DU.

No stage2, work in stage3 with 1a.

If the above 1a information is included in F1AP message from CU to DU, the DU may consider the potential mobility issue detected including ping-pong.

FFS on the extra indication for ping-pong case

These issues are particularly critical in 5G NG-RAN where CU-DU disaggregation may introduce visibility gaps across RRC layers, thus impacting handover efficiency and optimization. This contribution provides an evaluation of the down-selected solutions captured by the chair and offers recommendations on the most viable option(s) to proceed with.

In addition to CU–DU information exchange, UE-assisted distributed MRO suggests that selective, on-demand UE feedback on recent cell revisit patterns and mobility stability can further improve DU-local detection of ping-pong phenomena. This contribution therefore also notes potential synergies between CU-supplied UHI and opportunistic UE-assisted reporting, to be addressed as complementary enhancements.

# 2. Background and Problem Description

Early in LTE standardization, recurrent ping-pong handovers—frequent toggling of a UE between adjacent cells—highlighted the need to retain and forward historical mobility data across nodes. This challenge emerged with the shift to a flat architecture, where the absence of a central controller (RNC) limited visibility into the UE's mobility path. To address this, UE History Information (UHI) was introduced to transport context on recent cell visits, assisting target cells in anticipating and mitigating inefficient mobility outcomes.

While the Self-Organizing Network (SON) framework recognized the relevance of ping-pong detection from the outset, its general intra-system treatment was not explicitly captured in Stage 2 specifications. It was implicitly assumed that the node receiving UHI would interpret and act upon it. However, only inter-RAT ping-pong cases were formally addressed, due to the higher cost and complexity associated with inter-system handovers. For such scenarios, mechanisms like the Handover Report were introduced to inform the original source node when a UE returned to the same or a sibling cell post inter-RAT mobility.

With the expansion of dual connectivity in 5G, including SCG mobility scenarios, the need for more holistic historical mobility tracking resurfaced. The Master Node (MN), while typically responsible for aggregating and relaying UE history to the next MN, often lacks insight into intra-SN mobility. To overcome this, the Secondary Node (SN) plays a supporting role by collecting and reporting intra-SN movements back to the MN, ensuring that a complete and consolidated history is relayed to subsequent MNs. This unified record is especially valuable for avoiding redundant handovers and better profiling UE movement tendencies.

In the context of distributed gNB deployments, where the Central Unit (CU) handles mobility decisions based on measurement reports and policies, a new challenge emerges: the Distributed Unit (DU), lacking L1/L3 context, is often unable to autonomously detect ping-pong behavior. This limitation reduces the potential for local optimization and increases reliance on higher-layer corrective actions. In distributed gNB architecture, the CU typically makes mobility decisions based on UE measurement reports and network policies. However, when handover decisions lead to sub-optimal outcomes—such as repeated returns to the source cell or frequent toggling between neighboring cells (ping-pong)—these are often symptomatic of missing L1/L3 context at the DU level, which is critical for local optimization.

To address this, ongoing discussions (e.g., RAN3#128) have introduced a solution space whereby the CU shares enriched UHI—particularly a list of prior cell entries and their L1/L3 classification—with the DU via the CU-DU Cell Switch Notification. This enables the DU to gain a minimal yet sufficient history view to identify local ping-pong events and proactively suppress them. By empowering the DU with selective UE mobility context, the architecture can support more adaptive, responsive, and localized mobility management, reducing unnecessary signaling and improving user experience.

The proposed solution space focuses on transferring UHI-relevant information—specifically a list of previous cell entries and types (L1/L3)—from CU to DU using the CU-DU Cell Switch Notification. The goal is to allow the DU to locally identify and potentially mitigate ping-pong mobility behavior without relying on further central decisions.

By empowering the DU with selective UE mobility context, the architecture can support more adaptive, responsive, and localized mobility management, reducing unnecessary signaling and improving user experience.

Complementary to CU–DU coordination, the UE itself can act as an additional source of fine-grained mobility history. Since the UE has immediate awareness of recent serving cell changes, dwell times, and revisit counts, opportunistic UE-assisted reporting—triggered only under specific DU-configured conditions—can close short-term visibility gaps in ping-pong detection. This hybrid approach, combining CU-provided UHI and targeted UE feedback, may enable the DU to apply more precise suppression logic without excessive signaling overhead.

# 3. Discussion

The following options were captured in RAN3#128 and are briefly analyzed below:

- Option 1a: CU provides full UHI list with L1/L3 type to the DU (max 16 items)

The detection and mitigation of ping-pong mobility remains a persistent issue in distributed gNB architectures, where the separation of CU and DU functions limits the DU’s visibility into the UE’s prior mobility context. Drawing from earlier LTE and 5G discussions, it is evident that effective handover decisions—particularly in preventing rapid toggling between adjacent cells—depend significantly on the availability of timely and relevant UE History Information (UHI). Option 1a, which proposes transferring the full UHI list (up to 16 items) with L1/L3 classification from CU to DU via the Cell Switch Notification, represents a promising approach for enabling localized ping-pong mitigation.

The inclusion of detailed history allows the DU to assess short-term UE mobility trends and apply local logic to avoid repeated handovers. Especially in high-density deployments, where small cells operate under tight latency and load constraints, such visibility can empower the DU to suppress unnecessary transitions without escalating decisions back to the CU. Moreover, leveraging the existing UHI format ensures minimal protocol complexity, while extending it with L1/L3 tags allows the DU to apply differentiated strategies based on frequency layer or RAT.

Observation 1: The DU's ability to detect ping-pong behavior is significantly enhanced when it receives UE History Information (UHI) with L1/L3 type, allowing it to distinguish between intra-frequency and inter-frequency transitions. However, the relevance of each UHI entry to DU decision-making may vary depending on deployment (e.g., small cell vs macro), frequency layer, and DU capabilities.

Nevertheless, the approach raises considerations regarding signaling efficiency and practical relevance. Forwarding the full 16-entry list in every handover context may result in non-negligible overhead, particularly for UEs with fast mobility. Additionally, the presence of entries related to PScell changes may not always yield actionable information for the DU, especially when cross-frequency or inter-site transitions are involved. This highlights the need for policy-based filtering at the CU, ensuring that only entries relevant for DU-level evaluation are propagated. For instance, macro-layer entries or inter-frequency handovers could be selectively excluded where not actionable by the DU.

Observation 2: Transferring the full UHI list with up to 16 entries may introduce unnecessary signaling overhead, particularly in high-mobility scenarios or dense deployments, without proportionally increasing detection accuracy. Empirical thresholds for how many recent entries actually improve DU logic remain undefined.

An observation worth noting is that the value of UHI for DU logic may saturate beyond a certain number of recent entries; thus, a configurable cap (e.g., 6–8 items) based on deployment type could be considered. A further proposal is to explore delta-based signaling models where only changes to the UHI list are transmitted, rather than the full list, to minimize repetitive data transfer. This could be complemented by defining DU-specific heuristics or thresholds for ping-pong detection based on revisit frequency, cell IDs, or time-to-return metrics.

Observation 3: PScell-related entries may not always carry actionable value for DU-level decisions, particularly when inter-frequency or cross-RAT mobility is out of DU control. Their inclusion should be made configurable.

Proposal 1: Define an optional maximum effective UHI list length for DU usage, such as 6–8 recent entries, subject to operator configuration, to allow for vendors to implement DU-side ping-pong logic without incurring unnecessary processing or memory load.

Proposal 2: Introduce CU-side configurable filtering criteria for UHI entries included in the CU-DU Cell Switch Notification—e.g., to exclude entries older than a given time threshold, entries corresponding to inter-frequency or inter-RAT mobility and PScell changes, if the DU is not expected to act on macro-layer transitions.

Proposal 3: Evaluate a delta-based UHI update mechanism where only incremental changes in the UE’s mobility history are sent from CU to DU (rather than re-sending the full list every time) to reduce redundant signaling while maintaining timely DU awareness.

Overall, Option 1a provides a viable enhancement to distributed mobility handling, particularly in enabling DU-local intelligence without fundamental architectural change. Standardization of content granularity, filtering criteria, and update policy remains essential to balance detection effectiveness with signaling efficiency.

- Option 1b: CU provides full UHI list (excluding PScell) with L1/L3 type to the DU (max 16 items)

Option 1b proposes that the CU provides the DU with a filtered UHI list (excluding the PScell) annotated with L1/L3 type, delivered via the CU-DU Cell Switch Notification. This enables the DU to locally infer recent UE movements and autonomously detect potential ping-pong behavior, thus facilitating more proactive suppression measures without round-trip signaling to the CU. The approach is bounded by a maximum of 16 historical entries to maintain signaling efficiency and minimize overhead.

Observation 4: Excluding the PScell from the UHI list may simplify context processing and avoid duplicate data, particularly where the PScell is already known by the DU through configuration or implicit assignment. However, in scenarios where the PScell was also implicated in recent ping-pong behavior (e.g., due to reconfigurations or serving cell switches), its absence may lead to missed detection opportunities.

This solution enables more distributed and responsive mobility handling, reducing reliance on centralized decision-making while preserving NG-RAN architectural principles. Nevertheless, careful consideration must be given to the completeness and freshness of UHI data, the interpretation capabilities of the DU, and the definition of thresholds or suppression timers at DU level to ensure consistent behavior across vendors and deployments.

- Option 2a: CU provides last consecutive L1 entries to the DU

As stated previously in the current 5G RAN architecture, the Central Unit (CU) orchestrates mobility decisions based on UE measurement reports and handover policies. Nevertheless, it is the Distributed Unit (DU) that directly interacts with physical layer metrics and L1 mobility triggers. Option 2a is particularly appealing due to its simplicity and efficiency. By focusing exclusively on recent L1-level events, it allows the DU to perform lightweight pattern recognition (e.g., frequent reappearance of the same cells), which is often sufficient to detect basic ping-pong behavior. Furthermore, the list size remains small, thereby minimizing signaling overhead over the CU-DU interface and preserving real-time responsiveness. Such design supports scalable implementation, especially for dense deployments where bandwidth conservation and low-latency operations are essential.

However, the approach inherently trades off contextual richness. The absence of L3-type mobility classification (e.g., whether transitions were intra-frequency, inter-frequency, or inter-RAT) may limit the DU’s ability to interpret whether a repeated handover is problematic or expected (e.g., due to frequency-based reselection logic). Additionally, the short memory horizon may not capture deeper patterns such as oscillations across a broader timescale or cycles involving more than two cells.

Observation 5: The inability of DU to recognize the L3-type context of handover limits the potential for granular ping-pong classification, particularly when behavior arises from frequency-based inter-cell transitions rather than immediate L1 oscillation. Without L3 tagging, the DU may over- or under-suppress handovers.

Observation 6: Current Cell Switch Notification does not include any field structure for UE-specific historical mobility records, making the introduction of such lists implementation-specific and non-uniform across vendors.

While Option 2a provides a pragmatic and efficient means to empower DU-level ping-pong mitigation through CU-assistive UHI sharing, its limitations in historical depth and lack of L3-awareness may constrain its precision. The abstracted proposals herein aim to enhance Option 2a with minimal extensions that balance complexity, signaling overhead, and functional utility. These enhancements could form the basis for normative specification under the MRO enhancement work item in future RAN3 releases.

- Option 2b: CU provides FFS-defined number of last consecutive L1 entries to the DU

Ongoing RAN3 discussions have proposed transferring enriched UHI data from CU to DU via the CU-DU Cell Switch Notification, enabling the DU to locally analyze recent mobility patterns. Option 2b refines this approach by allowing the CU to provide a For Further Study (FFS)-defined number of last consecutive L1 entries to the DU. This flexibility in list length represents an enhancement over fixed-length approaches by permitting deployments to fine-tune the depth of mobility history shared, balancing signaling overhead against the precision of local ping-pong detection.

By limiting the shared context to L1 entries, Option 2b simplifies the information exchange and reduces complexity for DU processing, making it suitable for a wide range of deployment scenarios. The tunable list length can be adapted based on environment, UE speed, or network density, potentially increasing detection accuracy for oscillatory handovers without overwhelming the CU-DU interface.

Observation 7: The dynamic definition of the UHI list length (N) in Option 2b offers a tunable tradeoff between signaling overhead and decision accuracy, enabling deployments to optimize based on UE velocity, gNB density, or ping-pong sensitivity profiles. For instance, high-mobility UEs in ultra-dense networks may benefit from longer lists, whereas static users in rural macro scenarios may require minimal history depth.

However, the approach currently does not include explicit L3-type differentiation, which restricts the DU’s ability to discern the nature of handover transitions—whether intra-frequency, inter-frequency, or inter-RAT—thus limiting decision granularity and possibly causing either over-suppression or missed ping-pong events.

Observation 8: However, Option 2b—as currently described—focuses solely on L1-type entries, without explicitly encoding L3-type (intra- or inter-CU boundary) differentiation. This absence may limit the DU’s ability to distinguish between superficial radio-level re-entries and deeper logical mobility reversions, which are more indicative of systemic ping-pong behavior.

Consequently while Option 2b does not fully replicate the CU’s mobility intelligence, it allows the DU to operate with a lightweight, locally relevant subset of historical context, enabling proactive suppression of minor ping-pong events without increasing control plane dependency. For complex ping-pong cases involving inter-CU dynamics, coordination through existing UHI mechanisms at the CU level remains essential. Nevertheless, enabling DU-level awareness through controlled CU-to-DU UHI delegation is a promising step toward adaptive and scalable mobility optimization in future RAN deployments.

# 4. Proposed Implementation Option

Option 1a offers maximum visibility into the recent UE mobility trajectory, enabling the DU to apply localized logic to detect repetitive ping-pong behavior with higher accuracy. This rich context is particularly valuable in LTM scenarios where frequent handovers and short anchor durations can obscure mobility trends. While Option 1a incurs moderate signaling overhead, its benefits in proactive and granular ping-pong detection justify its inclusion, especially once the UHI list format is standardized.

Alternative options (1b, 2a, 2b) offer reduced complexity or lower overhead but at the expense of reduced historical context or decision precision. Therefore, this contribution supports progressing Option 1a in Stage 3, with further study on optimizing the signaling structure and possible inclusion of an extra ping-pong indication for enhanced DU-side logic.

Proposal 4: It is proposed to proceed with Option 1a for inclusion in the normative Stage 3 work, with the CU providing the full UHI list (maximum 16 entries) including PScell and L1/L3 types to the DU during Cell Switch Notification.

This approach provides a comprehensive foundation for DU-side ping-pong mitigation logic and supports enhanced MRO performance in LTM configurations.

# 5. UE-Assisted Distributed Ping-Pong Mitigation

Discussions in previous sections have primarily addressed CU–DU coordination enhancements through the transfer of enriched UE History Information (UHI) to enable DU-local ping-pong detection. The complementary concept of UE-assisted distributed MRO offers further opportunities to enhance detection granularity and reaction speed, particularly in LTM scenarios with frequent UHI events. UE supplements the DU’s view of mobility history by reporting additional contextual metrics (e.g., short-term serving cell revisit frequency, time-since-last-entry, or UE-perceived layer stability) that can assist DU-local decision-making. This approach leverages the UE’s intrinsic awareness of recent mobility oscillations without imposing continuous measurement reporting overhead on the CU–DU interface.

Observation 9: UE-side mobility context, when opportunistically shared with the DU, can provide higher temporal resolution for detecting rapid handover reversals (e.g., ping-pong events with dwell times shorter than the CU–DU signaling cycle). Such information can reduce false positives in DU logic by correlating cell change events with UE-reported layer stability or instantaneous SINR trends.

Observation 10: UE-assisted reporting can be selectively triggered based on DU-configured criteria—such as detection of a return to the same NR Cell Global ID within a predefined short time window—ensuring that the approach remains signaling-efficient and does not overload the DU with redundant metrics.

Observation 11: Incorporating UE-derived “time-to-return” or “short revisit count” metrics into DU-local ping-pong suppression logic enables better discrimination between normal intra-frequency load balancing and detrimental oscillations that degrade user experience.

Proposal 5: Introduce an optional UE-assisted MRO enhancement, whereby the DU may configure the UE to report mobility stability indicators (e.g., serving cell dwell time, revisit frequency within last N seconds) when specific return-to-cell patterns are detected. The CU–DU Cell Switch Notification framework may be complemented with these UE-triggered updates for more precise local suppression decisions.

Proposal 6: Consider hybrid operation where CU-provided UHI list (as per Option 1a) is augmented by recent UE-assisted metrics in DU-local decision loops, enabling the DU to refine suppression thresholds dynamically based on real-time UE feedback.

Proposal 7: FFS on conditional activation logic—DU triggers UE-assisted reporting only when CU-supplied UHI indicates multiple recent returns to the same cell or when DU detects threshold-exceeding oscillation rates—thus limiting activation to cases with a high likelihood of ping-pong degradation.

# 6. Conclusions

In summary, this contribution evaluates the down-selected solutions discussed during the previous RAN3 meeting regarding the use of UHI (UE History Information) list handling for detecting and mitigating ping-pong issues in LTM 5G mobility scenarios. Specifically, the options focus on enhancing CU-to-DU coordination via the Cell Switch Notification message by transmitting selected mobility history to the DU. Among the discussed alternatives (1a, 1b, 2a, 2b), Option 1a, wherein the CU provides the full UHI list with L1/L3 type (up to 16 entries), is proposed as the most effective candidate for Stage 3 normative work

This paper makes the following observations and proposals:

Observation 1: The DU's ability to detect ping-pong behavior is significantly enhanced when it receives UE History Information (UHI) with L1/L3 type, allowing it to distinguish between intra-frequency and inter-frequency transitions. However, the relevance of each UHI entry to DU decision-making may vary depending on deployment (e.g., small cell vs macro), frequency layer, and DU capabilities.

Observation 2: Transferring the full UHI list with up to 16 entries may introduce unnecessary signaling overhead, particularly in high-mobility scenarios or dense deployments, without proportionally increasing detection accuracy. Empirical thresholds for how many recent entries actually improve DU logic remain undefined.

Observation 3: PScell-related entries may not always carry actionable value for DU-level decisions, particularly when inter-frequency or cross-RAT mobility is out of DU control. Their inclusion should be made configurable.

Proposal 1: Define an optional maximum effective UHI list length for DU usage, such as 6–8 recent entries, subject to operator configuration, to allow for vendors to implement DU-side ping-pong logic without incurring unnecessary processing or memory load.

Proposal 2: Introduce CU-side configurable filtering criteria for UHI entries included in the CU-DU Cell Switch Notification—e.g., to exclude entries older than a given time threshold, entries corresponding to inter-frequency or inter-RAT mobility and PScell changes, if the DU is not expected to act on macro-layer transitions.

Proposal 3: Evaluate a delta-based UHI update mechanism where only incremental changes in the UE’s mobility history are sent from CU to DU (rather than re-sending the full list every time) to reduce redundant signaling while maintaining timely DU awareness.

Observation 4: Excluding the PScell from the UHI list may simplify context processing and avoid duplicate data, particularly where the PScell is already known by the DU through configuration or implicit assignment. However, in scenarios where the PScell was also implicated in recent ping-pong behavior (e.g., due to reconfigurations or serving cell switches), its absence may lead to missed detection opportunities.

Observation 5: The inability of DU to recognize the L3-type context of handover limits the potential for granular ping-pong classification, particularly when behavior arises from frequency-based inter-cell transitions rather than immediate L1 oscillation. Without L3 tagging, the DU may over- or under-suppress handovers.

Observation 6: Current Cell Switch Notification does not include any field structure for UE-specific historical mobility records, making the introduction of such lists implementation-specific and non-uniform across vendors.

Observation 7: The dynamic definition of the UHI list length (N) in Option 2b offers a tunable tradeoff between signaling overhead and decision accuracy, enabling deployments to optimize based on UE velocity, gNB density, or ping-pong sensitivity profiles. For instance, high-mobility UEs in ultra-dense networks may benefit from longer lists, whereas static users in rural macro scenarios may require minimal history depth.

Observation 8: However, Option 2b—as currently described—focuses solely on L1-type entries, without explicitly encoding L3-type (intra- or inter-CU boundary) differentiation. This absence may limit the DU’s ability to distinguish between superficial radio-level re-entries and deeper logical mobility reversions, which are more indicative of systemic ping-pong behavior.

Proposal 4: It is proposed to proceed with Option 1a for inclusion in the normative Stage 3 work, with the CU providing the full UHI list (maximum 16 entries) including PScell and L1/L3 types to the DU during Cell Switch Notification.

Observation 9: UE-side mobility context, when opportunistically shared with the DU, can provide higher temporal resolution for detecting rapid handover reversals (e.g., ping-pong events with dwell times shorter than the CU–DU signaling cycle). Such information can reduce false positives in DU logic by correlating cell change events with UE-reported layer stability or instantaneous SINR trends.

Observation 10: UE-assisted reporting can be selectively triggered based on DU-configured criteria—such as detection of a return to the same NR Cell Global ID within a predefined short time window—ensuring that the approach remains signaling-efficient and does not overload the DU with redundant metrics.

Observation 11: Incorporating UE-derived “time-to-return” or “short revisit count” metrics into DU-local ping-pong suppression logic enables better discrimination between normal intra-frequency load balancing and detrimental oscillations that degrade user experience.

Proposal 5: Introduce an optional UE-assisted MRO enhancement, whereby the DU may configure the UE to report mobility stability indicators (e.g., serving cell dwell time, revisit frequency within last N seconds) when specific return-to-cell patterns are detected. The CU–DU Cell Switch Notification framework may be complemented with these UE-triggered updates for more precise local suppression decisions.

Proposal 6: Consider hybrid operation where CU-provided UHI list (as per Option 1a) is augmented by recent UE-assisted metrics in DU-local decision loops, enabling the DU to refine suppression thresholds dynamically based on real-time UE feedback.

Proposal 7: FFS on conditional activation logic—DU triggers UE-assisted reporting only when CU-supplied UHI indicates multiple recent returns to the same cell or when DU detects threshold-exceeding oscillation rates—thus limiting activation to cases with a high likelihood of ping-pong degradation.