3GPP TSG-RAN WG3 Meeting #129	R3-255436

Bengaluru, India, 25 – 29 August, 2025

Agenda item:	17.3

Source:	Nokia

Title:	(TP for BLCR to TS 38.423, TS 38.473) Stage 3 Proposals for On-Demand SIB1

Document for:	Discussion and Decision

# 1	Introduction

Based on agreements taken at RAN3#128 we here discuss further updates to XnAP and F1AP.

# 2	Discussion

## 2.1 XnAP

We here list RAN3’s agreements relevant for the XnAP BLCR (numbering added by us for the purpose of this discussion):

- [1] UL WUS configuration transmission from NES Cell DU to NES Cell CU. NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface.

- [2] Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject).

- [3] The UL WUS configuration will be transferred over Xn via a new class1 defined procedure.

- [4] Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

- [5] The purpose of the new XnAP procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2.

- [6] The procedure uses non UE-associated signaling.

- [7] Cell A stores the UL WUS configuration information after it has received it.

- [8] A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

- [9] The procedure uses non UE-associated signalling.

- [10] After Cell A gNB has received and accepted the UL WUS Configuration request from the NES Cell gNB, it broadcasts in SIBx at the first opportunity.

- [11] From signalling point of view, in UL WUS Configuration Provision Request (naming to be further discussed) from NES Cell gNB to Cell A gNB, one option is “start with UL WUS Configuration”. This codepoint/choice is used to trigger Cell A to broadcast UL WUS configuration in SIBx.

- [12] The NES Cell gNB can “stop” (naming to be further discussed) Cell A to broadcast UL WUS configuration.

- [13] The message structure of UL WUS CONFIGURATION PROVISION REQUEST message includes one “OD-SIB1 config R19” defined in TS 38.331 (presence M), one NES Cell ID (presence M ), one Cell-A ID (presence O)

RAN3 127-bis had earlier agreed on adding NR-CGIs of the Cell A and NES cell in the definition of the message UL WUS CONFIGURATION PROVISION REQUEST.

RAN3 128 had agreed on having a single one “OD-SIB1 config R19” defined in TS 38.331 (presence M), one NES Cell ID (presence M ), one Cell-A ID (presence O)

This definition has a WUS Configuration being associated with a given pair of NR-CGIs (of Cell A and NES Cell) and one such pair per message. This means that for each such pair for which the WUS configuration has to be sent, a separate instance of the procedure UL WUS CONFIGURATION PROVISION (with its associated messages) will have to be initiated by the gNB-NES Cell. This would also mean that any subsequent UL WUS CONFIGURATION PROVISION STATUS UPDATE will have to associated to the right UL WUS CONFIGURATION PROVISION procedure.

This could potentially lead to a lot of Xn signalling because of the following reasons

- A given Cell A could be mapped to more than one NES Cell

- A given NES Cell could be mapped to more than one Cell A

- There could be multiple NES Cells that support OD-SIB1 at any given point in time

Observation-1: Based on the current definition of WUS CONFIGURATION PROVISION REQUEST, separate instances of the UL WUS CONFIGURATION PROVISION procedure has be initiated for each WUS Configuration that has to be shared, leading to potential signalling overload on Xn

These issues could be addressed if there are multiple cell IDs that are supported in a given WUS CONFIGURATION PROVISION procedure

Proposal 1: Enhance WUS CONFIGURATION PROVISION procedure to include multiple cell IDs for both NES Cells and Cell As along with their associated WUS Configuration.

If the WUS configuration for multiple cells can be given simultaneously, it leads to the possibility of the following scenarios to be handled.

- Case 1: All the WUS configurations sent by the gNB (NES Cell) were successfully handled by the gNB (Cell A)

- Case 2: Partial success – some of the WUS configurations sent by the gNB (NES Cell) were successfully handled by the gNB (Cell A)

- Case 3: None of the WUS configurations sent by the gNB (NES Cell) was successfully handled by the gNB (Cell A)

In order to handle these scenarios, we propose to enhance the WUS CONFIGURATION PROVISION RESPONSE message to have 2 lists – a list of pair of cells where the WUS configuration was successfully handled and a second list where the WUS configuration was not successfully handled. This should handle the case 1 and case 2 above. For the case 3, we propose to use the WUS CONFIGURATION PROVISION FAILURE message with an appropriate cause value.

Proposal-2: Enhance WUS CONFIGURATION PROVISION RESPONSE with 2 lists of cell ID pairs – one where the WUS configuration was successfully handled and another one where it was not successfully handled.

Proposal-3: Enhance WUS CONFIGURATION PROVISION FAILURE with an appropriate cause value to indicate the case where none of the WUS configurations provided was successfully handled at the gNB (Cell A).

Furthermore, RAN3 has not yet taken any agreement on whether there should be standardized support (OAM or network signalling) to help the NG-RAN node 1 to select an NG-RAN node 2 serving cells that can potentially serve as cell A.

Proposal 4: RAN3 to discuss whether there should be standardized support (OAM or network signalling) to help the NG-RAN node 1 to select an NG-RAN node 2 serving cells that can potentially serve as cell A.

Since this support is at the cell level, we propose to add a new flag in the Served Cell Information NR to indicate the support of Cell A functionality

Proposal 5: Add a new flag in the IE Served Cell Information NR to indicate if a given cell supports anchor cell functionality or not.

## 2.2 F1AP

The following agreements are relevant for the F1AP BLCR:

(10) Reuse the existing IE “Served Cell Information” IE to include UL WUS configuration from gNB-DU to gNB -CU in F1 Setup Request and gNB-DU Configuration Update procedures.

This agreement is covered by the F1AP BLCR that was agreed at last meeting. By the agreed signalling, the CU-CP may trigger the UL WUS Configuration Provisioning procedure over Xn. When the CU-CP can confirm successful response from the required cell A’s (both intra-gNB scenario and inter-gNB scenario should be supported), it should inform the gNB-DU so that it can activate the OD-SIB1 mode of operation. A simple solution for this could be to include an indicator in the Cells to be Activated List Item IE. However, in order to also support ‘discontinue’ and ‘stop’ scenarios, as discussed in section 2.1, we suggest that this indicator contains two codepoints e.g. as follows:

| >>SIB1 Operation Mode | O |  | ENUMERATED (on demand, periodic, …) | SIB1 operation mode as described in TS 38.300 [4]. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

This indicator should be included within the Cells to be Activated List Item IE in the following messages: F1 SETUP RESPONSE, GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE, GNB-CU CONFIGURATION UPDATE.

Proposal 6: Include a SIB1 Operation Mode indicator IE in the following messages: F1 SETUP RESPONSE, GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE, GNB-CU CONFIGURATION UPDATE.

Proposal 7: The SIB1 Operation Mode indicator IE is extendible with ENUMERATED type with the following codepoints: on demand, periodic.

# 3	Conclusion

We have made the following observations and proposals:

Observation-1: Based on the current definition of WUS CONFIGURATION PROVISION REQUEST, separate instances of the UL WUS CONFIGURATION PROVISION procedure has be initiated for each WUS Configuration that has to be shared, leading to potential signalling overload on Xn

Proposal-1: Enhance WUS CONFIGURATION PROVISION REQUEST message to include multiple cell IDs for both NES Cells and Cell As along with their associated WUS Configuration.

Proposal-2: Enhance WUS CONFIGURATION PROVISION RESPONSE with 2 lists of cell ID pairs – one where the WUS configuration was successfully handled and another one where it was not successfully handled.

Proposal-3: Enhance WUS CONFIGURATION PROVISION FAILURE with an appropriate cause value to indicate the case where none of the WUS configurations provided was successfully handled at the gNB (Cell A).

Proposal 4: RAN3 to discuss whether there should be standardized support (OAM or network signalling) to help the NG-RAN node 1 to select an NG-RAN node 2 serving cells that can potentially serve as cell A.

Proposal 5: Add a new flag in the IE Served Cell Information NR to indicate if a given cell supports anchor cell functionality or not

Proposal 6: Include a SIB1 Operation Mode indicator IE in the following messages: F1 SETUP RESPONSE, GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE, GNB-CU CONFIGURATION UPDATE.

Proposal 7: The SIB1 Operation Mode indicator IE is extendible with ENUMERATED type with the following codepoints: on demand, periodic.

# References

[1]		R3-255080, NES BL CR to TS 38.423 submitted to RAN3#129

[2]		R3-255081, NES BL CR to TS 38.473 submitted to RAN3#129

# Annex	- TP for NES BL CR TS 38.423

The TP is based on [1]

<<< start of changes >>>

# Text Proposal for BL CR for TS 38.423

Editor’s Note: The procedure, message and the IE names are FFS

Editors Note: How to address the role of the NG-RAN node1 and NG-RAN node2 to be further discussed.

### 8.x.x	UL WUS Configuration Provision (FFS)

#### 8.x.x.1	General

The purpose of the UL WUS Configuration Provision procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2 “and request NG-RAN node2 to transmit UL WUS configuration information (FFS)”.

“The procedure is also used to enable an NG-RAN node1 to request NG-RAN node2 to stop transmission of UL WUS configuration.  (FFS)”

The procedure uses non-UE -associated signaling.

#### 8.x.x.2	Successful Operation

Figure 8.x.x.2-1: UL WUS Configuration Provision procedure, successful operation

If the start is included in the Request Type IE in the UL WUS CONFIGURATION PROVISION REQUEST (Naming FFS) message, the NG-RAN node2 shall, if supported, broadcast UL WUS Configuration (Naming FFS) in SIBx (SIB naming FFS) and reply with the UL WUS CONFIGURATION PROVISION RESPONSE (Naming FFS) message. The NG-RAN node2 stores the UL WUS configuration information (Naming FFS).

If the Assisted Cell (naming FFS) is included in the UL WUS CONFIGURATION PROVISION REQUEST (Naming FFS) message, the NG-RAN node2 shall, if supported, broadcast the UL WUS Configuration information (Naming FFS) in the cell indicated by the assisted cell (naming FFS).

If the “stop” is included in the Request Type IE in the UL WUS CONFIGURATION PROVISION REQUEST (Naming FFS) message, the NG-RAN node2 shall stop broadcasting UL WUS Configuration (Naming FFS) for the given NG-Cell ID (Naming FFS) in SIBx (SIB naming FFS). NG-RAN node2 removes the received UL WUS Configuration.

#### 8.x.x.3	Unsuccessful Operation

Figure 8.x.x.3-1: UL WUS Configuration Provision procedure, unsuccessful operation

If the NG-RAN node2 cannot broadcast UL WUS Configuration in SIBx (Naming FFS) of any of the UL WUS Configurations requested by NG-RAN node1, or a failure occurs during UL WUS Configuration Provision (Naming FFS), the NG-RAN node2 shall send UL WUS CONFIGURATION PROVISION FAILURE (Naming FFS) message. The message shall contain the Cause IE with an appropriate value.

#### 8.x.x.4	Abnormal Conditions

Void.

### 8.x.y	UL WUS Configuration Transmission Status Update (FFS)

#### 8.x.y.1	General

This procedure is initiated by an NG-RAN node2 to inform a neighbouring NG-RAN node1 about a previously admitted UL WUS configuration provision being stopped. The procedure uses non-UE associated signalling.

#### 8.x.y.2	Successful Operation

Figure 8.x.y.2-1: UL WUS Configuration Transmission Status Update procedure, successful operation

The NG-RAN node2 shall indicate that one or more of previously admitted UL WUS configuration provision in the UL WUS CONFIGURATION PROVISION STATUS UPDATE message is being stopped.

#### 8.x.y.3	Unsuccessful Operation

Not applicable.

#### 8.x.y.4	Abnormal Conditions

Void.

*************** Next change ***************

#### 9.x.x.x	UL WUS CONFIGURATION PROVISION REQUEST

This message is sent by the NG-RAN node1 to the peer NG-RAN node2 to request or stop provisioning of an UL WUS configuration in one assisted cell.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| CHOICE Request Type | M |  |  |  | YES | reject |
| >Start |  |  |  |  |  |  |
| >> UL WUS Configuration List |  | 1 |  | List of UL WUS Configurations with their associated NES Cell | YES | ignore |
| >>> UL WUS Configuration Item |  | 1 .. <maxnoofCellsinNG-RAN node> |  |  | YES | ignore |
| >>>>UL WUS configuration (FFS to align to RAN2 spec) | M |  | OCTET STRING | Includes the XX IE (FFS) as specified in TS 38.331 [10]. | – |  |
| >>>>NR CGI at NG-RAN node1 (Naming FFS) | M |  | NR CGI 9.2.2.7 | Indicates the NR cell of the NG-RAN node1 supports the UL WUS configuration/On Demand SIB1(Naming FFS) | – |  |
| >>>>Anchor Cell List |  | 1 |  |  | YES | ignore |
| >>>>>Anchor Cell Item |  | 1 .. <maxnoofCellsinNG-RAN node> |  |  | YES | ignore |
| >>>>>NR CGI at NG-RAN node2 (Naming FFS) | O |  | NR CGI 9.2.2.7 | Indicates the NR cell of the NG-RAN node2 requested to broadcast the UL WUS configuration (Naming FFS). | – |  |
| >Stop |  |  |  |  |  |  |
| >>NES Cell List |  | 1 |  | Contains a list of NES Cells | YES | ignore |
| >>>NES Cell Item |  | 1 .. <maxnoofCellsinNG-RAN node> |  |  | YES | ignore |
| >>>>NR CGI at NG-RAN node1 (Naming FFS) | M |  | NR CGI 9.2.2.7 | Indicates the NR cell of the NG-RAN node1 supports the UL WUS configuration/On Demand SIB1(Naming FFS) | – |  |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

#### 9.1.3.y	UL WUS CONFIGURATION PROVISION RESPONSE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that one or more of the requested UL WUS configuration will be provided by the NG-RAN node2.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| UL WUS Configuration Success List |  | 1 |  | Contains a list of cell ID pairs where the UL WUS Configuration was handled successfully | YES | ignore |
| > UL WUS Configuration Success Item |  | 1 .. <maxnoofCellsinNG-RAN node> |  |  | YES | ignore |
| >>NR CGI at NG-RAN node1 (Naming FFS) | M |  | NR CGI 9.2.2.7 | Indicates the NR cell of the NG-RAN node1 supports the UL WUS configuration/On Demand SIB1(Naming FFS) | YES | reject |
| >>NR CGI at NG-RAN node2 (Naming FFS) | M |  | NR CGI 9.2.2.7 | Indicates the NR cell of the NG-RAN node2 which successfully handled a given UL WUS Configuration |  |  |
| UL WUS Configuration Failure List |  | 1 |  | Contains a list of cell ID pairs where the UL WUS Configuration was not handled successfully | YES | ignore |
| > UL WUS Configuration Failure Item |  | 1 .. <maxnoofCellsinNG-RAN node> |  |  | YES | ignore |
| >>NR CGI at NG-RAN node1 (Naming FFS) | M |  | NR CGI 9.2.2.7 | Indicates the NR cell of the NG-RAN node1 whose UL WUS configuration/On Demand SIB1(Naming FFS) was not successfully handled | YES | reject |
| >>NR CGI at NG-RAN node2 (Naming FFS) | M |  | NR CGI 9.2.2.7 | Indicates the NR cell of the NG-RAN node2 which did not successfully handle a given UL WUS Configuration |  |  |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

#### 9.1.3.z	UL WUS CONFIGURATION PROVISION FAILURE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that the UL WUS configuration cannot be provided in any of the requested cells of the NG-RAN node2.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| UL WUS Configuration Failure List |  | 1 |  | Contains a list of cell ID pairs where the UL WUS Configuration was not handled successfully | YES | ignore |
| > UL WUS Configuration Failure Item |  | 1 .. <maxnoofCellsinNG-RAN node> |  |  | YES | ignore |
| >>NR CGI at NG-RAN node1 (FFS) | M |  | NR CGI 9.2.2.7 | Indicates the NR cell of the NG-RAN node1 supports the UL WUS configuration/On Demand SIB1(Naming FFS) | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

#### 9.1.3.t	UL WUS CONFIGURATION PROVISION STATUS UPDATE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to report that an admitted UL WUS configuration provision is being stopped.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| UL WUS Configuration List |  | 1 |  | Contains a list of cells in NG-RAN node2 | YES | ignore |
| >UL WUS Configuration Item |  | 1 .. <maxnoofCellsinNG-RAN node> |  |  | YES | ignore |
| >NR CGI at NG-RAN node2 (FFS) | M (FFS) |  |  | RAN node1 supports the UL WUS configuration/On Demand SIB1(Naming FFS) |  |  |
| Provision Status | M |  | ENUMERATED (stopped, …) | Indicates the status of the UL WUS configuration transmission. | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

Editor’s Note: This message will be refined at the next meeting.

*************** Next change ***************

********Note in the below AS.1, the new procedure/IE names are subject to be refined ******

### 9.3.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

XnAP-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM XnAP-CommonDataTypes

HandoverRequest,

HandoverRequestAcknowledge,

HandoverPreparationFailure,

SNStatusTransfer,

UEContextRelease,

HandoverCancel,

NotificationControlIndication,

RANPaging,

RetrieveUEContextRequest,

RetrieveUEContextResponse,

RetrieveUEContextConfirm,

RetrieveUEContextFailure,

XnUAddressIndication,

SecondaryRATDataUsageReport,

SNodeAdditionRequest,

SNodeAdditionRequestAcknowledge,

SNodeAdditionRequestReject,

SNodeReconfigurationComplete,

SNodeModificationRequest,

SNodeModificationRequestAcknowledge,

SNodeModificationRequestReject,

SNodeModificationRequired,

SNodeModificationConfirm,

SNodeModificationRefuse,

SNodeReleaseRequest,

SNodeReleaseRequestAcknowledge,

SNodeReleaseReject,

SNodeReleaseRequired,

SNodeReleaseConfirm,

SNodeCounterCheckRequest,

SNodeChangeRequired,

SNodeChangeConfirm,

SNodeChangeRefuse,

RRCTransfer,

XnRemovalRequest,

XnRemovalResponse,

XnRemovalFailure,

XnSetupRequest,

XnSetupResponse,

XnSetupFailure,

NGRANNodeConfigurationUpdate,

NGRANNodeConfigurationUpdateAcknowledge,

NGRANNodeConfigurationUpdateFailure,

E-UTRA-NR-CellResourceCoordinationRequest,

E-UTRA-NR-CellResourceCoordinationResponse,

ActivityNotification,

CellActivationRequest,

CellActivationResponse,

CellActivationFailure,

ResetRequest,

ResetResponse,

ErrorIndication,

PrivateMessage,

DeactivateTrace,

TraceStart,

HandoverSuccess,

ConditionalHandoverCancel,

EarlyStatusTransfer,

FailureIndication,

HandoverReport,

ResourceStatusRequest,

ResourceStatusResponse,

ResourceStatusFailure,

ResourceStatusUpdate,

MobilityChangeRequest,

MobilityChangeAcknowledge,

MobilityChangeFailure,

AccessAndMobilityIndication,

CellTrafficTrace,

RANMulticastGroupPaging,

ScgFailureInformationReport,

ScgFailureTransfer,

F1CTrafficTransfer,

IABTransportMigrationManagementRequest,

IABTransportMigrationManagementResponse,

IABTransportMigrationManagementReject,

IABTransportMigrationModificationRequest,

IABTransportMigrationModificationResponse,

IABResourceCoordinationRequest,

IABResourceCoordinationResponse,

CPCCancel,

PartialUEContextTransfer,

PartialUEContextTransferAcknowledge,

PartialUEContextTransferFailure,

RachIndication,

DataCollectionRequest,

DataCollectionResponse,

DataCollectionFailure,

DataCollectionUpdate,

ULWUSConfigurationProvisionRequest,

ULWUSConfigurationProvisionResponse,

ULWUSConfigurationProvisionFailure,

ULWUSConfigurationTransmissionStatusUpdate

FROM XnAP-PDU-Contents

id-handoverPreparation,

id-sNStatusTransfer,

id-handoverCancel,

id-notificationControl,

id-retrieveUEContext,

id-rANPaging,

id-xnUAddressIndication,

id-uEContextRelease,

id-secondaryRATDataUsageReport,

id-sNGRANnodeAdditionPreparation,

id-sNGRANnodeReconfigurationCompletion,

id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation,

id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation,

id-mNGRANnodeinitiatedSNGRANnodeRelease,

id-sNGRANnodeinitiatedSNGRANnodeRelease,

id-sNGRANnodeCounterCheck,

id-sNGRANnodeChange,

id-activityNotification,

id-rRCTransfer,

id-xnRemoval,

id-xnSetup,

id-nGRANnodeConfigurationUpdate,

id-e-UTRA-NR-CellResourceCoordination,

id-cellActivation,

id-reset,

id-errorIndication,

id-privateMessage,

id-deactivateTrace,

id-traceStart,

id-handoverSuccess,

id-conditionalHandoverCancel,

id-earlyStatusTransfer,

id-failureIndication,

id-handoverReport,

id-resourceStatusReportingInitiation,

id-resourceStatusReporting,

id-mobilitySettingsChange,

id-accessAndMobilityIndication,

id-cellTrafficTrace,

id-RANMulticastGroupPaging,

id-scgFailureInformationReport,

id-scgFailureTransfer,

id-f1CTrafficTransfer,

id-iABTransportMigrationManagement,

id-iABTransportMigrationModification,

id-iABResourceCoordination,

id-retrieveUEContextConfirm,

id-cPCCancel,

id-partialUEContextTransfer,

id-rachIndication,

id-dataCollectionReportingInitiation,

id-dataCollectionReporting,

id-uLWUSConfigurationProvision,

id-uLWUSConfigurationProvisionStatus

FROM XnAP-Constants;

-- **************************************************************

--

-- Interface Elementary Procedure Class

--

-- **************************************************************

XNAP-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome				OPTIONAL,

&UnsuccessfulOutcome				OPTIONAL,

&procedureCode			ProcedureCode	UNIQUE,

&criticality			Criticality		DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE		&InitiatingMessage

[SUCCESSFUL OUTCOME		&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

PROCEDURE CODE			&procedureCode

[CRITICALITY			&criticality]

}

-- **************************************************************

--

-- Interface PDU Definition

--

-- **************************************************************

XnAP-PDU ::= CHOICE {

initiatingMessage	InitiatingMessage,

successfulOutcome	SuccessfulOutcome,

unsuccessfulOutcome	UnsuccessfulOutcome,

...

}

InitiatingMessage ::= SEQUENCE {

procedureCode	XNAP-ELEMENTARY-PROCEDURE.&procedureCode		({XNAP-ELEMENTARY-PROCEDURES}),

criticality		XNAP-ELEMENTARY-PROCEDURE.&criticality			({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			XNAP-ELEMENTARY-PROCEDURE.&InitiatingMessage	({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode	XNAP-ELEMENTARY-PROCEDURE.&procedureCode		({XNAP-ELEMENTARY-PROCEDURES}),

criticality		XNAP-ELEMENTARY-PROCEDURE.&criticality			({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			XNAP-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode	XNAP-ELEMENTARY-PROCEDURE.&procedureCode		({XNAP-ELEMENTARY-PROCEDURES}),

criticality		XNAP-ELEMENTARY-PROCEDURE.&criticality			({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			XNAP-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

-- **************************************************************

--

-- Interface Elementary Procedure List

--

-- **************************************************************

XNAP-ELEMENTARY-PROCEDURES XNAP-ELEMENTARY-PROCEDURE ::= {

XNAP-ELEMENTARY-PROCEDURES-CLASS-1			|

XNAP-ELEMENTARY-PROCEDURES-CLASS-2			,

...

}

XNAP-ELEMENTARY-PROCEDURES-CLASS-1 XNAP-ELEMENTARY-PROCEDURE ::= {

handoverPreparation										|

retrieveUEContext										|

sNGRANnodeAdditionPreparation							|

mNGRANnodeinitiatedSNGRANnodeModificationPreparation	|

sNGRANnodeinitiatedSNGRANnodeModificationPreparation	|

mNGRANnodeinitiatedSNGRANnodeRelease					|

sNGRANnodeinitiatedSNGRANnodeRelease					|

sNGRANnodeChange										|

xnRemoval												|

xnSetup													|

nGRANnodeConfigurationUpdate							|

e-UTRA-NR-CellResourceCoordination						|

cellActivation											|

reset													|

resourceStatusReportingInitiation						|

mobilitySettingsChange									|

iABTransportMigrationManagement							|

iABTransportMigrationModification						|

iABResourceCoordination									|

partialUEContextTransfer								|

dataCollectionReportingInitiation						|

uLWUSConfigurationProvision,

...

}

XNAP-ELEMENTARY-PROCEDURES-CLASS-2 XNAP-ELEMENTARY-PROCEDURE ::= {

sNStatusTransfer						|

handoverCancel							|

rANPaging								|

xnUAddressIndication					|

uEContextRelease						|

sNGRANnodeReconfigurationCompletion		|

sNGRANnodeCounterCheck					|

rRCTransfer								|

errorIndication							|

privateMessage							|

notificationControl						|

activityNotification					|

secondaryRATDataUsageReport				|

deactivateTrace							|

traceStart								|

handoverSuccess							|

conditionalHandoverCancel				|

earlyStatusTransfer						|

failureIndication						|

handoverReport							|

resourceStatusReporting					|

accessAndMobilityIndication				|

cellTrafficTrace						|

rANMulticastGroupPaging					|

scgFailureInformationReport				|

scgFailureTransfer						|

f1CTrafficTransfer						|

retrieveUEContextConfirm				|

cPCCancel								|

rachIndication							|

dataCollectionReporting					|

uLWUSConfigurationProvisionStatus,

...

}

*************** Next change ***************

-- **************************************************************

--

-- Interface Elementary Procedures

--

-- **************************************************************

handoverPreparation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverRequest

SUCCESSFUL OUTCOME		HandoverRequestAcknowledge

UNSUCCESSFUL OUTCOME	HandoverPreparationFailure

PROCEDURE CODE			id-handoverPreparation

CRITICALITY				reject

}

sNStatusTransfer	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SNStatusTransfer

PROCEDURE CODE			id-sNStatusTransfer

CRITICALITY				ignore

}

*************** Next change ***************

dataCollectionReportingInitiation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE				DataCollectionRequest

SUCCESSFUL OUTCOME				DataCollectionResponse

UNSUCCESSFUL OUTCOME			DataCollectionFailure

PROCEDURE CODE					id-dataCollectionReportingInitiation

CRITICALITY						reject

}

dataCollectionReporting	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DataCollectionUpdate

PROCEDURE CODE			id-dataCollectionReporting

CRITICALITY				ignore

}

uLWUSConfigurationProvision		XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE				ULWUSConfigurationProvisionRequest

SUCCESSFUL OUTCOME				ULWUSConfigurationProvisionResponse

UNSUCCESSFUL OUTCOME			ULWUSConfigurationProvisionFailure

PROCEDURE CODE					id-uLWUSConfigurationProvision

CRITICALITY						reject

}

uLWUSConfigurationProvisionStatus	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ULWUSConfigurationTransmissionStatusUpdate

PROCEDURE CODE			id-uLWUSConfigurationProvisionStatus

CRITICALITY				ignore

}

END

-- ASN1STOP

*************** Next change ***************

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ActivationIDforCellActivation,

AMF-Region-Information,

AMF-UE-NGAP-ID,

AS-SecurityInformation,

AssistanceDataForRANPaging,

*************** Next change ***************

id-S-CPAC-Request,

id-sk-Counter,

id-Source-M-NG-RANnodeID,

id-SourceSN-to-TargetSN-QMCInfo,

id-RegistrationRequestForDataCollection,

id-ReportCharacteristicsForDataCollection,

id-ReportingPeriodicityForDataCollection,

id-NodeAssociatedInfoResult,

id-SLPositioning-Ranging-Services-Info,

id-PDUSessionsListToBeReleased-UPError,

id-UserPlaneFailureIndication,

id-SRSPositioningConfigOrActivationRequest,

id-NRPPaPositioningInformation,

id-RequestType,

id-Node1-Cell-ID,

id-Node2-Cell-ID,

id-ProvisionStatus,

Anchor-Cell-Support,

maxnoofCellsinNG-RANnode,

maxnoofDRBs,

maxnoofPDUSessions,

maxnoofQoSFlows,

maxnoofServedCellsIAB,

maxnoofTrafficIndexEntries,

maxnoofTLAsIAB,

maxnoofBAPControlPDURLCCHs,

maxnoofServingCells,

maxnoofSSBAreas

FROM XnAP-Constants;

-- **************************************************************

--

#### -- HANDOVER REQUEST

--

-- **************************************************************

*************** Next change ***************

-- **************************************************************

--

#### -- DATA COLLECTION UPDATE

--

-- **************************************************************

DataCollectionUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{DataCollectionUpdate-IEs}},

...

}

DataCollectionUpdate-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NGRAN-Node1-Measurement-ID					CRITICALITY reject	TYPE Measurement-ID												PRESENCE mandatory}|

{ ID id-NGRAN-Node2-Measurement-ID					CRITICALITY reject	TYPE Measurement-ID												PRESENCE mandatory}|

{ ID id-CellMeasurementResultForDataCollection-List	CRITICALITY ignore	TYPE CellMeasurementResultForDataCollection-List		PRESENCE optional }|

{ ID id-UEAssociatedInfoResult-List					CRITICALITY ignore	TYPE UEAssociatedInfoResult-List							PRESENCE optional }|

{ ID id-NodeAssociatedInfoResult					CRITICALITY ignore	TYPE NodeAssociatedInfoResult								PRESENCE optional },

...

}

-- **************************************************************

--

#### -- UL WUS CONFIGURATION REQUEST

--

-- **************************************************************

ULWUSConfigurationProvisionRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ULWUSConfigurationProvisionRequest-IEs}},

...

}

ULWUSConfigurationProvisionRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-RequestType								CRITICALITY reject	TYPE RequestType								PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject	TYPE InterfaceInstanceIndication			PRESENCE optional },

...

}

RequestTypeChoice ::= CHOICE {

Start							ULWUSConfigurationInformationList,

Stop							NESCellList,

choice-extension				ProtocolIE-Single-Container { {RequestTypeChoice-ExtIEs} }

}

RequestTypeChoice-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

ULWUSConfigurationInformation ::= SEQUENCE {

uLWUSConfiguration					OCTET STRING,

node1-Cell-ID						NR-CGI,

node2-Cell-ID						NR-CGI						OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {ULWUSConfigurationInformation-ExtIEs} }	OPTIONAL,

...

}

ULWUSConfigurationInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- UL WUS CONFIGURATION RESPONSE

--

-- **************************************************************

ULWUSConfigurationProvisionResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ULWUSConfigurationProvisionResponse-IEs}},

...

}

ULWUSConfigurationProvisionResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-UL WUS Configuration Success List			CRITICALITY reject TYPE UL-WUS-Configuration-Success-List}|

{ ID id-UL WUS Configuration Failure List 			CRITICALITY reject TYPE UL-WUS-Configuration List}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject	TYPE InterfaceInstanceIndication			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- UL WUS CONFIGURATION FAILURE

--

-- **************************************************************

ULWUSConfigurationProvisionFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ ULWUSConfigurationProvisionFailure-IEs}},

...

}

ULWUSConfigurationProvisionFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-Cause									CRITICALITY ignore	TYPE Cause											PRESENCE mandatory}|

{ ID id-NES-Cell-List						CRITICALITY ignore	TYPE NES-Cell-List									PRESENECE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject	TYPE InterfaceInstanceIndication			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- UL WUS CONFIGURATION TRANSMISSION STATUS UPDATE

--

-- **************************************************************

ULWUSConfigurationTransmissionStatusUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ULWUSConfigurationTransmissionStatusUpdate-IEs}},

...

}

ULWUSConfigurationTransmissionStatusUpdate-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-UL-WUS-Configuration-List 			CRITICALITY	ignore	TYPE UL-WUS-Configuration-List		PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject	TYPE InterfaceInstanceIndication			PRESENCE optional },

...

}

ProvisionStatus ::= ENUMERATED {stopped, ...}

<<< next change >>>

NES-Cell-List	::= SEQUENCE (SIZE(1..maxnoofCellsInNGRANnode)) OF NR-CGI,

<<< next change >>>

UL-WUS-Configuration-List	::= SEQUENCE (SIZE(1..maxnoofCellsInNGRANnode)) OF UL-WUS-Configuration-Item

UL-WUS-Configuration-Item ::= SEQUENCE {

Node2_CellID								NR-CGI,

ProvisionStatus							ProvisionStatus,

...

}

<<< next change >>>

ServedCellInformation-NR ::= SEQUENCE {

nrPCI								NRPCI,

cellID								NR-CGI,

tac									TAC,

ranac								RANAC						OPTIONAL,

broadcastPLMN						BroadcastPLMNs,

nrModeInfo							NRModeInfo,

measurementTimingConfiguration		OCTET STRING,

connectivitySupport					Connectivity-Support,

iE-Extensions						ProtocolExtensionContainer { {ServedCellInformation-NR-ExtIEs} } OPTIONAL,

...

}

ServedCellInformation-NR-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-BPLMN-ID-Info-NR				CRITICALITY ignore	EXTENSION BPLMN-ID-Info-NR						PRESENCE optional }|

{ ID id-ConfiguredTACIndication			CRITICALITY ignore	EXTENSION ConfiguredTACIndication				PRESENCE optional }|

{ ID id-SSB-PositionsInBurst			CRITICALITY ignore	EXTENSION SSB-PositionsInBurst					PRESENCE optional }|

{ ID id-NRCellPRACHConfig				CRITICALITY ignore	EXTENSION NRCellPRACHConfig						PRESENCE optional }|

{ ID id-NPN-Broadcast-Information		CRITICALITY reject	EXTENSION NPN-Broadcast-Information			PRESENCE optional }|

{ ID id-CSI-RSTransmissionIndication	CRITICALITY ignore	EXTENSION CSI-RSTransmissionIndication		PRESENCE optional }|

{ ID id-SFN-Offset						CRITICALITY ignore	EXTENSION SFN-Offset							PRESENCE optional }|

{ ID id-Supported-MBS-FSA-ID-List		CRITICALITY ignore	EXTENSION Supported-MBS-FSA-ID-List			PRESENCE optional }|

{ ID id-NR-U-ChannelInfo-List			CRITICALITY ignore	EXTENSION NR-U-ChannelInfo-List					PRESENCE optional }|

{ ID id-Additional-Measurement-Timing-Configuration-List		CRITICALITY ignore	EXTENSION Additional-Measurement-Timing-Configuration-List		PRESENCE optional }|

{ ID id-Redcap-Bcast-Information		CRITICALITY ignore	EXTENSION Redcap-Bcast-Information			PRESENCE optional }|

{ ID id-eRedcap-Bcast-Information		CRITICALITY ignore	EXTENSION ERedcap-Bcast-Information			PRESENCE optional }|

{ ID id-MobileIABCell					CRITICALITY ignore	EXTENSION MobileIABCell							PRESENCE optional }|

{ ID id-XR-Bcast-Information			CRITICALITY ignore	EXTENSION XR-Bcast-Information					PRESENCE optional }|

{ ID id-BarringExemptionforEmerCallInfo	CRITICALITY ignore	EXTENSION BarringExemptionforEmerCallInfo	PRESENCE optional } |

{ ID id-Anchor-Cell-Support				CRITICALITY ignore 	ENTENSION Anchor-Cell-Support },

...

}

END

-- ASN1STOP

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-handoverPreparation											ProcedureCode ::= 0

id-sNStatusTransfer												ProcedureCode ::= 1

id-handoverCancel												ProcedureCode ::= 2

id-retrieveUEContext											ProcedureCode ::= 3

id-rANPaging													ProcedureCode ::= 4

id-xnUAddressIndication											ProcedureCode ::= 5

id-uEContextRelease												ProcedureCode ::= 6

id-sNGRANnodeAdditionPreparation								ProcedureCode ::= 7

id-sNGRANnodeReconfigurationCompletion							ProcedureCode ::= 8

id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 9

id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 10

id-mNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 11

id-sNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 12

id-sNGRANnodeCounterCheck										ProcedureCode ::= 13

id-sNGRANnodeChange												ProcedureCode ::= 14

id-rRCTransfer													ProcedureCode ::= 15

id-xnRemoval													ProcedureCode ::= 16

id-xnSetup														ProcedureCode ::= 17

id-nGRANnodeConfigurationUpdate									ProcedureCode ::= 18

id-cellActivation												ProcedureCode ::= 19

id-reset														ProcedureCode ::= 20

id-errorIndication												ProcedureCode ::= 21

id-privateMessage												ProcedureCode ::= 22

id-notificationControl											ProcedureCode ::= 23

id-activityNotification											ProcedureCode ::= 24

id-e-UTRA-NR-CellResourceCoordination							ProcedureCode ::= 25

id-secondaryRATDataUsageReport									ProcedureCode ::= 26

id-deactivateTrace												ProcedureCode ::= 27

id-traceStart													ProcedureCode ::= 28

id-handoverSuccess												ProcedureCode ::= 29

id-conditionalHandoverCancel									ProcedureCode ::= 30

id-earlyStatusTransfer											ProcedureCode ::= 31

id-failureIndication											ProcedureCode ::= 32

id-handoverReport												ProcedureCode ::= 33

id-resourceStatusReportingInitiation							ProcedureCode ::= 34

id-resourceStatusReporting										ProcedureCode ::= 35

id-mobilitySettingsChange										ProcedureCode ::= 36

id-accessAndMobilityIndication									ProcedureCode ::= 37

id-cellTrafficTrace												ProcedureCode ::= 38

id-RANMulticastGroupPaging										ProcedureCode ::= 39

id-scgFailureInformationReport									ProcedureCode ::= 40

id-ProcedureCode41-NotToBeUsed									ProcedureCode ::= 41

id-scgFailureTransfer											ProcedureCode ::= 42

id-f1CTrafficTransfer											ProcedureCode ::= 43

id-iABTransportMigrationManagement								ProcedureCode ::= 44

id-iABTransportMigrationModification							ProcedureCode ::= 45

id-iABResourceCoordination										ProcedureCode ::= 46

id-retrieveUEContextConfirm										ProcedureCode ::= 47

id-cPCCancel													ProcedureCode ::= 48

id-partialUEContextTransfer										ProcedureCode ::= 49

id-rachIndication												ProcedureCode ::= 50

id-dataCollectionReportingInitiation							ProcedureCode ::= 51

id-dataCollectionReporting										ProcedureCode ::= 52

id-uLWUSConfigurationProvision									ProcedureCode ::= 5x

id-uLWUSConfigurationProvisionStatus							ProcedureCode ::= 5y

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

*************** Next change ***************

id-CPAC-Preparation-Type																			ProtocolIE-ID ::= 468

id-UserPlaneFailureIndication																		ProtocolIE-ID ::= 469

id-MN-only-MDT-collection																			ProtocolIE-ID ::= 470

id-BarringExemptionforEmerCallInfo																	ProtocolIE-ID ::= 471

id-Transmission-Bandwidth-asymmetric																ProtocolIE-ID ::= 472

id-SRSPositioningConfigOrActivationRequest															ProtocolIE-ID ::= 473

id-NRPPaPositioningInformation																		ProtocolIE-ID ::= 474

id-RequestType																						ProtocolIE-ID ::= 4xx

id-Node1-Cell-ID																					ProtocolIE-ID ::= 4xx+1

id-Node2-Cell-ID																					ProtocolIE-ID ::= 4xx+2

id-ProvisionStatus																					ProtocolIE-ID ::= 4xx+3

id-Anchor-Cell-Support																					ProtocolIE-ID ::= 4xx+3

END

-- ASN1STOP